3GPP TSG-RAN WG2 Meeting #129bis	R2-2502183

Wuhan, China, 7 – 11 April 2025

Agenda item:	8.7.5

Source:	Apple

Title:	Views on Avoidance of Unnecessary RLC Retransmissions

WID/SID:	NR_XR_Ph3-Core

Document for:	Discussion / Decision

# Introduction

For the Rel-19 XR work item objective related to RLC-AM enhancements for packets with small delay budget [1], RAN2 has agreed to support avoidance of unnecessary RLC transmission/retransmission. Some of the agreements we have made include the following:

RAN2 #127bis Agreements on Avoidance of Unnecessary RLC Retransmission [2]: RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed) RAN2 #128 Agreements on Avoidance of Unnecessary RLC Retransmission [3]: Special handling to avoid PDCP control PDU discard is not needed. A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report. RAN2 #129 Agreements on Avoidance of Unnecessary RLC Retransmission [4]: When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. The new RLC timer at the RX is per RLC entity The duration of the new RLC timer is not lower than that of t-reassembly

In this paper, we aim to present some of our views on the following topics:

- Whether a Status Report should be triggered when the receiver determines that some RLC SDUs are already outdated.

- Whether packet discarding by the upper layer can influence the going polling procedure?

# Discussions

## Status Report for Indication of Abandoned RLC SDUs

Based on RAN2 agreements, once the receiver determines that one or more RLC SDUs should be abandoned based on a local timer, it can inform the transmitter to move the transmitting window forward, by deliberately acknowledging the abandoned RLC SDUs via a status report.

In our understanding, RAN2 adopts the approach based on RX-initiated window synchronization (instead of TX-initiated approach) to minimize the impacts to current specification, at an expense of latency performance. In particular, the receiver needs to go through two stages before it can detect that a RLC SDU has been discarded:

- Identify a SN gap in the received RLC SDU, and then start a timer.

- Wait until the expiration of the timer.

Thus, it can take quite a while for the receiver to determine that a RLC SDU has been discarded. Furthermore, in XR we need to deal with PDU Set discarding, where a large number of packets may be discarded simultaneously. In such cases, it may take even longer for the receiver to identify which RLC SDUs can be abandoned especially when the SNs of the discarded packets are not consecutive.

Even the transmitter would stop transmission or retransmission of RLC SDUs that have been discarded, it cannot move the transmitting window forward before the receiver provides a status report. With this in mind, we think the receiver should proactively provide the status report whenever it determines that some packets have been discarded, in order to minimize the overall latency.

Proposal 1: The receiver should proactively send a status report once it determines that some RLC SDUs have been discarded, in order to advance the transmitting window as soon as possible.

## Impacts of Discarding to Ongoing Polling Procedures

According to TS 38.322, when a poll is triggered and sent, the transmitter should start or restart t-PollRetransmit, and set the state variable POLL_SN to the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer.

Furthermore, the transmitter can stop and reset t-PollRetransmit when it receives a STATUS report that indicates the status of RLC SDU corresponding to POLL_SN:

- if the STATUS report comprises a positive or negative acknowledgement for the RLC SDU with sequence number equal to POLL_SN: - if t-PollRetransmit is running: - stop and reset t-PollRetransmit.

We keep the t-PollRetransmit running because the transmitter would like to continue the polling procedure until it knows if each of the RLC SDU with SN up to POLL_SN is successfully delivered. And therefore, if the transmitter gets a status report that includes at least POLL_SN, the transmitter can stop and resets t-PollRetransmit , as it now knows that each of the RLC SDU with SN up to POLL_SN is successfully delivered to the receiver based on such status report. Based on the information of NACK_SN in the status report, the transmitter can perform retransmission of RLC SDUs accordingly.

When a packet is discarded and considered outdated, the transmitter is no longer interested about whether the receiver has successfully received the discarded packets or not, because no retransmission will be performed for their RLC SDUs regardless of their status. Of course, the transmitter would still need to know whether the RLC SDU is positively acknowledged so it can advance the transmitting window, but the receiver will anyway provide such status report proactively (e.g. based on Proposal 1) when the outdated packets are detected. Thus, in cases where all RLC SDUs with SN between (and including) TX_Next_Ack and POLL_SN are already either positively/negatively acknowledged or outdated, we think it is questionable whether the transmitter should keep the t-PollRetransmit running or if we can just stop the timer, because such polling procedure may no longer be needed. In fact, excessive polling may result in some additional signaling overhead of unnecessary or pre-mature status reports.

It is worth pointing out that, the situation mentioned above may not be a corner case in XR, where a whole bunch of packets may become outdated simultaneously due to PDU Set discarding. Hence, we think RAN2 can further discuss the intended UE behavior in such cases.

Proposal 2: RAN2 should discuss if t-PollRetransmit should continue to run when all RLC SDUs with SNs between (and including) TX_Next_Ack and POLL_SN are already positively/negatively acknowledged or outdated.

# Conclusions

This paper presents some of our views on RLC-AM enhancements relating to unnecessary retransmission of outdated RLC SDUs. We have the following proposals:

Proposal 1: The receiver should proactively send a status report once it determines that some RLC SDUs have been discarded, in order to advance the transmitting window as soon as possible.

Proposal 2: RAN2 should discuss if t-PollRetransmit should continue to run when all RLC SDUs with SNs between (and including) TX_Next_Ack and POLL_SN are already positively/negatively acknowledged or outdated.

# References

- RP-241771, Revised WID on XR (eXtended Reality) for NR Phase 3, Nokia (Rapporteur), Sept. 2024, Melbourne, Australia.

- 3GPP RAN2 #127bis Chair’s Notes, Hefei, China, October 2024.

- 3GPP RAN2 #128 Chair’s Notes, Orlando, USA, November 2024.

- 3GPP RAN2 #129 Chair’s Notes, Athens, Greece, November 2024.