# 3GPP TSG RAN WG2 Meeting #129bis      	                           R2-2502188
Wuhan, China, Apirl 7-11, 2025

Agenda item:	8.13.2

Source:	Apple

Title:	Relay discovery and selection for Multi-hop UE-to-NW Relay

WID/SID:	NR_SL_relay_multihop – Release 19

Document for:	Discussion and Decision

# 1 Introduction

RAN2#127 [1] has made the following agreements on relay discovery and selection:

- From RAN2 perspective, models A and B can both be supported. - Reuse existing AS discovery protocol to transmit discovery message for multi-hop U2N relaying. - Reuse SL-SRB4 to transmit discovery message for multi-hop U2N relaying. - Reuse existing resource pools defined for discovery message transmission and reception - Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs; FFS mode 1 for remote UE. - Configuration can be provided by SIB/dedicated message or pre-configured as in Rel-17/18. - If the RSRP measurement of the serving cell is below a Uu threshold, or the Remote UE could not find a serving cell, the Remote UE can perform discovery transmission, as in Rel-17/18. - If the Uu RSRP measurement of the serving cell is above a low threshold and below a high threshold, the last relay UE can perform discovery transmission, as in Rel-17/18 (subject to how the gNB configures one or both thresholds). - FFS discovery conditions for the intermediate relay UEs.

RAN2#128[2] meeting has further reached the following agreements for relay discovery and selection:

- A lower bound of Uu RSRP is not required for the UE to operate as an intermediate Relay UE. - If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE (i.e., minimise number of hops when possible). FFS if operation as an intermediate relay UE towards a different cell would be supported. - The network can configure an upper bound of Uu RSRP for the UE to operate as an intermediate relay UE. If the upper bound is not configured, there is no threshold, but this does not override the previous agreement. FFS if there would be additional restrictions in the case of no upper bound. - The following cases are supported for L2 multi-hop relay: - - One last Relay UE can have two connections with one intermediate Relay UE and one Remote UE (the intermediate Relay UE and Remote UE are physically different UE). - - Two physically different Remote UE(s) can have each indirect path via the same intermediate Relay UE and the same last Relay UE. - FFS if the last relay UE can use the same L2ID for both of the connections in either case. - Cases with two indirect paths to the gNB for the same remote UE are not supported. - An Intermediate Relay UE can serve multiple multi-hop indirect paths of different Remote UEs. - If the intermediate Relay UE also is acting as a Remote UE, it cannot support different indirect paths to the gNB with same/different last/U2N/parent intermediate Relay UE(s) based on different PC5 unicast links. - RAN2 sends LS to ask SA2 to clarify whether an intermediate relay needs to establish PC5 link with the relay from which the discovery announcement message is received before sending/forwarding the discovery announcement message.

RAN2#129 [3] has made the following agreements on relay discovery and selection:

- The last relay UE over one cell cannot be operated as an intermediate relay UE towards a different cell. FFS if there can be exceptions for control plane approach 2 when the intermediate relay UEs are in idle/inactive. - From RAN2 perspective, when an intermediate/last relay UE serves two different downstream UEs, there is no assumption on whether it presents the same or different L2IDs for connection establishment. - Reuse the same Uu RSRP thresholds for discovery transmission as in the legacy (i.e., single hop L2 U2N relay) for L2 Remote UE and L2 U2N last Relay UE in multi-hop U2N relay. - Reuse the same discovery resource pool(s) and configurations as in the legacy (i.e., single hop L2 U2N relay) in multi-hop U2N relay. - For model A discovery, there are no additional AS criteria on discovery message forwarding (the existence of the PC5 link is sufficient). - For model B discovery, if the PC5 link on which the discovery message is received has already been established, no additional AS criteria on discovery message forwarding are applied. FFS if the PC5 link is not already established - For multi-hop U2N relay selection at the remote UE, it reuses the trigger condition of R17 single-hop U2N relay, i.e.: 1) Direct Uu signal strength of current serving cell of the multi-hop U2N Remote UE is below a configured signal strength threshold; 2) Indicated by upper layer of the U2N Remote UE. - For multi-hop U2N Relay reselection trigger at the remote UE, reuse the following R17 single-hop U2N Relay reselection trigger condition: - PC5 signal strength of current first relay UE is below a (pre)configured signal strength threshold. - When U2N Remote UE receives a PC5-S link release message from current first relay UE. - When U2N Remote UE detects PC5 RLF with the current first relay UE. - Indicated by upper layer. - L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design): - cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network. - release of RRC connection between an intermediate relay UE and the network. - PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s). - FFS: link quality degradation of the upstream links resulting in a notification. - FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE). - FFS on the signalling contents of the PC5-RRC indication from the first relay UE. - FFS applicability to L3.

In this paper, we continue to discuss some controversial issues and FFS(s) in relay discovery and selection aspects for Multi-hop UE-to-NW relay.

# 2 Discussion

### 2.1 AS criteria on discovery message forwarding in Model B

For Model B discovery, there is no requirements for intermediate relay UE to establish a link towards the “parent” relay node first. So, in this case, the relay UE only receives the discovery broadcast. Logically, the PC5 SD-RSRP threshold shall be used as an AS layer criterion to determine whether the discovery message (Model-B discovery request or Model B discovery response) need to be forwarded or not.

Proposal 1 	In case of no prior PC5 link establishment, PC5 SD-RSRP threshold are to be used as AS layer criteria for Model-B discovery message forwarding.

It seems redundant for relay UEs to apply this threshold checking in both discovery request and discovery response messages. The reception of discovery request message and discovery response message occur sequentially in a very short time period. If the model-B request has already meet the SD-RSRP threshold in a certain PC5 link, then the discovery response message is for sure to be able to meet the same threshold of that PC5 link, too. However, this is only true if the intermediate relay UEs are configured the same SD-RSRP threshold. If one of the relay UE follows its SIB12 and the other intermediate relay UE uses pre-configuration. Then, the SD-RSRP thresholds could be different. Therefore, it is safe to apply the checking in both directions.

Proposal 2 	Both the forwarding of model-B discovery request message and forwarding of model-B discovery response message are subject to AS layer criterion checking (e.g., meeting or exceeding PC5 RSRP threshold) if there is no link established yet .

### 2.2 Notification Trigger for Remote UE relay reselection

RAN2 has already agreed on the following scenarios that a PC5-RRC notification message is to be generated to trigger remote UE to conduct relay reselection.

- cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network.

- release of RRC connection between an intermediate relay UE and the network.

- PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s)

However, we think the notification message is only the last resort and should not be always used. For example, the degradation of upstream link is a link quality issue for the first intermediate relay UE, and the relay uE need to proactively solve this issue by switching to another relay UE instead of sending a PC5-RRC notification towards the remote UE.

Proposal 3 	Link quality degradation of the upstream links should not trigger a PC5-RRC notification towards the remote UE.

### 2.3 Intermediate Relay UE reselection

Any PC5 link of intermediate relay UE may fail. If such a RLF is detected (for example, as shown in Figure 4) To consider the forward extensibility of relay discovery design, we consider the scenario with two hops as an example:

Figure 3: PC5 RLF detected between 2nd intermediate relay UE and last relay UE

There are two options can be considered:

Option 1: The intermediate relay UE performs relay reselection and find a proper last relay UE.

Option 2: The intermediate relay UE sends notification message to remote UE, and let remote UE to re-initiate Model B discovery.

Option 2 has already been agreed by RAN2 in RAN2-129 meeting [3], but we think Option 1 is defineley a better approach than Option 2 and needs to be supported too. For the sake of fast recovery from scenarios like RLF or cell reselection, it is desirable to support intermediate relay UE to immediately start relay reselection procedure to find an alternative path towards gNB.

Similarly, if a Uu RLF is detected, the last relay UE will inform the Uu RLF to the intermediate relay UE. In this case, the intermediate relay UE shall also trigger relay re-selection for fast recovery.

Proposal 4	Support intermediate relay UE to trigger relay reselection in case of 1)cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure 2) RRC connection of relay UE is released 3) for upstream link(s), PC5-RLF is detected or PC5 link is released.

### 2.4 Uu threshold for operating as Intermediate relay UE

Regarding the Uu threshold issue for intermediate relay UE’s transmission of U2N relay discovery message, RAN2 has agreed that a lower bound is not needed and NW may optionally configure a single upper bound. RAN2 also agrees that if the upper bound is not provided, then the following agreement is still applicable as a restriction to intermediate relay UE:

If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE

However, is the above agreement still applicable if an upper bound is configured? We believe it should be applicable. too. However, depends on whether the upper bound is weaker or stronger than the Uu RSRP lower bound for being a “Last relay UE”, this may be confusing as a UE may be located in a region which is both eligible as a last relay UE and as an intermediate relay UE based on the RRC configuration. Therefore, we think RAN2 need to clarify this.

Proposal 5 	RAN2 clarify that the agreement “If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE “ always take precedence regardless of whether upper bound (for operation as intermediate relay UE) is configured or not.

Regarding the FFS issue “FFS if there would be additional restrictions in the case of no upper bound.”, we need to consider two cases:

- (a) There is no configured upper bound for being last relay UE, as shown in case (a) of Figure 1.

- (b) There is a configured upper bound (e.g., threshHighRelay-r17) for being the last relay UE, as shown in case (b) of Figure 1.

Figure 1: Relationship between intermediate relay UE location region and Uu RSRP

(a) w/o threshHighRelay-r17 configured vs (b) w/ threshHighRelay-r17 configured

If there is no configured upper bound for being the last relay UE, then it means any UE which satisfied the lower bound to being a relay UE will be acting as a relay UE. For this case, there is no additional restriction needed.

But for the other case, according to the current agreement, an intermediate relay UE may happen to locate very close to the centre of the cell as long as its Uu RSRP measurement is larger than threshHighRelay-r17. In this case, per the current RAN2 agreement, as the UE is not eligible to be a last relay UE, then it is eligible to be an in-coverage intermediate relay UE. But this is not a reasonable configuration, if the motivation to configure threshHighRelay-r17 is to prevent strong SL transmission in UL band from an in-coverage relay UE from interfering gNB reception, then this motivation is also applicable to the intermediate relay UE. Therefore, we think even if the upper bound for intermediate relay UE operation is not configured, there should always be an implicit restriction of keeping intermediate relay UE at least outside of the in-coverage last relay UE eligible region.

Proposal 6 	When Uu RSRP upper bound for operating as an intermediate relay UE is not configured, a relay UE only operate as intermediate relay UE towards the same cell if its Uu RSRP measurement is less than the configured lower bound of Uu RSRP for operating as a last relay UE.

### 2.5 Resource allocation for discovery transmission

Regarding the FFS in the following agreement [1]:

Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs; FFS mode 1 for remote UE.

We think for remote UE in Multi-hop R19 scenario, it does not have a direct Uu link and it cannot receive DCI signal transmitted by gNB. Therefore, mode 1 resource allocation is anyway crippled this remote UE as not all mode 1 mechanisms are working. It is much simpler to not support mode 1 and only allow remote UE to use resource allocation mode 2.

It is also worth clarifying that the agreement “Both of resource allocation mode 1 or mode 2 can be supported as in Rel-17/18 at least by relay UEs;” means for legacy R17/R18 relay which has to be the last relay UE. For intermediate relay UE which connects to the gNB via multiple hops, mode 1 support is also questionable. Hence, a much cleaner design is to only have mode 2 resource allocation for both remote UE and intermediate relay UEs.

Proposal 7 	Only mode 2 RA is supported for remote UE and intermediate relay UE.

# 3 Conclusion

In this contribution, we discuss the Relay discovery and selection for Multi-hop UE-to-NW Relay, and have the following proposals:

Proposal 1 	In case of no prior PC5 link establishment, PC5 SD-RSRP threshold are to be used as AS layer criteria for Model-B discovery message forwarding.

Proposal 2 	Both the forwarding of model-B discovery request message and forwarding of model-B discovery response message are subject to AS layer criterion checking (e.g., meeting or exceeding PC5 RSRP threshold) if there is no link established yet .

Proposal 3 	Link quality degradation of the upstream links should not trigger a PC5-RRC notification towards the remote UE.

Proposal 4	Support intermediate relay UE to trigger relay reselection in case of 1)cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure 2) RRC connection of relay UE is released 3) for upstream link(s), PC5-RLF is detected or PC5 link is released.

Proposal 5 	RAN2 clarify that the agreement “If a UE can operate as a last relay UE according to the restrictions configured by the network, it does not operate as an intermediate relay UE towards the same cell for any remote UE “ always take precedence regardless of whether upper bound (for operation as intermediate relay UE) is configured or not.

Proposal 6 	When Uu RSRP upper bound for operating as an intermediate relay UE is not configured, a relay UE only operate as intermediate relay UE towards the same cell, if its Uu RSRP measurement is less than the configured lower bound of Uu RSRP for operating as a last relay UE.

Proposal 7 	Only mode 2 RA is supported for remote UE and intermediate relay UE.

# 4 References

[1] Chairman’s notes, RAN2#127

[2] Chairman’s notes, RAN2#128

[3] Chairman’s notes RAN2#129