3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502195

Wuhan, CN, April 7-11, 2025

Agenda Item:	8.8.2

Source:	THALES

Title:	Discussion on cell barring for NR NTN downlink coverage enhancements

Document for:	Discussion and Decision

# 1	Introduction

The downlink coverage enhancement for Non-Terrestrial Networks (NTN) is being part of Rel-19 as approved in the NTN phase 3 Work Item [1]. For the system-level enhancements part, the SS/PBCH block periodicity is being extended up to 160 ms. It brings flexibility for power-sharing between satellite beams across the satellite footprint :

[…] - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms […]

In this contribution, we discuss the cell barring mechanism to apply in cells operating in downlink coverage enhancement mode.

# 2	Discussion

## Cell barring

In the RAN2#129 meeting [2] the cell barring to be applied for cells operating in downlink coverage enhancement mode has been discussed:

Proposal 6: It is up to NW implementation to set cellBarredNTN-r17 to barred to bar UEs not supporting R19 extended SSB periodicity. A new field cellBarredNTN-r19 (value is set to barred or notBarred) is introduced for future use. UEs supporting extended SSB periodicity ignore cellBarredNTN-r17 if the new cellBarredNTN-r19 is present, and follow cellBarredNTN-r17 if the new cellBarredNTN-r19 is absent. Continue in the next meeting

Two questions have to be solved to decide if a new cell barring procedure is needed in Rel-19 for downlink coverage enhancement:

- Is a cell operating with extended SS/PBCH block periodicity (i.e. different from 20 ms) is a different cell than a non-DL CE cell from the UE standpoint ?

- Is a Rel-19 cell supporting link level enhancements is backward compatible to legacy UE ?

The Rel-19 Downlink Coverage Enhancement feature includes two parts: a system-level (i.e. extension of the SSB periodicity) enhancement part and a link-level (i.e. PDCCH and PDSCH) enhancements part.

For the first question, what is new with the downlink coverage enhancement is the SSB periodicity extension for the initial access. SSB periodicites up to 160 ms are already supported in the specifications for secondary cells, for example [3] :

ssb-periodicityServingCell ENUMERATED { ms5, ms10, ms20, ms40, ms80, ms160, spare2, spare1 } OPTIONAL, -- Need S

Confirmed in RAN2#127 that there is no RAN2 impact on SSB configuration if the SSB periodicity is not larger than 160 ms:

From RAN2 point of view, if the SSB periodicity is no larger than 160ms, there is no RAN2 impact on SSB configuration (there might still be impacts on DTX aspects)

The SSB periodicity up to 160 ms is already supported in the specifications for Connected UE in a secondary cell, but not for initial access cell

The UE can get the SSB periodicity different from 20 ms after decoding SIB1 (e.g. ServingCellConfigCommonSIB).

For the initial access with cell extended SSB periodicity, RAN1#119 made the observation below:

Observation Backward compatibility for legacy UEs (i.e. Rel-17 and Rel-18 UEs) assuming a default SSB periodicity of 20ms is not guaranteed when SS/PBCH blocks periodicity is larger than 20ms within the cell used for initial frequency scan. Legacy UEs (i.e. Rel-17 and Rel-18 UEs) are not expected to be able to camp on a cell with SS/PBCH blocks periodicity larger than 160 ms.

The backward compatibility cannot be ensured for legacy UEs. However, if a UE is able to decode PBCH and SIB1, there is no reason to prevent it from connecting with a barring mechanism.

Based on RAN1 observation, legacy UEs may not be able to camp on Rel-19 cell with extended SSB periodicity

In all the cases, the UE that is not able to decode SIB1 will not be able to decode barring bit. In this case, the UE will just not be able to detect the cell.

If a UE is not able to decode SIB1 in a cell with extended periodicity, therefore even if cell barring is defined, the UE will not be able to decode it

In addition, from the satellite network operator standpoint, the dynamic and flexible power-sharing is critical to optimize the system capacity. If we go for preventing pre-Rel19 UE connecting to a cell that is operating in DL CE mode with extended SSB periodicity, it means that a cell transitioning to DL CE mode for whatever operator decision will have to reject the pre-Rel19 UE that was accepted before. This would limit the interest in the feature, even if it is essential for dynamic satellite deployment, so the management of Rel-19 DL CE cells and “legacy” cells is left to network implementation.

Up to the network to cope with compatibility issues for legacy UEs when the cell operates with extended SSB periodicity

A new barring procedure is not needed for a cell that is operating with extended SSB periodicity

For the second question, the WID objective does not precise a need for backward compatibility for link level enhancements (i.e. PDCCH and PDSCH). RAN1 still discuss the link level aspects for DL CE and possible backward compatibility. The conclusion may be to rely on UE capabilities as for uplink coverage enhancements in Rel-19. However, this discussion can be postponed until the RAN1 conclusion on the features list.

Postpone the discussion on DL CE cell barring to RAN1 conclusion on link level enhancements if the backward compatibility with pre-Rel19 UEs cannot be ensured

# 3	Conclusion

In thi contribution, we made the following observations and proposals:

<Cell barring>

- The Rel-19 Downlink Coverage Enhancement feature includes two parts: a system-level (i.e. extension of the SSB periodicity) enhancement part and a link-level (i.e. PDCCH and PDSCH) enhancements part.

- The SSB periodicity up to 160 ms is already supported in the specifications for Connected UE in a secondary cell, but not for initial access cell

- Based on RAN1 observation, legacy UEs may not be able to camp on Rel-19 cell with extended SSB periodicity

- If a UE is not able to decode SIB1 in a cell with extended periodicity, therefore even if cell barring is defined, the UE will not be able to decode it

- Up to the network to cope with compatibility issues for legacy UEs when cell operates with extended SSB periodicity

- A new barring procedure is not needed for a cell that is operating with extended SSB periodicity

- Postpone the discussion on DL CE cell barring to RAN1 conclusion on link level enhancements if the backward compatibility with pre-Rel19 UEs cannot be ensured

# References

RP-243300, Revised WID: Non-Terrestrial Networks (NTN) for NR Phase 3, RAN#106, 2024-12, Madrid, ES.

R2-2501333, Report from Break-out session on NR-NTN and IoT-NTN, RAN2#129, 2025-02, Athens, GR.

3GPP TS 38.331 “5G; NR; Radio Resource Control (RRC); Protocol specification”, Rel-18.