3GPP TSG RAN WG2#129bis	R2-2502213

Wuhan, China, 7th - 11th April 2025

Agenda item:	8.4.3

Source:	Nokia

Title:	RRM measurement relaxation in RRC_IDLE/INACTIVE

WID/SID:	NR_LPWUS-Core - Release 19

Document for:	Discussion and Decision

# 1	Introduction

In RAN#102 meeting, a new work item (RP-240801) was agreed for Low Power Wake Up Signal and Receivers designs. These designs are to be primarily targeted at delay and power-sensitive, small form-factor devices, such as industrial sensors, controllers and wearables. Unlike previous power saving study items, the objectives for this study encompasses new signals and receiver architectures.

In this contribution we focus on the following objective:

- For IDLE/INACTIVE modes

  - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

# 2	Discussion

As a part of the WID objectives the need to specify RRM relaxation. These procedures are mostly related to RAN2 and RAN4:

- For IDLE/INACTIVE modes - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

If MR measurements are relaxed, but not fully omitted, some joint evaluation of threshold based on measurements from both, MR and LR could be considered. It is not clear whether these would need to be accounted as joint quantity, or whether the evaluation could be done based on measurements from both, LR and MR.

Proposal 1: RRM relaxation entry and exit condition can include both LR and MR measurement evaluations. FFS whether LR and MR measurements are somehow combined or evaluated independently.

RAN2 received an LS [1] from RAN1 where it was indicated the UE may not support LP-WUS reception on all the bands supported by the UE. When LP-WUS is deployed it can be assumed that not all the cells will support LP-WUS. In some NW implementations LP-WUS support can be introduced in some specific cells or frequency(s). For the UE supporting LP-WUS it would be more saving friendly to camp on the cell which is supporting LP-WUS. It can be assumed that from system perspective it would be beneficial if the UE always camps and starts the access on the best cell of the frequency to avoid interference. Different cell (re)selection enhancements could be studied for enabling that the UE camps on the cell supporting LP-WUS more often, possibly considering adjusting or applying different priorities based on the LP-WUS support/coverage.

Proposal 2: UE supporting LP-WUS prioritizes frequencies supporting LP-WUS in the inter-frequency cell (re)selection procedure.

LP-WUS is intended to be used for the benefit of UE power saving. One approach for UE power saving is to offload some or all current measurements performed by the MR to the LR. At least in the rel-18, only serving cell measurements are performed by LR. There are currently various procedures, such as EMR (aka idle/inactive measurements) and cell reselection measurements, which aim to assist both network and UE to perform fast data transmission (e.g. with fast Scell activation) upon connection setup/resume.

The rel-18 work on fast CA/DC setup also expanded the idle-mode procedures to cover cell reselection measurements.  By using LP-WUS, the benefits of these features would be diminishing as there would be no measurements reports to provide to the network, since some or all measurements can be offloaded to LR and LR may not be able perform neighbour cell measurements. In offloading case even all MR measurements can be omitted when LP-WUS is used.

Proposal 3: Discuss what is the intended UE behaviour while MR relaxation or offloading is applied, and UE is configured with IDLE/INACTIVE measurements

The following was agreed in RAN#127 meeting for LP-WUS monitoring condition:

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

Since cell may not support both types (OFDM and OOK) of LRs similar separate entry/exit thresholds for OFDM-based and OOK-based WUR are needed also for serving cell relaxation and offloading:

Proposal 4: Separate entry/exit thresholds for serving cell measurement relaxation can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Proposal 5: Separate entry/exit thresholds for serving cell measurement offloading can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

# 3	Conclusion

The following is proposed based on the discussion:

Proposal 1: RRM relaxation entry and exit condition can include both LR and MR measurement evaluations. FFS whether LR and MR measurements are somehow combined or evaluated independently.

Proposal 2: UE supporting LP-WUS prioritizes frequencies supporting LP-WUS in the inter-frequency cell (re)selection procedure.

Proposal 3: Discuss what is the intended UE behaviour while MR relaxation or offloading is applied, and UE is configured with IDLE/INACTIVE measurements

Proposal 4: Separate entry/exit thresholds for serving cell measurement relaxation can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

Proposal 5: Separate entry/exit thresholds for serving cell measurement offloading can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs.

# 4	References

- R2-2407921 LS on LP-WUS operation in IDLE/INACTIVE mode