3GPP TSG-RAN WG2 Meeting #129bis	R2-2502215

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:		8.2.2

Source:		Futurewei

Title:		Further discussions on A-IoT paging

Document for:		Discussion and Decision

# Introduction

In RAN2#129, the following was agreed with respect to A-IoT paging [1]:

- Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

- 1 bit solution is excluded.   FFS the size.  Aim to have a reasonable size.

- RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

Agreements on paging ID

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.   FFS if there is really a need for visibility in the MAC layer

- Agreements - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

In this contribution, we further discuss some detailed design aspects of A-IoT paging.

# Discussions

## Device’s behavior when receiving different service request while engaging on another procedure

As a device is expected to perform only up to one procedure at a time, if the device is currently engaged on a procedure related to a service request (meaning that the device has not successfully completed the corresponding procedure yet) and receives a different service request (presumably with a different Transaction ID), different companies have expressed different opinions as to what the device should do: “Some thought this meant device ignores new request since it is not expected to perform another procedure, while others thought this means the device terminates the ongoing procedure and follows the new request (still performing only one procedure at a time)” [2].

First, in our view, the only viable deployment scenario for multiple readers to co-exist with overlapping coverage areas without suffering from interference is that these readers can well coordinate with each other, e.g., they are supported by the same gNB, with the coverage areas of different readers supported by different radio units (RUs) that are spread out within the service area. In this scenario, implementation solutions, e.g., through internal coordination within a common baseband unit, can ensure that readers with overlapping coverage areas carry out their respective procedures during different and non-overlapping time periods and that the values of the Transaction ID used by these readers are far apart from each other, provided that the Transaction ID field is sufficiently long. Therefore, in this paper, we focus on the case where the different service request is from the same reader.

We think there are legitimate use cases where the reader may terminate an old procedure related to an old service request and begin a new procedure related to the new service request without all the devices necessarily completing the old procedure successfully. For example, the reader may terminate an inventory procedure when the target percentage or target number of devices have been successfully inventoried. These targets may have been received from the AF through the CN as a service-level requirement when the inventory request is received. Therefore, it is entirely possible that some device(s) may have not successfully completed the procedure related to the inventory request when the reader terminates it. These devices may still monitor for a subsequent paging message with the same Transaction ID of the service request that they are currently engaged on but have not been successful yet. The question is whether and how the reader informs the devices the termination of a procedure.

In our view, in addition to indicating the new Transaction ID of the new service request in the new paging message, the reader does not need to include (nor separately send) any additional indication indicating that it has terminated the procedure related to the old service request. The new Transaction ID in the new paging message indicates that the procedure related to the old service request has ended and a new procedure related to a new service request has begun.

Proposal 1. In addition to indicating the new Transaction ID of the new service request in the new paging message, the reader does not need to include (nor separately send) any additional indication indicating that it has terminated the procedure related to the old service request.

Proposal 2. The new Transaction ID in the new paging message indicates that the procedure related to the old service request has ended and a new procedure related to a new service request has begun.

Since the device should always follow the instructions from the reader, the device should follow the newly received paging message and abandon the previous service request even if the device has not responded successfully yet. It is up to the reader to ensure that a paging message with a new Transaction ID is not sent until the procedure related to the old service request is terminated.

Proposal 3. The device follows the new paging message with a new Transaction ID and abandons the previous service request even if the device has not responded successfully to the previous service request yet.

## On Paging ID details

SA2 has defined a structure for A-IoT device’s permanent ID, which supports both operator allocation IDs and third-party allocated IDs, potentially with different lengths. According to the latest TR 23.700-13 [3], operator allocated ID and third-party allocated ID are shown in Figures 1 and 2, respectively.

Figure 1 (same as Figure 8.2.1-1: The example of Operator allocated ID in [3])

Figure 2 (same as Figure 8.2.1-2: The example third party allocated ID in [3])

The device’s permanent ID or a mask of it may be used in the A-IoT paging message as the paging ID for use cases and deploy environments where there is no security or privacy concern. Meanwhile, SA3 has agreed that a mechanism to protect A-IoT device ID based on the use of temporary ID shall be supported [4]. The length of the temporary ID, including whether the length may be variable, is unknown at this point. Given the options of using the permanent ID, or a mask of it, or the temporary ID as the paging ID in the paging message, RAN2 should design the paging ID field with the assumption that the paging ID may have a variable length.

There are two options for including a variable-length paging ID field in the paging message. The first option is to always mandate the paging ID be included as the last field in the paging message so that there may be no need to explicitly indicate the length of the paging ID. However, mandating such a variable-length field to always be the last field in the paging message has the following drawbacks:

- Although in Rel-19, the agreement so far is that there can be only one ID in the paging message, RAN2 may decide to support more than one IDs in the paging message in future releases. Without an ID length indication, it is hard to handled multiple IDs in the paging message.

- Mandating the paging ID to always be the last field in the paging message may be too restrictive for forward compatibility, when new information needs to be added to the paging message in future releases.

- If padding bit(s) are needed, the padding bit(s) are usually added at the end of the message. Then, there may be an ambiguity as to which bits belong to the paging ID field and which bits are the padding bits.

The second option is that an ID length field is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message, i.e., the paging-all case. The ID length field will help a recipient to retrieve the paging ID from the A-IOT paging message properly while supporting a flexibility of the placement of the paging ID in an A-IoT paging message.

In our view, the first option is not viable due to the drawbacks described above. Therefore, we propose that an ID length field is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

Proposal 4. An ID length field is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

## On temporary device ID

The temporary (device) ID is to be designed by SA3. We just need to focus on its impacts, if any, to RAN2’s aspects.

As discussed above, when used as the paging ID in an A-IoT paging message, the length of the temporary ID also needs to be indicated in the A-IoT paging message. Therefore, there is a need to know whether the temporary ID has a fixed length or a variable length, and if variable-length, whether the CN can provide the ID length information to the reader. RAN2 can send LS to SA3/SA2 to seek answers to these questions.

Although the design is up to SA3, we can expect that individual temporary ID assigned for security and privacy purpose should appear as random as possible. Therefore, a pre-specified group of devices will unlikely share a common mask of their individual temporary IDs to be used as their group ID. However, there may be a possibility that a temporary ID can be generated and assigned to a group of devices as the group ID of the group and be used as the paging ID in an A-IoT paging message for paging the group of devices. However, to assign a group ID to a group of device requires a group management procedure, which should be carried out at upper layers, e.g., at the A-IoT NAS layer, and hence is out of the scope of RAN2. However, RAN2 can send LS to SA2/SA3 to seek their feedback on whether this is possible.

Observation: Individual temporary IDs assigned for security and privacy purpose are expected to appear as random as possible. Therefore, a pre-specified group of devices will unlikely share a common mask of their individual temporary IDs to be used as their group ID. However, there may be a possibility that a temporary ID can be generated and assigned to a group of devices as the group ID of the group and be used as the paging ID in an A-IoT paging message for paging the group of devices.

Proposal 5. Send an LS to SA3 and SA2 ask them: 1) whether the temporary ID has a fixed length or a variable length, and if variable-length, whether the CN can provide the ID length information to the reader; and 2) whether a temporary ID can be generated and assigned to a group of devices as the group ID of the group and be used as the paging ID in an A-IoT paging message for paging the group of devices.

## Information for triggering device(s) to re-access

RAN2 has decided, during Rel-19 SI phase, that (ACK/NACK-based) RLC-like retransmission is not supported for A-IoT. Meanwhile, after failing to receive an expected D2R response from a recognized device, the reader is allowed to send another R2D trigger message including the same Command Request to cause the device to retransmit the Command Response, as devices always follow the command from the reader. However, if we do not mandate such behaviors (i.e., immediately re-sending the same Command Request after a failure) on all readers, and when there is no further R2D data transmission to be sent to the device, there is still ambiguity on the device side as to whether its previous D2R data transmission has been received successfully or not (and whether it needs to re-access or not) if the device does not receive anything back from the reader.

To address the ambiguity issue mentioned above, we propose that after all transmissions triggered by a current paging (or similar R2D trigger) message are complete, whether successfully or unsuccessfully, the reader sends a subsequent paging (or similar R2D trigger) message with the same Transaction ID. This subsequent paging message can include the RN16s of the device(s), if any, from which the Msg1 has been received but the Msg3 or subsequent D2R data transmission, which was expected during the previous paging round, has not been received successfully by the reader. In response to seeing their respective RN16s being listed, these devices can make a re-access attempt either during this subsequent paging round or a later paging round with the same Transaction ID. Devices, which had transmitted during the previous paging round and the RN16 of which is not indicated in this subsequent paging (or similar R2D trigger) message considers its previous D2R transmission to the reader has been successful and therefore will not respond to any subsequent paging (or similar R2D trigger) message with the same Transaction ID. The subsequent paging (or similar R2D trigger) message may include information to trigger additional devices, which were unable to participate in the previous paging rounds with the same Transaction ID, to participate in the current paging round.

Proposal 6. A subsequent paging (or similar R2D trigger) message with a same Transaction ID can include the RN16s of the device(s), if any, from which the Msg1 has been received but the Msg3 or subsequent D2R data transmission, which was expected during the previous paging round, has not been received successfully by the reader. In response, these devices can make a re-access attempt either during this subsequent paging round or a later paging round triggered with the same Transaction ID.

Proposal 7. Devices, which had transmitted during the previous paging round and the RN16 of which is not indicated in this subsequent paging (or similar R2D trigger) message considers its previous D2R transmission to the reader has been successful and therefore will not respond to any subsequent paging (or similar R2D trigger) message with the same Transaction ID.

# Conclusions

We propose the following:

Proposal 1. In addition to indicating the new Transaction ID of the new service request in the new paging message, the reader does not need to include (nor separately send) any additional indication indicating that it has terminated the procedure related to the old service request.

Proposal 2. The new Transaction ID in the new paging message indicates that the procedure related to the old service request has ended and a new procedure related to a new service request has begun.

Proposal 3. The device follows the new paging message with a new Transaction ID and abandons the previous service request even if the device has not responded successfully to the previous service request yet.

Proposal 4. An ID length field is always included together with the paging ID field in the A-IoT paging message, except the case where no ID is included in the A-IoT paging message.

Observation: Individual temporary IDs assigned for security and privacy purpose are expected to appear as random as possible. Therefore, a pre-specified group of devices will unlikely share a common mask of their individual temporary IDs to be used as their group ID. However, there may be a possibility that a temporary ID can be generated and assigned to a group of devices as the group ID of the group and be used as the paging ID in an A-IoT paging message for paging the group of devices.

Proposal 5. Send an LS to SA3 and SA2 ask them: 1) whether the temporary ID has a fixed length or a variable length, and if variable-length, whether the CN can provide the ID length information to the reader; and 2) whether a temporary ID can be generated and assigned to a group of devices as the group ID of the group and be used as the paging ID in an A-IoT paging message for paging the group of devices.

Proposal 6. A subsequent paging (or similar R2D trigger) message with a same Transaction ID can include the RN16s of the device(s), if any, from which the Msg1 has been received but the Msg3 or subsequent D2R data transmission, which was expected during the previous paging round, has not been received successfully by the reader. In response, these devices can make a re-access attempt either during this subsequent paging round or a later paging round triggered with the same Transaction ID.

Proposal 7. Devices, which had transmitted during the previous paging round and the RN16 of which is not indicated in this subsequent paging (or similar R2D trigger) message considers its previous D2R transmission to the reader has been successful and therefore will not respond to any subsequent paging (or similar R2D trigger) message with the same Transaction ID.

# References

R2_129_ChairNotes_25-02-21_final.

R2-2502211, Email discussion report: [POST129][035][AIoT] Paging, Qualcomm (Rapporteur).

TR 23.700-13 V1.3.0.

R2-2501502_S3-251055, Reply LS on security aspects of Ambient IoT (with attachment of S3-251048), SA3.