3GPP TSG-RAN WG2 Meeting #129bis	R2-2502218

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:		8.7.5

Source:		Futurewei

Title:		On RLC Status report after discarding outdated AMD PDU

Document for:		Discussion and Decision

# Introduction

During [POST129] e-mail discussions [508] on XR Stage-2 CR [1] and [512] on XR RLC CR [2], companies expressed different opinions regarding whether “The receiving side of an AM RLC entity shall trigger a STATUS report when t-RxDiscard expires.”

In this contribution, we discuss why the receiving side of the AM RLC entity should not immediately trigger and shall not autonomously trigger a STATUS report when t-RxDiscard expires.

# Discussions

## Existing agreements on avoiding unnecessary RLC retransmissions

In RAN2#127bis, the following was agreed with respect to avoiding unnecessary RLC retransmissions [3]:

Agreements on avoiding unnecessary retransmissions - RAN2 confirm the previous baseline assumption: the RLC receiving window always advances to any given RLC SN before the transmitting window does. - RAN2 will adopt a “combined” approach for avoiding unnecessary RLC retransmissions, i.e. - TX side stops transmissions of an outdated SDU - RX side abandons the SDU based on a local timer - Rx informs Tx side about the abandoned SDUs, as a baseline we assume existing SR can be reused unless issues are identified - FFS if some C-PDU handling is needed to avoid C-PDU discard - FFS if some indication is sent from Tx to Rx. The assumption is this is not a full status report, but something simple (if needed)

In RAN2#128, the following was further agreed with respect to avoiding unnecessary RLC retransmissions [4]:

Agreements on unnecessary RLC retransmissions avoidance - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced

A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer.

The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

In RAN2#129, the following was further agreed with respect to avoiding unnecessary RLC retransmissions [5]:

Agreements on RLC enhancements Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly - Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

One argument made during the e-mail discussion for immediately sending the SR is that RAN2#128 made the following agreement: “The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.”

However, this agreement essentially states that, if an SR is to be reported (e.g., due to any of the legacy triggers), ACK should be used for reporting those RX-side-discarded SDUs. But it doesn’t necessarily mean that an SR should be immediately triggered due to the discarding event. In fact, the earlier agreement made in meeting#127bis, as highlighted above, emphasizes that the existing SR is to be reused. In our view, that includes reusing the existing triggers if they alone can satisfy the purpose of sending this SR.

In the next, we will examine why we send this SR in the first place.

## On the reason for sending this SR

RAN2 agreed, in meeting#129, that the TX side discards outdated SDU (and avoid the retransmission thereof) when receiving a discard indication of that SDU from PDCP. So, clearly, the SR is sent not for the purpose of stop the retransmission of outdated AMD PDU.

One original concern was that if there is no SR, the TX window may be stalled after a while by the SN gap created by the discarding. Since RAN2 have not specified how to prevent the TX window stalling due to RX-side SDU discarding, one argument used for immediately sending the SR is to prevent TX window stalling. However, in our view, there is no need for RAN2 to specify how to prevent TX window stalling at the gNB (given specification on the RX-side SDU discarding is only on the UE), for the following reason:

First, based on our analysis in our previously submitted paper [6], there is no urgency to immediately send an SR, because the time duration it takes for the SN gap to stall the TX window is about 156 msec (with 150Mbps data rate and 12-bit RLC SN), much longer than any of t-Reassembly and t-RxDiscard, given the short PDB/PSDB (10 msec, typically) of XR. Therefore, there are plenty opportunities that an SR triggered by the legacy triggers will do the job.

Secondly, even if there is no SR that can be timely triggered by any of the legacy triggers or the SR triggered by the expiry of t-Reassembly may be premature, the gNB still can poll (i.e., by using the Polling bit in the RLC header of AMD PDU) the UE to send back an SR, given the gNB is the TX side that has discarded (and abandoned the retransmission of) the outdated SDUs in the first place, the gNB is the one that has configured the t-RxDiscard on the UE, and the gNB has a quite wide time margin to poll the UE before the TX window stalling may happen. Given the existing specification supports it, when and for what reason the gNB polls the UE to send RLC SR is an implementation issue.

In addition to these standard-based solutions, in the next section, we will describe a gNB implementation based solution, which can work as a standalone solution or work together with the standard-based solutions.

## gNB implementation based solution without using OTA signaling

It is understood that the RX window should advance before the TX window. Given that the time for an SN gap created by the discarded PDUs to stall the TX window is so much longer than the PDB/PDSB and the t-RxDiscard, the gNB, by implementation, can push its TX window forward and over the SN gap before the stalling can happen and yet after the RX window has been advanced over the SN gap with a great level of confidence. For example, since the t-RxDiscard value is configured by the gNB, the gNB can push its TX window forward and over the SN gap after a time period that is sufficiently longer than t-RxDiscard and yet before the stalling can happen, after the gNB discards obsolete RLC PDUs that are pending a retransmission.

Observation1: Both the standard based solutions and the gNB implementation based solution, as described above, can ensure that TX window stalling will not happen in case of discarding outdated AMD PDU.

## Difference from the case of t-Reassembly expiry

TS 38.322 specifies that “The receiving side of an AM RLC entity shall trigger a STATUS report when t-Reassembly expires” [7]. Allowing the UE to autonomously and immediately trigger an SR, in the case of t-Reassembly expiry, is because when the UE detects a reception failure of an AMD PDU, we want the UE to report a NACK for the failed SDU/PDU as soon as possible so that an RLC retransmission can be attempted as soon as possible.

However, in the case of t-RxDiscard expiry, both the TX side and the RX side have abandoned the (re)transmission of outdated SDU/PDU. Whether to send the SR sooner or later bears no effect on the abandoning for both sides. As we have discussed above, there is no urgency to send the SR immediately after t-RxDiscard expiry to prevent TX window stalling either. Therefore, the UE shall not autonomously send a Status Report when t-RxDiscard expires.

Observation 2. Triggering an SR after t-RxDiscard expiry has different purpose and different requirement on timeliness than triggering an SR after t-Reassembly expiry.

Proposal. The UE shall not autonomously send a Status Report when t-RxDiscard expires.

# Conclusions

Observation 1: Both the standard based solutions and the gNB implementation based solution, as described above, can ensure that TX window stalling will not happen in case of discarding outdated AMD PDU.

Observation 2. Triggering an SR after t-RxDiscard expiry has different purpose and different requirement on timeliness than triggering an SR after t-Reassembly expiry.

Therefore, we propose the following:

Proposal. The UE shall not autonomously send a Status Report when t-RxDiscard expires.

# References

R2-250xxxx Draft Stage 2 CR XR Rel-19 v21 Rapporteur.

R2-250xxxx_Running RLC CR for XR_v24_Rapp.

R2-2409215, Report from session on R18 MBS, R18 QoE and R19 XR, Session Chair (Huawei).

R2-2410915, Report from session on R18 MBS, R18 QoE and R19 XR, Session Chair (Huawei).

R2-2501335, Report from session on R18 MBS, R18 QoE and R19 XR, Session Chair (Huawei).

R2-2407511, Discussions on RLC enhancements for Rel-19 XR, Futurewei.

TS 38.322 v18.2.0.