3GPP TSG-RAN WG2 Meeting#129bis	    			                                                   R2-2502220

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.2.2

Source:	ETRI

Title:	Discussion on A-IoT Paging

Document for:	Discussion and Decision

Introduction

In R19, a new WI: Solutions for Ambient IoT (Internet of Things) in NR was setup and the objective for the random access is as below[1]:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

In this paper, we would like to discuss the issues on A-IoT paging.

Discussions

Device behavior during simultaneous paging

In the RAN2#127 meeting, the following agreements were made for simultaneous paging procedures:

• Parallel service requests by the same reader are not supported. • The device is expected to perform only one procedure at a time. Finalization of device behavior in case multiple requests are received in parallel is still for further study (FFS), if needed.

However, there was no agreement on the device behavior when multiple requests are received in parallel. A device may receive multiple paging messages when services are simultaneously requested by the CN/AF. Each request can be classified as either a re-request of an already triggered service or a new service request. A re-request occurs when the CN/AF does not receive a response to a requested service and attempts to request the same service again. A new service request occurs when the CN or AF first requests one service from the device, then requests a different one.

These scenarios can be further divided into the following:

Single-reader Scenario

In this scenario, even if the reader receives a new service request from the CN/AF, it does not trigger a new paging procedure while an existing paging is ongoing. Instead, it waits until the ongoing paging procedure is completed, and then performs a new paging based on the subsequent request.  Since the device does not receive a new paging during an ongoing procedure, there is no impact on the device’s behavior. However, to support this mechanism, the reader must be capable of suspending any new service requests from the CN/AF until the current paging is completed, and only then proceed with the newly received request.

Observation 1. In a single reader scenario, simultaneous paging does not occur.

Multi-readers Scenario

This scenario refers to a case where the CN/AF requests the same service from multiple readers. In this case, the device may receive multiple paging messages for the same service from different readers simultaneously. Since coordination between readers is not assumed, this kind of scenario is possible. This is particularly problematic in dense A-IoT deployments where many readers are located in close proximity. Multiple paging messages for the same service may lead to inefficient use of radio resources and increased transmission delay due to frequent collisions. Therefore, a mechanism is required to prevent redundant procedures for the same service in this scenario.

As discussed in the previous RAN2 meeting, this issue was to be addressed through the design of the transaction ID. Therefore, the Transaction ID design should take this scenario into account.

Observation 2. In a multiple reader scenario, simultaneous paging may occur, and this should be considered in the design of the transaction ID.

Following mechanisms can be considered for handling multiple paging messages from different readers.

Option 1: Selective Handling (Selecting one paging message)

  - Option 1-1: Prioritize the ongoing paging

This approach continues the currently ongoing paging and discards the newly received paging message.

  - Option 1-2: Prioritize the newly received paging

This approach stops the ongoing paging and processes the newly received paging instead.

Option 2: Buffering of Newly Received Paging

This approach buffers new paging and processes them only after the ongoing paging procedure is completed.

Given that most A-IoT services are not delay-sensitive, option 1 can be compensated for through a service re-request by the CN/AF or re-triggered paging from the reader. In addition, since the ongoing paging has already started, option 1-1 is more efficient than option 1-2 in terms of resource usage. Option 2 requires the device to have memory to store the received paging. When processing the buffered paging, the device should check whether the necessary resources are available for the procedure, which leads to increased device complexity.

Therefore, based on above discussion, it is feasible to prioritize ongoing procedure when a device receives multiple paging messages.

Proposal 1. it is feasible to prioritize ongoing procedure when a device receives multiple paging messages.

Transaction ID

The Transaction ID enables the device to identify the paging message associated with the service request from the CN/AF. In the previous RAN2 meeting, it was agreed that the device performs only one procedure at a time. Therefore, in a single-reader scenario, a 1-bit indicator such as an NDI-type is sufficient to distinguish between an ongoing procedure and a new one.

Proposal 2. In a single reader scenario, a 1-bit transaction ID is sufficient.

However, in a multi-reader scenario where there is no coordination among readers, the device may receive a paging message with same transaction ID from different readers. Therefore, the Transaction ID should be designed with consideration for the maximum number of services that a device may support. For example, if up to four services are expected, the Transaction ID can include 2 bits to indicate each service.

Proposal 3. In a multi-reader scenario, an identifier indicating the service requested by CN/AF should be included in the transaction ID, based on the maximum number of services.

Considering the scenarios above, the transaction ID could consist of an identifier indicating a paging change and another indicating service. Therefore, assuming a maximum of N services, the total size of the transaction ID would be log₂N + 1 bits.

Proposal 4. The Transaction ID consists of a 1-bit service identifier and a service identifier of log₂N bits, resulting in a total size of 1 + log₂N bits.

Paging ID visibility at MAC layer

The paging ID is a device identifier included in the paging message, and it can be either a single device ID or a group device ID. In the previous RAN2 meeting, there was a discussion on whether the paging ID should be visible to the MAC layer, but no agreement was reached.

If the Paging ID is visible to the MAC layer, the MAC layer can filter the received paging and determine whether it should be processed or discarded by the device. This allows the MAC layer to reduce unnecessary upper-layer processing by filtering out irrelevant paging messages. However, since the Paging ID is generated by the NAS layer, decapsulating and handling it at the MAC layer may raise concerns from a protocol design perspective.

If the Paging ID is not visible to the MAC layer, the MAC forwards the paging message to the upper layers, where it is determined whether to process or discard it. In this case, since processing occurs in the upper layers, some delay may be introduced, and additional battery consumption is expected. However, since most A-IoT services are not real-time, the delay is not a significant issue. As for energy consumption, it is relatively marginal compared to MAC-level filtering, given the involvement of upper-layer handling. Therefore, it is feasible for the paging ID not to be visible to the MAC layer.

Proposal 5. The paging ID is not visible to the MAC layer.

Resource scheduling for msg1 transmission

Resource size for msg1 transmission

The size of Msg1 can vary depending on the RA(Random Access) type. In 3-step CBRA, Msg1 contains only the RN16, whereas in 2-step CBRA, Msg1 may include both the RN16 and the upper-layer device ID. Consequently, the size of the D2R resource required to transmit Msg1 varies depending on the type of random access.

As a result, the paging message needs to include information about RA type, indicating whether the random access is 2-step or 3-step CBRA. However, in the previous RAN2 meeting, it was agreed that only 3-step CBRA will be considered for Rel-19. Therefore, the paging message does not need to include information about the RA type.

Although only 3-step CBRA is considered, the resource size for Msg1 transmission can still vary depending on physical layer transmission parameters. Therefore, the paging message should include the resource size for Msg1 transmission. Even when physical layer transmission parameters are fixed, the device still needs to be informed of the MSG1 resource size (e.g., the number of access occasions).

Proposal 6. The paging message includes the resource size required for Msg1 transmission (e.g. the number of access occasions).

Resource selection for msg1 transmission

Upon receiving a paging message, the device selects D2R resources for Msg1 transmission. In CFRA, the device requires resource configuration information for Msg1 transmission, and which should be included in the paging message.

Proposal 7. In CFRA, the paging message includes resource configuration information for Msg1 transmission.

In CBRA, the device selects the resources for Msg1 transmission based on the Slotted Aloha. The device generates a random number based on the Q value-like information for resource selection and decrements it at each access occasion. The device generates a random number for resource selection and decrements it at each access occasion. When the number reaches zero, the device transmits Msg1. To facilitate this procedure, the reader should provide Q value-like information in the paging message.

Proposal 8. In CBRA, the paging message includes Q value-like information to support resource selection for Msg1 transmission.

Indication of Msg1 resource(s)

The reader provides information that allows the device to recognize each access occasion. Regarding this issue, following two options were agreed in the last RAN2#129 meeting.

- From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: - Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or - Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field

Since the information indicating access occasion is time-critical, it is considered feasible to transmit it via L1 signaling. However, if a MAC message is considered, we prefer defining a new R2D message similar to the QueryRep-like in RFID, rather than using the paging message.  This is because paging is generally intended to notify the device that a service request has been received from the CN/AF. Therefore, using it to indicate each access occasion would not align with the original purpose of paging.

Proposal 9. If the MAC message is considered, defining a new R2D message is preferable indicating D2R resource (e.g. access occasion).

Conclusions

Based on the discussion in the contribution, we have the following observations and proposals:

Observation 1. In a single reader scenario, simultaneous paging does not occur.

Observation 2. In a multiple reader scenario, simultaneous paging may occur, and this should be considered in the design of the transaction ID.

Proposal 1. it is feasible to prioritize ongoing procedure when a device receives multiple paging messages.

Proposal 2. In a single reader scenario, a 1-bit transaction ID is sufficient.

Proposal 3. In a multi-reader scenario, an identifier indicating the service requested by CN/AF should be included in the transaction ID, based on the maximum number of services.

Proposal 4. The Transaction ID consists of a 1-bit service identifier and a service identifier of log₂N bits, resulting in a total size of 1 + log₂N bits.

Proposal 5. The paging ID is not visible to the MAC layer.

Proposal 6. The paging message includes the resource size required for Msg1 transmission (e.g. the number of access occasions).

Proposal 7. In CFRA, the paging message includes resource configuration information for Msg1 transmission.

Proposal 8. In CBRA, the paging message includes Q value-like information to support resource selection for Msg1 transmission.

Proposal 9. If the MAC message is considered, defining a new R2D message is preferable indicating D2R resource (e.g. access occasion).

References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, Dec. 2024

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #129”, Feb. 2025