3GPP TSG-RAN WG2 Meeting#129bis	    			                                                   R2-2502221

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.2.3

Source:	ETRI

Title:	Considerations for re-access in Ambient IoT

Document for:	Discussion and Decision

Introduction

In R19, a new WI: Solutions for Ambient IoT (Internet of Things) in NR was setup and the objective for the random access is as below[1]:

- RAN2 scope: - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission: - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

In this paper, we would like to discuss the issues in re-access in A-IoT.

Discussions

Re-access conditions

The following cases of D2R access errors can be considered in relation to inventory and inventory + command services:

- Case 1: Msg1 transmission failure for inventory (including collision)

- Case 2: Msg3 transmission failure for inventory

- Case 3: transmission failure of response for command

These D2R access errors can be recovered through re-access or retransmission. Re-access refers to the device restarting from Msg1 transmission, triggered by a paging message or a similar command. In contrast, retransmission refers to resending a specific message that was not successfully received, based on an ACK/NACK indicating the reader’s reception status. Generally, re-access consumes more resources and results in longer delays compared to retransmission. Therefore, it is more efficient to resolve D2R access errors rather than re-access.

Based on RAN2 agreements, among the above cases, case 1 and case 2 can trigger re-access. However, the trigger for re-access in case 3 may not have been discussed. Our understanding is that case 2 and case 3 can be resolved by retransmission, except for case 1. However, if the retransmission is not permitted in the A-IoT, the re-access procedure should be performed for all of the above cases.

Therefore, RAN2 needs to discuss the conditions for initiating re-access.

Proposal 1. RAN2 needs to discuss the conditions for initiating re-access.

Figure 1. D2R access error cases

D2R access failure determination

The reader waits for a message reception from the device after sendng the paging. However, the reader may fail to receive response messages due to collisions or transmission errors. In such cases, the reader should determine that a D2R access failure has occurred and followings can be considered:

- Option 1: CN/AF’s Re-request

If the CN/AF does not receive a response for the service, it will re-request the service. In this case, D2R access failure is determined based on the CN/AF’s re-initiation.

- Option 2: Reader’s Timer

D2R access failure is determined based on a specific timer maintained by reader.

Option 1 relies on the CN/AF to initiate a re-request, which may introduce significant processing delays. In contrast, Option 2 enables faster failure detection and response, but introduces some complexity due to the need for timer management. However, since the reader functionality in A-IoT is expected to be implemented in gNB or UE, the complexity is likely to be minimal.

Therefore, it is considered feasible for the reader to determine D2R access failure based on a timer.

Proposal 2. It feasible for the reader to determine D2R access failure based on a timer.

Message for re-access triggering

If the reader decides to trigger re-access, it is necessary to discuss which message should be used to instruct the device. This issue was discussed in RAN2#127bis, and the followings have been identified:

- Paging message: Re-access is triggered by the same paging message as used for initial access.

- New message: A new message is defined specifically to trigger re-access.

If the device needs to distinguish between initial access and re-access, introducing a new message would be necessary. However, if such distinction is not required, reusing the paging message is a feasible option. Paging message may include parameters such as device ID, type of random access, resource configuration for Msg1 transmission, and resource selection information (e.g., Q-like value). These parameters can be used in both initial access and re-access. Furthermore, re-access is essentially a procedure to re-initiate a service, similar to initial access. Thus, the paging message can be reused for re-access rather than introducing a new message, which is preferable in terms of reducing device complexity.

Therefore, the paging message is used to trigger both initial access and re-access.

Proposal 3. Paging message is used to trigger both initial access and re-access.

Re-access opportunities

D2R resources can be composed of access rounds and access occasions. An access round may contain one or more access occasions, and the device uses these to attempt re-access: Followings can be considered for the number of paging messages within an access round:

- Option 1: Single paging opportunity

- Option 2: Multiple paging opportunities

Option 1 has a simple. However, since only one service can be allowed per access round, a long access round may result in service delay. Option 2 allows for multiple service requests within a single access round through multiple paging messages, enabling more flexible service handling. However, in this option, the reader provides reception timing of each paging to the devices, which increases complexity.

Therefore, if appropriate access round configurations can accommodate service delay, a single paging per access round is more advantageous.

Proposal 4. A single paging per access round is preferable.

Figure 2. Two options for re-access opportunities

Resources structure for re-access

The D2R resource structure for re-access can be considered in the following forms:

- Option 1: Shared resource pool

- Option 1: Separate resource pool

Option 1 uses a shared resource pool for both initial access and re-access. In this structure, initial access and re-access share the same resource, so the configuration information in the paging message remains simple. That is, a single resource configuration can be used for both initial access and re-access. However, since only one resource pool is configured, it may be difficult to manage optimal resource sizes for each access. In contrast, Option 2 separates the resource areas for initial access and re-access. This enables independent resource configuration based on the reader's policy. However, because initial access and re-access utilize different resource structures and access parameters, the resource configuration information included in the paging message becomes more complex and increases in size. Therefore, a shared resource pool for initial access and re-access is preferable in the D2R resource structure.

Therefore, the use of a shared resource pool for both initial access and re-access is preferable within the D2R resource structure.

Proposal 5. The use of a shared resource pool for both initial access and re-access is preferable within the D2R resource structure.

Figure 3. Resource configurations for initial access and re-access

Conclusions

Based on the discussion in the contribution, we have the following proposals:

Proposal 1. RAN2 needs to discuss the conditions for initiating re-access.

Proposal 2. It feasible for the reader to determine D2R access failure based on a timer.

Proposal 3. Paging message is used to trigger both initial access and re-access.

Proposal 4. A single paging per access round is preferable.

Proposal 5. The use of a shared resource pool for both initial access and re-access is preferable within the D2R resource structure.

References

- RP-243326, New Work Item: Solutions for Ambient IoT (Internet of Things) in NR, Dec. 2024

- Chair’s Note, “3GPP TSG-RAN WG2 Meeting #129”, Feb. 2025