3GPP TSG-RAN WG2 Meeting #129bis	R2-2502222

Wuhan, China, April 7th – 11st, 2025

Agenda Item:	8.7.5

Source:	Huawei, HiSilicon

Title:	Discussion on RLC AM enhancements

Document for: Discussion and Decision

# 1	Introduction

In the Rel-19 XR WID approved in RAN#103 [1], one of the objectives is about the RLC re-transmission enhancements for RLC AM. In this contribution, we will provide our views on unnecessary retransmission avoidance and timely RLC retransmission.

Besides, there was some discussion on the handling of SN gap report considering the RLC enhancements. We will also analyze the issue to see whether it needs to be solved.

# 2	Discussion on avoiding unnecessary transmission

In RAN2#127-bis [2], the combined approach was agreed to avoid unnecessary RLC AM retransmission for the outdated RLC SDU (segment), in which the Tx RLC entity stops retransmitting the outdated data and the Rx RLC entity stops waiting for the outdated data based on a new timer. It is important for the Tx RLC and Rx RLC entities to know how to determine the outdated data respectively.

## 2.1	Determination of the outdated data at Rx RLC entity

As agreed in RAN2#129 [3] shown below, a single new timer can be configured at the Rx RLC entity:

- The new RLC timer at the RX is per RLC entity

Additionally, upon the expiry of the new timer t-RxDiscard, the variable for maintaining receiving window, e.g RX_Next will be updated accordingly. For the sake of synchronizing the transmitting window timely, a Status Report can be thereby triggered at the Rx RLC entity in order to update the transmitting window for Tx RLC entity in time. Thus, we propose that RLC Status Report can be triggered upon expiry of the new timer t-RxDiscard.

Proposal 1: When the new timer expires, RLC Status Report is triggered.

## 2.2	Determination of the outdated data at Tx RLC entity

- 2.2.1 discardTimer based Tx RLC entity

For the sake of avoiding resource waste, RAN2#129 [3] agreed to allow the Tx RLC entity to stop the retransmission of the outdated RLC SDU when based on the expiry of the associated PDCP discardTimer, as shown below:

- When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs.

For initial transmission, the RLC SDU that has not been submitted to the MAC layer can be directly discarded when the discard indication is received as per legacy. While for retransmission, when the discard indication corresponding to an RLC SDU is received from the associated PDCP entity, the RLC SDU will be considered as outdated and no longer retransmitted.

Furthermore, there also remains an issue whether the Tx RLC entity discards the associated RLC SDU that has been submitted to the MAC layer once the discard indication is received. Considering the fact that discarding the transmitted RLC SDU brings no benefit but only introduces additional specification impact, it is suggested to maintain the specification unchanged and reuse the legacy discarding condition. Moreover, since it was agreed that the Rx RLC entity will send ACK for the outdated RLC SDU, the transmitting window can be updated appropriately. To be more specific, when the RLC SDU is indicated by discard indication from PDCP transmitting entity, the Tx RLC entity should not discard RLC SDU (segment) that has been submitted to the lower layer. To stop the retransmission for the outdated RLC SDU, the Tx RLC entity can further consider whether the discard indication is received when the retransmission condition is met. For example, the Tx RLC entity will not consider the outdated RLC SDU for retransmission even if its NACK is received after discard indication has been received.

Proposal 2: RAN2 keeps the legacy discarding procedure that does not discard the outdated RLC SDU if either the RLC SDU or a segment thereof has been submitted to the lower layers upon the reception of the discard indication.

- 2.2.2 Discarding of Important XR Data

In the combined approach, the Tx RLC entity has the knowledge of the outdated RLC SDU based on their remaining time and hence determines whether to stop the retransmission. However, according to TS 26.522[4], the RLC SDU should not be randomly discarded, as given below:

In general, whenever the RAN needs to discard packets (e.g., under congestion situations), it is better to discard packets of lower importance rather than discarding packets randomly. If a discarded packet is critical for the media stream, the QoE may be severely degraded. For this reason, the PDU Set Importance (PSI) field can be used to mark PDU Sets with their importance level. The PSI field can then be used by the RAN to discard PDU sets. The PSI field can then be used by the RAN to discard PDU sets. In case of congestion, PDU Sets with higher PSI values are more likely to be discarded.

In other words, we can conclude that some frames can significantly contribute to the decoding of their successive frames that have dependency on them. In such case, their successive frames cannot be successfully decoded and displayed even if they are transmitted correctly. Since the new timer at Rx RLC entity starts later than the time point of actual transmission and it usually is configured with sufficient value for avoiding mistaken discarding, there remain chances for retransmission of the high importance RLC SDU, which is capable of securing the reliability for the entire XR traffic. Therefore, we believe that it is beneficial that the Tx RLC entity keeps the retransmission of the higher PSI RLC SDU (segment) regardless of its remaining time, until its associated acknowledgement is received. The specification impact of TP is provided in Appendix A.

Observation 1: Discarding critical packets for the media stream will severely degrade the QoE.

Proposal 3: Tx RLC entity can determine whether to stop retransmitting an outdated RLC SDU dependent on its importance, e.g. stopping retransmission of lower importance RLC SDU, as given in Appendix A.

# 3. Discussion on timely RLC retransmission enhancement

An agreement on analysing solutions to ensure timely RLC retransmission has been reached in RAN2#129 [3] meeting, as given below:

- Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - We have separate thresholds for autonomous reTx and for polling

## 3.1	Enhancement on autonomous retransmission

Two potential solutions to achieving the autonomous retransmission are considered according to the agreement, including reusing discardTimer and introducing a new timer at the RLC.

Option 1 discardTimer based solution: An RLC SDU (segment) that has been submitted to the MAC entity can be considered for autonomous retransmission when the corresponding discardTimer of the RLC SDU becomes below a threshold before the associated feedback is received, as shown in Figure 3. To be more specific, when the remaining time of the running discardTimer of an PDCP SDU becomes below the configured threshold, the transmitting PDCP entity can send an indication to the Tx RLC entity for triggering autonomous retransmission. Upon reception of the indication, the Tx RLC entity can consider the transmitted RLC SDU (segment) for which the associated feedback is not received for retransmission.



Figure 3. Retransmitting SDU#1 when its remaining time below a threshold and no associated feedback is received

Option 2 new timer based solution: The introduced timer can be started once the associated RLC SDU has been submitted to the lower layer. The Tx RLC entity can consider the associated RLC SDU (segment) for which the associated feedback is not received for retransmission, upon the expiry of the timer, as shown in Figure 4. Moreover, to reduce the capacity loss, if any feedback is received before the expiry of the timer, the timer can be stopped and the autonomous retransmission will thereby not be triggered.



Figure 4. Retransmitting SDU#1 when no any associated feedback is received within a certain period

The major concern imposed by the autonomous retransmission is the network capacity loss. In Option 1, the reliability of the RLC SDU (segment) can be ensured, at the expense of tremendous capacity loss. For example, due to the uncertainty of uplink resources allocation, the hundreds of RLC SDUs within a PDU Set may be transmitted in various time points, and the unacknowledged late-transmitted RLC SDUs may be experiencing HARQ procedures. It is highly likely that retransmitting these RLC SDUs brings no benefit for the users` experience but inevitably deteriorates the system`s capacity. For example, assuming that the threshold for autonomous retransmission is 10ms and an RLC SDU is transmitted right before its remaining time reaching the threshold, e.g. at remaining time of 11ms, there is only 1ms for the Tx RLC entity to send Status Report or for the network to schedule the HARQ retransmission.

Observation 2: Autonomous retransmission relying on the discardTimer introduces inevitable capacity loss, especially for the RLC SDUs transmitted right before the remaining time reaching the threshold.

By contrast, Option 2 mitigates the capacity impact by allowing the RLC SDU to experience a complete HARQ procedure. To be more specific, the autonomous retransmission can be triggered upon the expiry of the timer which is only started after the SDU is submitted to RLC layer and gets an opportunity for initial transmission, which avoids the potential redundant retransmission between the HARQ and ARQ. Although there may be little chance for autonomously retransmitting the late-transmitted RLC SDU, with the aid of the BSR or DSR, the network has the knowledge of the remaining data volume and the remaining time, and hence can allocate more robust resource, e.g. low-order MCS, to guarantee the robustness. Moreover, Option 2 also allows for the time for the Status Report reception at the Tx RLC entity, further avoiding the unnecessary autonomous retransmission for the positively acknowledged RLC SDUs. The specific TP is provided in Appendix B.

Observation 3: The capacity loss imposed by the autonomous retransmission can be mitigated if it allows for time for HARQ procedure and Status Report reception for each transmitted RLC SDU.

Proposal 4: RAN2 introduces a new timer for autonomous retransmission, which is started when the associated RLC SDU is submitted to lower layers, as specified in Appendix B.

Proposal 5: Upon the expiry of the autonomous retransmission timer, the associated RLC SDU can be considered for retransmission.

## 3.2 Enhancement on polling triggering

The polling based method tends to reuse the existing Status Report based RLC retransmission mechanism, but applies a more proactive mechanism to request a frequent Status Report. Based on the agreement in RAN2#129 [3], two potential methods can also be discussed for polling triggering.

Option 1 discardTimer based solution: An RLC SDU that has been submitted to the MAC entity can trigger polling when the corresponding discardTimer of the RLC SDU becomes below the configured threshold before the associated feedback is received. To be more specific, when the remaining time of the running discardTimer of an PDCP SDU becomes below a configured threshold, the transmitting PDCP entity can send an indication to the Tx RLC entity for polling triggering. Upon reception of the indication, the Tx RLC entity can include a poll in an AMD PDU pending for transmission.

Option 2 new timer based solution: The introduced timer can be started once the associated RLC SDU has been submitted to the lower layer. Upon the expiry of the timer, the Tx RLC entity can include a poll in an AMD PDU. If discardTimer of the associated RLC SDU expires, the new timer can be stopped accordingly.

Unlike the autonomous retransmission that introduces capacity loss by transmitting RLC SDUs, sending polling information only needs to set Poll bit to ‘1’ in the RLC header of an AMD PDU, which introduces no resource waste.  Thus, we observe no sound reason to introduce a new timer described in Option 2 for polling triggering.

As for Option 1, even if the RLC SDU is transmitted just right before the indication for triggering polling is received, sending polling information by simply including a Poll bit to ‘1’ brings no explicit capacity loss. At the Rx RLC entity, since whether to trigger Status Report upon the reception of poll is up to gNB implementation, we believe that the gNB is capable of avoiding frequently transmitting Status Reports or sending premature feedback to the Tx RLC entity. Thus, reusing discardTimer for polling triggering is preferred.

Proposal 6: RAN2 supports reusing the PDCP discardTimer for polling triggering.

Once the definition of remaining time is clarified, the further issue would be how to trigger polling based on remaining time. As a simple solution, when the remaining time of the transmitted RLC SDU that the associated feedback has not been received becomes below the configured threshold, polling can be triggered and the Tx RLC entity can include a poll in the AMD PDU.

Proposal 7: RAN2 supports to trigger polling when the shortest remaining time of a PDCP SDU among the PDCP SDUs buffered in the RLC transmitting entity that have been transmitted and have not been positively acknowledged drops below a configured threshold.

Furthermore, regardless of which Options are selected, there exists a case that no available RLC SDU can be transmitted when the polling is triggered based on the remaining time, e.g. both the transmission and retransmission buffer are empty. As this issue is relatively similar to that in t-PollRetransmit expiry, we believe that the similar operation can be reused for reference. For example, when a poll is triggered based on the remaining time, and if there is no available RLC SDU to be transmitted, the Tx RLC entity can consider a specific RLC SDU for retransmission to include the poll. Finally, a TP is provided in the Appendix C.

Proposal 8: If there is no available RLC SDU to convey the poll when a poll is triggered based on the remaining time, a specific RLC SDU can be considered for retransmission, as given in Appendix C.

# 4. Discussion on SN gap report under RLC enhancements

In the last meeting, there was an LS from RAN3 asking RAN2 to evaluate the alternatives for SN gap report handling during UE handover, to address the issue that the target gNB is unaware of discarded PDCP SDUs. During the discussion, RAN2 thought there is some issue for the PDCP SN gap report in the context of RLC enhancements.

For Rel-19, RAN2 will discuss whether the problem is a critical issue in the context of RLC enhancements. If RAN2 concludes the issue needs to be solved in Rel-19, further discussions are needed on the solutions considering but not limiting to the three alternatives as in the RAN3 LS.

With the enhancements to avoid unnecessary RLC retransmission, it would be possible that the gNB cannot receive the PDCP SN gap report sent by the UE since it may be regarded as outdated at the receiving RLC entity before it is successfully received. However, there was already an agreement in RAN2#128 meeting which excludes the enhancements to handle PDCP control PDU specially in case of RLC AM enhancements.

- Special handling to avoid PDCP control PDU discard is not needed.

Observation 4: RAN2 already agreed there is no need to introduce special handling to avoid PDCP control PDU discard.

Technically, we also think it is not worth considering the issue. To illustrate our claim, we use the following figure as an example. As shown in the figure, packets #4,5,6 are discarded before transmitting, and a corresponding SN gap report is generated. The SN gap report will be submitted to the transmitting RLC layer before the subsequent packet #8 and be assigned with RLC SN 5. If the transmission of the SN gap report fails, the receiving RLC entity will start a timer after receiving packet #8 since SN 5 is missing. When the timer expires, the RLC entity regards the packet associated with SN 5 as outdated, which means the SN gap report is lost. However, before the RLC timer expires, the PDCP entity has start the reordering timer upon receiving packet #7, which is used to decide whether to wait for the packets associated with SN 4,5,6. It is highly possible that the reordering timer expires before the RLC layer decides to abandon the SN gap report, so that there is little benefit to figure out a solution to compensate for the loss of SN gap report.

Figure 5. Illustration for SN gap report loss in case of R19 RLC enhancements.

Observation 5: There is little benefit to address the loss of PDCP SN gap report caused by the Rel-19 RLC AM enhancements (reducing RLC retransmissions).

Proposal 9: RAN2 to conclude there is no need to solve the issue of PDCP SN gap report with the context of RLC AM enhancements in R19.

# 4. Conclusion

This contribution makes the following proposals:

Avoiding unnecessary transmission

Observation 1: Discarding critical packets for the media stream will severely degrade the QoE.

Proposal 1: When the new timer expires, RLC Status Report is triggered.

Proposal 2: RAN2 keeps the legacy discarding procedure that does not discard the outdated RLC SDU if either the RLC SDU or a segment thereof has been submitted to the lower layers upon the reception of the discard indication.

Proposal 3: Tx RLC entity can determine whether to stop retransmitting an outdated RLC SDU dependent on its importance, e.g. stopping retransmission of lower importance RLC SDU, as given in Appendix A.

Timely RLC retransmission enhancement

Observation 2: Autonomous retransmission relying on the discardTimer introduces inevitable capacity loss, especially for the RLC SDUs transmitted right before the remaining time reaching the threshold.

Observation 3: The capacity loss imposed by the autonomous retransmission can be mitigated if it allows for time for HARQ procedure and Status Report reception for each transmitted RLC SDU.

Proposal 4: RAN2 introduces a new timer for autonomous retransmission, which is started when the associated RLC SDU is submitted to lower layers, as specified in Appendix B.

Proposal 5: Upon the expiry of the autonomous retransmission timer, the associated RLC SDU can be considered for retransmission.

Proposal 6: RAN2 supports reusing the PDCP discardTimer for polling triggering.

Proposal 7: RAN2 supports to trigger polling when the shortest remaining time of a PDCP SDU among the PDCP SDUs buffered in the RLC transmitting entity that have been transmitted and have not been positively acknowledged drops below a configured threshold.

Proposal 8: If there is no available RLC SDU to convey the poll when a poll is triggered based on the remaining time, a specific RLC SDU can be considered for retransmission, as given in Appendix C.

SN gap report

Observation 5: There is little benefit to address the loss of PDCP SN gap report caused by the Rel-19 RLC AM enhancements (reducing RLC retransmissions).

Proposal 9: RAN2 to conclude there is no need to solve the issue of PDCP SN gap report with the context of RLC AM enhancements in R19.

# 5. Reference

[1] RP-240791 Revised WID on XR (eXtended Reality) for NR Phase 3, 3GPP TSG RAN Meeting #103

[2] Chair note of RAN2#127-bis, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_127bis/Inbox/Chair_Notes/R2_127bis%20R18%20MBS%2C%20R18%20QoE%20and%20R19%20XR%20session%20notes%20(Dawid)_EOM.docx

[3] Chair note of RAN2#129, https://www.3gpp.org/ftp/tsg_ran/WG2_RL2/TSGR2_129/Inbox/Chair_Notes/R2_129%20R18%20MBS%2C%20R18%20QoE%20and%20R19%20XR%20session%20notes%20(Dawid)_EOM.docx

[4] 3GPP TS 26.522

- Appendix A

## 5.3	ARQ procedures

### 5.3.1	General

ARQ procedures are only performed by an AM RLC entity.

### 5.3.2	Retransmission

The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following:

- STATUS PDU from its peer AM RLC entity.

When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall:

- if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer,

- if stopRxTxObsoleteSDU is not configured; or

- if stopRxTxObsoleteSDU is configured and psi-BasedStopObsoleteSDU is not configured, and the corresponding RLC SDU has not been indicated as discard from upper layer; or

- if psi-BasedStopObsoleteSDU is configured and the corresponding RLC SDU belongs to a high importance PDU set; or

- if psi-BasedStopObsoleteSDU is configured and the corresponding RLC SDU belongs to a low importance PDU set and has not been indicated as discard from upper layer:

      - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission.

NOTE:	Identification of PSI of a PDU Set and determination of low importance PDU Set are left up to UE implementation.

## 7.4	Configurable parameters

The following parameters are configured by TS 38.331 [5]:

a) maxRetxThreshold

This parameter is used by the transmitting side of each AM RLC entity to limit the number of retransmissions corresponding to an RLC SDU, including its segments (see clause 5.3.2).

b) pollPDU

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollPDU PDUs (see clause 5.3.3).

c) pollByte

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollByte bytes (see clause 5.3.3).

d) stopRxTxObsoleteSDU

This parameter is used by the transmitting side of each AM RLC to stop the retransmission for an RLC SDU which has been indicated as discard from upper layer. (see clause 5.3.2).

e) psi-BasedStopObsoleteSDU

This parameter is used by the transmitting side of each AM RLC to stop the retransmission for an RLC SDU which belong to the low importance PDU set and has been indicated as discard from upper layer. (see clause 5.3.2).

- Appendix B

## 5.2	Data transfer procedures

### 5.2.3	AM data transfer

#### 5.2.3.1	Transmit operations

##### 5.2.3.1.1	General

The transmitting side of an AM RLC entity shall prioritize transmission of RLC control PDUs over AMD PDUs. The transmitting side of an AM RLC entity shall prioritize transmission of AMD PDUs containing previously transmitted RLC SDUs or RLC SDU segments over transmission of AMD PDUs containing not previously transmitted RLC SDUs or RLC SDU segments.

The transmitting side of an AM RLC entity shall maintain a transmitting window according to the state variable TX_Next_Ack as follows:

-	a SN falls within the transmitting window if TX_Next_Ack <= SN < TX_Next_Ack + AM_Window_Size;

-	a SN falls outside of the transmitting window otherwise.

The transmitting side of an AM RLC entity shall not submit to lower layer any AMD PDU whose SN falls outside of the transmitting window.

For each RLC SDU received from the upper layer, the AM RLC entity shall:

-	associate a SN with the RLC SDU equal to TX_Next and construct an AMD PDU by setting the SN of the AMD PDU to TX_Next;

-	increment TX_Next by one.

When submitting an AMD PDU that contains a segment of an RLC SDU, to lower layer, the transmitting side of an AM RLC entity shall:

-	set the SN of the AMD PDU to the SN of the corresponding RLC SDU;

- if the AMD PDU includes the last byte of an RLC SDU:

  - start the autonomousRetxTimer for the corresponding RLC SDU, if configured.

The transmitting side of an AM RLC entity can receive a positive acknowledgement (confirmation of successful reception by its peer AM RLC entity) for an RLC SDU by the following:

-	STATUS PDU from its peer AM RLC entity.

When receiving a positive acknowledgement for an RLC SDU with SN = x, the transmitting side of an AM RLC entity shall:

-	send an indication to the upper layers of successful delivery of the RLC SDU;

-	set TX_Next_Ack equal to the SN of the RLC SDU with the smallest SN, whose SN falls within the range TX_Next_Ack <= SN <= TX_Next and for which a positive acknowledgment has not been received yet;

- if the autonomousRetxTimer for the RLC SDU is running:

  - stop the autonomousRetxTimer for the RLC SDU.

## 5.3	ARQ procedures

### 5.3.1	General

ARQ procedures are only performed by an AM RLC entity.

### 5.3.2	Retransmission

The transmitting side of an AM RLC entity can receive a negative acknowledgement (notification of reception failure by its peer AM RLC entity) for an RLC SDU or an RLC SDU segment by the following:

        - STATUS PDU from its peer AM RLC entity.

When receiving a negative acknowledgement for an RLC SDU or an RLC SDU segment by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall:

- if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer:

  - consider the RLC SDU or the RLC SDU segment for which a negative acknowledgement was received for retransmission,

- if a negative acknowledgement is for the RLC SDU and the autonomousRetxTimer for the corresponding RLC SDU is running:

  - stop the autonomousRetxTimer for the RLC SDU.

When the autonomousRetxTimer for an RLC SDU expires, the transmitting side of the AM RLC entity shall:

- if the SN of the corresponding RLC SDU falls within the range TX_Next_Ack <= SN < = the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer:

  - consider the corresponding RLC SDU or the RLC SDU segment which is neither positively nor negatively acknowledged for retransmission.

When an RLC SDU or an RLC SDU segment is considered for retransmission, the transmitting side of the AM RLC entity shall:

- if the RLC SDU or RLC SDU segment is considered for retransmission for the first time:

  - set the RETX_COUNT associated with the RLC SDU to zero.

- else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU:

  - increment the RETX_COUNT.

- if RETX_COUNT = maxRetxThreshold:

  - indicate to upper layers that max retransmission has been reached.

## 5.4	SDU discard procedures

When indicated from upper layer (e.g. PDCP) to discard a particular RLC SDU, the transmitting side of an AM RLC entity or the transmitting UM RLC entity shall discard the indicated RLC SDU and stop the autonomousRetxTimer for the RLC SDU if it is running, if neither the RLC SDU nor a segment thereof has been submitted to the lower layers. The transmitting side of an AM RLC entity shall not introduce an RLC SN gap when discarding an RLC SDU.

## 7.3	Timers

The following timers are configured by TS 38.331 [5]:

a) t-PollRetransmit

This timer is used by the transmitting side of an AM RLC entity in order to retransmit a poll (see clause 5.3.3).

b) t-Reassembly

This timer is used by the receiving side of an AM RLC entity and receiving UM RLC entity in order to detect loss of RLC PDUs at lower layer (see clauses 5.2.2.2 and 5.2.3.2). If t-Reassembly is running, t-Reassembly shall not be started additionally, i.e. only one t-Reassembly per RLC entity is running at a given time.

c) t-StatusProhibit

This timer is used by the receiving side of an AM RLC entity in order to prohibit transmission of a STATUS PDU (see clause 5.3.4).

d) autonomousRetxTimer

This parameter is used by the transmitting side of each AM RLC entity to consider the retransmission for the associated RLC SDU or the associated RLC SDU segment when its associated timer expires (see clause 5.3.2).

- Appendix C

## 5.3	ARQ procedures

### 5.3.1	General

ARQ procedures are only performed by an AM RLC entity.

### 5.3.3	Polling

#### 5.3.3.1	General

An AM RLC entity can poll its peer AM RLC entity in order to trigger STATUS reporting at the peer AM RLC entity.

#### 5.3.3.2	Transmission of a AMD PDU

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall:

-	increment PDU_WITHOUT_POLL by one;

-	increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU;

-	if PDU_WITHOUT_POLL >= pollPDU; or

-	if BYTE_WITHOUT_POLL >= pollByte:

-	include a poll in the AMD PDU as described below.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall:

-	if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or

-	if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling);

-	include a poll in the AMD PDU as described below.

NOTE:	Empty RLC buffer (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) should not lead to unnecessary polling when data awaits in the upper layer. Details are left up to UE implementation.

If PollingThreshold is configured, the transmitting side of an AM RLC entity shall:

- If the smallest remaining time of the running PDCP discardTimers among all the PDCP SDUs buffered in the transmitting side of the AM RLC entity that have been submitted to the lower layers and have not been positively acknowledged becomes below PollingThreshold

  - if both the transmission buffer and the retransmission buffer are empty (excluding transmitted RLC SDU or RLC SDU segment awaiting acknowledgements); or

  - if no new RLC SDU or RLC SDU segment can be transmitted (e.g. due to window stalling)

- consider the RLC SDU with the highest SN among the RLC SDUs submitted to lower layer for retransmission; or

- consider any RLC SDU which has not been positively acknowledged for retransmission.

  - include a poll in an AMD PDU as described below.

To include a poll in an AMD PDU, the transmitting side of an AM RLC entity shall:

-	set the P field of the AMD PDU to "1";

-	set PDU_WITHOUT_POLL to 0;

-	set BYTE_WITHOUT_POLL to 0.

Upon submission of an AMD PDU including a poll to lower layer, the transmitting side of an AM RLC entity shall:

-	set POLL_SN to the highest SN of the AMD PDU among the AMD PDUs submitted to lower layer;

-	if t-PollRetransmit is not running:

-	start t-PollRetransmit.

-	else:

-	restart t-PollRetransmit.

## 7.4	Configurable parameters

The following parameters are configured by TS 38.331 [5]:

a) maxRetxThreshold

This parameter is used by the transmitting side of each AM RLC entity to limit the number of retransmissions corresponding to an RLC SDU, including its segments (see clause 5.3.2).

b) pollPDU

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollPDU PDUs (see clause 5.3.3).

c) pollByte

This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for every pollByte bytes (see clause 5.3.3).

d) PollingThreshold

- This parameter is used by the transmitting side of each AM RLC entity to trigger a poll for the RLC SDUs with remaining time below the threshold (see clause 5.