3GPP TSG-RAN WG2 Meeting #129bis			     R2-2502223

Wuhan, China, 7-11 April, 2025

Agenda Item:	8.7.5 (NR_XR_Ph3-Core)

Source: 	LG Electronics Inc.

Title:         	Remaining issues on RLC enhancements for XR

Document for: 	Discussion and Decision

Introduction

This contribution discusses remaining issues on unnecessary retransmission and timely RLC retransmissions.

Discussion

Unnecessary retransmissions avoidance

[FFS - Detection of obsolescence of an AMD PDU: - The receiving side of an AM RLC entity shall trigger a STATUS report when t-RxDiscard expires.]

During the RLC running CR discussion, one company argues that triggering a STATUS report upon expiry of t-RxDiscard is not needed since none of the existing RAN2 agreements regarding this topic mandate the UE to send an SR immediately, and instead gNB implementation, besides the legacy polling mechanisms, can ensure that Tx window stalling will not happen in case of RLC SDU discarding. Finally, the rapporteur changes it as FFS as shown in the above box.

We would like to explain two points why this change is needed. The first point is that, as per the current RLC specification, if the receiving RLC entity receives an RLC SDU which falls outside of the receiving window, this RLC SDU is discarded. For this reason, the transmitting window needs to be synchronized with the receiving window, and is updated based on the STATUS report which is constructed after updating the receiving window.

The second point is that, based on the agreement (i.e., The duration of the new RLC timer is not lower than that of t-reassembly), the STATUS report may be triggered after expiry of t-reassembly. When the STATUS report triggered by t-reassembly expiry is transmitted, t-RxDiscard is still running and the Rx entity cannot know whether the AMD PDU is outdated, then this STATUS report includes NACK for this AMD PDU. After a while, t-RxDiscard expires and this AMD PDU becomes outdated, and this should be positively acknowledged in the STATUS report as soon as possible to make TX entity move Tx window forward. In this situation, just relying on legacy STATUS triggering condition would cause additional delay since the receiving RLC entity should wait for a next poll or expiry of t-Reassembly to trigger the next STATUS report. This delay would be inappropriate for XR service considering that XR data is delay sensitive data and all RLC enhancements are to satisfy this tight latency requirement. This is why RAN2 made the following agreements.

- The RLC receiving window always advances to any given RLC SN before the transmitting window does.

- Rx informs Tx side about the abandoned SDUs.

- The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report.

Thus, the STATUS report should be triggered after expiry of t-RxDiscard upon detection of obsolescence of an AMD PDU, and RAN2 don’t need to introduce additional Tx side operation regarding Tx window, i.e. Tx window advancement is purely based on RLC status report, as in legacy.

Proposal 1. Remove the bracket and FFS in 5.3.4	Status reporting of the RLC running CR, i.e., the STATUS report is triggered after expiry of t-RxDiscard.

[FFS - Detection of obsolescence of an AMD PDU:

- The receiving side of an AM RLC entity shall trigger a STATUS report when t-RxDiscard expires.]

Timely RLC retransmissions

Autonomous retransmissions and polling enhancements - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC

One FFS is whether remaining time is determined based on 1) the discardTimer at PDCP layer or 2) the new timer at RLC layer. The key issue here is that how to determine whether remaining time of a RLC SDU falls below a threshold. The first option is that PDCP indicates this based on the discardTimer at PDCP and the second option is that RLC determines this based on its own new timer at RLC without indication from the PDCP. More specifically, according to the contributions in the last meeting, the proponent of a new timer at RLC want to start the new timer once an RLC SDU is transmitted and then perform retransmission of the RLC SDU after expiry of the new timer. With this in mind, we have following concerns on the new timer at RLC.

- Too early retransmission problem: Even though a RLC SDU having long remaining time is transmitted, the new timer is started for the RLC SDU and too early retransmission can be performed after expiry of the new timer. To avoid this problem, there should be an additional condition/mechanism, i.e., 1) the new timer is not started when the remaining time of discardTimer at PDCP is large enough or 2) different timer value is applied to the new timer considering actual remaining time of discardTimer at PDCP.

- Unnecessary retransmission problem: when a RLC SDU is transmitted, if the length of a new timer is longer than the remaining time of discardTimer at PDCP, unnecessary retransmission of the RLC SDU if performed after expiry of the new timer. To avoid this problem, 1) the new timer is not started when the remaining time of discardTimer is smaller than the length of a new timer or 2) the new timer is stopped upon receiving a discard indication from the PDCP or 3) the NW should configure unnecessary retransmission avoidance at Tx side.

As explained above, even though the new timer at RLC is introduced, checking the remaining time of discardTimer at PDCP would be needed to avoid too early/unnecessary retransmission problem.

Observation 1. A new timer at RLC can trigger too early or unnecessary retransmission.

Observation 2. Even though the new timer at RLC is introduced, interaction between PDCP and RLC is still needed to avoid too early/unnecessary retransmission problem.

In addition, if the new timer at RLC is started once an RLC SDU is transmitted as in autonomous retransmission, it is unclear how to trigger remaining time based polling. This is because the new timer is not started until an RLC SDU is transmitted and there may be no running timer to determine remaining time based polling even when new transmission of a delay-critical RLC SDU is submitted to lower layer. To avoid this ambiguity, different timer start condition would be considered. For example, another new timer at RLC is started when a RLC SDU is received from the PDCP, and then the RLC entity needs to check whether remaining time of another new timer for polling is lower than a threshold.

Observation 3. If the new timer at RLC is started once an RLC SDU is transmitted, it is unclear how to trigger remaining time based polling since there may be no running timer when new transmission of a delay-critical RLC SDU is submitted to lower layer.

However, according to the PDCP specification, at reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall start a discardTimer for the PDCP SDU. It is obvious that when the UE receives an uplink grant, the PDCP entity submits a PDCP PDU to a RLC layer, but when to submit a PDCP PDU to a RLC layer before receiving an uplink grant is up to implementation. There should be unavoidable discrepancy between remaining time of discardTimer at PDCP and remaining time of new timer for polling at RLC. Same problems, i.e., early/unnecessary polling, cannot be avoided and interaction between PDCP and RLC is also needed to avoid this problem.

Observation 4. Even though another new timer for polling at RLC is introduced to avoid problems in the observation 3, early/unnecessary polling, would not be avoided and interaction between PDCP and RLC would be needed to avoid this problem.

On the other hand, if autonomous retransmission and polling are triggered based on the discardTimer at PDCP, all we need are just two new thresholds configured and we don’t need to worry about too early/unnecessary retransmission/polling since PDCP can indicate necessary indication to the RLC at the exact timing based on the remaining time and configured threshold. Considering a discard indication and delay-critical indication, as well as newly introduced delay-reporting indication, are already indicated by PDCP, we don’t think that adding two more indications for autonomous retransmission/polling enhancement are big problem.

Observation 5. If autonomous retransmission and polling triggered based on the discardTimer at PDCP, there is no too early/unnecessary retransmission/polling problem and those can be triggered at the exact timing based on the remaining time and configured threshold.

Proposal 2. Autonomous retransmission and remaining time based polling is performed by an indication from the PDCP based on the remaining time of discardTimer at PDCP and configured threshold.

Another remaining issue is how to control RETX_COUNT for autonomous retransmission.

- Editor’s Note: It is still open how Autonomous Retransmission coexists with ARQ procedures, i.e. whether/how to increment the RETX_COUNT for Autonomous Retransmission.

When the retransmission is indicated by a STATUS PDU, the RLC entity increments RETX_COUNT value per RLC SDU. If RETX_COUNT value reaches the configured maximum retransmission threshold, the RLC entity indicates to RRC that max retransmission has been reached and then RLF would be declared by RRC. Considering that autonomous retransmission is performed to satisfy tight latency requirement, if autonomous retransmission increments RETX_COUNT and maximum retransmission threshold is reached, RLF would cause RRC connection re-establishment and this requires long interruption which makes autonomous retransmission useless. In addition, RAN2 agreed that “Only a single autonomous retransmission will be triggered per RLC SDU.”, and skipping one time of RETX_COUNT increment by autonomous retransmission would not be a big problem. Thus, we prefer to keep the current mechanism and RETX_COUNT increment by autonomous retransmission is not needed. Note that RETX_COUNT is not only way to detect RLF and there are other mechanisms to detect RLF in a UE.

Proposal 3. Autonomous retransmission does not increment RETX_COUNT, but a simple text is added in the RLC specification as in the following TP.

When an RLC SDU or an RLC SDU segment is considered for retransmission by a STATUS PDU from its peer AM RLC entity, the transmitting side of the AM RLC entity shall: - if the RLC SDU or RLC SDU segment is considered for retransmission for the first time: - set the RETX_COUNT associated with the RLC SDU to zero. - else, if it (the RLC SDU or the RLC SDU segment that is considered for retransmission) is not pending for retransmission already and the RETX_COUNT associated with the RLC SDU has not been incremented due to another negative acknowledgment in the same STATUS PDU: - increment the RETX_COUNT. - if RETX_COUNT = maxRetxThreshold: - indicate to upper layers that max retransmission has been reached.

The next remaining issue is about excessive polling as shown editor’s note below.

- Editor’s Note: It is still open on how to avoid excessive polling for the polling enhancement, e.g. only one polling or multiple.

In LTE, concatenation is performed at RLC and only one AMD PDU including one poll bit is submitted for one transmission opportunity. In NR, however, concatenation is performed at MAC and the RLC entity can submit multiple AMD PDUs for one transmission opportunity. In this condition, if pollPDU or pollByte is configured with a lower value, it is already possible to submit multiple AMD PDUs including poll for one transmission opportunity. This means that multiple polling can be already transmitted for one transmission opportunity and there is no problem so far since even if status reporting was triggered several times before receiving the first transmission opportunity indicated by lower layer, one STATUS PDU is constructed and transmitted. In this sense, we think that multiple polling is already possible and it is not an issue. Thus, no special handling to avoid multiple polling is needed.

Observation 6. In 5G NR, multiple polling is already transmitted for one transmission opportunity and it is not a problem so far.

Proposal 4. Even if polling is included based on the remaining time, no special handling to avoid multiple polling is introduced.

As per the current RLC running CR, remaining time based polling can be checked only when the AMD PDU containing a new transmitted RLC SDU or RLC SDU segment as shown yellow highlight.

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - increment PDU_WITHOUT_POLL by one; - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte; or - if the remaining time of an RLC SDU or an RLC SDU segment falls below the [specified threshold for polling enhancement] [as indicated from PDCP]: - include a poll in the AMD PDU as described below;

However, the intention of remaining time based polling is to receive the STATUS report after the delay-critical RLC SDU transmission to receive another retransmission opportunity and satisfy the PDB of the delay-critical RLC SDU. Thus, when the delay-critical RLC SDU is submitted to the lower layer for transmission, the RLC entity should include a poll regardless of whether the delay-critical RLC SDU is new transmission or retransmission.

Proposal 5. If the remaining time of an RLC SDU falls below a specified threshold, the RLC entity should include a poll even when the RLC SDU is retransmitted. (Related TP is given below)

Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission such that the AMD PDU contains either a not previously transmitted RLC SDU or an RLC SDU segment containing not previously transmitted byte segment, the transmitting side of an AM RLC entity shall: - increment PDU_WITHOUT_POLL by one; - increment BYTE_WITHOUT_POLL by every new byte of Data field element that it maps to the Data field of the AMD PDU; - if PDU_WITHOUT_POLL >= pollPDU; or - if BYTE_WITHOUT_POLL >= pollByte; - include a poll in the AMD PDU as described below; Upon notification of a transmission opportunity by lower layer, for each AMD PDU submitted for transmission, the transmitting side of an AM RLC entity shall: - if both the transmission buffer and the retransmission buffer becomes empty (excluding transmitted RLC SDUs or RLC SDU segments awaiting acknowledgements) after the transmission of the AMD PDU; or - if no new RLC SDU can be transmitted after the transmission of the AMD PDU (e.g. due to window stalling); or - if the remaining time of an RLC SDU or an RLC SDU segment falls below the [specified threshold for polling enhancement] [as indicated from PDCP]: - include a poll in the AMD PDU as described below.

Conclusion

Based on the above discussions, we present the following observations and proposals:

Observation 1. A new timer at RLC can trigger too early or unnecessary retransmission.

Observation 2. Even though the new timer at RLC is introduced, interaction between PDCP and RLC is still needed to avoid too early/unnecessary retransmission problem.

Observation 3. If the new timer at RLC is started once an RLC SDU is transmitted, it is unclear how to trigger remaining time based polling since there may be no running timer when new transmission of a delay-critical RLC SDU is submitted to lower layer.

Observation 4. Even though another new timer for polling at RLC is introduced to avoid problems in the observation 3, early/unnecessary polling, would not be avoided and interaction between PDCP and RLC would be needed to avoid this problem.

Observation 5. If autonomous retransmission and polling triggered based on the discardTimer at PDCP, there is no too early/unnecessary retransmission/polling problem and those can be triggered at the exact timing based on the remaining time and configured threshold.

Observation 6. In 5G NR, multiple polling is already transmitted for one transmission opportunity and it is not a problem so far.

Proposal 1. Remove the bracket and FFS in 5.3.4	Status reporting of the RLC running CR, i.e., the STATUS report is triggered after expiry of t-RxDiscard.

[FFS - Detection of obsolescence of an AMD PDU:

- The receiving side of an AM RLC entity shall trigger a STATUS report when t-RxDiscard expires.]

Proposal 2. Autonomous retransmission and remaining time based polling is performed by an indication from the PDCP based on the remaining time of discardTimer at PDCP and configured threshold.

Proposal 3. Autonomous retransmission does not increment RETX_COUNT, but a simple text is added in the RLC specification as in the following TP.

Proposal 4. Even if polling is included based on the remaining time, no special handling to avoid multiple polling is introduced.

Proposal 5. If the remaining time of an RLC SDU falls below a specified threshold, the RLC entity should include a poll even when the RLC SDU is retransmitted. (Related TP is given below)