3GPP TSG-RAN WG2 Meeting #129bis			         R2-2502227

Wuhan, China, Apr. 7th – 11th , 2025

Agenda Item:	8.4.2

Source: 	LG Electronics Inc.

Title:         	Remaining issues on LP-WUS in RRC IDLE or INACTIVE

Document for: 	Discussion and Decision

Introduction

In this contribution, we will discuss remaining issues on the LP-WUS operation in RRC_IDLE/INACTIVE.

Discussion

- LP-WUS configuration

There has been extensive discussion on whether the entry/exit condition of using LP-WUS is mandatory or optional present over the past several meetings, but RAN2 have yet to reach a conclusion. The proponents of the optional entry/exit condition for using LP-WUS are divided into three camps, each with a different intended UE behaviour when the exit/entry condition is absent in the LP-WUS related configuration, as follows:

- Camp 1. Leave it up to UE implementation, if the entry/exit conditions are absent

- Camp 2. Use default entry/exit condition, if the entry/exit conditions are absent

- Camp 3. Assume that the entry condition is always met, if the entry/exit conditions are absent

In fact, this is not simply an issue of whether to support the optional configuration. Rather, we need to discuss separately whether to support UE behaviour that each camp considers when the entry condition is absent.

Issue #1: Whether the decision on the entry/exit condition of using LP-WUS can be up to UE

The first camp argues that smart UEs can make sensible UE implementation when to start/stop LP-WUS monitoring. However, if UE conservatively determines the entry/exit condition to ensure reliable LP-WUS reception, the MR may lose the opportunity to enter the ultra-deep-sleep and the power saving gain from using LP-WUS will somewhat decreases. Conversely, if the threshold for the entry condition is set too low, or if the threshold for the exit condition is set too high, the UE will monitor LP-WUS with low reliability and may miss paging. Since only a few bits are expected to configure the entry/exit condition, it seems unnecessary to tolerate such potential issues.

Observation 1	The entry/exit condition determined by UE implementation may cause a decrease in UE power saving through LP-WUS monitoring and a degradation in paging reception performance.

Proposal 1	The entry/exit condition for using LP-WUS cannot be decided by UE implementation.

Issue #2: Whether to support default entry/exit condition of using LP-WUS

Since the LP-WUS transmission operates on a cell basis, the appropriate entry/exit conditions may vary for each cell. We are not sure a single default condition can be used in any cell. Considering that only 8bits are needed to configure an RSRP/RSRQ threshold and the entry/exit condition would consist of up to four RSRP/RSRQ thresholds, i.e. MR RSRP, MR RSRQ, LR RSRP and LR RSRQ, signalling gain that can be achieved by not configuring the entry/exit conditions would be trivial. Furthermore, although it is FFS which SIB to use to convey the LP-WUS related configuration, it will be transmitted in on-demand manner.

Proposal 2	Do not support default entry/exit condition for using LP-WUS.

Issue #3: Need for entry/exit condition of using LP-WUS, when LP-WUS and the cell have the same coverage

If the LP-WUS coverage is the same as or larger than the cell coverage and LP-WUS is transmitted on the intra-frequency, the network is not required to provide the entry/exit condition, and UE can assume the entry condition is always met.

Proposal 3	If the entry/exit condition for using LP-WUS is absent in a cell, UE considers the entry condition is always met while camping on that cell.

Regarding the LP-WUS related configuration, there is another FFS if dedicated configuration is needed. It is obvious the LP-SS/LP-WUS configuration is cell specific. There is no reason to support dedicated signalling-based configuration for them. However, the LP-WUS configuration may need to be activated/de-activated per UE. For instance, if a UE has a very high paging probability, the power saving gain the UE can achieve through LP-WUS monitoring would be minimal.

Proposal 4	Do not support LP-WUS related configuration using dedicated signalling.

- Entry/exit condition of using LP-WUS

RAN2#127bis

If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

The LPWUS monitoring exit condition does not include MR measurements.

Although LR threshold is included in the entry condition of using LP-WUS, it does not seem reasonable to force UE to perform LR measurements upon acquisition of the entry condition. The power consumed for LR measurements may not be significant, so some UEs might start LR measurements as soon as it receives the entry condition, meanwhile other UEs that want to minimize the LR measurement may decide whether to perform the LR measurement based on the MR measurement results, e.g. if MR measurement becomes better than the MR threshold in the entry condition. Therefore, we propose to leave when to start the LR measurement outside the LP-WUS coverage up to UE implementation.

Proposal 5	It is up to UE implementation when to start the LR measurements.

MR can wake up while performing LP-WUS monitoring using LR for various reasons, e.g. due to the detection of LP-WUS, for performing RRM measurement, for SI request/reception, or for transitioning to RRC_CONNECTED. While MR is active, LR may not need to monitor LP-WUS. However, except the case when transitioning to RRC_CONNECTED, the activated MR will rapidly accomplish its task, e.g. monitoring PO or RRM measurement, then return to sleep. Hence, UE does not need to stop monitoring LP-WUS in such cases. If LR is turned off while MR is active, and then is turned back on after the MR activity ends, MR cannot transition to sleep mode until LR is ready to monitor LP-WUS. This could delay the entry of MR into sleep state. Therefore, unless significant benefit of stopping LP-WUS monitoring during MR activity is revealed, MR activity should not be specified as an exit condition of LP-WUS monitoring.

Proposal 6	If the entry condition of using LP-WUS is met, MR is only allowed to skip PEI-O/PO monitoring based on the LP-WUS, but still should perform other existing activities, such as RRM measurements or UL transmission.

If the proposal 6 is accepted, it is obvious that MR wakes up for other activities even though exit condition is met. Therefore, other activities of MR do not need to be specified as exit condition.

Proposal 7	The MR activation caused by RRM measurements or UL transmission is not specified as exit condition of using LP-WUS.

Some companies want to have a unified condition for using LP-WUS and RRM offloading. The main reason of that is MR can maintain a longer sleep state, maximizing power saving gain, if it does not perform both paging monitoring and serving cell measurements. Considering that the RRM offloading/relaxation is not suitable for UEs at cell edge, the LP-WUS coverage will be larger than the RRC offloading/relaxation coverage. Using the same condition for using LP-WUS and RRM offloading/relaxation implies that if RRM offloading is not suitable, LP-WUS would also not be used even though it is still reliable. However, the relaxed requirement for serving cell measurement is being defied in RAN4 and in case 3 MR can have more opportunities to sleep by not monitoring PEI-O/PO.

Proposal 8	The condition for using LP-WUS is not associated with the conditions for RRM offloading/relaxation.

- LP-WUS subgrouping

In the PEI subgrouping, the CN assigned subgroup is prioritized over the UE_ID based subgroup. That is, UE determines the UE_ID based subgroup ID only when the CN assigned subgroup ID is not assigned, or the serving cell support the UE_ID based subgrouping. There is no difference between the PEI subgrouping and the LP-WUS subgrouping, except they are transmitted on different channel, so the existing principles used for PEI subgrouping can be directly applied to the LP-WUS subgrouping.

Proposal 9	For LP-WUS subgrouping, following principles of the PEI subgrouping are reused:

      - If the LP-WUS subgroup ID is not assigned by CN, or if the serving cell supports only UE_ID based LP-WUS subgrouping, UE uses a LP-WUS subgroup ID based on UE_ID.

      - If a LP-WUS subgroup ID is assigned by CN, and if the serving cell supports the CN assigned LP-WUS subgrouping, UE uses the LP-WUS subgroup ID assigned by CN.

Conclusion

LP-WUS configuration:

Proposal 1	The entry/exit condition for using LP-WUS cannot be decided by UE implementation.

Proposal 2	Do not support default entry/exit condition for using LP-WUS.

Proposal 3	If the entry/exit condition for using LP-WUS is absent in a cell, UE considers the entry condition is always met while camping on that cell.

Proposal 4	Do not support LP-WUS related configuration using dedicated signalling.

The entry/exit condition of using LP-WUS:

Proposal 5	It is up to UE implementation when to start the LR measurements.

Proposal 6	If the entry condition of using LP-WUS is met, MR is only allowed to skip PEI-O/PO monitoring based on the LP-WUS, but still should perform other existing activities, such as RRM measurements or UL transmission.

Proposal 7	The MR activation caused by RRM measurements or UL transmission is not specified as exit condition of using LP-WUS.

Proposal 8	The condition for using LP-WUS is not associated with the conditions for RRM offloading/relaxation.

LP-WUS subgrouping

Proposal 9	For LP-WUS subgrouping, following principles of the PEI subgrouping are reused:

      - If the LP-WUS subgroup ID is not assigned by CN, or if the serving cell supports only UE_ID based LP-WUS subgrouping, UE uses a LP-WUS subgroup ID based on UE_ID.

      - If a LP-WUS subgroup ID is assigned by CN, and if the serving cell supports the CN assigned LP-WUS subgrouping, UE uses the LP-WUS subgroup ID assigned by CN.