3GPP TSG-RAN WG2 Meeting #129bis	R2-2502232

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.13.3

Source: 	China Telecom

Title:	Discussion on control plane aspects for NR sidelink multi-hop relay

WID/SID:	NR_SL_relay_multihop

Document for:	Discussion and Decision

- Introduction

In last few RAN2 meetings, control plane procedures and SRAP impact in multi-hop relay were widely discussed with much progress made. However, there are still a few open issues on RLF handling, SI and paging related aspects for multi-hop U2N relay. In this contribution, we would like to share our opinions on control plane remaining issues for multi-hop U2N relay.

- Discussion

General aspects on supporting multi-hop U2N relay

In previous RAN2 meeting, it was agreed to support a baseline procedure for the case in which the intermediate relay UEs for a remote UE all transition to RRC_CONNECTED (if not already there) when the remote UE goes to RRC_CONNECTED. Whether/how to support the case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment remain FFS.

For single hop U2N relay, the following aspects on RRC state of U2N relay UE are specified in TS 38.300[2].

Copied from TS 38.300 A U2N Relay UE shall be in RRC_CONNECTED to perform relaying of unicast data. For L2 U2N Relay operation, the following RRC state combinations are supported: - Both L2 U2N Relay UE and L2 U2N Remote UE shall be in RRC_CONNECTED to perform transmission/reception of relayed unicast data; and - The L2 U2N Relay UE can be in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED as long as all the L2 U2N Remote UE(s) that are connected to the L2 U2N Relay UE are either in RRC_INACTIVE or in RRC_IDLE.

Regarding multi-hop U2N relay, the related U2N relays on the relay path shall follow the same principle as single hop U2N relay scenario. All the U2N Relay UEs on the relay path shall be in RRC_CONNECTED to perform relaying of unicast data. All the related multi-hop U2N relays and U2N remote UE shall be in RRC_CONNECTED to perform transmission/reception of relayed unicast data. The multi-hop U2N relay can be in RRC_IDLE, RRC_INACTIVE or RRC_CONNECTED as long as all the L2 U2N Remote UE(s) or L2 U2N relay(s) that are connected to the L2 U2N Relay UE are either in RRC_INACTIVE or in RRC_IDLE.

The agreed baseline procedure in RAN2#128 meeting follows the same principle as single hop U2N relay quite well. On top of the baseline procedure, we see no strong motivation to support additional case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment. Furthermore, if we support such optimisation for multi-hop U2N relay, it will bring huge spec impact on almost all the related control plane aspects, including SI, paging, SRAP design, etc.. Moreover, it may also bring extra impacts on the support of service continuity with multi-hop indirect path switch involving intermediate relay UEs in RRC_INACTIVE or in RRC_IDLE. Otherwise, service continuity with multi-hop indirect path switch involving intermediate relay UEs in RRC_INACTIVE or in RRC_IDLE would not be supported without additional enhancement. If the optimisation case is only supported for connection establishment for simplicity, the user experience will be really bad since the service continuity is not guaranteed. Considering the limited TU for Rel-19 multi-hop U2N relay, we suggest RAN2 not support the optimisation case.

Proposal 1: The case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment is not supported.

Control plane aspects on supporting multi-hop U2N relay

Radio link failure

For radio link failure in multi-hop U2N relay scenario, the Uu RLM and RLF handling for remote UE and relay UEs can follow the same behaviour as single hop U2N relay scenario. For the intermediate relay UE, upon detecting PC5 RLF, some enhancement might be needed to indicate the RLF to the related nodes in the other direction that are not suffering the RLF. PC5-RRC message could be used to carry the indication of PC5 RLF between the relay UEs and the remote UE. The NotificationMessageSidelink message could be reused in this case. Whether to use the indication of relayUE-PC5-RLF for U2U relay or introduce a new indication for multi-hop U2N relay case could be further discussed in stage-3 CR drafting.

Proposal 2: For the multi-hop U2N relay UE, upon detecting PC5 RLF, the relay UE may send the NotificationMessageSidelink message to the remote UE or next hop relay UE in other direction to indicate the PC5 RLF.

Proposal 3: Whether to reuse the indication of relayUE-PC5-RLF for U2U relay or introduce a new indication for multi-hop U2N relay case could be further discussed in stage-3.

System information

For single hop U2N relay, the in-coverage L2 U2N Remote UE is allowed to acquire any necessary SIB(s) over Uu interface irrespective of its PC5 connection to L2 U2N Relay UE. The L2 U2N Remote UE can also receive the system information from the L2 U2N Relay UE after PC5 connection establishment with L2 U2N Relay UE. The L2 U2N Remote UE in RRC_IDLE or RRC_INACTIVE can inform L2 U2N Relay UE of its requested SIB type(s) via PC5-RRC message. Then, L2 U2N Relay UE triggers on-demand SI/SIB acquisition procedure according to its own RRC state (if needed) and sends the acquired SI(s)/SIB(s) to L2 U2N Remote UE via PC5-RRC message.

For multi-hop U2N relay, the same principle can be followed and it was agreed that the U2N Remote UE acquires the SI of the cell of the Last Relay UE, which is forwarded via the Intermediate Relay UE(s) in RAN2#128 meeting. In last RAN2 meeting, lots of agreements were reached on the details of system information handling for remote UE and relay UE. For intermediate relay UE, the following issues highlighted in yellow were left for further discussion.

Agreements and open issues in RAN2#129 - The PC5-RRC message containing the required SI that is transmitted by the remote UE or by the intermediate relay UE to the parent node contains at least the requested SIB list. FFS if the intermediate relay UE can respond directly instead of forwarding the request to the parent node if it has the requested SI. - When an intermediate relay UE is in RRC_IDLE/RRC_INACTIVE it can obtain the SI required by it or requested by the remote UE by requesting SI from the parent relay UE in PC5-RRC (e.g., using RemoteUEInformationSidelink). FFS if it can also receive it directly from SIB broadcast by the last relay UE’s serving cell on Uu (when the intermediate relay UE is in coverage of the same cell that serves the last relay UE). - The intermediate relay UE sends SI request in PC5-RRC (e.g., in RemoteUEInformationSidelink) to the parent relay (intermediate relay or last relay): - upon reception of new/changed required SI received from a remote UE/child relay UE (where the concerned SI was not previously requested) - FFS when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu (e.g., moving in/out of coverage) to initiate/cancel SI forwarding by the parent relay.

One issue is whether the intermediate relay UE in RRC_IDLE/RRC_INACTIVE can receive SI directly from SIB broadcast by the cell on Uu. From intermediate relay UE’s perspective, it is actually a remote UE as well. Thus, it shall follow the remote UE behaviour on SI handling as legacy. There is no strong motivation to enhance the intermediate relay UE to keep on tracking the broadcasted SI in Uu for multi-hop U2N relay, which may also bring extra complexity to the intermediate relay UE. Besides, based on the agreement made in RAN2#128 meeting, an upper bound of Uu RSRP is configured to determine the operation as an intermediate relay UE. Thus, the intermediate relay UE is very likely to be at the edge of the cell or even out of coverage. The case that the intermediate relay UE can receive SI directly from SIB broadcast by the cell on Uu is more a corner case. Moreover, to guarantee the consistency between the SIBs received directly from Uu and from the parent relay UE, further enhancement might be needed, such as the third open issue listed above. Thus, we suggest not to support it.

Proposal 4: Regarding the SI receiving of an intermediate relay UE, RAN2 confirms that intermediate relay UE receiving SI directly from SIB broadcast by the cell on Uu is not supported.

If proposal 4 is agreed, there is no need to support the new trigger condition on sending SI request in PC5-RRC to the parent relay when there is a change in the ability of the intermediate UE.

Proposal 5: Regarding the sending of SI request by an intermediate relay UE to the parent relay, RAN2 confirms not to support sending SI request when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu.

Whether an intermediate relay UE can forward available SI directly (rather than retrieving it from the last relay UE) remains FFS. If proposal 4 is agreed, there is no inconsistency issue of the SI stored in the intermediate relay UE. Allowing intermediate relay UE to forward available SI directly when it has available SI can help reduce the PC5 signalling overhead and reduce the time for remote UE to get the available SI. Assuming the remote UE requires SIBx and SIBy while the intermediate relay UE only has available SIBx. In such case, the intermediate relay UE could forward the SI request only with SIBy to the parent relay or last relay and respond the SIBx directly.

Proposal 6: RAN2 confirms that the intermediate relay UE can respond directly if it has the requested SI. For the requested SI that the intermediate relay UE doesn’t have, it forwards the requested SI list to the parent relay or last relay.

- Conclusion

In this contribution, we discuss control plane related issues for multi-hop U2N relay. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: The case that intermediate relay UEs do not all move to RRC_CONNECTED when the remote UE triggers connection establishment is not supported.

Proposal 2: For the multi-hop U2N relay UE, upon detecting PC5 RLF, the relay UE may send the NotificationMessageSidelink message to the remote UE or next hop relay UE in other direction to indicate the PC5 RLF.

Proposal 3: Whether to reuse the indication of relayUE-PC5-RLF for U2U relay or introduce a new indication for multi-hop U2N relay case could be further discussed in stage-3.

Proposal 4: Regarding the SI receiving of an intermediate relay UE, RAN2 confirms that intermediate relay UE receiving SI directly from SIB broadcast by the cell on Uu is not supported.

Proposal 5: Regarding the sending of SI request by an intermediate relay UE to the parent relay, RAN2 confirms not to support sending SI request when there is a change in the ability of the intermediate UE to receive SIB broadcast on Uu.

Proposal 6: RAN2 confirms that the intermediate relay UE can respond directly if it has the requested SI. For the requested SI that the intermediate relay UE doesn’t have, it forwards the requested SI list to the parent relay or last relay.

- Reference

RP-241609, New WID on NR sidelink multi-hop relay, LG Electronics, InterDigital, FirstNet

3GPP TS 38.300 18.2.0: "NR; Radio Resource Control (RRC) protocol specification".