3GPP TSG-RAN WG2 Meeting #129bis	R2-2502237

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:	8.6.4

Source: 	China Telecom

Title:	Discussion on conditional intra-CU LTM

WID/SID:	NR_Mob_Ph4-Core

Document for:	Discussion and Decision

- Introduction

In last RAN2 meeting, the Rel-19 WI of Mobility enhancement was widely discussed and much progress has been made. Regarding conditional intra-CU LTM, the following agreements were reached.

Agreements on C-LTM:

- From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

- The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

- For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

- Do not support DG-based first UL transmission for RACH-less conditional LTM.

- For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

- For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

- CLTM TAT is started/re-started when UE receives TA value for the candidate cell sent by the MAC CE from current serving cell.

- If the CLTM TAT associated with one candidate cell is running, UE considers the TA for the candidate cell is valid; if CLTM TAT associated with one candidate cell expires, UE considers the TA for the candidate cell is invalid.

- If UE has valid TA value at the time CLTM execution, UE performs RACH-less CLTM, otherwise UE performs RACH base CLTM.

- The CLTM TAT is per candidate configuration.

- FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.

- Absolute TA value is included in the MAC CE for CLTM early TA acquisition.

- The candidate configuration ID is included in the MAC CE for CLTM early TA acquisition. And one configuration ID for one TA in one MAC CE.

In this contribution, we would like to share our opinions on remaining issues on TA handling for CLTM.

- Discussion

Handling of TAT timer for PTAG upon executing a CLTM

In last RAN2 meeting, how to handle the TAT timer for PTAG upon executing a CLTM was discussed with no consensus reached. The following potential options were listed in the Chair notes.

-	Option 1 (revert the offline agreement): start the timeAlignmentTimer associated with PTAG as in Rel-18;

-	Option 2 (keep the offline agreement): discuss the options below in the MAC running CR

o	Option 2.1: clarify the running TAT to act as the timeAlignmentTimer associated with PTAG by a note in the MAC running CR;

o	Option 2.2: define that UE can perform UL tx when the TAT for the target cell is running. Besides, we need to add a note that, if SCell and SpCell are in the different TAG, SCell may not work when this TAT for target cell is running.

o	Option 2.3: the UE stops the CLTM timer and starts the PTAG using the remaining time of the CLTM timer.

According to the current spec, to start or restart the timeAlignmentTimer associated with PTAG (Option 1) is a legacy UE behaviour upon a legacy HO or LTM. For CLTM, we see no strong motivation to introduce a different behaviour upon the execution of CLTM. For Option 2, either of those sub options has to introduce extra spec impact and different UE behaviour from the legacy. Some companies may have concern on the TA validity of Option 1. In our opinion, the TA validity after CLTM could be maintained by the legacy Timing Advance Command MAC CE. The NW can adjust the TA based on the following UL transmission if needed. The enhancement in Option 2 is not needed. Thus, we prefer to go for option 1. Furthermore, the CLTM TAT timer associated with the target (candidate) cell could be stopped upon CLTM execution.

Proposal 1: Upon CLTM execution, the UE starts or restarts the timeAlignmentTimer associated with PTAG and stops the CLTM TAT timer associated with the target cell.

Whether to release the CLTM TAs for other candidate cells

Another open issue is whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution. As agreed in previous RAN2 meetings, the validity of CLTM TAs is maintained by CLTM TAT configured per candidate cell. From the UE’s perspective, the CLTM TA could be treated as valid as long as the associated TAT timer is running. On the other hand, the TA of candidate cell is provided by the source gNB from the candidate cell. Thus, although CLTM is executed, the previous calculated TAs by other candidate cells are still valid if the associated timers have not been expired. Keeping the CLTM TAs for other candidate cells upon CLTM execution is beneficial for potential subsequent RACH-less CLTM and could help to reduce unnecessary early sycn as well. Thus, we prefer to keep the CLTM TAs for other candidate cells.

Proposal 2: Upon CLTM execution, the received CLTM TAs of other candidate cells is not released at UE side.

Proposal 3: Upon CLTM execution, the CLTM TA timers associated to other candidate cells are not stopped at UE side.

One thing that the target cell may not know is the left time for the CLTM TAT timer, since the CLTM TA command is send by the source gNB. However, from the new source cell’s point of view (i.e. the target cell), there is no need to consider the left time of the CLTM TAT timers. The gNB can trigger the early sycn with other candidate cell when needed based on the L1 or L3 measurement reports from the UE. The UE could follow the NW’s indication to perform early sycn with the candidate cell indicated by the gNB, no matter whether the CLTM TAT associated to the candidate cell is running or not.

Proposal 4: After CLTM execution, UE considers the TA for other candidate cell as valid as long as the associated CLTM TAT are running.

Proposal 5: After CLTM execution, the UE follows the PDCCH order from the new source cell to perform early sycn with the indicated candidate cell no matter whether the CLTM TAT associated to the candidate cell is running or not.

- Conclusion

In this contribution, we discuss the remaining issues on TA handling for CLTM. We kindly ask RAN2 to consider the corresponding proposals listed as below.

Proposal 1: Upon CLTM execution, the UE starts or restarts the timeAlignmentTimer associated with PTAG and stops the CLTM TAT timer associated with the target cell.

Proposal 2: Upon CLTM execution, the received CLTM TAs of other candidate cells is not released at UE side.

Proposal 3: Upon CLTM execution, the CLTM TA timers associated to other candidate cells are not stopped at UE side.

Proposal 4: After CLTM execution, UE considers the TA for other candidate cell as valid as long as the associated CLTM TAT are running.

Proposal 5: After CLTM execution, the UE follows the PDCCH order from the new source cell to perform early sycn with the indicated candidate cell no matter whether the CLTM TAT associated to the candidate cell is running or not.

- Reference

RP-240299	Revised Work Item: NR mobility enhancements Phase 4

RP-24xxxxx	draft Report of 3GPP TSG RAN WG1 meeting #116bis

3GPP TS 38.331 18.0.0: "NR; Radio Resource Control (RRC) protocol specification".

R2-2403731	Report from session on V2X/SL, R19 NES and MOB