3GPP TSG-RAN WG2 Meeting #129bis	R2-2502238

Wuhan, China, Apr. 7th – 11st, 2025

Agenda item:	8.1.3

Source:	China Telecom

Title:	Discussion on NW side data collection

WID/SID:	NR_AIML_Air-Core– Release 19

Document for:	Discussion and Decision

# 1 Introduction

In the previous meetings, NW-side data collection was discussed, and the following agreements were made with several FFSs.

Agreements (RAN2#127)

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

- UE stores the logged training data at AS layer with a minimum AS layer memory size supported by the UE. FFS on the memory size.  This is across all use cases

- When UE reaches its buffer limitation the UE stops measurement for data collection purposes and logging.

- Measurements for data collection purposes and logging based can be controlled based on power state of the UE.  It is up to UE implementation how the UE determines power state.  FFS whether the UE stops autonomously or if it reports to the network .

- FFS whether AS buffer event based reporting is supported.  FFS if we send availability indication or full report if it is supported

- FFS on event based data collection/logging

- On-demand request from the network is supported.   FFS details on signalling

Agreements (RAN2#127b)

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.  FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

Agreements (RAN2#128)

- Focus on the following three radio condition event based logging

-  L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2)

-  Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value)

-  L1 beam level measurement

- Measurements on aperiodic CSI resources are not reported for NW sided data collection.

- Data collection is controlled by the network.   The UE will not autonomously stop when low power state is detected.

- The UE reports to the network when the power state is low.  We will not specify how the UE determines low power state.   The network should de-configure the data collection (this can be captured in stage 2).

- The UE reports to the network when buffer is or may become full.  FFS when it reports (before and/or after).

- The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory).  FFS how this is signalled and if the reporting can be part of availability indication.

Agreements (RAN2#129)

- Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log

- Low power bit indication is supported

- Data availability indication is supported.  FFS when this would be triggered

- As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

- UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

- FFS how to handle idle/inactive and RLF cases and whether we have a unified.

In this contribution, we will focus on the following remaining issues of data availability indication:

- Data availability indication is supported. FFS when this would be triggered.

- UE retains logged data during handover (HO). FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed.

- FFS how to handle idle/inactive and RLF cases and whether we have a unified.

# 2 Discussion

## 2.1 Definition/Trigger of data availability indication

In RAN2#129 meeting, RAN2 agreed that data availability indication is supported. The data availability indication is used to tell the network that there is data available. But what the available data means and when the indication is triggered were left as FFS. From our side, available data refers to data that UE has logged and ready for transmission, which means that there is data (i.e. the amount of data is greater than 0), and the data matches some constraints. Therefore, the amount of collected data should be considered for determining whether the data is available. We think that the data can be determined as available based on the following:

- Network configurations

When the amount of collected data exceeds the configured threshold from NW, the data is available.

- UE’s internal status

When the AS buffer is above a threshold or the AS buffer is full, the data is available.

Proposal 1: Whether the data is available can be determined based on NW configurations and UE’s internal status:

- Based on network configurations: when the amount of collected data exceeds the configured threshold from NW, the data is available.

- Based on UE’s internal status: when the AS buffer is above a threshold or the AS buffer is full, the data is available.

Additionally, the data availability indication can be triggered as follows:

- The data availability indication can be triggered when the amount of collected data is above a threshold configured by NW.

- The data availability indication can be triggered when the AS buffer is above a threshold, or the AS buffer is full.

- The data availability indication can be triggered when receiving the request from NW, i.e., NW may request UE to report data availability indication.

In our opinion, UE should tell the network that there is data available as soon as it becomes available. However, in order to ensure the continuity of data collection, it is essential to report the logged data before the buffer is full and make room for new collected data. Thus, it is reasonable to trigger availability indication before AS buffer is full.

Observation 1: It is late to trigger the data availability indication when the AS buffer is full.

Based on observation 1, we found that the threshold for amount of collected data needs to be less than AS buffer size. Therefore, based on proposal 1 and observation 1, we have following proposal:

Proposal 2: The data availability indication can be triggered as follows:

- The data availability indication can be triggered when the amount of collected data is above a threshold configured by NW.

- The data availability indication can be triggered when the AS buffer is above a threshold. The threshold needs to be less than AS buffer size.

- The data availability indication can be triggered when receiving the request from NW, i.e., NW may request UE to report data availability indication.

## 2.2 UE retains logged data during handover (HO)

In RAN2#129 meeting, RAN2 agreed that UE retains logged data during handover (HO). if data is retained in the UE , UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message). However, it is still FFS whether there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed.

From our side, there could be scenarios where UE releasing the data during HO makes sense.

- Memory Constraints on the UE

  - The UE has limited storage capacity, and the logged data exceeds this limit. During HO, retaining large datasets might strain resources, especially if new logs are being generated. In this case, the UE might need to release older or less critical data to free up space for ongoing logging or critical operations during HO.

  - This could be controlled by network configuring a threshold or policy (e.g., via RRC signaling) to guide when to discard data.

- Data Relevance or Validity Expires

  - The logged data is tied to the source gNB and becomes irrelevant after HO to target gNB, which operates in a different area or configuration. In this case, the logged data is no longer useful. Releasing it during HO could optimize UE resources and reduce unnecessary signaling post-HO.

  - The source gNB could indicate UE to discard data as part of the HO command.

- Security or Privacy Policy

  - The logged data includes sensitive information (e.g., location data) that shouldn’t be retained across network boundaries (e.g., moving to a different operator’s gNB in roaming). In this case, to comply with privacy regulations or operator policies, the UE might need to release data during HO to avoid transferring it to an unauthorized gNB.

  - This would require network signaling instructing the UE to discard data upon HO.

- Network-Initiated Request

  - The target gNB doesn’t need the old data and prefers the UE to start fresh logging post-HO, perhaps to align with its own optimization goals. In this case, the UE might release the data to comply with the target gNB’s request.

  - The target gNB could include a command in the HO configuration (via the source gNB’s RRCReconfiguration) or post-HO configuration from target gNB, telling the UE to discard old data.

Proposal 3: There are scenarios where the UE might need to release logged data during handover:

- Scenarios:

  - The UE has limited storage capacity, and the logged data exceeds this limit.

  - The logged data is related to the source gNB and becomes irrelevant after HO to target gNB.

  - The logged data includes sensitive information (e.g., location data) that shouldn’t be retained across network boundaries.

  - The target gNB doesn’t need the old data and prefers the UE to start fresh logging post-HO.

- Network control:

  - UE discards the logged data via network control, such as via signaling in RRCReconfiguration or post-HO messages.

## 2.3 UE retains logged data across RRC states

From our side, network need to collect all the logged data before transmitting the RRCRelease message. In addtion, considering the UE mobility, the UE may moves to another area and the logged data is not useful anymore, since the data may targets to specific area. Therefore, it is not beneficial to retain logged data across RRC states.

Proposal 4: Network collects the logged data before transmitting an RRCRelease message, which makes UE enter RRC_IDLE/RRC_INACTIVE state from RRC_CONNECTED state.

Proposal 5: When UE enters RRC_IDLE/RRC_INACTIVE state from RRC_CONNECTED state, the UE discards the logged data.

## 2.4 Signaling for data availability indication

In legacy logged MDT, the UE indicates the availability of logged measurements within RRC complete messages (i.e., RRCReconfigurationComplete / RRCSetupComplete / RRCResumeComplete / RRCReestablishmentComplete). For instance, considering a handover case, if the network's data collection configuration is kept in the UE, the UE must inform the new gNB of its availability. This notification allows the new gNB to request retrieval of the UE’s logged data as needed. Moreover, as the availability indication information is UE side assistance information which may be helpful to network, so we assume UAI can be also used to deliver this information to network.

Proposal 6: Support both the RRC complete messages and UAI message for reporting the data availability indication.

# 3 Conclusion

In this contribution, we further discuss the remaining issues related to data availability for NW-side data collection. We have the following proposals:

Proposal 1: Whether the data is available can be determined based on NW configurations and UE’s internal status:

- Based on network configurations: when the amount of collected data exceeds the configured threshold from NW, the data is available.

- Based on UE’s internal status: when the AS buffer is above a threshold or the AS buffer is full, the data is available.

Observation 1: It is late to trigger the data availability indication when the AS buffer is full.

Proposal 2: The data availability indication can be triggered as follows:

- The data availability indication can be triggered when the amount of collected data is above a threshold configured by NW.

- The data availability indication can be triggered when the AS buffer is above a threshold. The threshold needs to be less than AS buffer size.

- The data availability indication can be triggered when receiving the request from NW, i.e., NW may request UE to report data availability indication.

Proposal 3: There are scenarios where the UE might need to release logged data during handover:

- Scenarios:

  - The UE has limited storage capacity, and the logged data exceeds this limit.

  - The logged data is related to the source gNB and becomes irrelevant after HO to target gNB.

  - The logged data includes sensitive information (e.g., location data) that shouldn’t be retained across network boundaries.

  - The target gNB doesn’t need the old data and prefers the UE to start fresh logging post-HO.

- Network control:

  - UE discards the logged data via network control, such as via signaling in RRCReconfiguration or post-HO messages.

Proposal 4: Network collects the logged data before transmitting an RRCRelease message, which makes UE enter RRC_IDLE/RRC_INACTIVE state from RRC_CONNECTED state.

Proposal 5: When UE enters RRC_IDLE/RRC_INACTIVE state from RRC_CONNECTED state, the UE discards the logged data.

Proposal 6: Support both the RRC complete messages and UAI message for reporting the data availability indication.