3GPP TSG RAN WG2 #129bis		  R2-2502264

Wuhan, CN, Apr 7 – 11, 2025

Title:	Data Collection for NW-Sided Model Training for BM

Agenda item:	8.1.3

Source:	Google

Document for:	Discussion and Decision

# 1 Introduction

In RAN2#129 meeting, there were some agreements and FFSs for nw-side data collection.

All agreements for NW side data collection

- Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log

- Low power bit indication is supported

- Data availability indication is supported.  FFS when this would be triggered

- As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

- UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

- UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

- FFS how to handle idle/inactive and RLF cases and whether we have a unified.

In RAN2#127bis meeting, both periodic and event-triggered data logging are supported for NW side data collection.

Agreements on NW side data collection

- Periodic logging is supported for training data collection procedure in R19

- Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

- Periodic reporting of logged data is not supported.

- On-demand reporting of the logged measurements will be specified

- UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

- Low priority SRB will be used.    FFS new SRB or use of SRB4

- For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

In this paper, we further discuss the remaining FFSs and give related considerations.

# 2 Discussion

## Data Collection Configuration

In RAN2#128 meeting, it was agreed that the NW only configures periodic CSI resources for measurements for NW-sided data collection.

- Measurements on aperiodic CSI resources are not reported for NW sided data collection.

In RAN2#127bis, periodical logging was supported for NW-side data collection. Literally, for periodic logging, the UE logs data periodically. There is an issue on the setting of the logging interval for periodic logging scheme.

One approach is to implicitly set the periodic logging interval to the period of the configured CSI resource. However, this approach lacks flexibility. For example, if the NW wants to adjust the logging interval for the previous CSI resource, it would need to modify the existing CSI resource configuration to the new period. This would involve additional signalling overhead as well.

Alternatively, to enable flexible periodic data logging for a configured CSI resource, we propose introducing explicit logging interval configuration. This is similar to the legacy L3 periodic reporting configuration. This would allow the NW to change the logging interval without modifying the pervious CSI resource.

Proposal 1: Define an explicit logging interval for periodic data logging.

In legacy L3 measurement procedure, the NW provides measurement configurations including measurement object configurations and reporting configurations. In measurement object configurations, it can include a list of objects for the UE to measure. In reporting configurations, it can include a list of reporting configurations. For the reporting configurations, it can provide detailed parameters for periodical reporting or event-triggered reporting. In addition, a list of measurement identities are introduced, where each measurement identity links one measurement object with one reporting configuration, e.g., one measurement ID linking one measurement object ID with one reporting configuration ID. By configuring multiple measurement identities, each measurement object can be linked with more than one reporting configuration. Concurrently, more than one measurement object can be linked to the same reporting configuration. This structure provides a flexible way to correlate measurement objects with reporting configurations.

In the existing draft CR, it introduces CSI-LoggedMeaurementConfigId and CSI-ResourceConfigId, similar as the legacy measurement ID and measurement object ID, irrespectively. Detailed logging configuration parameters are defined per CSI-LoggedMeaurementConfigId. If the NW intends to apply the same set of event-triggered logging configuration parameters to multiple CSI resource configurations, it must repeat these parameters for each CSI-LoggedMeaurementConfigId, which results in significant signalling overhead. Thus, the current structure is inefficient.

CSI-LoggedMeasurementConfig-r19 ::=          SEQUENCE {

csi-LoggedMeasurementConfigId-r19         CSI-LoggedMeasurementConfigId-r19,

csi-LoggedResourceConfig-r19              CSI-ResourceConfigId,

eventTriggeredConfig-r19                  EventTriggeredConfig-r19                    OPTIONAL, --Need R

...

}

Taking into consideration the above analysis, it is desirable to similarly apply the flexible principle used in the legacy L3 measurement framework to the structure of the CSI logged measurement configuration. Specifically, we need to define data logging configurations where each data logging configuration assigned with a unique ID, e.g., DataLoggingConfigId, corresponds to a set of parameters. For each logging configuration associated with a CSI-LoggedMeaurementConfigId, the NW can directly configure a DataLoggingConfigId, instead of defining detailed parameters. This approach allows the flexibility as the legacy L3 measurement framework, reducing complexity and signalling overhead.

Proposal 2: Define data logging configurations where each data logging configuration includes a set of parameters and is associated with a unique ID, e.g., DataLoggingConfigId.

For the data logging configurations, it is straightforward to support both periodic and event-triggered data logging schemes.

Proposal 3: The data logging configurations supports parameters for both periodic and event-triggered data logging.

## Data Collection Reporting

Following the progress in RAN2#129 meeting, it is understood that the UE will send a data availability indication. The specific triggering mechanism is still FFS. In legacy logged MDT, the UE performs data logging in idle or inactive state. During data logging, the UE doesn’t have chance to send the data availability indication. Unlike legacy logged MDT, for this data logging feature, the UE is always in connected mode. This provides more opportunities for the UE to trigger the data availability indication. For efficient logged data reporting, it is preferable that the UE triggers the data availability indication when necessary.

In one approach, the NW configures a minimum size for the logged data. When the volume of the logged data exceeds this minimum size, the UE is allowed to trigger the data availability indication. The exact timing of the trigger, after the minimum size is reached, is left to UE implementation. This can avoid frequent reporting of logged data.

Alternatively, the NW configures a memory usage threshold, e.g., a percentage value. When the UE detects that its memory usage exceeds this threshold, the UE immediately triggers the data availability indication.

Proposal 4: The UE triggers a data availability indication:

- Opt1: the UE is allowed to trigger the indication if the logged data size meets or exceeds a configured minimum size; the specific trigger timing is up to UE implementation

- Opt2: the UE immediately triggers the indication if its memory usage for logging exceeds a configured threshold.

As discussed in the above sections, it is known that the UE may support multiple data logging tasks simultaneously. It is possible that for certain tasks, the UE cannot log sufficient data due to various reasons. For example, for event-triggered tasks, the triggering conditions for the events might not be satisfied at all, or be satisfied infrequently. In addition, the UE may lack sufficient memory to store all the data required for a specific data logging task. Moreover, there might be abnormal UE state, e.g., low power, overheating, which may cause the UE to stop data logging potentially based on NW control. Furthermore, RAN2 agreed that the UE can retain logged data during handover. Generally, the UE would stop the data logging configured by the source cell. For all the above cases, they may result in no or insufficient logged data for some configured asks at UE side.

Therefore, When the UE reports the logged data to the NW, it is essential to include a cause indication for any task where the data logging is incomplete or the logged data is limited.

Proposal 5: The UE includes a cause indication (e.g., event not triggered, event triggered infrequently, low memory, low power state, overheating, or mobility) for the CSI resources with no or insufficient logged data.

## Mobility, Idle/Inactive and RLF Cases

There might be the case that a UE, configured with data collection configurations, transits into idle or inactive state. The state transition may occur in two ways. One is that the NW initiatively releases the connection and configures UE to idle or inactive state, e.g., due to normal RRC state management or abnormal network congestion. In the other case, the UE detects RLF with the previous serving cell and enters idle state subsequently. For normal RRC state management case, there might be sufficient time for the NW to retrieve the logged data from the UE before the state transition. For the remaining cases, it might be difficult for the original serving NW to retrieve the logged data.

To handle these cases, separate solutions would bring significant complexity for both NW and UE. Therefore, a unified solution is preferable. As a potential unified solution, when the UE disconnects from the cell that configured data collection, the UE shall retain the logged data, if available.

Proposal 6: The UE retains logged data when entering idle/inactive or detecting RLF.

As discussed in the above part, the UE may report logged data in a cell different from the one which originally configured the data collection. To address potential data leakage issue, one solution is to introduce a reporting area for the logged data reporting. If the cell the UE is currently camped on is within the reporting area, the UE can initiate the logged data reporting procedure; otherwise, the UE refrains from reporting logged data to the camping cell.

Proposal 7: The UE checks whether the camping cell is within the valid area for logged data reporting.

For the above cases, when the UE determines the camping cell is suitable for logged data reporting, the UE initiates data availability to the camping cell. When the camping cell retrieves the logged data, the UE should provide the data collection cell information alongside the logged data, in order to assist the delivery of these data from the current camping cell to the one which originally configured the data collection.

Proposal 8: The UE includes data collection cell information with the logged data when reporting it in a different cell.

# 3 Conclusion

In this contribution, we discussed the potential solutions on the remaining FFSs and gave more considerations on W-side data collection or BM. The related proposals are summarized below:

Proposal 1: Define an explicit logging interval for periodic data logging.

Proposal 2: Define data logging configurations where each data logging configuration includes a set of parameters and is associated with a unique ID, e.g., DataLoggingConfigId.

Proposal 3: The data logging configurations supports parameters for both periodic and event-triggered data logging.

Proposal 4: The UE triggers a data availability indication:

- Opt1: the UE is allowed to trigger the indication if the logged data size meets or exceeds a configured minimum size; the specific trigger timing is up to UE implementation

- Opt2: the UE immediately triggers the indication if its memory usage for logging exceeds a configured threshold.

Proposal 5: The UE includes a cause indication (e.g., event not triggered, event triggered infrequently, low memory, low power state, overheating, or mobility) for the CSI resources with no or insufficient logged data.

Proposal 6: The UE retains logged data when entering idle/inactive or detecting RLF.

Proposal 7: The UE checks whether the camping cell is within the valid area for logged data reporting.

Proposal 8: The UE includes data collection cell information with the logged data when reporting it in a different cell.