3GPP TSG-RAN WG2 Meeting #129 bis		R2-2502267

Hubei, China, 7th – 11st April. 2025

Source:	China Telecom

Title:	Discussion on UE initiated beam reporting

Agenda Item:	8.12.3

Document for:	Discussion and Decision

- Introduction

Working assumption: RAN2 assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification. The final decision is up to RAN1, we can revisit this if R1 has a different decision.

No need to send a LS to R1 on this.

According to last meeting’s progress as above, we will discuss the RAN2 aspects related to UEIBM.

- Discussion

In the last meeting, RAN2 agreed to assume the event evaluation for UE-initiated beam reporting is captured in 38.321, where the evaluation is based on indications from measurements described in a RAN1 specification.

Refer to the beam failure detection. The MAC layer utilizes BFI_COUNTER, beamFailureInstanceMaxCount, and beamFailureDetectionTimer to determine beam failure and trigger BFR.

The MAC entity shall for each Serving Cell configured for beam failure detection:

1>	if the Serving Cell is configured with two BFD-RS sets:

2>  if beam failure instance indication for a BFD-RS set has been received from lower layers:

3>  start or restart the beamFailureDetectionTimer of the BFD-RS set;

3>  increment BFI_COUNTER of the BFD-RS set by 1;

3>  if BFI_COUNTER of the BFD-RS set >= beamFailureInstanceMaxCount:

4>trigger a BFR for this BFD-RS set of the Serving Cell;

For event detection of UEIBM, a similar approach can be adopted. When an instance is detected and if the timer is not running, start the timer, and the value of the timer can be EventDetectionTimeWindowLength-r19 configured by RRC and increase the Event Instance Counter. If the Event Instance Counter reaches EventInstanceCount-r19 configured by RRC before the timer expiry, trigger a report.

- Proposal 1: Refer to beam failure detection, introduce a timer and a counter for event detection of UEIBM. FFS whether any optimization is needed.

- Conclusion

- Proposal 1: Refer to beam failure detection, introduce a timer and a counter for event detection of UEIBM. FFS whether any optimization is needed.