3GPP TSG-RAN WG2 Meeting #129-bis	R2-2502281

Wuhan, China, April 7th – April 11th, 2025

Agenda item:	8.3.2.2

Source:	Qualcomm Incorporated

Title:	Simulation Results for Model Generalization

WID/SID:	FS_NR_AIML_Mob – Release 19

Document for:	Discussion and Decision

# Introduction

In the previous two RAN2 meetings (RAN2 #128 [1] and RAN2 #129 [2]), the simulation framework, assumptions, and parameters for Model Generalization for RRM measurement prediction were discussed, and it was agreed to study AI/ML model generalization in multiple cases. As captured in the Chair notes for the respective meetings, it was agreed in meeting RAN2 #128 to study model generalization across UE speeds, and it was agreed in RAN2 #129 to study generalization across simulation scenarios.

To study Model Generalization across UE speeds, it was agreed in [1] to study AI/ML performance for the following cases:

- Baseline: The AI/ML model is trained using the dataset with Configuration #B and tested using the dataset with Configuration #B​

- Generalization Case #1 (GC#1): The AI/ML model is trained using the dataset with Configuration #A but tested using the dataset with Configuration #B​

- Generalization Case #2 (GC#2): The AI/ML model is trained using mixed datasets with both configurations and tested using the dataset with Configuration #B

It was agreed to consider datasets in which mobile UEs move with speeds of 30 km/h, 60 km/h, and 90 km/h. Table 1 and Table 2 summarize the agreed UE speed simulation combinations for FR1 and FR2 generalization study, respectively.

Table 1: Agreed simulation combinations for FR1 study for Model Generalization over UE speeds

|  | UE speeds in Training dataset |  |  | UE speeds in Inference dataset |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 30 km/h | 60 km/h | 90 km/h | 30 km/h | 60 km/h | 90 km/h |
| Baseline | Yes |  |  | Yes |  |  |
| GC #1 |  | Yes |  | Yes |  |  |
| GC #1 |  |  | Yes | Yes |  |  |
| GC #2 | Yes | Yes | Yes | Yes |  |  |
| Baseline |  | Yes |  |  | Yes |  |
| GC #1 | Yes |  |  |  | Yes |  |
| GC #1 |  |  | Yes |  | Yes |  |
| GC #2 | Yes | Yes | Yes |  | Yes |  |
| Baseline |  |  | Yes |  |  | Yes |
| GC #1 | Yes |  |  |  |  | Yes |
| GC #1 |  | Yes |  |  |  | Yes |
| GC #2 | Yes | Yes | Yes |  |  | Yes |

Table 2: Agreed simulation combinations for FR2 Model Generalization study

|  | UE speeds in Training dataset |  |  | UE speeds in Inference dataset |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 30 km/h | 60 km/h | 90 km/h | 30 km/h | 60 km/h | 90 km/h |
| Baseline | Yes |  |  | Yes |  |  |
| GC #1 |  | Yes |  | Yes |  |  |
| GC #1 |  |  | Yes | Yes |  |  |
| GC #2 | Yes | Yes | Yes | Yes |  |  |
| Baseline |  | Yes |  |  | Yes |  |
| GC #1 | Yes |  |  |  | Yes |  |
| GC #1 |  |  | Yes |  | Yes |  |
| GC #2 | Yes | Yes | Yes |  | Yes |  |
| Baseline |  |  | Yes |  |  | Yes |
| GC #1 | Yes |  |  |  |  | Yes |
| GC #1 |  | Yes |  |  |  | Yes |
| GC #2 | Yes | Yes | Yes |  |  | Yes |

To study Model Generalization across scenarios, it was agreed in [2] to study AI/ML performance over two sets of configurations, one with a UMi deployment scenario and one with a UMa deployment scenario. Table 3 summarizes the agreed sets of configurations for scenario generalization in the FR1 case (for FR2, the columns are swapped between Configuration #1 and Configuration #2):

Table 3: Agreed configurations for Model Generalization over scenarios

| Parameter | Configuration #1 | Configuration #2 |
| --- | --- | --- |
| Deployment scenario | UMi | UMa |
| ISD (m) | 200 | 500 |
| BS antenna height (m) | 10 | 25 |
| BS Tx power (dBm) | 40 | 44 |

In this contribution, we provide simulation evaluations for Model Generalization across UE speeds for the FR1 and FR2 Generalization Case #1 and Generalization Case #2 simulation combinations summarized in Table 1 and Table 2. We also provide simulation evaluations for FR1 Model Generalization across scenarios.

# Simulation assumptions

In this section, we discuss assumptions applicable to both FR1 and FR2 data generation simulations, detailing the methodology for UE trajectory generation, soft line-of-sight (LoS) updates, and shadow fading updates.

## UE trajectory generation formulation

This section details the chosen model for generating random trajectories for mobile UEs in data generation simulations. The chosen model is based on Option 2 in TR 38.343 [3], Section 6.3.1, with modifications based on Option 4 in [4] and a minor modification for extended mobility.

A UE has initial position (x0,y0, z0), constant forward velocity s, and initial azimuthal velocity angle ϕv,0. Temporal granularity tgran is defined as the smallest unit of time considered in random trajectory generation and mean temporal step size tstep is defined as the mean travel time for walk steps along the random trajectory. tstep is chosen such that it will be a whole-number multiple of the temporal granularity:

tstep=Nsubstepstgran, where

- Nsubsteps∈Z+ is the mean number of substeps per walk step.

- Maximum walk time tmax≫tstep is likewise chose to be a whole-number multiple of tgran.

- Distance granularity dgran is computed as dgran=stgran.

For the purposes of random trajectory generation for urban macro and urban micro simulations, a geometric sector SNb is defined as a geometric region consisting of the intersection of two regions: the first region consists of one third of the hexagonal cell surrounding a gNB and symmetric with respect to the gNB’s azimuthal orientation ϕNb, and the second region consists of the area outside a circle with radius dNb centered on the gNB. Figure 1 provides an example of a geometric sector.

Figure 1 Geometric sector defined for UE mobility constraints

The overall valid UE trajectory region Svalid is the geometric region in which a UE trajectory may be generated and consists of the union of all geometric sectors corresponding to each gNB in the simulation:

Svalid=i=1NNbSi

Where NNb is the number of gNBs in the simulation. Figure 2 visualizes Svalid for a 2-tier UMa simulation with ISD 200.

Figure 2 Geometric sectors for every gNB in a 2-tier Urban Macro simulation. Each geometric sector is labelled with the MAC ID of its corresponding gNB. The valid UE trajectory region Svalid consists of the union of all sectors.

Given the above setup, the following algorithm is used to generate a random trajectory for a particular UE:

- Initialize total sub-step counter m=0.

- For walk step k=1,2,…:

  - Draw number of sub-steps for the kth walk step Nsubsteps,k from a geometric distribution with probability of success 1Nsubsteps.

  - Draw azimuthal velocity angle delta for the kth walk step Δϕv,k from a uniform distribution in the range [-45°, 45°].

  - Compute sub-step velocity angle delta: Δϕv,k,substep=Δϕv,kNsubsteps,k

  - For each sub-step n=1, …,Nsubsteps,k:

    - Increment total sub-step counter m=m+1. If the UE’s total travel time thus far mtgran is greater than tmax, terminate the walk. Otherwise, continue.

    - Update the UE’s sub-step velocity angle: ϕv,m=ϕv,m-1+Δϕv,k,substep.

    - Compute potential updated UE position: (xm,ym,zm)=xm-1+dgrancosϕv,m,ym-1+dgransinϕv,m,z0.

If xm,ym,zm  is outside the valid UE trajectory region Svalid, terminate the walk. Otherwise, update the UE’s position.

Figure 3 shows an example set of UE trajectories generating using the above algorithm.

Figure 3 Example random trajectories for 21 UEs within a 2-Tier UMa model

## Channel modelling: grid-based soft line-of-sight and shadow fading

For studies with extended (inter-cell) UE trajectories, we use a grid-based approach to update line-of-sight (LoS) and shadow fading (SF) values for mobile UEs. This section details this grid-based procedure.

### Soft LoS updates

This section details the approach used to update line-of-sight (LoS) states for mobile UEs in data generation simulations. Discussion of the motivation for adopting this soft LoS model has been provided in [4].

In the soft LoS model provided in TR 38.901 [5], Section 7.6.3.3, each gNB-UE link maintains a soft LoS state LOSsoft∈[0, 1], calculated as

LOSsoftG,d=12+1πarctan20λG+Fd, where

- G is a spatially consistent Gaussian R.V. with correlation distance dcor according to 38.901 Table 7.6.3.1-2,

- λ is the wavelength of the center frequency fc (in meters), and

- Fd=2erf-12PrLOSd-1.

To apply this soft LoS state, for each link, both LoS and NLoS channel coefficients HLOS and HNLOS are generated per the 38.901 Fast Fading Model (Section 7.5, Steps 2-12), and the overall channel is calculated per 38.901 Equation 7.6-18 as

HLOSsoft=HLOSLOSsoft+HNLOS1-LOSsoft2.

Although TR 38.901 provides correlation distances for LoS random variables, a specific correlation function is not provided. To correlate the Gaussian random variables G used in soft LoS state generation in Option B, we use exponential correlation:

RLOSΔd=exp-Δddcor, where

- Δd is the distance between any two points in the xy-plane, and

- dcor is the correlation distance provided in Table 7.6.3.1-2.

As discussed in [4], this exponential correlation function is chosen to be consistent with the exponential autocorrelation function provided for shadow fading in 38.901, Section 7.4.4, as well as with the spatial correlation procedure provided in WINNER II [6], Section 3.3.1. We also note that the distribution statistics of the Gaussian random variables G are not specified in TR 38.901; for simplicity, we choose G to be standard normal Gaussians.

We assume that exponential correlation is applied on a per-gNB-location basis; that is, the Gaussians used to generate the soft LoS states for all UE-gNB links associated with a particular set of co-located gNBs are correlated per RLOSΔd, and there is no inter-gNB LoS correlation. We also assume that the generated LoS states for given xy-locations are fixed in time, meaning UEs will only experience LoS state transitions as a result of mobility. With these assumptions, the following procedure is used to calculate soft LoS states for mobile UEs:

- For each gNB in the simulation n=1,2,…,NNB:

  - Establish a grid of location points Dn that covers the area in which mobile UE trajectories will be constrained.

  - Over Dn generate a grid Gn of standard normal Gaussian realizations at each grid point correlated in distance by RLOSΔd=exp-Δddcor.

  - Using the soft LoS state procedure outlined in TR 38.901, Section 7.6.3.3, generate a grid of soft LoS realizations Ln= LOSsoft(Gn, Dn) from Gn and PrLOSd for the given scenario.

- During the channel update for each mobile UE i=1,2,…,NUE:

  - For each gNB in the simulation n=1,2,…,NNB, interpolate the soft LoS state for the UE-gNB link from Ln given the mobile UE’s xy-location.

When generating datasets with inter-site UE mobility, for each set of 3 co-located gNBs a unique soft LoS grid is used to model line-of-sight states for mobile UEs. For a dataset using 2-tier UMi/Uma models, a total of 7 LoS grids are generated for the entire dataset. Figure 4 shows example soft LoS grids used for a 2-tier FR1 UMa model with 500-meter ISD.

Figure 4 Soft LoS grids for a 2-tier FR1 model, ISD = 500m, fc = 4 GHz. Left: grid for gNBs located at (0, 0) m. Right: grid for gNBs located at (0, 500) m.

To demonstrate the impact of soft LoS updates on generated channels for mobile UEs, Figure 5 provides a comparison between simulation results for a mobile UE with and without soft LoS updates enabled. In the simulation with LoS updates disabled (orange line), the mobile UE has a constant LoS = 1 state with its serving gNB, while in the simulation with LoS updates enabled (blue line), the LoS state is updated as a function of the UE’s position. As the mobile UE enters NLoS regions, the maximum RSRP across all beam pair links in the simulation with LoS updates tends to drop significantly compared to the maximum RSRP for the simulation where the LoS state remains 1.

Figure 5 Simulation comparison for a mobile UE with and without soft LoS updates enabled. Top: maximum RSRP for mobile UE with and without soft LoS updates. Bottom: LoS state for mobile UE with and without LoS updates. A value of 0 corresponds to a completely NLoS link, while a value of 1 corresponds to a completely LoS link.

### Shadow fading updates

This section describes the model used for updating shadow fading values for mobile UEs in data generation simulations. Similarly to soft LoS state updates, we use a grid-based approach to update shadow fading values in a spatially consistent manner. Discussion of the motivation for adopting this approach has been provided in [4].

As with soft LoS values, we assume that shadow fading values are fixed in time for given xy-locations and independently calculated on a per-gNB basis. We also assume that a gNB’s shadow fading values are autocorrelated exponentially as per TR 38.901 [5], Section 7.4.4, with autocorrelation function

RΔx=exp-Δxdcor, where

- Δx is the 2D-distance between any two locations in the simulation, and

- dcor is the environment-specific correlation distance provided in TR 38.901, Table 7.5-6.

With these assumptions, the following procedure is used to update shadow fading values for mobile UEs:

- For each gNB in the simulation n=1,2,…,NNB:

  - Establish a grid of location points Dn that covers the area in which mobile UE trajectories will be constrained.

  - On top of Dn generate a grid of correlated shadow fading values SFn~N0, σsf, where σsf is the scenario/LoS-state-specific shadow fading standard deviation provided in TR 38.901, Table 7.5-6, and the R.V.s at each grid point are correlated in distance by RLOSΔd=exp-Δddcor .

- During the channel update for each mobile UE i=1,2,…,NUE:

  - For each gNB in the simulation n=1,2,…,NNB, interpolate the LoS and NLoS shadow fading values for the UE-gNB link from SFn given the mobile UE’s xy-location and UE-gNB link LoS.

Similarly to soft LoS grids a unique pair of LoS/NLoS SF grids are generated for each set of co-located gNBs in a layout. Figure 6 shows two example pairs of SF grids for a 2-tier UMa layout with 500-meter ISD.

Figure 6 Shadow fading grids for 2-tier FR1 UMa model with 500-meter ISD. Top: LoS (left) and NLoS (right) shadow fading grids for gNBs located at (0, 0) m. Bottom: LoS (left) and NLoS (right) shadow fading grids for gNBs located at (0, 500) m.

## Channel modelling: disabling wraparound

To facilitate modelling of inter-site UE mobility, we disable wraparound for UMa and UMi models. Figure 7 demonstrates the impact of disabling wraparound on the channel modelling parameters on the arrival and departure LoS cluster angles between UEs in the bottom sector and gNBs in the upper-left sector of a 2-tier model. Disabling wraparound ensures that the channels between mobile UEs and distant gNBs do not experience instantaneous changes as the UEs draw nearer.

Figure 7 Effect of disabling wraparound on LoS cluster angles between UEs and gNBs in separate sectors. Left: when wraparound is enabled, the UE LoS arrival angles (blue arrows) point toward a “wrapped” version of gNB 40 (not shown). Right: when wraparound is disabled, the UE LoS arrival angles point toward the non-wrapped version of gNB 40.

## Derivation of cell-level metrics

For a given UE-gNB link, given measured RSRPs across all Tx-Rx beam pairs and polarizations, the cell-level metric for RRM prediction is derived from Tx-Rx beam pair RSRPs via the following procedure:

- RSRPs are summed across Rx polarizations, and the maximum RSRP is taken across Tx polarizations.

- For k gNBs, the cell level metric Mn is calculated as

Mn=m1,n,m2,n,…,mk,n, where

- mi,n=1Nbj=1Nbri,j,n,

- ri,j,n is the RSRP of the jth top SSB for the ith sector at measurement instance n.

By taking the maximum RSRP across Rx beams, we assume a genie Rx beam selection algorithm that provides an upper-bound approximation for a realizable Rx beam selection algorithm.

The number of top beams averaged for this metric Nb is chosen to be Nb=3 for FR2 simulations and Nb=1 for FR1 simulations.

The resulting metrics are filtered in time via the Layer 3 filtering procedure outlined in 3GPP TS-38.331 [7], Section 5.5.3.2:

F0=M0

Fn=1-aFn-1+aMn, where

- Fn is the filtered cell metric result at measurement instance n,

- a=12k4, where k is the filter coefficient for the corresponding measurement quantity.

For the simulations discussed in this document, a filtering coefficient of k=4 is used to filter cell-level metrics. Figure 8 shows the effect of L3 filtering on calculated cell-level metrics for a given UE trajectory and 3 co-located gNBs.

Figure 8 Cell-level metrics with and without L3 filtering, k=4

# AI/ML methodology

In this section, we detail the AI/ML methodology used in Case A and Case B intra-cell, inter-cell temporal domain RRM prediction.

### Temporal domain Case A

In the RRM cell-level metric Case A prediction problem, given measured and filtered cell-level metrics at a set of time instances within a sliding observation window (OW), the model predicts future cell-level metric values at time instances within a sliding prediction window (PW). Figure 9 visualizes the observation/prediction windows in the Case A prediction problem.

Figure 9 Illustration of Case A prediction

Given the above setup, the Case A prediction problem can be stated as follows: given past cell metric measurements taken at all beam management cycles within the OW, predict cell level metrics for the next y prediction cycles within the PW.

### Temporal domain Case B

For the RRM cell-level metric Case B prediction problem, we consider a repeating measurement pattern in which cell-level metrics are measured at x contiguous time instances followed by y contiguous time instances at which no measurements are taken, referred to as an “MxPy” measurement pattern. For given values of x and y, the corresponding measurement reduction rate can be calculated as yx+y.

Figure 10 visualizes cell-level metric prediction within an MxPy beam measurement pattern framework.

Figure 10 Illustration of Case B prediction within an MxPy measurement pattern framework

In this setting, the observation window (OW) can be defined as the period of time from the present stretching into the past containing all measurements used by the predictor, and the prediction window (PW) can be defined as the period of time containing the next y predicted beam management cycles.

Given the above setup, the Case B prediction problem can be stated as follows: given past cell metric measurements taken at measured beam management cycles within the OW, predict cell level metrics for the next y prediction cycles within the PW.

### Predictor performance evaluation

To evaluate predictor performance, we compute the following metrics:

- Average L3 cell level RSRP difference (dB): the mean absolute error in dB between the predicted and measured cell-metric values.

- Average L3 cell level RSRP difference at prediction window edge (dB): the mean absolute error in dB between the predicted and measured cell-metric values for the final predicted instance in the prediction window (the prediction window “edge”).

- Top cell prediction accuracy across prediction window (%): the average prediction rate accuracy for the cell with the top RSRP across all predicted instances in the prediction window.

- Top cell prediction accuracy at prediction window edge (%): the average prediction rate accuracy for the cell with the top RSP for the final predicted instance in the prediction window (the prediction window “edge”).

For a non-AI/ML baseline, we compare AI/ML prediction results to prediction results from a sample-and-hold prediction scheme, where the latest measured cell-level metrics are used as predictions for future metric values.

# Simulation Evaluation for Model Generalization over UE speeds

In this section, we provide simulation evaluation results for model generalization over UE speeds for both FR1 Case B and FR2 Case A intra-frequency, inter-cell temporal domain RRM prediction.

## FR1 intra-frequency, inter-cell temporal domain Case B

In this section, we discuss the details of the simulation assumptions, parameters, and methodology for FR1 intra-frequency, temporal domain Case B model generalization over UE speeds and discuss the simulation results.

### Trajectory generation

For FR1 intra-frequency, temporal domain Case B model generalization dataset generation, we use the trajectory generation formulation outlined in Section 2.1 of this document. Table 4 lists the parameters used in this case.

Table 4: UE trajectory generation parameters for FR1 intra-frequency, intra-cell temporal Case B model generalization over UE speeds

| Parameter | Value |
| --- | --- |
| tstep | 1 s |
| tgran | 0.01 s |
| tmax | 160 s |
| s | 30, 60, 90 km/h |
| dNb | 35 m |
| ISD | 500 m |

For this dataset, UE trajectories span all 21 geometric sectors corresponding to each gNB in the 2-tier model.

### Summary of simulation assumptions

Table 5 summarizes FR2 simulation assumptions and parameters.

Table 5: Simulation assumptions for FR1 intra-frequency, intra-cell, temporal domain case B

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 @ 4 GHz; SCS: 30 kHz |
| Deployment | 500m ISD, 7 site, 3 sectors/cells per site |
| Channel model | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. Fast fading is considered for all the RRM sub-cases. Do not consider Oxygen absorption (7.6.1), Time-varying Doppler shift (7.6.6), Explicit ground reflection model (7.6.8), and blockage (7.6.4), for channel modelling (38.901). Soft LoS and Shadow Fading aspects of Channel modelling are discussed in the sections above. |
| System BW | 20 MHz |
| UE Speed | 30 km/h, 60 km/h, 90 km/h |
| UE distribution | 100% outdoor. UEs are randomly dropped within a cell with uniform distribution. |
| BS Antenna Configuration | 1 TX antenna per panel 1 panel per sector |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi |
| UE Antenna Configuration | 1 RX antenna per panel 2 panels |
| UE Antenna radiation pattern | Omni-direction |
| BS Tx Power | 44dBm |
| Maximum UE Tx Power | 23dbm |
| BS receiver Noise Figure | 5db |
| UE receiver Noise Figure | 9dB |
| Inter site distance | 500m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5m |
| Spatial consistency | Procedure A in TR38.901 |
| UE trajectory model | Based on Option 2 in TR 38.343 [3], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility. Refer to section 2.1.1 for a detailed discussion of the trajectory model. |
| L1 sampling period | 40ms |
| Prediction windows | 400ms |

### Simulation dataset evaluation

For FR1 intra-frequency, temporal domain Case B model generalization, we have generated 3 datasets in which mobile UEs travel at 30 km/h, 60 km/h, and 90 km/h, respectively. Table 6 provides statistics for the trajectories within the datasets; Figure 11 shows CDFs of the trajectory lengths.

Table 6: FR1 model generalization dataset statistics

| Parameter | Dataset @30 km/h | Dataset @60 km/h | Dataset @90 km/h |
| --- | --- | --- | --- |
| # of trajectories (total) | 1205 | 2436 | 2854 |
| Total data time (hr.) | 27.52 | 29.8 | 21.48 |
| Training/test split | 964/241 | 1948/488 | 2283/571 |
| Mean trajectory length (s) | 82.21 | 44.1 | 27.1 |
| Median trajectory length (s) | 73.08 | 29.72 | 19.6 |
| Min. trajectory length (s) | 0.08 | 0.04 | 0.04 |
| Max. trajectory length (s) | 159.96 | 159.96 | 159.96 |

Figure 11 Empirical CDFs of FR1 model generalization dataset trajectory lengths. Left: 30 km/h dataset. Center: 60 km/h dataset. Right: 90 km/h dataset.

To evaluate the generated dataset for FR1 model generalization, in Figure 12 we plot the cell level metrics for each sector as a function of UE position across the dataset for two example gNBs (22 and 25). These plots demonstrate the impacts of UE-gNB distance, gNB panel azimuthal pointing angle, and soft LoS state on the cell-level metric values.

Figure 12 Cell-level metrics for each cell as functions of UE position, FR1 intra-frequency, inter-cell temporal case B dataset. Left: cell metric data for gNB 22. Right: cell metric data for gNB 25.

### AI/ML model and methodology

For FR1 model generalization studies, we consider the Case B prediction problem outlined in Section 3.1.2. We consider an M1P9 measurement pattern (90% measurement reduction rate.)

Our AI/ML predictors are encoder-only transformer models: one trained individually on each of the 30, 60, and 90 km/h training datasets, and one trained on all three together. We compare performance of each of these four models on the 30, 60 and 90 km/h test datasets.

We evaluate these predictors on the set of RRM prediction metrics described in Section 3.1.3.

### Simulation results

Table 7 summarizes simulation assumptions for FR1 intra-frequency, inter-cell, temporal domain Case B prediction model generalization across UE speeds, and Table 8 provides simulation results.

Table 7: Simulation assumptions, FR1 Case B, model generalization across UE speeds

| Report Parameters |  | Qualcomm |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option | Option 2 in TR 38.343 [3], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility |
|  | UE trajectory boundary processing option | Trajectory termination |
|  | UE speed (30, 60, 90, 120 km/h) | 30 km/h, 60 km/h, 90 km/h |
|  | Whether soft LoS is modelled or not | Yes |
|  | Spatial consistency option (A or B) | A |
|  | Number of Tx beams | 1 |
|  | Number of Rx beams | 1 |
|  | Measurement reduction rate | 90% |
|  | Observation window | <=1280 ms |
|  | Prediction window | 400 ms |
| Data size (number of samples) | Training/validity | 771/193 (30 km/h), 1588/390 (60 km/h), 1826/457 (90 km/h) |
|  | Testing | 241 (30 km/h), 488 (60 km/h), 571 (90 km/h) |
| AI/ML model input/output | Model input | Past measured L3-filtered cell-level metric values |
|  | Model output | Prediction for future L3-filtered cell metric values |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer, …) | Transformer |
|  | Model complexity in number of parameters | 1279K |
|  | Model complexity in model size (e.g., Mbyte) | 9.8 Mb |

Table 8: Simulation results, FR1 Case B, model generalization across UE speeds

| Average L3 cell level RSRP difference across prediction window (dB) | Predictor | Inference on 30 km/h | Inference on 60 km/h | Inference on 90 km/h |
| --- | --- | --- | --- | --- |
|  | AI/ML trained on 30 km/h dataset | 1.64 | 2.10 | 2.46 |
|  | AI/ML trained on 60 km/h dataset | 1.70 | 2.12 | 2.47 |
|  | AI/ML trained on 90 km/h dataset | 1.79 | 2.17 | 2.50 |
|  | AI/ML trained on 30, 60, and 90 km/h datasets | 1.65 | 2.09 | 2.44 |
|  | Baseline (sample-and-hold) | 1.63 | 2.10 | 2.48 |
| Average L3 cell level RSRP difference at prediction window edge (dB) | Predictor | Inference on 30 km/h | Inference on 60 km/h | Inference on 90 km/h |
|  | AI/ML trained on 30 km/h dataset | 2.22 | 2.94 | 3.48 |
|  | AI/ML trained on 60 km/h dataset | 2.33 | 2.97 | 3.47 |
|  | AI/ML trained on 90 km/h dataset | 2.47 | 3.05 | 3.51 |
|  | AI/ML trained on 30, 60, and 90 km/h datasets | 2.24 | 2.92 | 3.43 |
|  | Baseline (sample-and-hold) | 2.20 | 2.95 | 3.52 |
| Top cell prediction accuracy across prediction window (%) | Predictor | Inference on 30 km/h | Inference on 60 km/h | Inference on 90 km/h |
|  | AI/ML trained on 30 km/h dataset | 85.97 | 83.22 | 80.65 |
|  | AI/ML trained on 60 km/h dataset | 85.65 | 83.22 | 80.79 |
|  | AI/ML trained on 90 km/h dataset | 85.12 | 82.86 | 80.65 |
|  | AI/ML trained on 30, 60, and 90 km/h datasets | 86.01 | 83.38 | 80.91 |
|  | Baseline (sample-and-hold) | 85.94 | 83.08 | 80.39 |
| Top cell prediction accuracy at prediction window edge (%) | Predictor | Inference on 30 km/h | Inference on 60 km/h | Inference on 90 km/h |
|  | AI/ML trained on 30 km/h dataset | 82.26 | 78.07 | 74.55 |
|  | AI/ML trained on 60 km/h dataset | 81.72 | 78.05 | 74.88 |
|  | AI/ML trained on 90 km/h dataset | 80.91 | 77.47 | 74.73 |
|  | AI/ML trained on 30, 60, and 90 km/h datasets | 82.31 | 78.38 | 75.04 |
|  | Baseline (sample-and-hold) | 82.22 | 77.81 | 74.10 |

Figure 13 shows the top average L3 cell-level RSRP difference for each predictor, both across the prediction window and at the prediction window edge, for the 30 km/h, 60 km/h, and 90 km/h test datasets. Similarly, Figure 14 shows the top cell prediction accuracy for each predictor, both across the prediction window and at the prediction window edge, for the 30 km/h, 60 km/h, and 90 km/h test datasets.

Figure 13 L3-cell level RSRP differences for FR1 Case B model generalization across UE speeds. Left: RSRP difference across prediction window. Right: RSRP difference at prediction window edge.

Figure 14 Top cell prediction accuracies for FR1 Case B model generalization across UE speeds. Left: accuracy across prediction window. Right: accuracy at prediction window edge.

We note that, as with the 3-cell FR1 intra-frequency temporal cell metric prediction Case B results reported in Section 4.6 of [8], the sample-and-hold baseline is roughly competitive with the AI/ML predictors in terms of top cell prediction accuracy, with all predictors demonstrating high accuracy for a 400-ms observation window. Comparing AI/ML to baseline performance across UE speeds, AI/ML achieves an increasing advantage over sample-and-hold prediction as UE speed increases, with the best AI/ML predictor achieving a 0.94% increase over sample-and-hold for top-cell prediction accuracy at prediction window edge for the 90 km/h UE test dataset.

All predictors display similar prediction performance for all three test datasets, demonstrating that an AI/ML cell-metric predictor trained on a certain UE speed has similar capability in predicting cell metrics for UEs at a different speed as a predictor that has been specifically trained for UEs at that different speed; however, we note a modest improvement in top cell prediction % accuracy for the cell-metric predictor trained on all three training datasets achieves a modest improvement in top cell prediction accuracy over the predictors trained on individual speed training datasets in the case the 90 km/h test dataset. Finally, we note that predictor performance shows a negative relationship with UE speed in inference, demonstrating that cell-metric prediction becomes increasingly difficult with increasing UE speed.

Proposal 1: Adopt the following Observation: In the case of FR1 Case B model generalization over UE speeds, AI/ML achieves an increasing advantage over sample-and-hold prediction for increasing UE speeds.

Observation 1: As observed by other companies and concluded by RAN2, in the case of FR1 Case B model generalization over UE speeds, an AI/ML cell-metric predictor trained on a dataset with UEs moving at 60 km/h has similar prediction performance for a 90 km/h test dataset as a cell-metric predictor trained on a dataset with UEs moving at 90 km/h (and vice versa), demonstrating an ability for cell-metric predictors to generalize across UE speeds.

Observation 2: As observed by other companies and concluded by RAN2, in the case of FR1 Case B model generalization over UE speeds, performance for 90-km/h UEs is worse than that for 60-km/h UEs, demonstrating that cell-metric prediction becomes increasingly difficult with increasing UE speed.

Observation 3: As observed by other companies and concluded by RAN2, in the case of FR1 Case B model generalization over UE speeds, a modest improvement in top-1 % accuracy is achieved through training on 30, 60, and 90 km/h datasets together in comparison to training on these datasets individually.

## FR2 intra-frequency, inter-cell temporal domain Case A

In this section, we discuss the details of the simulation assumptions, parameters, and methodology for FR2 intra-frequency, temporal domain Case A model generalization over UE speeds and discuss the simulation results.

### Trajectory generation

For FR2 intra-frequency, temporal domain Case A model generalization dataset generation, we use the trajectory generation formulation outlined in Section 2.1 of this document. Table 9 lists the parameters used in this case.

Table 9: UE trajectory generation parameters for FR2 intra-frequency, intra-cell temporal Case A model generalization over UE speeds

| Parameter | Value |
| --- | --- |
| tstep | 1 s |
| tgran | 0.01 s |
| tmax | 160 s |
| s | 30, 60, 90 km/h |
| dNb | 10 m |
| ISD | 200 m |

For this dataset, UE trajectories span all 21 geometric sectors corresponding to each gNB in the 2-tier model.

### Summary of simulation assumptions

Table 10 summarizes FR2 simulation assumptions and parameters.

Table 10: Simulation assumptions for FR2 intra-frequency, intra-cell, temporal domain Case A

| Parameter | Value |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz; SCS: 120 kHz |
| Deployment | 200m ISD, 7 site, 3 sectors/cells per site |
| Channel model | UMi with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. Fast fading is considered for all the RRM sub-cases. Do not consider Oxygen absorption (7.6.1), Time-varying Doppler shift (7.6.6), Explicit ground reflection model (7.6.8), and blockage (7.6.4), for channel modelling (38.901). Soft LoS and Shadow Fading aspects of Channel modelling are discussed in the sections above. |
| System BW | 80 MHz |
| UE Speed | 30 km/h, 60 km/h, 90 km/h |
| UE distribution | 100% outdoor. UEs are randomly dropped within a cell with uniform distribution. |
| BS Antenna Configuration | Antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ. Number of BS beams: 8 SSB beams per sector Beam pattern: 8 azi. beams with 22.5-deg. separation |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6. |
| UE Antenna Configuration | Antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Number of UE beams: 4 Rx per UE panel Beam pattern: 4 beams with 45-deg. separation |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8. |
| BS Tx Power | 40dBm |
| Maximum UE Tx Power | 23dBm |
| BS receiver Noise Figure | 7dB |
| UE receiver Noise Figure | 10dB |
| Inter site distance | 200m |
| BS Antenna height | 10m |
| UE Antenna height | 1.5m |
| Spatial consistency | Procedure A in TR38.901 |
| UE trajectory model | Based on Option 2 in TR 38.343 [3], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility. Refer to section 2.1.1 for a detailed discussion of the trajectory model. |
| L1 sampling period | 20ms |
| Prediction window | 400ms |

### Simulation dataset evaluation

For FR2 intra-frequency, temporal domain Case A model generalization, we have generated 3 datasets in which mobile UEs travel at 30 km/h, 60 km/h, and 90 km/h, respectively. Table 11 provides statistics for the trajectories within the datasets; Figure 15 shows CDFs of the trajectory lengths.

Table 11: FR2 model generalization dataset statistics

| Parameter | Dataset @30 km/h | Dataset @60 km/h | Dataset @90 km/h |
| --- | --- | --- | --- |
| # of trajectories (total) | 2589 | 4051 | 3717 |
| Total data time (hr.) | 26.04 | 17.41 | 9.71 |
| Training/test split | 2071/518 | 3240/811 | 2973/744 |
| Mean trajectory length (s) | 36.21 | 15.47 | 9.4 |
| Median trajectory length (s) | 24.22 | 11.72 | 7.3 |
| Min. trajectory length (s) | 0.02 | 0.02 | 0.02 |
| Max. trajectory length (s) | 159.98 | 113.5 | 60.28 |

Figure 15 Empirical CDFs of FR2 model generalization dataset trajectory lengths. Left: 30 km/h dataset. Center: 60 km/h dataset. Right: 90 km/h dataset.

To evaluate the generated dataset for FR1 model generalization, in Figure 16 we plot the cell level metrics for each sector as a function of UE position across the dataset for two example gNBs (22 and 25). These plots demonstrate the impacts of UE-gNB distance, gNB panel azimuthal pointing angle, and soft LoS state on the cell-level metric values.

Figure 16 Cell-level metrics for each cell as functions of UE position, FR2 intra-frequency, inter-cell temporal case A dataset. Left: cell metric data for gNB 22. Right: cell metric data for gNB 25.

### AI/ML model and methodology

For FR2 model generalization studies, we consider the Case A prediction problem outlined in Section 3.1.1, with a PW length of 400 ms.

Our AI/ML predictors are encoder-only transformer models: one trained individually on each of the 30, 60, and 90 km/h training datasets, and one trained on all three together. We compare performance of each of these four models on the 30, 60 and 90 km/h test datasets.

We evaluate these predictors on the set of RRM prediction metrics described in Section 3.1.3.

### Simulation results

Table 12 summarizes simulation assumptions for FR1 intra-frequency, inter-cell, temporal domain Case B prediction model generalization across UE speeds, and Table 13 provides simulation results.

Table 12: Simulation assumptions, FR2 Case A, model generalization across UE speeds

| Report Parameters |  | Qualcomm |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option | Option 2 in TR 38.343 [3], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility |
|  | UE trajectory boundary processing option | Trajectory termination |
|  | UE speed (30, 60, 90, 120 km/h) | 30 km/h, 60 km/h, 90 km/h |
|  | Whether soft LoS is modelled or not | Yes |
|  | Spatial consistency option (A or B) | A |
|  | Number of Tx beams | 8 SSB beams |
|  | Number of Rx beams | 4 beams per panel × 2 panels = 8 Rx beams |
|  | Measurement reduction rate | N/A |
|  | Observation window | <= 640 ms |
|  | Prediction window | 400 ms |
| Data size (number of samples) | Training/validity | 1657/414 (30 km/h), 2592/648 (60 km/h), 2378/595 (90 km/h) |
|  | Testing | 518 (30 km/h), 811 (60 km/h), 744 (90 km/h) |
| AI/ML model input/output | Model input | Past measured L3-filtered cell-level metric values |
|  | Model output | Prediction for future L3-filtered cell metric values |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer, …) | Transformer |
|  | Model complexity in number of parameters | 1280k |
|  | Model complexity in model size (e.g., Mbyte) | 9.77 Mb |

Table 13: Simulation results, FR2 Case A, model generalization across UE speeds

| Average L3 cell level RSRP difference across prediction window (dB) | Predictor | Inference on 30 km/h | Inference on 60 km/h | Inference on 90 km/h |
| --- | --- | --- | --- | --- |
|  | AI/ML trained on 30 km/h dataset | 2.67 | 3.53 | 4.15 |
|  | AI/ML trained on 60 km/h dataset | 2.78 | 3.54 | 4.10 |
|  | AI/ML trained on 90 km/h dataset | 2.94 | 3.63 | 4.14 |
|  | AI/ML trained on 30, 60, and 90 km/h datasets | 2.69 | 3.52 | 4.11 |
|  | Baseline (sample-and-hold) | 2.67 | 3.60 | 4.25 |
| Average L3 cell level RSRP difference at prediction window edge (dB) | Predictor | Inference on 30 km/h | Inference on 60 km/h | Inference on 90 km/h |
|  | AI/ML trained on 30 km/h dataset | 3.90 | 5.09 | 5.92 |
|  | AI/ML trained on 60 km/h dataset | 4.05 | 5.06 | 5.79 |
|  | AI/ML trained on 90 km/h dataset | 4.29 | 5.18 | 5.83 |
|  | AI/ML trained on 30, 60, and 90 km/h datasets | 3.92 | 5.05 | 5.84 |
|  | Baseline (sample-and-hold) | 3.94 | 5.25 | 6.15 |
| Top cell prediction accuracy across prediction window (%) | Predictor | Inference on 30 km/h | Inference on 60 km/h | Inference on 90 km/h |
|  | AI/ML trained on 30 km/h dataset | 83.75 | 78.88 | 76.07 |
|  | AI/ML trained on 60 km/h dataset | 83.46 | 79.01 | 76.25 |
|  | AI/ML trained on 90 km/h dataset | 83.20 | 78.85 | 76.23 |
|  | AI/ML trained on 30, 60, and 90 km/h datasets | 83.73 | 78.94 | 76.20 |
|  | Baseline (sample-and-hold) | 83.63 | 78.57 | 75.72 |
| Top cell prediction accuracy across prediction window (%) | Predictor | Inference on 30 km/h | Inference on 60 km/h | Inference on 90 km/h |
|  | AI/ML trained on 30 km/h dataset | 78.11 | 71.30 | 66.88 |
|  | AI/ML trained on 60 km/h dataset | 77.75 | 71.52 | 67.29 |
|  | AI/ML trained on 90 km/h dataset | 77.42 | 71.47 | 67.41 |
|  | AI/ML trained on 30, 60, and 90 km/h datasets | 78.14 | 71.38 | 67.17 |
|  | Baseline (sample-and-hold) | 77.91 | 70.63 | 66.25 |

Figure 17 shows the RSRP differences for FR2 Case A prediction across the prediction window and at the prediction window edge (400 ms), and Figure 18 shows the top cell prediction accuracy.

Figure 17 L3-cell level RSRP differences for FR2 Case A model generalization across UE speeds. Left: RSRP difference across prediction window. Right: RSRP difference at prediction window edge.

Figure 18 Top cell prediction accuracies for FR2 Case A model generalization across UE speeds. Left: accuracy across prediction window. Right: accuracy at prediction window edge.

We note that AI/ML achieves a modest but increasing advantage over baseline as UE speeds increase. We also note that, as with FR1 Case B, predictor performance shows a negative relationship with UE speed in the test dataset.

In comparison to FR1 Case B, we see increased differentiation between performance of AI/ML predictors trained on different UE speed datasets. In particular, we note that in terms of L3-cell level RSRP differences, the predictor trained on the 90 km/h training dataset performs poorly on the lower speed test datasets, whereas the predictors trained on lower speed training datasets do not suffer comparable performance degradation on the 90 km/h test dataset. This trend possibly indicates that predictors only trained on high-speed datasets may generalize more poorly than predictors trained on low-speed datasets.

As with FR1 Case B, the predictor trained on all three training datasets performs well in terms of top cell prediction accuracy for all three test datasets, demonstrating that training on heterogeneous datasets can lead to an improvement in predictor performance.

Proposal 2: Adopt the following Observation: In the case of FR2 Case A model generalization over UE speeds, AI/ML achieves an increasing advantage over sample and hold as UE speed in the test dataset increases.

Observation 4: As observed by other companies and concluded by RAN2, in the case of FR2 Case A model generalization over UE speeds, predictor performance shows a negative relationship with UE speed, demonstrating that cell metric prediction is more difficult for rapidly moving UEs.

Observation 5: As observed by other companies and concluded by RAN2, in the case of FR2 Case A model generalization over UE speeds, a predictor trained on datasets with multiple UE speeds shows competitive performance in comparison predictors trained on a single speed, demonstrating the advantage of a heterogeneous training dataset.

# Simulation Evaluation for Model Generalization across scenarios

In this section we discuss simulation evaluation for model generalization across UMa and UMi scenarios.

## FR1 intra-frequency, inter-cell temporal domain Case B

### Trajectory generation

For FR1 intra-frequency, temporal domain Case B dataset generation, we use the trajectory generation formulation outlined in Section 2.1 of this document. (In offline discussion, it was agreed to use UE speeds of 30 km/h for the FR1 case and 60 km/h in the FR2 case.) Table 14 lists the parameters used in the FR1 case.

Table 14: UE trajectory generation parameters, FR1 Case B, model generalization over scenarios

| Parameter | Value |
| --- | --- |
| tstep | 1 s |
| tgran | 0.01 s |
| tmax | 160 s |
| s | 30 km/h |
| dNb | 10 m |
| ISD | 200 m |

For this dataset, UE trajectories span all 21 geometric sectors corresponding to each gNB in the 2-tier model.

### Summary of simulation assumptions

Table 15 summarizes simulation assumptions and parameters for FR1 model generalization across scenarios. Note that for both Configuration #1 (UMi) and Configuration #2 (UMa), we have re-used all existing parameters for UMa FR1 RRM datasets aside from the agreed parameter differences listed in Table 3.

Table 15: Simulation assumptions, FR1 Case B, model generalization over scenarios

| Parameter | Value |
| --- | --- |
| Frequency Range | FR1 @ 4 GHz; SCS: 30 kHz |
| Deployment | 200m ISD (UMi), 500m ISD (UMa), 7 site, 3 sectors/cells per site |
| Channel model | UMi, UMa, with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. Fast fading is considered for all the RRM sub-cases. Do not consider Oxygen absorption (7.6.1), Time-varying Doppler shift (7.6.6), Explicit ground reflection model (7.6.8), and blockage (7.6.4), for channel modelling (38.901). Soft LoS and Shadow Fading aspects of Channel modelling are discussed in the sections above. |
| System BW | 20 MHz |
| UE Speed | 30 km/h |
| UE distribution | 100% outdoor. UEs are randomly dropped within a cell with uniform distribution. |
| BS Antenna Configuration | 1 TX antenna per panel 1 panel per sector |
| BS Antenna radiation pattern | 3-sector antenna radiation pattern, 8 dBi |
| UE Antenna Configuration | 1 RX antenna per panel 2 panels |
| UE Antenna radiation pattern | Omni-direction |
| BS Tx Power | 44dBm |
| Maximum UE Tx Power | 23dbm |
| BS receiver Noise Figure | 5db |
| UE receiver Noise Figure | 9dB |
| Inter site distance | 200m (UMi), 500m (UMa) |
| BS Antenna height | 10m (UMi), 25m (UMa) |
| UE Antenna height | 1.5m |
| Spatial consistency | Procedure A in TR38.901 |
| UE trajectory model | Based on Option 2 in TR 38.343 [3], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility. Refer to section 2.1.1 for a detailed discussion of the trajectory model. |
| L1 sampling period | 40ms |
| Prediction windows | 200 ms (M1P4), 400 ms (M1P9) |

### Simulation dataset evaluation

For FR1 Case B scenario generalization, we have generated two datasets corresponding to Configurations #1 and #2. Table 16 provides statistics for the trajectories within the datasets; Figure 19 shows CDFs of the trajectory lengths.

Table 16: Dataset statistics, FR1 Case B, model generalization over scenarios

| Parameter | Configuration # 1 Dataset (UMi) | Configuration # 2 Dataset (UMa) |
| --- | --- | --- |
| # of trajectories (total) | 1531 | 1205 |
| Total data time (hr.) | 15.36 | 27.52 |
| Training/test split | 1223/308 | 964/241 |
| Mean trajectory length (s) | 36.12 | 82.21 |
| Median trajectory length (s) | 25.28 | 73.08 |
| Min. trajectory length (s) | 0.04 | 0.08 |
| Max. trajectory length (s) | 159.96 | 159.96 |

Figure 19 Empirical CDFs of dataset trajectory lengths for FR1 model generalization over scenarios. Left: UMi dataset. Right: UMa dataset.

To evaluate the generated dataset for FR1 model generalization over scenarios, in Figure 20 we plot the cell level metrics for each sector as a function of UE position across the dataset for two example gNBs (22 and 25) for both the Configuration #1 and Configuration #2. These plots demonstrate the impacts of UE-gNB distance, gNB panel azimuthal pointing angle, and soft LoS state on the cell-level metric values.

Figure 20 Cell-level metrics for each cell as functions of UE position, FR1 datasets for scenario generalization. Top left: cell metric data for gNB 22, Configuration #1. Top right: cell metric data for gNB 25, Configuration #1. Bottom left: cell metric data for gNB 22, Configuration #2. Bottom right: cell metric data for gNB 25, Configuration #2.

### AI/ML model and methodology

For FR1 scenario generalization studies, we consider the Case B prediction problem outlined in Section 3.1.2. We consider measurement patterns of M1P4 and M1P9.

Our AI/ML predictors are encoder-only transformer models. For each measurement pattern, we have trained three models: one trained individually on the UMi training dataset, one trained individually on the UMa training dataset, and one trained on both together. To evaluation the ability of AI/ML models to generalize across scenarios, we compare performance of each of these three models on both the UMi and UMa test datasets.

We evaluate these predictors on the set of RRM prediction metrics described in Section 3.1.3.

### Simulation results

Table 17 summarizes simulation assumptions for FR1 Case B, Model Generalization across scenarios. Table 18 provides simulation results for M1P4, and Table 19 provides simulation results for M1P9.

Table 17: Summary of simulation assumptions, FR1 Case B, Model Generalization across scenarios

| Report Parameters |  | Qualcomm |
| --- | --- | --- |
| Reported simulation assumptions | UE trajectory option | Option 2 in TR 38.343 [3], Section 6.3.1, with modifications based on Option 4 and a minor modification for extended mobility |
|  | UE trajectory boundary processing option | Trajectory termination |
|  | UE speed (30, 60, 90, 120 km/h) | 30 km/h |
|  | Whether soft LoS is modelled or not | Yes |
|  | Spatial consistency option (A or B) | A |
|  | Number of Tx beams | 1 |
|  | Number of Rx beams | 1 |
|  | Measurement reduction rate | 80% (M1P4), 90% (M1P9) |
|  | Observation window | <=3200 ms (M1P4), <=6400 ms (M1P9) |
|  | Prediction window | 200 ms (M1P4), 400 ms (M1P9) |
| Data size (number of samples) | Training/validity | 978/245 (Configuration #1), 771/193 (Configuration #2) |
|  | Testing | 308 (Configuration #1), 241 (Configuration #2) |
| AI/ML model input/output | Model input | Past measured L3-filtered cell-level metric values |
|  | Model output | Prediction for future L3-filtered cell metric values |
| AI/ML model description | Model type (e.g., LSTM, CNN, transformer, …) | Transformer |
|  | Model complexity in number of parameters | 1275k |
|  | Model complexity in model size (e.g., Mbyte) | 9.8 Mb |

Table 18: Simulation results, FR1 Case B, model generalization across scenarios (M1P4)

| Average L3 cell level RSRP difference across prediction window (dB) | Predictor | Inference on UMi dataset | Inference on UMa dataset |
| --- | --- | --- | --- |
|  | AI/ML trained on UMi dataset | 1.83 | 1.27 |
|  | AI/ML trained on UMa dataset | 1.82 | 1.21 |
|  | AI/ML trained on UMi and UMa datasets | 1.83 | 1.21 |
|  | Baseline (sample-and-hold) | 1.83 | 1.20 |
| Average L3 cell level RSRP difference at prediction window edge (dB) | Predictor | Inference on UMi dataset | Inference on UMa dataset |
|  | AI/ML trained on UMi dataset | 2.46 | 1.61 |
|  | AI/ML trained on UMa dataset | 2.46 | 1.53 |
|  | AI/ML trained on UMi and UMa datasets | 2.46 | 1.53 |
|  | Baseline (sample-and-hold) | 2.46 | 1.53 |
| Top cell prediction accuracy across prediction window (%) | Predictor | Inference on UMi dataset | Inference on UMa dataset |
|  | AI/ML trained on UMi dataset | 86.90 | 89.30 |
|  | AI/ML trained on UMa dataset | 86.87 | 89.37 |
|  | AI/ML trained on UMi and UMa datasets | 86.88 | 89.37 |
|  | Baseline (sample-and-hold) | 86.87 | 89.37 |
| Top cell prediction accuracy at prediction window edge (%) | Predictor | Inference on UMi dataset | Inference on UMa dataset |
|  | AI/ML trained on UMi dataset | 85.18 | 88.02 |
|  | AI/ML trained on UMa dataset | 85.14 | 88.12 |
|  | AI/ML trained on UMi and UMa datasets | 85.16 | 88.10 |
|  | Baseline (sample-and-hold) | 85.16 | 88.10 |

Figure 21 shows the RSRP errors for FR1 Case B model generalization across scenarios with a measurement pattern of M1P4, and Figure 22 shows the top cell prediction accuracy.

Figure 21 L3-cell level RSRP differences for FR1 Case B model generalization across scenarios, M1P4. Left: RSRP difference across prediction window. Right: RSRP difference at prediction window edge.

Figure 22 Top cell prediction accuracy for FR1 Case B model generalization across scenarios, M1P4. Left: top cell prediction accuracy across prediction window. Right: top cell prediction accuracy at prediction window edge.

Table 19: Simulation results, FR1 Case B, model generalization across scenarios (M1P9)

| Average L3 cell level RSRP difference across prediction window (dB) | Predictor | Inference on UMi dataset | Inference on UMa dataset |
| --- | --- | --- | --- |
|  | AI/ML trained on UMi dataset | 2.65 | 1.90 |
|  | AI/ML trained on UMa dataset | 2.69 | 1.64 |
|  | AI/ML trained on UMi and UMa datasets | 2.65 | 1.64 |
|  | Baseline (sample-and-hold) | 2.68 | 1.63 |
| Average L3 cell level RSRP difference at prediction window edge (dB) | Predictor | Inference on UMi dataset | Inference on UMa dataset |
|  | AI/ML trained on UMi dataset | 3.75 | 2.66 |
|  | AI/ML trained on UMa dataset | 3.84 | 2.22 |
|  | AI/ML trained on UMi and UMa datasets | 3.74 | 2.23 |
|  | Baseline (sample-and-hold) | 3.82 | 2.20 |
| Top cell prediction accuracy across prediction window (%) | Predictor | Inference on UMi dataset | Inference on UMa dataset |
|  | AI/ML trained on UMi dataset | 82.41 | 85.69 |
|  | AI/ML trained on UMa dataset | 82.17 | 85.98 |
|  | AI/ML trained on UMi and UMa datasets | 82.43 | 85.98 |
|  | Baseline (sample-and-hold) | 82.25 | 85.94 |
| Top cell prediction accuracy at prediction window edge (%) | Predictor | Inference on UMi dataset | Inference on UMa dataset |
|  | AI/ML trained on UMi dataset | 77.70 | 81.78 |
|  | AI/ML trained on UMa dataset | 77.28 | 82.29 |
|  | AI/ML trained on UMi and UMa datasets | 77.77 | 82.25 |
|  | Baseline (sample-and-hold) | 77.35 | 82.22 |

Figure 23 shows the RSRP errors for FR1 Case B model generalization across scenarios with a measurement pattern of M1P9, and Figure 24 shows the top cell prediction accuracy.

Figure 23 L3-cell level RSRP differences for FR1 Case B model generalization across scenarios, M1P9. Left: RSRP difference across prediction window. Right: RSRP difference at prediction window edge.

Figure 24 Top cell prediction accuracy for FR1 Case B model generalization across scenarios, M1P9. Left: top cell prediction accuracy across prediction window. Right: top cell prediction accuracy at prediction window edge.

We observe that for both M1P4 and M1P9 measurement patterns, AI/ML and baseline display superior performance on the UMa test dataset compared to the UMi test dataset. As UEs are moving at the same speed (30 km/h) in both datasets, the lower top cell accuracy in the UMi case may be due to the smaller ISD resulting in relatively rapid fluctuations in the top cell.

In all cases, we see that the sample-and-hold baseline is roughly on-par with AI/ML prediction; the best AI/ML predictor achieves a modest (~0.42%) gain in top cell prediction accuracy at prediction window edge on the UMi test dataset for M1P9. When comparing AI/ML predictors in the M1P9 case, we see that each predictor trained on an individual scenario dataset tests to slightly outperform the other individually trained predictor on its matched test dataset, while the predictor trained on both UMi and UMa datasets shows strong performance in both cases. This trend aligns with the generalization trend seen in the UE speed generalization studies in which predictors trained on heterogeneous datasets tend to show strong performance across all test datasets.

Proposal 3: Adopt the following Observation: In the case of FR1 Case B model generalization over scenarios, RRM predictor top cell prediction accuracy performance is worse in the UMi case than the UMa case, which may be due to the smaller ISDs in the UMi case.

Proposal 4: Adopt the following Observation: In the case of FR1 Case B model generalization over scenarios, AI/ML predictors trained on one scenario tend to slightly outperform predictors trained on the other scenario on their matched test datasets, while predictors trained on both datasets perform strongly in both cases.

# Conclusions

The following Observations were made in the case of FR1 Case B generalization over UE speeds:

Proposal 1: Adopt the following observation: In the case of FR1 Case B model generalization over UE speeds, AI/ML achieves an increasing advantage over sample-and-hold prediction for increasing UE speeds.

Observation 1: As observed by other companies and concluded by RAN2, in the case of FR1 Case B model generalization over UE speeds, an AI/ML cell-metric predictor trained on a dataset with UEs moving at 60 km/h has similar prediction performance for a 90 km/h test dataset as a cell-metric predictor trained on a dataset with UEs moving at 90 km/h (and vice versa), demonstrating an ability for cell-metric predictors to generalize across UE speeds.

Observation 2: As observed by other companies and concluded by RAN2, in the case of FR1 Case B model generalization over UE speeds, performance for 90-km/h UEs is worse than that for 60-km/h UEs, demonstrating that cell-metric prediction becomes increasingly difficult with increasing UE speed.

Observation 3: As observed by other companies and concluded by RAN2, in the case of FR1 Case B model generalization over UE speeds, a modest improvement in top-1 % accuracy is achieved through training on 30, 60, and 90 km/h datasets together in comparison to training on these datasets individually.

The following Observations were made in the case of FR2 Case A generalization over UE speeds:

Proposal 2: Adopt the following observation: In the case of FR2 Case A model generalization over UE speeds, AI/ML achieves an increasing advantage over sample and hold as UE speed in the test dataset increases.

Observation 4: As observed by other companies and concluded by RAN2, in the case of FR2 Case A model generalization over UE speeds, predictor performance shows a negative relationship with UE speed, demonstrating that cell metric prediction is more difficult for rapidly moving UEs.

Observation 5: As observed by other companies and concluded by RAN2, in the case of FR2 Case A model generalization over UE speeds, a predictor trained on datasets with multiple UE speeds shows competitive performance in comparison predictors trained on a single speed, demonstrating the advantage of a heterogeneous training dataset.

The following Observations were made in the case of FR1 Case B generalization over scenarios:

Proposal 3: Adopt the following Observation: In the case of FR1 Case B model generalization over scenarios, RRM predictor top cell prediction accuracy performance is worse in the UMi case than the UMa case, which may be due to the smaller ISDs in the UMi case.

Proposal 4: In the case of FR1 Case B model generalization over scenarios, AI/ML predictors trained on one scenario tend to slightly outperform predictors trained on the other scenario on their matched test datasets, while predictors trained on both datasets perform strongly in both cases.

# References

| [1] | Chair notes, RAN2 #128 meeting. |
| --- | --- |
| [2] | Chair notes, RAN2 #129 meeting. |
| [3] | 3GPP, “3GPP TR 38.843 V18.0.0: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface,” January 2024. |
| [4] | 3GPP, “3GPP RAN2 2405205: Considerations for mobile UE trajectory generation and channel modelling for simulation evaluation,” 2024. |
| [5] | 3GPP RAN Plenary, “3GPP TR 38.901 V17.0.0: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Study on channel model for frequencies from 0.5 to 100 GHz (Release 17),” January 2024. |
| [6] | IST-WINNER II Deliverable 1.1.2 v.1.2, “WINNER II Channel Models,” 2007. |
| [7] | 3. R. Plenary, “3GPP TR 38.331 V18.2.0: 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Radio Resource Control (RRC) protocol specification (Release 18),” June 2024. |
| [8] | 3GPP, “3GPP RAN2 2410774: Simulation Results for RRM Measurement Prediction,” November 2024. |