3GPP TSG-RAN WG2 #129bis	R2-2502287

Wuhan, China, April 7th – 11th, 2025

Source:		NEC

Agenda item:		8.6.4	Conditional intra-CU LTM

Title:	Remaining Open Issues for CLTM

Document for:		Discussion and decision

- Introduction

During the last RAN2 meeting, the following agreements were made for the support of CLTM [1]:

Agreements on C-LTM:

- From a given source cell, network configures either L1 execution condition or L3 execution condition for a CLTM candidate cell.

- The UE starts execution condition evaluation once it receives CLTM configuration including C-LTM execution condition.

- For conditional LTM based on L1 measurements (LTM3/5-like event), the evaluation is per candidate beam. For conditional LTM based on L3 measurements (condA3/condA5 events), the evaluation is per candidate cell (i.e., with consolidation of multiple beams of the candidate cell).

- When the C-LTM execution condition is satisfied with a candidate beam (any beam from the candidate RS set) / cell, UE performs RACH-less C-LTM (with that triggered beam for L1 based C-LTM case) if UE has a valid TA for the associated candidate cell. Otherwise, UE performs RACH-based C-LTM.

- For L1 based C-LTM (assuming single beam fulfils C-LTM execution condition), beam selection for RACH-less LTM with CG is based on the beam meets C-LTM execution condition.

- For L1 based C-LTM events, the C-LTM execution is triggered when at least one beam fulfills the C-LTM event condition. When multiple candidate beams satisfy the C-LTM condition, it is up to UE implementation to select a beam and perform C-LTM.

- Do not support DG-based first UL transmission for RACH-less conditional LTM.

- For RACH-based conditional LTM procedure, CFRA can be supported when CFRA resource is included in candidate cell configurations, otherwise CBRA is performed.

- For intra-CU CLTM, it is up to NW implementation to ensure that only the candidate cells belonging to the same CU as the current serving cell are provided with the execution conditions.

- Support the CLTM fast failure recovery, which reuses R18 LTM failure recovery mechanism (i.e. based on CBRA).

In this paper, we provide our consideration on the remaining issues of conditional intra-CU LTM.

- Discussion

During last meeting, it was agreed that beam selection for RACH-less CLTM is based on the beam which meets C-LTM execution condition. In Rel-18 LTM, the beam selected is also used to determine whether the CG resource is a valid CG resource. The CG resource validation of Rel-19 CLTM, similarity, should be also based on the selected beam. For example, if the beam which fulfills the execution condition is an SSB, the UE shall select the SSB. And if the SSB associated with a CG resource is the same as the selected SSB, the UE shall consider the CG resource as valid.

Proposal 1. If the SSB associated with a CG resource is the same as the SSB which meets the execution condition and selected by the UE, the UE shall consider the CG resource as valid.

One issue to be discussed is that CSI-RS could be applicable beam of an event, however currently the CG resource is only associated with SSBs, hence how to CG resource selection that the beam fulfilling the execution condition is a CSI-RS.

There can be two options for to handle the case that beam meets the execution condition is CSI-RS:

Option 1. The UE determine whether a CG resource is valid based on the SSB associated with the CSI-RS which fulfills the execution condition

In our understanding, this option is aligned with Rel-18 handling for RACH-less LTM. In Rel-18, the TCI state indicated in the LTM cell switch command MAC CE can be associated with SSB or CSI-RS.

Agreement of RAN1 #114: In R18 LTM, on the QCL source of the TCI state before/during the cell switch command, - SSB or TRS can be configured in a TCI state for the candidate cell(s) before/during cell switch command - Whether the TRS can be used for the candidate cell(s) before/during cell switch command is up to UE capability

And if the TCI state is associated with a CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS of the TCI state, selects the SSB and indicates to the lower layer the SSB. More specifically, the UE needs to check the corresponding NZP-CSI-RS-Resource of the CSI-RS. If the NZP-CSI-RS-Resource indicates a further TCI state of which the reference signal is an SSB, the UE shall determine whether the CG resource is valid or not based on the SSB. Otherwise, if the reference signal provided by the further TCI state is another CSI-RS, the UE needs to further look in the TCI state indicated by the another CSI-RS, and so on, until the UE finds an SSB.

TS 38.213 v18.4.0 clause 21.1:

## 21.1 Configured-grant PUSCH transmission in RACH-less LTM cell switch <skipped> If the UE is provided cg-LTM-Configuration, the UE performs configured grant Type 1 PUSCH transmission on the valid PUSCH occasions associated with the SS/PBCH block indexes same as the SS/PBCH block indexes provided by or associated with QCL RS of the CandidateTCI-State or CandidateTCI-UL-State indicated by the LTM Cell Switch Command MAC CE.

Therefore, similar as in Rel-18, if the beam fulfilling the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS which fulfills the execution condition. In other words, if the SSB associated with the CG resource is the same as the SSB associated with the CSI-RS fulfilling the execution condition, the UE shall consider the CG resource as valid, and indicate the SSB to the lower layer.

Option 2. Extend CG resource for LTM to be associated with CSI-RS

For this option, if the CSI-RS associated with the CG resource is the same as the beam fulfilling the execution condition, the UE consider the CG resource as valid, and indicate the CSI-RS to the lower layer. To support this, the CG resource configuration needs to be extended, and some additional efforts are needed to specify how to support the association between CG resource and CSI-RS, including impact to RAN2 signalling and RAN1 specification.

And since the option 1 works, and it is aligned with the handling of Rel-18 RACH-less LTM, we see no need to further enhance CG resource to be associated with CSI-RS.

Proposal 2. For the case that the beam meeting the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS, i.e., if the SSB associated with the CG resource is the same as the SSB associated with the CSI-RS meeting the execution condition, the UE considers the CG resource as valid.

During last meeting, the TA maintenance was discussed, however there are many aspects remain to be controversial, including how to handle the TA for the selected candidate cell, for PTAG and for the other candidate cells:

FFS on whether the received CLTM TAs of other candidate cells is released or still valid at UE side upon CLTM execution.  CLTM TAT for target cell is not stopped upon CLTM cell switch execution to the target cell. Rapporteur suggests to discuss how/whether to start the timeAlignmentTimer associated with PTAG upon CLTM cell switch execution to the target cell during CB session: - Option 1 (revert the offline agreement): start the timeAlignmentTimer associated with PTAG as in Rel-18; - Option 2 (keep the offline agreement): discuss the options below in the MAC running CR o Option 2.1: clarify the running TAT to act as the timeAlignmentTimer associated with PTAG by a note in the MAC running CR; o Option 2.2: define that UE can perform UL tx when the TAT for the target cell is running. Besides, we need to add a note that, if SCell and SpCell are in the different TAG, SCell may not work when this TAT for target cell is running. o Option 2.3: the UE stops the CLTM timer and starts the PTAG using the remaining time of the CLTM timer. Will be revisited next meeting.

Firstly, for TA maintenance of other candidate cells upon CLTM cell switch execution to a target cell, we think that since the UE may perform subsequent CLTM to one of the other candidate cells in the future, the TA for the other candidate cell can be used for RACH-less CLTM if the associated TAT is still running. Therefore, we think the TA of the other candidate cells can be maintained by the UE upon CLTM cell switch execution, i.e., the TA is not released and the TAT associated is kept running.

Proposal 3. The TAT of the other candidate cells is not stopped by the UE upon CLTM cell switch execution.

For TAT of the target cell, similarly as the TAT for the other candidate cell, the UE may perform a subsequent CLTM back to the target cell again. And the TA can be reused for the subsequent RACH-less CLTM to the same target cell. Therefore, we think there is no need to stop the TAT of the target cell.

Proposal 4. The TAT of the target cell is not stopped by the UE upon CLTM cell switch execution.

For the TA to use for the access to the target cell and the TA to use after access to the target cell including for SCells, companies’ view is diverse.

For Option 1 that UE starts the timeAlignmentTimer associated with PTAG upon CLTM execution, it can work but it is not good because the TAT associated with the target cell is already running for a while, and if we apply the TA of the target cell for PTAG and start the timeAlignmentTimer for PTAG, it means that the validity time of the TA is extended, and the UE may continue using a TA which is already invalid during the CLTM execution procedure which results in LTM failure.

For option 2.1 that the TAT for the target cell act as the timeAlignmentTimer associated with PTAG. In our understanding the TAT for the target cell and the timeAlignmentTimer for the PTAG are different timers, it is better not to mess them up.

For option 2.2 that the UE can perform UL transmission when the TAT for the target cell is running. Since TA/TAT for the target cell and TA/TAT for PTAG are different concepts, mixing them is not a good choice. We also need to discuss whether UE also performs UL transmission on the SCells of PTAG based on the TA/TAT of the target cell. And the network may need to transmits another Absolute TA command MAC CE or TA command MAC CE to activate TA/TAT for PTAG later after the completion of the cell switch.

For Option 2.3 the UE stops the CLTM timer and starts the PTAG using the remaining time of the CLTM timer. We think this option also works, but we usually start/restarts the timer from 0, we don’t have the behavior of starting a timer from a value other than 0 before.

Based on the above analysis, we think none of them are a perfect solution, and we come up with another alternative approach which can avoid the issues of the options above:

- Upon CLTM execution, the UE applies the TA of the target cell for PTAG, and starts TAT for PTAG. If the TA of the target cell expires during the RACH-less CLTM procedure, the UE also consider the TAT for PTAG as expired.

For example, the maximum TAT value associated with the target candidate cell is 500ms, and the maximum TAT value associated with the PTAG is 1280ms. When CLTM execution is initiated, suppose that the TAT associated with the target candidate cell is 300ms. The UE shall apply the TA associated with the target cell for PTAG, start the TAT for PTAG, and continue the running of TAT for target candidate cell. When the TAT for the target candidate cell expires during the RACH-less CLTM procedure, which is 200ms later, the UE also considers the TAT for the PTAG as expired (although it is only 200ms and not reached to 1280ms).

Proposal 5. Upon CLTM execution, the UE applies the TA of the target cell for PTAG, and starts TAT for PTAG. If the TAT of the target cell expires during the RACH-less CLTM procedure, the UE also considers the TAT for the PTAG as expired.

If the solution in proposal 5 is not agreeable, we prefer to start the PTAG using the remaining time of the TAT timer for the target cell as in Option 2.3, as this is the only option does not change the original concept of TA/TAT for candidate cell and TA/TAT for PTAG, and does not extend the TA validity time for the TA, either.

Proposal 6. If proposal 5 is not agreeable, we prefer to start the TAT for PTAG using the remaining time of the TAT timer for the target cell.

# 3. Conclusion

In this paper, we discussion on the remaining issues of RACH-less CLTM, and have the following proposals:

Proposal 1. If the SSB associated with a CG resource is the same as the SSB which meets the execution condition and selected by the UE, the UE shall consider the CG resource as valid.

Proposal 2. For the case that the beam meeting the execution condition is CSI-RS, the UE determines whether a CG resource is valid based on the SSB associated with the CSI-RS, i.e., if the SSB associated with the CG resource is the same as the SSB associated with the CSI-RS meeting the execution condition, the UE considers the CG resource as valid.

Proposal 3. The TAT of the other candidate cells is not stopped by the UE upon CLTM cell switch execution.

Proposal 4. The TAT of the target cell is not stopped by the UE upon CLTM cell switch execution.

Proposal 5. Upon CLTM execution, the UE applies the TA of the target cell for PTAG, and starts TAT for PTAG. If the TAT of the target cell expires during the RACH-less CLTM procedure, the UE also considers the TAT for the PTAG as expired.

Proposal 6. If proposal 5 is not agreeable, we prefer to start the TAT for PTAG using the remaining time of the TAT timer for the target cell.

# Reference

[1] meeting report of RAN2 #129.