3GPP TSG-RAN WG2 Meeting #129bis		     R2-2502290

Wuhan, CN: 7th Apr – 11th Apr 2025

Source:	Qualcomm Incorporated

Title:	On LCM for UE-sided Models for Beam Management

Agenda Item:	8.1.2.2 LCM for UE-sided model for Beam Management use case

Document for:	Discussion and Decision

# Introduction & Background

The Rel-18 study on AI/ML Air Interface [1] has been concluded. The study has been captured in the Technical Report TR 38.843 [2]. Based on the Rel-18 study outcome, RAN #102, a new work item on artificial intelligence/machine learning for NR air interface has been approved in [3].

This paper discusses the “LCM of UE-sided models for beam management use case” considering recent RAN1/RAN2 agreements.

# Discussion on LCM of UE-sided Models for Beam Management

## 2.1 Applicability Signaling Aspects

### 2.1.1 Signaling Detail for Inference Related Parameter Set

In the RAN2#129 meeting [4], RAN2 had the following agreements

Agreements

- Inference configuration/parameters can be signalled in step 3 and/or Inference configuration can be signalled in step 5 (i.e. option a and option b from RAN1).

- The full inference configuration is sent in CSI-ReportConfig.

- Upon receiving a full inference configuration, the UE sends the initial applicability report in RRCReconfigurationComplete. UAI can be sent to update applicability.

- FFS signaling details for option B (e.g. whether it is signaling in CSI-Report Config or otherconfig)

Note that the UE side trains the AI/ML models for one or more CSI resource/resource sets and associated IDs signaled (e.g., using CSI-ReportConfig) by the gNB during the data collection. Furthermore, the UE model may be trained to produce the prediction result based on reporting configuration or type (e.g., predict a fixed number of top beams, a fixed number of prediction instances, or prediction at a fixed periodicity). To determine the applicability, the UE needs to check whether the UE has trained models for RS configurations (i.e., [set A, set B] RS sets), associated IDs, and reporting type/configuration.

Observation 1: To determine the applicability, the UE needs to check whether the UE has trained models for RS configurations (i.e., [set A, set B] RS sets), associated IDs, and reporting type/configuration.

The ongoing discussions present two ways to provide the inference related parameters (for applicability) or full inference configurations (for applicability and inference): either combined or in separate lists. The combined approach is advantageous for multiple reasons:

- The message structure is simplified and reduced, as the same structures are not repeated in two places.

- The logic at the network and UE are simplified, as they do not have to synchronize the two lists, performing checks for duplication or conflict between them.

- The number of reconfigurations is potentially reduced, by allowing future optimized signalling to move a resource between the two states without sending a full reconfiguration.

Thus, to simplify the standard and UE and NW implementations, RAN2 should conclude on a common solution for configuring UE with inference related parameters (for applicability) or full inference configurations (for applicability and inference).

Proposal 1:  To simplify the standard and UE and NW implementations, RAN2 should conclude on a common solution for configuring UE with inference related parameters (for applicability) or full inference configurations (for applicability and inference).

### 2.1.2 Handling of non-applicable periodic CSI-ReportConfig containing inference configuration

- In RAN2#129 meeting [4], RAN2 agreed on following for periodic CSI-ReportConfigs

If option A is configured in Step 3, for periodic CSI reporting, the UE autonomously activate the applicable functionalities upon reporting applicable functionalities via RRCReconfigurationComplete in step 4 (i.e. without need to wait RRCReconfiguration in Step 5).

- UE behavior for non-applicable periodic CSI-ReportConfig is not clear, i.e., what is UE expected to do if periodic CSI-ReportConfig is not applicable at the time of sending RRCReconfigurationComplete? Due to limited memory at the UE, downloading the required model may be required in most cases. In such cases, if UE is not allowed to report the applicability of period CSI-ReportConfig later using UE Assistance Information, then configuring UE with CSI-ReportConfig may not be useful. Therefore, UE should be allowed to activate periodic CSI-ReportConfig and send its applicability in the UE Assistance Information, when a periodic CSI-ReportConfig becomes applicable after sending RRCReconfigurationComplete containing applicability information,

- Observation 2:  UE behavior for non-applicable periodic CSI-ReportConfig is not clear, i.e., what is UE expected to do if periodic CSI-ReportConfig is not applicable at the time of sending RRCReconfigurationComplete?

Observation 3: Due to limited memory at the UE, downloading the required model may be required in most cases. In such cases, if UE is not allowed to report the applicability of period CSI-ReportConfig later using UE Assistance Information, then configuring UE with CSI-ReportConfig may not be useful.

Proposal 2: The UE shall autonomously activate applicable periodic CSI-ReportConfig upon sending the UE Assistance Information (when a periodic CSI-ReportConfig for inference becomes applicable after sending RRCReconfigurationComplete).

### 2.1.3 Indicating the cause of inapplicability

In RAN2#127 meeting [5], RAN2 agreed on the following

- UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE) and model availability in device. FFS whether other configuration can considered by UE (e.g. inference configuration).  FFS  how the applicable functionality is decided if NW-side additional condition is not provided in step 3.

Observation 4: In the RAN2#127 meeting RAN2 agreed that UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE), and model availability in the device. How UE determines the applicability is left up to the UE implementation.

When the UE or UE side does not have a trained model, the UE may not download the model for configured CSI-ReportConfig. In another case, the UE or UE side can have a trained model for the configured CSI-ReportConfig, but may not be available on the device. We are open to distinguishing the aforementioned two cases where the configured CSI-ReportConfig (containing inference config or inference related parameter set). Other information such as inapplicability due to memory, power, etc. are UE’s proprietary information and they are not required to be signaled.

Proposal 3: The UE indicates to the network the cause of inapplicability as either:

- Temporary inapplicable: For the case when the UE side has a trained model for the configured CSI-ReportConfig (containing inference config or inference related parameter set), but the model is not be available for inference.

- Inapplicable: For the case when UE side does not have a trained model for the configured CSI-ReportConfig (containing inference config or inference related parameter set). The model will not be available on this device for the forseeable future.

Proposal 4: Other reasons for inapplicability, such as due to memory, power, are UE’s proprietary information and shall not be signaled.

### 2.1.3 Clarification on initial applicability report

At the RAN2#129 meeting [4], RAN2 agreed on the following

Support the explicit reporting of applicability/inapplicability in the initial report and subsequent reporting it reports only the applicability it changed.   FFS if we report explicit cause

For the applicability report, the following aspects should be further discussed

- What should be the reference for initial applicability reporting?

- How should the UE report the applicability of the full inference configuration configured by the earlier RRCReconfiguration?

Observation 5:  For the applicability/inapplicability reporting, the following aspects should be further discussed

- What should be the reference for initial applicability/inapplicability reporting?

- How should the UE report the applicability of the configuration provided by earlier RRCReconfiguration messages?

#### 2.1.3.1	Determining Initial Applicability Report

RAN needs to clarify what is the reference for initial applicability/inapplicability reporting. The following can be considered as initial applicability and applicability reporting,

- The first applicability/inapplicability report sent after each RRCReconfiguration is considered the initial applicability/inapplicability report if the RRCReconfiguration contains one or more full inference configurations for applicability determination (option A) or one or more inference-related parameter sets for applicability determination (option B), or

- The first applicability/inapplicability report sent after each RRCReconfiguration containing fullConfig is considered the initial applicability/inapplicability report if the RRCReconfiguration contains one or more full inference configurations for applicability determination (option A) or inference-related parameter sets for applicability determination (option B), or

- The first applicability/inapplicability report sent after each RRCReconfiguration containing HO Command is considered the initial applicability/inapplicability report if the RRCReconfiguration contains one or more full inference configurations for applicability determination (option A) or one or more inference-related parameter sets for applicability determination (option B).

Observation 6: The following can be considered as initial applicability and applicability reporting,

- The first applicability/inapplicability report sent after each RRCReconfiguration is considered the initial applicability/inapplicability report if the RRCReconfiguration contains one or more full inference configurations for applicability determination (option A) or one or more inference-related parameter sets for applicability determination (option B), or

- The first applicability/inapplicability report sent after each RRCReconfiguration containing fullConfig is considered the initial applicability/inapplicability report if the RRCReconfiguration contains one or more full inference configurations for applicability determination (option A) or inference-related parameter sets for applicability determination (option B), or

- The first applicability/inapplicability report sent after each RRCReconfiguration containing HO Command is considered the initial applicability/inapplicability report if the RRCReconfiguration contains one or more full inference configurations for applicability determination (option A) or one or more inference-related parameter sets for applicability determination (option B).

In option 2 and option 3, if the RRCReconfiguration containing fullConfig or RRCReconfiguration containing HO Command does not contain one or more full inference configurations for applicability determination (option A) or one or more inference related parameter sets for applicability determination (option B), then the reference of initial applicability/inapplicability reporting is not clear. Therefore, we think that the first applicability/inapplicability report sent after each RRCReconfiguration is considered as the initial applicability/inapplicability report if the RRCReconfiguration contains one or more full inference configurations for applicability determination (option A) or one or more inference related parameter sets for applicability determination (option B). Furthermore, if the RRCReconfiguration does not contain a full inference configuration for applicability determination (option A) or inference-related parameter set for applicability determination (option B) or contains full inference configuration (option B), then the first applicability/inapplicability report sent after such RRCReconfiguration is not considered as an initial applicability/inapplicability report. UE can send the change in applicability in the RRCReconfigurationComplete (for one or more full inference configurations for applicability determination (option A) or one or more inference related parameter sets for applicability determination (option B) configured by earlier RRCReconfigurations).

Proposal 5: The first applicability/inapplicability report sent after each RRCReconfiguration is considered an initial applicability/inapplicability report if the RRCReconfiguration contains one or more full inference configurations for applicability determination (option A) or one or more inference-related parameter sets for applicability determination (option B).

- If the RRCReconfiguration does not contain full inference configuration for applicability determination (option A) or inference related parameter set for applicability determination (option B), or contains full inference configuration (option B), then the first applicability/inapplicability report sent after such RRCReconfiguration is not considered as the initial applicability/inapplicability report.

#### 2.1.3.2 	Initial Applicability/Inapplicability Report Contents

Note that UE may have one or more inference configurations for applicability determination (option A), one or more inference configurations for inference (option B), or one or more inference related parameter sets for applicability determination (option B) configured by previous RRCReconfigurations. When sending the first applicability/inapplicability report after the latest RRCReconfiguration, the UE can report the change in the applicability of one or more inference configurations for applicability determination (option A), one or more inference configurations for inference (option B), or inference related parameter sets for applicability determination (option B) configured by previous RRCReconfigurations in RRCReconfigurationComplete. UE shall explicitly report the applicability/inapplicability of one or more full inference configurations for applicability determination (option A) or one or more inference related parameter sets for applicability determination (option B) configured in the latest RRCReconfiguration.

Proposal 6: UE reports the following in the first applicability report after each RRCReconfiguration

- Change in applicability of one or more inference configurations for applicability determination (option A), one or more inference configurations for inference (option B), or one or more inference related parameter sets for applicability determination (option B) configured by previous RRCReconfigurations and not released, and

- Explicity report applicability/inapplicability of one or more full inference configurations for applicability determination (option A) or one or more inference-related parameter sets for applicability determination (option B) configured in the latest RRCReconfiguration.

Proposal 7: UE should be allowed to report changes in the applicability of one or more inference configurations for applicability determination (option A), one or more inference configurations for inference (option B), or one or more inference-related parameter sets for applicability determination (option B) configured by previous RRCReconfigurations and not released in the RRCReconfigurationComplete,

- Even if the latest RRCReconfiguration does not contain full inference configuration for applicability determination (option A) or inference related parameter set for applicability determination (option B), or contains full inference configuration (option B), and UE is configured to report applicability using UAI.

# Conclusion

Observation 1: To determine the applicability, the UE needs to check whether the UE has trained models for RS configurations (i.e., [set A, set B] RS sets), associated IDs, and reporting type/configuration.

Proposal 1:  To simplify the standard and UE and NW implementations, RAN2 should conclude on a common solution for configuring UE with inference related parameters (for applicability) or full inference configurations (for applicability and inference).

- Observation 2:  UE behavior for non-applicable periodic CSI-ReportConfig is not clear, i.e., what is UE expected to do if periodic CSI-ReportConfig is not applicable at the time of sending RRCReconfigurationComplete?

Observation 3: Due to limited memory at the UE, downloading the required model may be required in most cases. In such cases, if UE is not allowed to report the applicability of period CSI-ReportConfig later using UE Assistance Information, then configuring UE with CSI-ReportConfig may not be useful.

Proposal 2: The UE shall autonomously activate applicable periodic CSI-ReportConfig upon sending the UE Assistance Information (when a periodic CSI-ReportConfig for inference becomes applicable after sending RRCReconfigurationComplete).

Observation 4: In the RAN2#127 meeting RAN2 agreed that UE decides the applicable functionalities based on NW-side additional conditions (if provided), UE-side additional conditions (internally known by UE), and model availability in the device. How UE determines the applicability is left up to the UE implementation.

Proposal 3: The UE indicates to the network the cause of inapplicability as either:

- Temporary inapplicable: For the case when the UE side has a trained model for the configured CSI-ReportConfig (containing inference config or inference related parameter set), but the model is not be available for inference.

- Inapplicable: For the case when UE side does not have a trained model for the configured CSI-ReportConfig (containing inference config or inference related parameter set). The model will not be available on this device for the forseeable future.

Proposal 4: Other reasons for inapplicability, such as due to memory, power, are UE’s proprietary information and shall not be signaled.

Observation 5:  For the applicability/inapplicability reporting, the following aspects should be further discussed

- What should be the reference for initial applicability/inapplicability reporting?

- How should the UE report the applicability of the configuration provided by earlier RRCReconfiguration messages?

Observation 6: The following can be considered as initial applicability and applicability reporting,

- The first applicability/inapplicability report sent after each RRCReconfiguration is considered the initial applicability/inapplicability report if the RRCReconfiguration contains one or more full inference configurations for applicability determination (option A) or one or more inference-related parameter sets for applicability determination (option B), or

- The first applicability/inapplicability report sent after each RRCReconfiguration containing fullConfig is considered the initial applicability/inapplicability report if the RRCReconfiguration contains one or more full inference configurations for applicability determination (option A) or inference-related parameter sets for applicability determination (option B), or

- The first applicability/inapplicability report sent after each RRCReconfiguration containing HO Command is considered the initial applicability/inapplicability report if the RRCReconfiguration contains one or more full inference configurations for applicability determination (option A) or one or more inference-related parameter sets for applicability determination (option B).

Proposal 5: The first applicability/inapplicability report sent after each RRCReconfiguration is considered an initial applicability/inapplicability report if the RRCReconfiguration contains one or more full inference configurations for applicability determination (option A) or one or more inference-related parameter sets for applicability determination (option B).

- If the RRCReconfiguration does not contain full inference configuration for applicability determination (option A) or inference related parameter set for applicability determination (option B), or contains full inference configuration (option B), then the first applicability/inapplicability report sent after such RRCReconfiguration is not considered as the initial applicability/inapplicability report.

Proposal 6: UE reports the following in the first applicability report after each RRCReconfiguration

- Change in applicability of one or more inference configurations for applicability determination (option A), one or more inference configurations for inference (option B), or one or more inference related parameter sets for applicability determination (option B) configured by previous RRCReconfigurations and not released, and

- Explicity report applicability/inapplicability of one or more full inference configurations for applicability determination (option A) or one or more inference-related parameter sets for applicability determination (option B) configured in the latest RRCReconfiguration.

Proposal 7: UE should be allowed to report changes in the applicability of one or more inference configurations for applicability determination (option A), one or more inference configurations for inference (option B), or one or more inference-related parameter sets for applicability determination (option B) configured by previous RRCReconfigurations and not released in the RRCReconfigurationComplete,

- Even if the latest RRCReconfiguration does not contain full inference configuration for applicability determination (option A) or inference related parameter set for applicability determination (option B), or contains full inference configuration (option B), and UE is configured to report applicability using UAI.

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, RAN #102, Dec. 2023

- RAN2#129 meeting report

- RAN2#127 meeting report

- RAN1#118 meeting report

- RAN2#127bis meeting report

- RAN2#128 meeting report

- RAN1#118bis meeting report