3GPP TSG-RAN WG2 Meeting #129bis		     R2-2502291

Wuhan, CN: 7th Apr – 11th Apr 2025

Source:	Qualcomm Incorporated

Title:	On UE Side Data Collection

Agenda Item:	8.1.4 UE side data collection

Document for:	Discussion and Decision

# Introduction & Background

In the RAN#106 meeting, a way forward for UE-side data collection was endorsed in RP-243307 [1] with the following guidance for RAN2

- Study RAN aspects related to data transfer over UP

- Discuss level of NG-RAN involvement in the control and configuration of UE side data collection.

- Discuss NG-RAN involvement in the data transfer of UE side data collection (if any) (including visibility discussion).

- Discuss aspects/solutions from RAN perspective that enable the data transfer to CN domain or OAM domain.

- Discuss on the scalability aspects of CP

- RAN2 can Liaise with other WGs if/when needed

This paper discusses RAN2 FFS topics assigned to RAN2 in RP-243307 [1] for RAN-centric (e.g., beam management, CSI prediction, two-sided CSI feedback, AI/ML-based mobility) and LMF-centric use cases (e.g., positioning accuracy enhancements).

# On the UE side Data Collection

Based on a way forward for UE-side data collection endorsed in RP-243307 [1], RAN2 should further study

- How UP-based solution 2 and UP-based solution 3 would work (e.g., with or without NG-RAN involvement),

- Issues associated with transferring collected data using the user plane and control plane in Solution 2 and Solution 3.

RAN2 is not expected to prioritize a solution in its further study.

Observation 1:  Based on a way forward for UE-side data collection endorsed in RP-243307 [1], RAN2 should further study

- How UP-based solution 2 and UP-based solution 3 would work (e.g., with or without NG-RAN involvement),

- Issues associated with transferring collected data using the user plane and control plane in Solution 2 and Solution 3.

Observation 2: RAN2 is not expected to prioritize or deprioritize a solution in its further study.

## 2.1	UE-side data collection for RAN-centric use cases

### 2.1.1	RAN Aspects Related to Data Transfer over UP

In the legacy procedures, the NG-RAN is responsible for admission control of the PDU based on information/parameters provided by the 5GC. Following the legacy procedures, the NG-RAN can perform the admission control of the PDU session for the data collection report over UP. However, the following issues should be left to SA2 to determine

- Whether the data collection report can share the PDU session with other traffic?

- What information/parameters are used by the NG-RAN for the admission control of PDUs corresponding to the UE-side data collection report?

Proposal 1: Following the legacy procedures, the NG-RAN is responsible for admission control of the PDU based on information/parameters provided by the 5GC.

Proposal 2: SA2 should further study the following:

- Whether the data collection report can share the PDU session with other traffic?

- What information/parameters are used by the NG-RAN for the admission control of PDUs corresponding to the UE-side data collection report?

### 2.1.2	NG-RAN Involvement in Configuration and Data Transfer

#### 2.1.2.1 Signaling for UE Side Data Collection

In the RAN2#129 meeting [13], RAN2 agreed to extend the agreements on beam management as a general framework for data collection configuration

- Agreements - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

Based on the above agreement in the RAN2#129 meeting, it is FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. This section discusses signaling for UE side data collection configuration (see Fig. 2.1.2.1-1).

Fig 2.1.2.1-1: Signaling for UE side data collection configuration.

Proposal 3: RAN2 is requested to consider Fig. 2.1.2.1-1 as signaling for UE side data collection configuration.

Proposal 4: For the UE side data collection, the signaling involves the following steps:

- STEP 1: The network sends UE Capability Enquiry to request UE side data collection-related UE capabilities.

- STEP 2: UE provides the UE capability information containing its capabilities for UE side data collection (including the maximum number of SSB/CSI-RS measurements that UE can perform, and the maximum number of CSI-RS UE can predict)

- STEP 3: In the RRCReconfiguration, the network may provide a  UAI configuration (e.g., a START indication) allowing the UE to request UE-side data collection configuration.

- STEP 4: In the RRCReconfigurationComplete / UAI, the UE can requesting the network to provide choices for data collection configuration (e.g., by sending an START indication).

- STEP 5: The network may provide one or more CSI-ReportConfig as choices for UE side data collection configuration in the RRCReconfiguration.

- STEP 6: The UE indicates one or more selected CSI-ReportConfig for UE side data collection in the RRCReconfigurationComplete / UAI.

- STEP 7: Activation/deactivation (e.g., START/STOP indication for configured data collection configurations at the UE) of the data collection configuration can be performed based on the network decision or the UE requirements.

Proposal 5: Activation/deactivation of the data collection (STEP 7) can be performed by

- The network where the network can reconfigure or send MAC CE to activate/deactivate the CSI-ReportConfig for UE side data collection; based on network decisions (network wants to STOP transmitting RS or if the network determines configured resources/resource sets are not suitable), or

- UE Indication; if the UE wants to STOP collecting the training data or if the UE determines that configured resources/resource sets are not suitable.

#### 2.1.2.2	NG-RAN Involvement in Data Transfer

In RAN Plenary LS RP-242389 [3], the requirements outlined for the data transfer were

- The MNO has full control of the standardized data collection transfer process and can manage data transfer to the server for UE-side data collection, without the need of SLA for this purpose. This includes initiating, terminating, and fully managing data transfer.

- MNO has full visibility for standardized data.

As discussed earlier, the NG-RAN may be involved in PDU admission control in user plane-based data transfer based on information/parameters configured by 5GC to NG-RAN, as in legacy admission control for PDU sessions. The legacy RRC and NAS-based procedure can be reused, where the NG-RAN can send an indication to the UE for the UE-side data collection report in a control-plane-based data transfer.

For both user and control plane-based data transfer, the visibility requirement does not need to be performed by the NG-RAN, rather, it can be performed by the first terminating entity (5GC in solution 2 and OAM in solution 3) before the training data is transferred/delivered to the server for the UE side training data collection / OTT server.

Proposal 6: For user plane-based data transfer, the NG-RAN may perform PDU admission control based on information/parameters configured by 5GC to NG-RAN to achieve controllability requirements for data transfer, as in legacy PDU admission control.

Proposal 7: The legacy RRC and NAS-based procedure can be reused by NG-RAN to achieve controllability requirements for data transfer in the control-plane-based data transfer.

Proposal 8: The first terminating entity (5GC in solution 2 and OAM in solution 3) can achieve visibility requirements (in both control and user plane-based data transfer) before the training data is transferred/delivered to the server for the UE side training data collection / OTT server. NG-RAN involvement for visibility is not required.

### 2.1.3	Enabling Data Transfer to CN domain and OAM domain

#### 2.1.3.1	Enabling User Plane based Data Transfer to CN and OAM Domain

Procedures for enabling user plane-based data transfer to the CN and OAM domains should be the same. This may involve

- Procedure for UE to establish a PDU session with the network entity in the CN or OAM domain, and

- Configuring or providing information/parameters for PDU admission control.

Proposal 9: SA WGs (e.g., SA2, SA5) can discuss the required procedures for user-plane solutions in the CN domain and OAM domain may, for example,

- Procedure to enable the UE to establish a PDU session with the network entity in the CN or OAM domain, and

- Configuring or providing information/parameters for PDU admission control to NG-RAN.

#### 2.1.3.2	Enabling Control Plane based Data Transfer to CN and OAM Domain

Several aspects need to be considered to enable control plane-based data transfer, for example,

- UE memory requirements: the training data may need to be stored in the access stratum buffer (AS) for the control plane-based data transfer. Allocating a large buffer for training data collection at the UE AS buffer may not be feasible.

- Segmentation for UE side data collection. The following options may need to be supported

  - UE segments the data collection report such that it can fit into one PDCP PDU, or

  - Alternatively, RRC segmentation is required.

- Continuity of the training data reporting: RAN3 / NG-AP signaling enhancements may be required for the continuity of the data reporting. Note that due to different reasons the reporting of the UE data may need to be paused/reinitiated. A few examples can be

  - Network overloaded

  - Radio link failure, handover procedures, handover failures, and others

    - Handling during handover/transition from supporting node to non-supporting node

  - RRC state transitions

    - Following the SON/MDT principle, the UE should not be kept in the RRC Connected state for data collection reporting,

    - The UE identity is temporary at the gNB. The new gNB should know the TCE ID or NF ID to forward the UE-reported training data.

- Risk of leakage of proprietary information to other vendors

  - Procedure required for correct forwarding of data collection report based on UE identity

    - NG-RAN to request TCE ID or NF ID from 5GC by providing UE identity

    - Response message from 5GC to provide TCE or NF ID based on UE identity.

  - Alternatively, the NG-RAN should be able to derive UE vendor identity based on temporary UE identity. Supporting such a feature at NG-RAN is against UE privacy and also impractical.

Proposal 10: At least the following aspects need to be considered to enable control plane-based data transfer.

- UE memory requirements: the training data may need to be stored in the access stratum buffer (AS) for the control plane-based data transfer. Allocating a large buffer for training data collection at the UE AS buffer may not be feasible.

- Segmentation for UE side data collection. The following options may need to be supported

  - UE segments the data collection report such that it can fit into one PDCP PDU, or

  - Alternatively, RRC segmentation is required.

- Continuity of the training data reporting: RAN3 / NG-AP signaling enhancements may be required for the continuity of the data reporting. Note that due to different reasons the reporting of the UE data may need to be paused/reinitiated. A few examples can be

  - Network overloaded

  - Radio link failure, handover procedures, handover failures, and others

    - Handling during handover/transition from supporting node to non-supporting node

  - RRC state transitions

    - Following the SON/MDT principle, the UE should not be kept in the RRC Connected state for data collection reporting,

    - The UE identity is temporary at the gNB. The new gNB should know the TCE ID or NF ID to forward the UE-reported training data.

- Risk of leakage of proprietary information to other vendors

  - Procedure required for correct forwarding of data collection report based on UE identity

    - NG-RAN to request TCE ID or NF ID from 5GC by providing UE identity

    - Response message from 5GC to provide TCE or NF ID based on UE identity.

  - Alternatively, the NG-RAN should be able to derive UE vendor identity based on temporary UE identity. Supporting such a feature at NG-RAN is against UE privacy and also impractical.

### 2.1.4	Scalability Aspects of CP

In Rel-18, 3GPP introduced three new RAN-centric use cases (e.g., Beam management, CSI prediction, and two-sided CSI feedback) and one LMF-centric use case (e.g., positioning accuracy enhancements) for the study [4] [5]. In Rel-19, the 3GPP further introduced AI/ML for Mobility as a new use case with different objectives for the study [6]. Note that the UE side data collection is required for all of these use cases.

Observation 3: Throughout Rel-18 [4][5] and Rel-19 [6], the 3GPP has introduced new use cases for study.

Note that one of the requirements for UE data collection is that “the design is futureproof and extendable”. More AI/ML-enabled use cases are anticipated to be introduced in the future, the control plane-based data collection will not remain futureproof or extendable.

Observation 4: One of the requirements for the UE side data collection is

- The design is futureproof and extendable.

Proposal 11: With more AI/ML-enabled use cases anticipated to be introduced in the future, the control plane-based data collection will not remain futureproof or extendable.

## 2.2   Data Collection for UE-side Model for Positioning Enhancements

The Data Collection for UE-side Model for Positioning Enhancements can make use of the existing LCS framework and procedures (e.g., (deferred) MT-LR, MO-LR, etc.).  The "server for data collection" could act as LCS Client which initiates and terminates the reporting of collected data. Figure 2.2-1 shows an MT-LR example as specified in TS 23.273 [7] clause 6.1.2. When data collection is needed, the "server for data collection" sends an LCS Service Request at Step 1 to a GMLC to request AI/ML Model Training Data. The Request may be for one or more UEs identified by its e.g., GPSI, SUPI, or PEI and includes any details defining the request (which may be generic request information together with vendor specific/private information). The GMLC may verify any privacy settings at Step 2 before forwarding the request to the serving AMF at Step 3. If the UE is in CM IDLE state, the AMF initiates a network triggered Service Request procedure at Step 4 and performs the privacy verification procedure at Steps 5/6, if indicated in the request from Step 3. The AMF then selects a proper LMF at Step 7 and forwards the data collection request to the LMF at Step 8. The LMF then performs any UE and NW supported positioning procedure as specified in TS 38.305 [8] to obtain the UE location ("ground-truth") if needed and to obtain the necessary data for model training. The LMF then reports the information to the "server for data collection" at Steps 10-12.

Fig. 2.2-1: MT-LR for UE-side data collection.

The signalling between UE and LMF for positioning and data collection at Step 9 may use NAS control plane (CP) transport or secure TCP/TLS user plane (UP) transport, as already specified in TS 38.305 [8] and TS 23.273 [7]. The desired transport option can also be requested in the LCS Service Request at Step 1.

The supported protocols between LMF and UE (for both, CP and UP transport options) are LPP [9] and LCS Supplementary Services [10] (e.g., Event Reports, e.g., with embedded LPP PDUs). If periodic or event triggered reporting is required, deferred MT-LR can be used which – when user plane reporting is used/allowed – can provide the collected data via user plane directly to the "server for data collection" as illustrated in Fig. 2.2-2 below (please see TS 23.273 [7] clause 6.16 for details where the "LCS Client" in our case is the "server for data collection").

Fig. 2.2-2: Deferred MT-LR for UE-side data collection with UP Event Reporting directly to the "server for data collection".

Note, if the UE would need (additional) assistance data for data collection and/or "ground-truth" label determination at e.g., Step 3 in Fig. 2.2-2, the UE can send a "Cumulative Event Report" at Step 7 (which can also happen before Step 3) with an embedded LPP Request Assistance Data PDU, as specified in TS 23.273, clause 6.16 [7].

Corresponding procedures for UE-side and NW-side cancellation (e.g. if the UE is powered off or if cancellation is required based on user's input, etc.) are also already specified in Clause 6.3.4 of TS 23.273 [7].

The data collection could also be initiated by the UE via MO-LR, if needed, with transfer to 3rd party as specified in TS 23.273 [7], clause 6.2, where the '3rd party client' in our case is the "server for data collection".

Observation 5: For UE-side data collection in Positioning Enhancements Case 1, existing LCS and UE positioning procedures (e.g., (deferred) MT-LR, MO-LR) can be used to obtain the desired training data by a "server for data collection" from a UE.

Observation 6: The "training data" can be sent from a UE to the "server for data collection" when a secure user plane connection is established directly from the UE to the "server for data collection" (LCS Client). Corresponding procedures are already specified in TS 23.273, clause 6.16.

In all these LCS procedures, the actual UE positioning/data collection steps (e.g., Step 9 in Figure 2.2-1) would be the same and may have RAN2 impacts. There needs to be a "Request" and a "Provide" message for the collected data. As mentioned above, there are generally two protocol options available:

- LPP,

- LCS Supplementary Services.

However, the LCS Supplementary Services messages typically include embedded LPP PDUs for the positioning information.

It is assumed that the actual data requested/provided are (at least partly) UE vendor specific and are carried as a private OCTET STRING within the messages. For LPP, a private (vendor specific) OCTET STRING could be included in each of the 3 LPP transaction types (capability, assistance data, location information). Such a mechanism is already available in LPP in form of EPDU's (EPDU-Sequence IE) in each message [9]. For example, the ePDU-ID and ePDU-Name [9] could identify the specific vendor. In addition, or alternatively, the existing LPPe EPDU could be re-used, since LPPe also supports vendor specific private messages [11].

Note, such vendor specific private messages are commonly supported in positioning specifications. For example, LPPe [11] supports private messages similar to RTCM, where "Vendor IDs" are globally managed by the individual SDO. In LPP [9], ePDU-ID and ePDU-Name fields are available if e.g., LPPe is not being used. Also, NRPPa supports (vendor specific) private messages [12].

For Event Reports, the actual payload could be an embedded LPP PDU (as today), or directly added to the Event Report.

Proposal 12: For UE-side data collection in Positioning Enhancements Case 1, the actual data/information requested and provided can be carried in existing LPP and LCS Supplementary Services messages as (at least partly) vendor specific private OCTET STRINGs by using an LPP EPDU-Sequence.

The Request/Provide OCTET STRINGs could be carried in all network messages. In the example of Fig. 2.2-1, this would be Steps 1, 3, 8 for the "Request EPDU", and Steps 10, 11, 12 for the "Provide EPDU".

In order to perform the data collection at the UE/PRU, the UE/PRU would need assistance data from the network (LMF). The assistance data needed can be split into 2 parts:

(a) Assistance Data required to obtain the UE position ("ground-truth label").

(b) Assistance Data to obtain the data for model training (e.g., measurements).

The Assistance Data required to obtain the UE position are already specified. A UE could use any of the supported positioning methods, such as GNSS or DL-TDOA, etc. to determine a UE position for the "ground-truth label". Existing MO-LR LCS procedures could be used by the UE to request the required assistance data when needed.

The Assistance Data required to obtain the data for model training are likely the same as needed for the positioning (inference) operation, incl. any information to ensure consistency between training and inference.

Observation 7: In order to perform the data collection at the UE/PRU, the UE/PRU would need assistance data from the network (LMF), which, however, are mostly existing assistance data elements (e.g., (on-demand) DL-PRS assistance data, position calculation assistance data, etc.). Any additional assistance data depends on RAN1 conclusions.

Therefore, to enable data collection for UE-side positioning model training at a "server for data collection" the RAN2 specification impacts could be rather small and may be the same as required for the LCM/inference/positioning operation anyhow.

Observation 8: To enable data collection for UE-side positioning model training at a "server for data collection" the RAN2 specification impacts seem rather small and are mainly the same as required for the LCM/inference/positioning operation.

Similar, the required changes in SA/CT specifications may also be rather small and comprise updates to the service request/response messages/operations (e.g., indication that the request is for "data collection" instead of positioning) and inclusion of the private Request/Provide OCTET STRINGs (e.g., LPP EPDUs) in the various messages and operations.

# Conclusion

UE-side data collection for RAN-centric use cases

Observation 1:  Based on a way forward for UE-side data collection endorsed in RP-243307 [1], RAN2 should further study

- How UP-based solution 2 and UP-based solution 3 would work (e.g., with or without NG-RAN involvement),

- Issues associated with transferring collected data using the user plane and control plane in Solution 2 and Solution 3.

Observation 2: RAN2 is not expected to prioritize or deprioritize a solution in its further study.

Proposal 1: Following the legacy procedures, the NG-RAN is responsible for admission control of the PDU based on information/parameters provided by the 5GC.

Proposal 2: SA2 should further study the following:

- Whether the data collection report can share the PDU session with other traffic?

- What information/parameters are used by the NG-RAN for the admission control of PDUs corresponding to the UE-side data collection report?

Proposal 3: RAN2 is requested to consider Fig. 2.1.2.1-1 as signaling for UE side data collection configuration.

Proposal 4: For the UE side data collection, the signaling involves the following steps:

- STEP 1: The network sends UE Capability Enquiry to request UE side data collection-related UE capabilities.

- STEP 2: UE provides the UE capability information containing its capabilities for UE side data collection (including the maximum number of SSB/CSI-RS measurements that UE can perform, and the maximum number of CSI-RS UE can predict)

- STEP 3: In the RRCReconfiguration, the network may provide a  UAI configuration (e.g., a START indication) allowing the UE to request UE-side data collection configuration.

- STEP 4: In the RRCReconfigurationComplete / UAI, the UE can requesting the network to provide choices for data collection configuration (e.g., by sending an START indication).

- STEP 5: The network may provide one or more CSI-ReportConfig as choices for UE side data collection configuration in the RRCReconfiguration.

- STEP 6: The UE indicates one or more selected CSI-ReportConfig for UE side data collection in the RRCReconfigurationComplete / UAI.

- STEP 7: Activation/deactivation (e.g., START/STOP indication for configured data collection configurations at the UE) of the data collection configuration can be performed based on the network decision or the UE requirements.

Proposal 5: Activation/deactivation of the data collection (STEP 7) can be performed by

- The network where the network can reconfigure or send MAC CE to activate/deactivate the CSI-ReportConfig for UE side data collection; based on network decisions (network wants to STOP transmitting RS or if the network determines configured resources/resource sets are not suitable), or

- UE Indication; if the UE wants to STOP collecting the training data or if the UE determines that configured resources/resource sets are not suitable.

Proposal 6: For user plane-based data transfer, the NG-RAN may perform PDU admission control based on information/parameters configured by 5GC to NG-RAN to achieve controllability requirements for data transfer, as in legacy PDU admission control.

Proposal 7: The legacy RRC and NAS-based procedure can be reused by NG-RAN to achieve controllability requirements for data transfer in the control-plane-based data transfer.

Proposal 8: The first terminating entity (5GC in solution 2 and OAM in solution 3) can achieve visibility requirements (in both control and user plane-based data transfer) before the training data is transferred/delivered to the server for the UE side training data collection / OTT server. NG-RAN involvement for visibility is not required.

Proposal 9: SA WGs (e.g., SA2, SA5) can discuss the required procedures for user-plane solutions in the CN domain and OAM domain may, for example,

- Procedure to enable the UE to establish a PDU session with the network entity in the CN or OAM domain, and

- Configuring or providing information/parameters for PDU admission control to NG-RAN.

Proposal 10: At least the following aspects need to be considered to enable control plane-based data transfer.

- UE memory requirements: the training data may need to be stored in the access stratum buffer (AS) for the control plane-based data transfer. Allocating a large buffer for training data collection at the UE AS buffer may not be feasible.

- Segmentation for UE side data collection. The following options may need to be supported

  - UE segments the data collection report such that it can fit into one PDCP PDU, or

  - Alternatively, RRC segmentation is required.

- Continuity of the training data reporting: RAN3 / NG-AP signaling enhancements may be required for the continuity of the data reporting. Note that due to different reasons the reporting of the UE data may need to be paused/reinitiated. A few examples can be

  - Network overloaded

  - Radio link failure, handover procedures, handover failures, and others

    - Handling during handover/transition from supporting node to non-supporting node

  - RRC state transitions

    - Following the SON/MDT principle, the UE should not be kept in the RRC Connected state for data collection reporting,

    - The UE identity is temporary at the gNB. The new gNB should know the TCE ID or NF ID to forward the UE-reported training data.

- Risk of leakage of proprietary information to other vendors

  - Procedure required for correct forwarding of data collection report based on UE identity

    - NG-RAN to request TCE ID or NF ID from 5GC by providing UE identity

    - Response message from 5GC to provide TCE or NF ID based on UE identity.

  - Alternatively, the NG-RAN should be able to derive UE vendor identity based on temporary UE identity. Supporting such a feature at NG-RAN is against UE privacy and also impractical.

Observation 3: Throughout Rel-18 [4][5] and Rel-19 [6], the 3GPP has introduced new use cases for study.

Observation 4: One of the requirements for the UE side data collection is

- The design is futureproof and extendable.

Proposal 11: With more AI/ML-enabled use cases anticipated to be introduced in the future, the control plane-based data collection will not remain futureproof or extendable.

Data Collection for UE-side Model for Positioning Enhancements

Observation 5: For UE-side data collection in Positioning Enhancements Case 1, existing LCS and UE positioning procedures (e.g., (deferred) MT-LR, MO-LR) can be used to obtain the desired training data by a "server for data collection" from a UE.

Observation 6: The "training data" can be sent from a UE to the "server for data collection" when a secure user plane connection is established directly from the UE to the "server for data collection" (LCS Client). Corresponding procedures are already specified in TS 23.273, clause 6.16.

Proposal 12: For UE-side data collection in Positioning Enhancements Case 1, the actual data/information requested and provided can be carried in existing LPP and LCS Supplementary Services messages as (at least partly) vendor-specific private OCTET STRINGs by using an LPP EPDU-Sequence.

Observation 7: In order to perform the data collection at the UE/PRU, the UE/PRU would need assistance data from the network (LMF), which, however, are mostly existing assistance data elements (e.g., (on-demand) DL-PRS assistance data, position calculation assistance data, etc.). Any additional assistance data depends on RAN1 conclusions.

Observation 8: To enable data collection for UE-side positioning model training at a "server for data collection" the RAN2 specification impacts seem rather small and are mainly the same as required for the LCM/inference/positioning operation.

# References

- RP-243307, “Way forward on AIML UE sided data collection,” Dec 2024

- RAN2#128 meeting report

- RP-242389, “LS on AIML data collection,” Sep 2024

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, TSG RAN, RAN#94-e, Dec. 2021

- TR 38.843 v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18),” Dec. 2023

- RP-234055, “Study on AI (Artificial Intelligence)/ML (Machine Learning) for mobility in NR,” Dec. 2023

- 3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

- 3GPP TS 38.305: "Stage 2 functional specification of User Equipment (UE) positioning in NG-RAN".

- 3GPP TS 37.355: "LTE Positioning Protocol (LPP)".

- 3GPP TS 24.571: "Control plane Location Services (LCS) procedures; Stage 3".

- OMA-TS-LPPe, "LPP Extensions Specification", Open Mobile Alliance.

- 3GPP TS 38.455: "NG-RAN; NR Positioning Protocol A (NRPPa)".

- RAN2#129 meeting report