3GPP TSG-RAN WG2 Meeting #129bis	R2-2502300

Wuhan, China, 07th – 11th April, 2025

Agenda item:	8.7.4.1

Source: 	Huawei, HiSilicon

Title: 	Discussion on additional priority based LCP enhancements in XR

Document for:	Discussion and Decision

# 1. Introduction

For additional priority based LCP enhancements, RAN2#129 further discussed the related issues and had the following agreements:

- Only one additional priority is configured to an LCH for LCP enhancement. - We keep an existing agreement (remaining time threshold is configured per LCH) - There is no impact on BSR/SR/DSR triggering and reporting due to adjusted priority. - Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact - FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination - No additional PBR is needed for priority adjusted data - FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

In this contribution, we will further discuss remaining issues of the additional priority handling based on the above agreements.

# 2.	Discussion

## 2.1	Bj enhancement

In existing LCP procedure, resource allocation procedure is performed with consideration of Bj, i.e., tokens in the bucket. In the Initial resource allocation stage, only LCH(s) with Bj > 0 are allocated resources in a decreasing priority order, which means that the LCH priority-adjusted data may not be allocated with resources in the Initial resource allocation procedure if Bj is smaller than the LCH priority-adjusted data volume. In such case, the LCH priority-adjusted data may be further delayed or even be discarded because of the Bj limitation if there are no remaining resources.

To avoid the Bj limitation impact on LCH priority-adjusted data transmission, RAN2#129 discussed and agreed the following:

6. No additional PBR is needed for priority adjusted data 7. FFS Allow an LCH with an upgraded priority to be transmitted even if Bj is negative (if configured by the network), while the remaining time is less the configured threshold.

Considering that the PBR and BSD parameters are configured by NW with consideration of the traffic on each LCH, it may be a corner case that the configured PBR and BSD differ greatly from the buffered LCH priority-adjusted data volume. Therefore, even if the LCH priority-adjusted data cannot be allocated resources because of Bj limitation, the difference between Bj and LCH priority-adjusted data is small. If any enhancement is needed, it should not impact UE complexity or fairness.

Observation 1:	LCH priority-adjusted data may not get through if Bj is not enough and could cause further delay.

Observation 2:	For Bj enhancement, fairness and low complexity need to be considered.

To ensure that LCH with LCH priority-adjusted data can be allocated with resources, the following approach with consideration of fairness and complexity can be considered. Specifically, at the start of the Initial resource allocation, UE can select an LCH as long as there is buffered LCH priority-adjusted data, regardless of the value of Bj, through which the LCH(s) with LCH priority-adjusted data would have opportunity to obtain resources.

Furthermore, in current spec, Bj can be negative as described in 38.321 below:

##### 5.4.3.1.3 Allocation of resources 1> allocate resources to the logical channels as follows: 2> logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s); [Initial resource allocation] 2> decrement Bj by the total size of MAC SDUs served to logical channel j above; 2> if any resources remain, all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally. [Remaining resource allocation] NOTE 1: The value of Bj can be negative.

During the Initial resource allocation, Bj may be updated to a value less than 0 if the volume of LCH priority-adjusted data is larger than initial value of Bj. But considering that the spec already supports Bj with a negative value, so UE can keep performing the resource allocation for an LCH with LCH priority-adjusted data in the Initial resource allocation stage, even if Bj < 0 and there is still LCH priority-adjusted data in the buffer. After that, Bj will be increased as time elapses, which has less impact on data on other LCH without LCH priority-adjusted data. In addition, UE doesn't need to decide which value the Bj should be temporarily raised, which doesn't introduce complexity for UE.

Proposal 1:	In the initial resource allocation, resource should be allocated for logical channel selected for an UL grant containing LCH priority-adjusted data regardless of the value of Bj.

Based on the above Observations and Proposals, a corresponding text procedure is provided in Appendix 1.

Proposal 2:	Consider the TP in Appendix 1 for Bj enhancement.

## 2.2	Impact of additional priority handling on Intra-UE prioritization

With respect of impact on intra-UE prioritization, RAN2#129 discussed the issues for new transmission, retransmission and SR, and have the following agreement and FFS:

4. Intra-UE prioritization shall also use the additional LCP priority for UL grant priority determination. FFS whether this has specifications impact 5. FFS Intra-UE prioritization shall also use the additional LCP priority for SR priority determination

### 2.2.1	Priority determination of UL grant for intra-UE prioritization

In legacy intra-UE prioritization procedure, for the MAC entity configured with lch-basedPrioritization, the following mechanism is used to determine the priority of an UL grant as defined in 38.321.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

Based on the above highlighted text, the priority handling on intra-UE prioritization affects both the new transmission (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) and retransmission (i.e. the MAC PDU to transmit is already stored in the HARQ buffer). This should be extended to additional priority.

Observation 3:	In existing intra-UE prioritization, the UL grant priority determination depends on the highest priority among priorities of the logical channels that are multiplexed (i.e. retransmission) or have data available that can be multiplexed (i.e. new transmission).

Proposal 3:	Priority determination in intra-UE prioritization shall also consider both new transmission and retransmission as in legacy.

However, existing intra-UE prioritization does not consider the remaining time of the data that is multiplexed within the MAC PDU (retransmission) or to be multiplexed within a MAC PDU (new transmission) and hence the time used for determining the priority does not matter since it is not related to remaining time. With additional priority (i.e., priority of logical channel linked with remaining time of data) introduced in Rel-19, it is essential to also consider the remaining time when the priority for UL grant is determined by intra-UE prioritization.

Observation 4:	With additional priority (i.e., priority of logical channel linked with remaining time of data) introduced in Rel-19, it is essential to also consider the remaining time of the data when the priority for UL grant is determined by intra-UE prioritization.

Proposal 4:	When determining the priority of the LCH and the priority of the associated UL grant in the intra-UE prioritization procedure, the MAC entity should take into account the remaining time of the data multiplexed in or to be multiplexed in a MAC PDU

For new transmission and retransmission, the priority determination of UL grant for intra-UE prioritization based on the highest priority among priorities of the logical channels that are multiplexed (i.e. retransmission) or have data available that can be multiplexed (i.e. new transmission) should consider the actual transmission point of time of the UL grant.

Proposal 5:	The priority determination of UL grant for intra-UE prioritization based on the highest priority among priorities of the logical channels that are multiplexed (i.e. retransmission) or have data available that can be multiplexed (i.e. new transmission) should consider the actual transmission point of time of the UL grant.

In the running MAC spec [1], the following is added for UL grant with an Editor’s note:

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. In this determination, the priority of a data logical channel shall be the highest priority applied for it in the LCP procedure for this uplink grant (see clause 5.4.3.1.3). The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR. Editor’s Note: FFS whether this change is needed. If it is, FFS whether priority of a MAC PDU may change over time, if it includes data from a logical channel configured with additionalPriority.

This probably covers the new transmission case, as the additional priority determination in the LCP is evaluated at the time of the first symbol of this new transmission. However, this does not cover the actual transmission time of UL grant associated to retransmission since LCP is only for new transmission, while for the retransmission case the priority of a MAC PDU may change over time as the remaining time may become below the additionalPriorityThreshold.

Observation 5:	In the retransmission case, when the MAC PDU to transmit is already stored in the HARQ buffer, it is possible that the remaining time of the multiplexed data in the MAC PDU may change and thus priority may change between non-adjusted priority and adjusted-priority.

We have proposed the following TP to cover both the new transmission and retransmission case below (and in the Appendix 2):

For the MAC entity configured with lch-basedPrioritization, the MAC entity considers the value of additionalPriority, if configured, as the priority for the LCH for which the running PDCP discardTimer of an PDCP SDU of the LCH that is multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or can be multiplexed(i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU has the remaining value below additionalPriorityThreshold at the time of the first symbol of the PUSCH transmission that includes this MAC PDU.

Considering the impact of additional priority on intra-UE prioritization for new transmission and retransmission, a unified approach to determining the priority of the UL grant in the procedure should be supported and specified. To elaborate more, when determining the priority of the LCH and the priority of the associated UL grant in the intra-UE prioritization procedure, the MAC entity should take into account the status of the MAC PDU at its actual transmission point of time, including both the initial transmission and retransmission.

Proposal 6:	To consider the actual transmission point of time of the UL grant when MAC entity determines the priority of the UL grant in intra-UE prioritization for new transmission and retransmission, as given in the TP in Appendix 2.

### 2.2.2	Impact of additional priority handling on SR

In legacy specification, the intra-UE prioritization procedure also considers the scenario of the collision between the resource of SR and resource of MAC PDU, where the priority of the SR is determined by the priority of the LCH that triggered SR. For example, if the priority of the LCH that triggered an SR is higher than that of an overlapping uplink grant, the MAC entity shall consider the SR transmission as a prioritized SR transmission. As the additional priority is introduced, there exists ambiguity about which priority should be applied to determine the priority of the SR transmission, especially when there is LCH priority-adjusted data buffered in the LCH that triggered the SR.

Based on our analysis, we observe the benefit of applying the additional priority for the SR transmission when there is LCH priority-adjusted data buffered. Considering two LCHs, LCH#1 and LCH#2, LCH#1 is configured with additional priority, where the default priority of LCH#1 is lower than that of LCH#2, but the additional priority is higher than that of LCH#2. Due to LCP restrictions in LCH#1 and LCH#2, the data buffered in LCH#2 is multiplexed in MAC PDU. However, the LCH#1 may trigger an SR transmission when there is buffered data becoming delay-critical, where the transmission time of the SR is overlapped with that of the MAC PDU. Since the default priority of LCH#1 is lower than that of LCH#2 (and also the UL grant), the SR transmission will be deprioritized if the SR resource is overlapped with the PUSCH carrying data from LCH#2. As a consequence, the delay-critical data cannot be successfully transmitted within PDB, which is significantly deteriorating the experience of XR users. By contrast, if the additional priority of LCH#1 can be applied for determining the priority of the SR transmission, the transmitted SR can request more resources to transmit both the DSR MAC CE and the delay-critical data. Therefore, we believe that there is evident benefit of applying additional priority of the LCH for determining the priority of the SR transmission triggered by the LCH.

Moreover, we believe that whether the additional priority of the LCH triggering an SR is applied to determine the priority of the SR should consider the actual transmission time of the SR. To be more specific, when determining the priority of the SR, if there is LCH priority-adjusted data buffered in the LCH that triggered the SR at the time of SR transmission, the MAC entity should apply the additional priority of the LCH for determining the priority of the SR.

Proposal 7:	The additional priority should also be applied to the priority for SR transmission in intra-UE prioritization procedure, where the actual SR transmission time should be considered to determine whether to apply the additional or default priority of the LCH that triggered the SR, as given in the TP in Appendix 2.

## 2.4	Additional priority fallback

RAN2#128 discussed the issues related to additional priority fallback and had the following agreements:

- As a baseline, the additional LCH priority is applied to both the first round and the second round of the LCP procedure. The UE does not fallback to the default LCH priority in the second round even if there is no more LCH priority-adjusted data after the first round. - As an optional capability, the UE can also support to fallback to default priority in the 2nd round of LCP.

With the introduction of the additional priority fallback capability, UE can fallback to default priority when there is no LCH priority-adjusted data in the 2nd round of LCP procedure. However, when fallback condition is met, it is currently FFS whether UE can enable the fallback capability by itself or that it needs gNB to enable it. Considering that the LCP enhancement related configuration, e.g., additional priority and remaining time threshold for LCP enhancement, are configured by gNB, it is more reasonable for UE to perform the fallback capability only if gNB enables it.

Proposal 8:	If UE supports the capability of additional priority fallback, UE can perform the additional priority fallback after NW enables it via RRC configuration.

## 2.5	Other issues

In the MAC running CR [1], there is issue about additional priority-based LCH enhancement, as described in the below Editor’s Note

Editor’s Note: The above conditions on priority adjustment may need further clarifications, due to a corner case which has not been discussed yet. In this case, the data with the smallest remaining discard timer is queued behind non-delay-critical data and the UL grant is not large enough to include it in the transmission. It is FFS whether this logical channel should apply its additionalPriority or its Priority in this case.

Our understanding is similar to other companies during the MAC running CR review that additional priority should be applied when the smallest remaining time of the LCH is less than the priorityAdjustmentThreshold for the LCH, regardless of whether there are non-delay critical data in front of the delay critical data. Note that we have also agreed the following during RAN2#126:

LCP prioritization within a logical channel will not be considered in RAN2 discussions

Observation 6:	RAN2 agreed that “LCP prioritization within a logical channel will not be considered in RAN2 discussions”.

We further analyse the occurrence of the case. In the case where DSR is triggered prior to the LCP applying the additional priority (i.e. priorityAdjustmentThreshold for the LCH < remainingTimeThreshold of the LCG), the DSR will include both the delay critical data and the non-delay critical data ahead in the buffer status calculation to the NW and the NW can schedule enough UL grant based on the reported R19 DSR MAC CE, so it is corner case that the UL grant is not enough to carry all the data, e.g., LCH-adjusted data and the other data ahead of LCH-adjusted data. It only occurs when the DSR is not triggered prior to the LCP applying the additional priority (i.e. priorityAdjustmentThreshold for the LCH > remainingTimeThreshold of the LCG), in this case the UL grant may not be large enough. However, we do not see any issue here, as the UE will trigger the DSR to request for more UL grant later, if the previous grant is not large enough.

Observation 7:	The case occurs only when DSR is not triggered prior to the LCP applying the additional priority (i.e. priorityAdjustmentThreshold for the LCH > remainingTimeThreshold of the LCG) and the UL grant is not enough to cover both the delay-critical data and the non-delay critical data ahead. The delay critical data (if not completely transmitted in the previous UL grant) will trigger the DSR to request for more UL grant later.

Therefore, there is no need to consider the case where the LCH priority-adjusted data with the smallest remaining time of discardTimer is queued behind other data (e.g., non-delay-critical data) and the UL grant is not large enough to include the LCH priority-adjusted data for transmission.

Proposal 9:	Additional priority should be applied on the LCH if the smallest remaining time is less than the priorityAdjustmentThreshold for the LCH as agreed also for the case where the LCH priority-adjusted data with the smallest remaining time of discardTimer is queued behind other data (e.g., non-delay-critical data) and the UL grant is not large enough to include the LCH priority-adjusted data for transmission.

# 3. Conclusion

In this contribution, we analyzed the potential aspects for LCP enhancements. The following observations and proposals were made:

Bj enhancement

Observation 1:	LCH priority-adjusted data may not get through if Bj is not enough and could cause further delay.

Observation 2:	For Bj enhancement, fairness and low complexity need to be considered.

Proposal 1:	In the initial resource allocation, resource should be allocated for logical channel selected for an UL grant containing LCH priority-adjusted data regardless of the value of Bj.

Proposal 2:	Consider the TP in Appendix 1 for Bj enhancement.

Priority determination of UL grant for intra-UE prioritization

Observation 3:	In existing intra-UE prioritization, the UL grant priority determination depends on the highest priority among priorities of the logical channels that are multiplexed (i.e. retransmission) or have data available that can be multiplexed (i.e. new transmission).

Observation 4:	With additional priority (i.e., priority of logical channel linked with remaining time of data) introduced in Rel-19, it is essential to also consider the remaining time of the data when the priority for UL grant is determined by intra-UE prioritization.

Observation 5:	In the retransmission case, when the MAC PDU to transmit is already stored in the HARQ buffer, it is possible that the remaining time of the multiplexed data in the MAC PDU may change and thus priority may change between non-adjusted priority and adjusted-priority.

Proposal 3:	Priority determination in intra-UE prioritization shall also consider both new transmission and retransmission as in legacy.

Proposal 4:	When determining the priority of the LCH and the priority of the associated UL grant in the intra-UE prioritization procedure, the MAC entity should take into account the remaining time of the data multiplexed in or to be multiplexed in a MAC PDU

Proposal 5:	The priority determination of UL grant for intra-UE prioritization based on the highest priority among priorities of the logical channels that are multiplexed (i.e. retransmission) or have data available that can be multiplexed (i.e. new transmission) should consider the actual transmission point of time of the UL grant.

Proposal 6:	To consider the actual transmission point of time of the UL grant when MAC entity determines the priority of the UL grant in intra-UE prioritization for new transmission and retransmission, as given in the TP in Appendix 2.

Impact of additional priority handling on SR

Proposal 7:	The additional priority should also be applied to the priority for SR transmission in intra-UE prioritization procedure, where the actual SR transmission time should be considered to determine whether to apply the additional or default priority of the LCH that triggered the SR, as given in the TP in Appendix 2.

Additional priority fallback

Proposal 8:	If UE supports the capability of additional priority fallback, UE can perform the additional priority fallback after NW enables it via RRC configuration.

Other issues

Observation 6:	RAN2 agreed that “LCP prioritization within a logical channel will not be considered in RAN2 discussions”.

Observation 7:	The case occurs only when DSR is not triggered prior to the LCP applying the additional priority (i.e. priorityAdjustmentThreshold for the LCH > remainingTimeThreshold of the LCG) and the UL grant is not enough to cover both the delay-critical data and the non-delay critical data ahead. The delay critical data (if not completely transmitted in the previous UL grant) will trigger the DSR to request for more UL grant later.

Proposal 9:	Additional priority should be applied on the LCH if the smallest remaining time is less than the priorityAdjustmentThreshold for the LCH as agreed also for the case where the LCH priority-adjusted data with the smallest remaining time of discardTimer is queued behind other data (e.g., non-delay-critical data) and the UL grant is not large enough to include the LCH priority-adjusted data for transmission.

# 4. Reference

[1]. R2-250xxxx [POST129][509][XR] MAC running CR (Qualcomm)

# 5. Appendix

## 5.1 Appendix 1

#### 5.4.3.1	Logical Channel Prioritization

##### 5.4.3.1.1	General

The Logical Channel Prioritization (LCP) procedure is applied whenever a new transmission is performed.

RRC controls the scheduling of uplink data by signalling for each logical channel per MAC entity:

-	priority where an increasing priority value indicates a lower priority level;

-	prioritisedBitRate which sets the Prioritized Bit Rate (PBR);

-	bucketSizeDuration which sets the Bucket Size Duration (BSD).

RRC additionally controls the LCP procedure by configuring mapping restrictions for each logical channel:

-	allowedSCS-List which sets the allowed Subcarrier Spacing(s) for transmission;

-	maxPUSCH-Duration which sets the maximum PUSCH duration allowed for transmission;

-	configuredGrantType1Allowed which sets whether a configured grant Type 1 can be used for transmission;

-	allowedServingCells which sets the allowed cell(s) for transmission;

-	allowedCG-List which sets the allowed configured grant(s) for transmission;

-	allowedPHY-PriorityIndex which sets the allowed PHY priority index(es) of a dynamic grant for transmission;

-	allowedHARQ-mode which sets the allowed UL HARQ mode for transmission.

The following UE variable is used for the Logical channel prioritization procedure:

-	Bj which is maintained for each logical channel j.

The MAC entity shall initialize Bj of the logical channel to zero when the logical channel is established.

For each logical channel j, the MAC entity shall:

1>	increment Bj by the product PBR × T before every instance of the LCP procedure, where T is the time elapsed since Bj was last incremented;

1>	if the value of Bj is greater than the bucket size (i.e. PBR × BSD):

2>	set Bj to the bucket size.

NOTE:	The exact moment(s) when the UE updates Bj between LCP procedures is up to UE implementation, as long as Bj is up to date at the time when a grant is processed by LCP.

##### 5.4.3.1.2	Selection of logical channels

The MAC entity shall, when a new transmission is performed:

1>	select the logical channels for each UL grant that satisfy all the following conditions:

2>	the set of allowed Subcarrier Spacing index values in allowedSCS-List, if configured, includes the Subcarrier Spacing index associated to the UL grant; and

2>	maxPUSCH-Duration, if configured, is larger than or equal to the PUSCH transmission duration associated to the UL grant; and

2>	configuredGrantType1Allowed, if configured, is set to true in case the UL grant is a Configured Grant Type 1; and

2>	allowedServingCells, if configured, includes the Cell information associated to the UL grant. Does not apply to logical channels associated with a DRB configured with PDCP duplication within the same MAC entity (i.e. CA duplication) when CA duplication is deactivated for this DRB in this MAC entity; and

2>	allowedCG-List, if configured, includes the configured grant index associated to the UL grant; and

2>	allowedPHY-PriorityIndex, if configured, includes the priority index (as specified in clause 9 of TS 38.213 [6]) associated to the dynamic UL grant; and

2>	allowedHARQ-mode, if configured, includes the allowed UL HARQ mode for the HARQ process associated to the UL grant.

NOTE:	The Subcarrier Spacing index, PUSCH transmission duration, Cell information, and priority index are included in Uplink transmission information received from lower layers for the corresponding scheduled uplink transmission.

##### 5.4.3.1.3	Allocation of resources

Before the successful completion of the Random Access procedure initiated for DAPS handover, the target MAC entity shall not select the logical channel(s) corresponding to non-DAPS DRB(s) for the uplink grant received in a Random Access Response or the uplink grant for the transmission of the MSGA payload. The source MAC entity shall select only the logical channel(s) corresponding to DAPS DRB(s) during DAPS handover.

The MAC entity shall, when a new transmission is performed:

1>	allocate resources to the logical channels as follows:

2>	logical channels selected in clause 5.4.3.1.2 for the UL grant with Bj > 0 or with LCH priority-adjusted data regardless of the value of Bj are allocated resources in a decreasing priority order. If the PBR of a logical channel is set to infinity, the MAC entity shall allocate resources for all the data that is available for transmission on the logical channel before meeting the PBR of the lower priority logical channel(s);

2>	decrement Bj by the total size of MAC SDUs served to logical channel j above;

2>	if any resources remain,  all the logical channels selected in clause 5.4.3.1.2 are served in a strict decreasing priority order (regardless of the value of Bj) until either the data for that logical channel or the UL grant is exhausted, whichever comes first. Logical channels configured with equal priority should be served equally.

NOTE 1:	The value of Bj can be negative.

If the MAC entity is requested to simultaneously transmit multiple MAC PDUs, or if the MAC entity receives the multiple UL grants within one or more coinciding PDCCH occasions (i.e. on different Serving Cells), it is up to UE implementation in which order the grants are processed.

The UE shall also follow the rules below during the scheduling procedures above:

-	the UE should not segment an RLC SDU (or partially transmitted SDU or retransmitted RLC PDU) if the whole SDU (or partially transmitted SDU or retransmitted RLC PDU) fits into the remaining resources of the associated MAC entity;

-	if the UE segments an RLC SDU from the logical channel, it shall maximize the size of the segment to fill the grant of the associated MAC entity as much as possible;

-	the UE should maximise the transmission of data;

-	if the MAC entity is given a UL grant size that is equal to or larger than 8 bytes (when eLCID is not used) or 10 bytes (when eLCID is used) while having data available and allowed (according to clause 5.4.3.1) for transmission, the MAC entity shall not transmit only padding BSR and/or padding.

The MAC entity shall:

1>	if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

1>	else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant:

2>	if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and

2>	if the MAC PDU includes zero MAC SDUs; and

2>	if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR:

3>	not generate a MAC PDU for the HARQ entity.

Logical channels shall be prioritised in accordance with the following order (highest priority listed first):

-	MAC CE for C-RNTI, or data from UL-CCCH;

-	MAC CE for (Enhanced) BFR, or MAC CE for Configured Grant Confirmation, or MAC CE for Multiple Entry Configured Grant Confirmation;

-	MAC CE for Sidelink Configured Grant Confirmation;

-	MAC CE for LBT failure;

-	MAC CE for SL LBT failure according to clause 5.31.2;

-	MAC CE for Timing Advance Report;

-	MAC CE for Delay Status Report;

-	MAC CE for SL-BSR prioritized according to clause 5.22.1.6;

-	MAC CE for SL-PRS Resource Request;

-	MAC CE for (Extended) BSR, with exception of BSR included for padding;

-	MAC CE for (Enhanced) Single Entry PHR, or MAC CE for (Enhanced) Multiple Entry PHR or MAC CE for Single Entry PHR with assumed PUSCH, or MAC CE for Multiple Entry PHR with assumed PUSCH, or MAC CE for Enhanced Single Entry PHR for multiple TRP or MAC CE for Enhanced Multiple Entry PHR for multiple TRP, or MAC CE for Enhanced Single Entry PHR for multiple TRP STx2P or MAC CE for Enhanced Multiple Entry PHR for multiple TRP STx2P;

-	MAC CE for Positioning Measurement Gap Activation/Deactivation Request;

-	MAC CE for the number of Desired Guard Symbols;

-	MAC CE for Case-6 Timing Request;

-	MAC CE for (Extended) Pre-emptive BSR;

-	MAC CE for SL-BSR, with exception of SL-BSR prioritized according to clause 5.22.1.6 and SL-BSR included for padding;

-	MAC CE for IAB-MT Recommended Beam Indication, or MAC CE for Desired IAB-MT PSD range, or MAC CE for Desired DL Tx Power Adjustment;

-	data from any Logical Channel, except data from UL-CCCH;

-	MAC CE for Recommended bit rate query;

-	MAC CE for BSR included for padding;

-	MAC CE for SL-BSR included for padding.

NOTE 2:	Prioritization among MAC CEs of same priority is up to UE implementation.

The MAC entity shall prioritize any MAC CE listed in a higher order than 'data from any Logical Channel, except data from UL-CCCH' over NR sidelink transmission.

## 5.2 Appendix 2

### 5.4.1	UL Grant reception

…… Omitted……

For configured uplink grants configured with cg-RetransmissionTimer, the UE implementation selects an HARQ Process ID among the HARQ process IDs available for the configured grant configuration. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize the HARQ Process ID with the highest priority, where the priority of HARQ process is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. If the MAC entity is configured with intraCG-Prioritization, for HARQ Process ID selection among initial transmission and retransmission with equal priority, the UE shall prioritize retransmissions before initial transmissions. The priority of a HARQ Process for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than the priority of a HARQ Process for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU. If the MAC entity is not configured with intraCG-Prioritization, for HARQ Process ID selection, the UE shall prioritize retransmissions before initial transmissions. The UE shall toggle the NDI in the CG-UCI for new transmissions and not toggle the NDI in the CG-UCI in retransmissions.

NOTE 1:	If a configured uplink grant is associated with a multi-PUSCH configured grant, CURRENT_symbol refers to the symbol index of the first transmission occasion in the first configured uplink grant within the same periodicity. Otherwise, CURRENT_symbol refers to the symbol index of the first transmission occasion of a bundle of configured uplink grant.

NOTE 2:	A HARQ process is configured for a configured uplink grant where neither harq-ProcID-Offset nor harq-ProcID-Offset2 is configured, if the configured uplink grant is activated and the associated HARQ process ID is less than nrofHARQ-Processes. A HARQ process is configured for a configured uplink grant where harq-ProcID-Offset2 is configured, if the configured uplink grant is activated and the associated HARQ process ID is greater than or equal to harq-ProcID-Offset2 and less than sum of harq-ProcID-Offset2 and nrofHARQ-Processes for the configured grant configuration.

NOTE 3:	If the MAC entity receives a grant in a Random Access Response (i.e. MAC RAR or fallbackRAR), or addressed to Temporary C-RNTI or determines a grant as specified in clause 5.1.2a for MSGA payload and if the MAC entity also receives an overlapping grant for its C-RNTI or CS-RNTI, requiring concurrent transmissions on the SpCell, the MAC entity may choose to continue with either the grant for its RA-RNTI/Temporary C-RNTI/MSGB-RNTI/the MSGA payload transmission or the grant for its C-RNTI or CS-RNTI.

NOTE 4:	In case of unaligned SFN across carriers in a cell group, the SFN of the concerned Serving Cell is used to calculate the HARQ Process ID used for configured uplink grants.

NOTE 5:	If cg-RetransmissionTimer is not configured, a HARQ process is not shared between different configured grant configurations in the same BWP.

For the MAC entity configured with lch-basedPrioritization, the MAC entity considers the value of additionalPriority, if configured, as the priority for the LCH for which the running PDCP discardTimer of an PDCP SDU of the LCH that is multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or can be multiplexed(i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU has the remaining value below additionalPriorityThreshold at the time of the first symbol of the PUSCH transmission that includes this MAC PDU.

For the MAC entity configured with lch-basedPrioritization, priority of an uplink grant is determined by the highest priority among priorities of the logical channels that are multiplexed (i.e. the MAC PDU to transmit is already stored in the HARQ buffer) or have data available that can be multiplexed (i.e. the MAC PDU to transmit is not stored in the HARQ buffer) in the MAC PDU, according to the mapping restrictions as described in clause 5.4.3.1.2. The priority of an uplink grant for which no data for logical channels is multiplexed or can be multiplexed in the MAC PDU is lower than either the priority of an uplink grant for which data for any logical channels is multiplexed or can be multiplexed in the MAC PDU or the priority of the logical channel triggering an SR.

For the MAC entity configured with lch-basedPrioritization, if the corresponding PUSCH transmission of a configured uplink grant is cancelled by CI-RNTI as specified in clause 11.2A of TS 38.213 [6] or cancelled by a high PHY-priority PUCCH transmission as specified in clause 9 of TS 38.213 [6], this configured uplink grant is considered as a de-prioritized uplink grant. If this de-prioritized uplink grant is configured with autonomousTx, the configuredGrantTimer for the corresponding HARQ process of this de-prioritized uplink grant shall be stopped if it is running. If this de-prioritized uplink grant is configured with autonomousTx, the cg-RetransmissionTimer for the corresponding HARQ process of this de-prioritized uplink grant shall be stopped if it is running.

When the MAC entity is configured with lch-basedPrioritization, the MAC entity considers the value of additionalPriority, if configured, as the priority for the logical channel triggering an SR, if the running PDCP discardTimer of an PDCP SDU buffered for the LCH has the remaining value below additionalPriorityThreshold at the time of the SR transmission.

When the MAC entity is configured with lch-basedPrioritization, for each uplink grant delivered to the HARQ entity and whose associated PUSCH can be transmitted by lower layers, the MAC entity shall:

1>	if this uplink grant is received in a Random Access Response (i.e. in a MAC RAR or fallback RAR), or addressed to Temporary C-RNTI, or is determined as specified in clause 5.1.2a for the transmission of the MSGA payload:

2>	consider this uplink grant as a prioritized uplink grant.

1>	else if this uplink grant is addressed to CS-RNTI with NDI = 1 or C-RNTI:

2>	if there is no overlapping PUSCH duration of a configured uplink grant which was not already de-prioritized, in the same BWP, whose priority is higher than the priority of the uplink grant; and

2>	if there is no overlapping PUCCH resource with an SR transmission which was not already de-prioritized and the simultaneous transmission of the SR and the uplink grant is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, and the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant:

3>	consider this uplink grant as a prioritized uplink grant;

3>	consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s);

3>	consider the other overlapping SR transmission(s), if any, as a de-prioritized SR transmission(s), except for the SR transmission(s) whose simultaneous transmission is allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup;

3>	if the de-prioritized uplink grant(s) is a configured uplink grant configured with autonomousTx whose PUSCH has already started:

4>	stop the configuredGrantTimer for the corresponding HARQ process of the de-prioritized uplink grant(s);

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process of the de-prioritized uplink grant(s).

1>	else if this uplink grant is a configured uplink grant:

2>	if there is no overlapping PUSCH duration of another configured uplink grant which was not already de-prioritized, in the same BWP, whose priority is higher than the priority of the uplink grant; and

2>	if there is no overlapping PUSCH duration of an uplink grant addressed to CS-RNTI with NDI = 1 or C-RNTI which was not already de-prioritized, in the same BWP, whose priority is higher than or equal to the priority of the uplink grant; and

2>	if there is no overlapping PUCCH resource with an SR transmission which was not already de-prioritized and the simultaneous transmission of the SR and the uplink grant is not allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup, and the priority of the logical channel that triggered the SR is higher than the priority of the uplink grant:

3>	consider this uplink grant as a prioritized uplink grant;

3>	consider the other overlapping uplink grant(s), if any, as a de-prioritized uplink grant(s);

3>	if the de-prioritized uplink grant(s) is a configured uplink grant configured with autonomousTx whose PUSCH has already started:

4>	stop the configuredGrantTimer for the corresponding HARQ process of the de-prioritized uplink grant(s);

4>	stop the cg-RetransmissionTimer for the corresponding HARQ process of the de-prioritized uplink grant(s).

3>	consider the other overlapping SR transmission(s), if any, as a de-prioritized SR transmission(s), except for the SR transmission(s) whose simultaneous transmission is allowed by configuration of simultaneousPUCCH-PUSCH or simultaneousPUCCH-PUSCH-SecondaryPUCCHgroup or simultaneousSR-PUSCH-diffPUCCH-Groups or simultaneousPUCCH-PUSCH-SamePriority or simultaneousPUCCH-PUSCH-SamePriority-SecondaryPUCCHgroup.

NOTE 6:	If the MAC entity is configured with lch-basedPrioritization and if there is overlapping PUSCH duration of at least two configured uplink grants whose priorities are equal, the prioritized uplink grant is determined by UE implementation.

NOTE 7:	If the MAC entity is not configured with lch-basedPrioritization and if there is overlapping PUSCH duration of at least two configured uplink grants, it is up to UE implementation to choose one of the configured uplink grants.

NOTE 8:	If the MAC entity is configured with lch-basedPrioritization, the MAC entity does not take UCI multiplexing according to the procedure specified in TS 38.213 [6] into account when determining whether the PUSCH duration of an uplink grant overlaps with the PUCCH resource for an SR transmission.