3GPP RAN WG2 Meeting #129bis	R2-2502315

Wuhan, China, April 7th – 11th 2025

Agenda Item:	8.8.2

Source:	InterDigital

Title:	Downlink coverage enhancement for NTN

Document for:	Discussion, Decision

# Introduction

An objective for Rel-19 NR non-terrestrial networks (NTN) is to study DL coverage impacts and modifications to support dynamic power sharing among satellite beams. Enhancements can be considered at both the link level (e.g., improving the link margin of selected physical channels) and system level (e.g., supporting dynamic and flexible power sharing between beams). Detailed objectives include the following [1]:

- Specify downlink coverage enhancements targeting support for additional reference satellite payload parameters covering both GSO and NGSO constellations operating in FR1-NTN or FR2-NTN [RAN1, RAN2, RAN4] - Define additional reference satellite payload parameters assuming power sharing among satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint, such that satellite beams may not all be simultaneously active or may be active below the nominal EIRP density per satellite beam (see section 6.1.1 in TR 38.821) due to limited power and limited feeder link bandwidth. - Define the corresponding power sharing assumptions and necessary link level and system level evaluation methodology and relevant KPIs for evaluations of the coverage, to allow for identification of physical channels/signals and system-level aspects that need enhancements and the corresponding needed improvements. - Specify solutions, including link level enhancements for FR1-NTN and system level enhancements for FR1-NTN and FR2-NTN, allowing dynamic and flexible power sharing between satellite beams or different satellite beam patterns/size (i.e. wide or narrow) across the satellite footprint. - Link level enhancements are to be specified for the following channels: - PDCCH at least for CSS (except for Type-3) via PDCCH repetition - PDSCH with Msg4 via PDSCH repetition - PDSCH with SIB1 via 2 PDSCH repetitions within 20 ms duration - System-level enhancements are to be specified for the following: - Support of extended periodicity of the half frames with SS/PBCH blocks assumed by UE during initial access. - The maximum of the additional default value (apart from the existing 20ms value) is 160 ms - Notes for this objective: - SSB channel enhancement other than SSB periodicity extension is not considered - RAN1 should consider issues such as UE’s cell search complexity and impact to initial cell selection, latency and success rate, for the above extension - The SSB periodicity enhancements potentially defined in this WID only apply to NTN operation - Antenna gain of UE shall be assumed to be -5.5dBi in case of smartphone in FR1-NTN, the UE is assumed to be a full duplex UE, and at least 2Rx are considered at the UE - NGSO to be considered in priority: LEO Set-1 @ 600 km

This contribution discusses access control, specifically Cell barring, reselection to a cell oeparting with DL coverage enhancements (CE).

# Discussion

The following was agreed in RAN2#128 [2] regarding access control for UEs supporting Rel-19 NTN downlink coverage enhancements:

Agreements:

1.	If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.)

Then, RAN2 has some discussions on the cell barring mechanism after RAN2#128 meeting and RAN2 concluded to continue discussion on the cell barring mechanism in the coming RAN2 meeting (i.e. RAN2#129bis).

RAN1 have already concluded that the SSB periodicity won’t be extended to longer than 160ms and so from ASN.1 point of view, the legacy UEs can be aware of the SSB configuration for the Rel-19 DL-CE enabled cell. However, there is a possibility that legacy UEs may not work properly with the long SSB periodicity like 160ms and so it’s still better to consider having a cell barring for DL-CE cell to prevent legacy UEs from accessing the DL-CE enabled NTN cell. Thus, we propose;

Proposal 1:	Confirm that UEs supporting Rel-19 NTN DL-CE will not consider the existing NTN barring bit if Rel-19 NTN DL-CE barring IE is present in the system information.

# Conclusion

In this contribution the following observations and proposals are made regarding DL coverage enhancements:

Proposal 1:	Confirm that UEs supporting Rel-19 NTN DL-CE will not consider the existing NTN barring bit if Rel-19 NTN DL-CE barring IE is present in the system information.

# References

RP-243300 – Non-Terrestrial Networks (NTN) for NR Phase 3 – Thales, CATT

R2-2500002 RAN2#128 Meeting Report, MCC