3GPP TSG-RAN WG2 Meeting #129bis	R2-2502316

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.11.2

Source:	Nokia

Title:	Random Access Operation of SBFD

WID/SID:	NR_duplex_evo-Core - Release 19

Document for:	Discussion and Decision

# Introduction

In RAN#102 meeting, a Work Item (WI) on Evolution of NR duplex operation: Subband full duplex (SBFD) was approved. SBFD is being specified in Rel-19 as a new mode of operation in unpaired spectrum where a separate set of RBs within a TDD carrier are used for UL and DL transmission simultaneously. The intention is to allow more UL transmission opportunities compared to the typical DL-heavy TDD configurations used in today’s TDD deployments. This bring benefits in terms of UL coverage and UL latency, especially in wide-area macro networks where many UEs are in power-limited conditions in the UL direction. The work item description has been updated in RAN#104 meeting and the objectives are mentioned in  RP-241614.

RAN2 start discussing on SBFD random access procedures and in RAN2#129, and the following agreements are made:

Agreements When a SBFD aware UE supporting one or both SBFD RACH configuration options accesses a cell, the UE can apply the supported SBFD RACH configuration option in the cell. When a SBFD aware UE supporting a SBFD RACH configuration option accesses a cell configured with a different SBFD RACH configuration option, the UE applies the legacy RA operation, and does not apply the SBFD RACH configuration. For initial RA transmission, the network can indicate the RO type (legacy RO or additional RO) to the SBFD-aware UE for the case of CBRA. Detailed signalling is FFS. If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold. FFS whether RO type selection is performed before or after the RA type selection. FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

In this contribution, we further discuss different aspects of the SBFD random access operation.

# SBFD Random Access Operation

For initial access, the UE selects the index of the preferred SSB index and decodes the associated PBCH for MIB, and obtains relevant information for decoding the SIB1, from where it obtains the RACH configuration for sending Msg1. The RACH configuration contains, among others, the PRACH configuration index, the frequency domain multiplexing of the RACH occasions (ROs), the frequency offset of the ROs, and information about the power ramp up during the RACH procedure. For UEs in RRC_CONNECTED, dedicated RACH configuration, including CFRA, can be indicated by the network. If the CFRA configuration is missing, UEs in RRC_CONNECTED use the common RACH configuration for CBRA.

To enable support of initial access in SBFD, i.e., in the UL subband, RAN1 has been discussing the above aspects of random access procedures in SBFD since RAN1#116. In fact, RAN1 observed that support of initial access in SBFD provides at least one or more following benefits: reduced random access latency, reduced PRACH collision probability or allowing more contiguous frequency resources for PUSCH in UL slots, improved coverage of PRACH with sparse UL resources, and increased cell range of PRACH with sparse UL resources. As mentioned in section 1, WI was updated in RAN#104 [RP-241614] to specify SBFD operation to support random access in SBFD symbols by UEs in RRC_IDLE/INACTIVE mode, along with RRC_CONNECTED mode in RAN1 and RAN2.

In section 3, we present the various aspects of random access in RRC Connected mode, and in RRC Idle/Inactive mode, and the conclusion is provided in section 4.

# Various Aspects of SBFD random access

In this section, we discuss the RAN2 aspects of the SBFD random access procedure for RRC_CONNECTED and IDLE/INACTIVE operations.  RAN1 agreements on SBFD random access is listed in Appendix 1.

## PRACH resource configuration

To enable random access for SBFD symbols in SBFD-aware UEs, the gNB must set up a new set of PRACH resources which includes RO configuration, SSB-RO mapping, the random access configuration table, and PRACH repetition.  Since Option 2 necessitates support for two RACH configurations, it introduces some overhead, especially when redundant information is present in these configurations. Hence to reduce the signaling overhead, if the legacy ROs and the additional ROs have the same value for one of the RACH configuration parameters it would not be needed to include those parameters in the additional RACH configuration. In such a scenario, inclusion in the legacy RACH configuration would be enough.

Proposal 1: For Option 2, in case the legacy ROs and the additional ROs have the same value for one of the RACH configuration mandatory parameters, only the legacy RACH configuration will be configured with this parameter.  For Option 2, in case the SBFD aware UEs did not receive a dedicated mandatory parameter in the additional RACH configuration, the SBFD-aware UEs determine this dedicated parameter from the legacy RACH configuration.

## RACH Occasion selection and prioritization

For both RACH configuration Option 1 and Option 2 agreed in RAN1, SBFD-aware UEs will have the possibility of transmitting RACH on legacy ROs and SBFD ROs, while legacy UEs would only be capable of using legacy ROs.

In the last meeting, we agreed on the UE selection between the legacy and the additional ROs based on certain specified/configured conditions/prioritizations.

RAN2#129 If no RO type indication is provided by the NW, a UE selects RO type based on a SSB RSRP threshold. FFS whether NW can further indicate whether to select the additional RO type below or above this SSB RSRP threshold.

In the following we present some of these conditions:

RO selection based on load balancing and coverage

For load balancing purposes, it is beneficial that SBFD-aware UEs prioritize as much as possible RACH transmissions on SBFD ROs and avoid consuming the quite-limited capacity of legacy RACH occasions. Considering that UE-to-UE CLI may only be a problem in SBFD ROs but not in legacy ROs, and may be especially critical in cell-edge areas (where the UL transmission power is high, while the DL received signal power is low), the selection between legacy ROs and SBFD ROs may also be based UE’s coverage conditions. For instance, a cell-centre UEs transmit RACH on SBFD ROs, while a cell-edge SBFD-aware UE uses ROs belonging to the legacy RACH configuration.

Observation 1: For CBRA from the SBFD-aware UE perspective and to have better load balancing across different RACH resources,  SBFD ROs shall have higher priority than legacy (ROs). The priority between SBFD and legacy ROs can be based on the UE’s coverage conditions.

Since the RACH configuration Option 1 is focused on reducing the latency of RA for SBFD UEs in good coverage area, we propose SBFD UEs selects the additional RO for initial access when the measured RSRP is above the RSRP threshold.

Proposal 2: For RACH configuration Option 1, the UE selects the additional RO for initial access when the measured RSRP is above the RSRP threshold.

As the RACH configuration Option 2 focuses on increasing the coverage, due to the possibility of supporting of two PRACH formats with different lengths, SBFD-aware UEs have the possibility to select a suitable preamble format based on measured RSRP.  In Figure 2, one UE (UE1) is within cell range 1, while the other (UE2) is in cell range 2. UE1, located closer to the network with strong SS-RSRP, benefits from using the short-length PRACH format for access. In contrast, UE2, situated in a coverage-deficient area (cell range 2), requires the long-length PRACH format to access the network effectively. Due to the support of two PCIs in Option 2, these two UEs can select the best PRACH format types based on the SS-RSRP.Since the RACH configuration Option 2 is focused  on extending coverage of RA for SBFD UEs, we propose SBFD UEs selects the additional RO for initial access when the measured RSRP is below the RSRP threshold.

Proposal 3: For RACH configuration Option 2, in case the additional prachConfigurationIndex indicates long PRACH preamble formats, the UE selects the additional ROs for initial access when the measured RSRP is below the RSRP threshold.

Figure 2 SBFD aware UEs select the best PRACH preamble format based on their location regarding the NW.

## RA-RNTI collision in SBFD

For the additional RACH configuration in option 2, designed for SBFD-aware UEs, a challenge is avoiding RA-RNTI confusion caused by having two sets of RACH resources. The RA-RNTI is used by a UE to receive the Random Access Response (RAR) once the PRACH is transmitted. Different RACH occasions within a system frame are assigned unique RA-RNTI values to ensure that the RAR for a given PRACH transmission is directed only to the UE that used that specific RACH occasion. If two RACH occasions share the same RA-RNTI value, there is a risk that two UEs using different RACH occasions could incorrectly act on the same RAR message.

More specifically, the UE calculates the RA-RANTI when sending the Msg 1 of the RA procedure. According to TS38.321, the RA-RNTI is computed as

RA-RNTI = 1 + s_id + 14 × t_id + 14 × 80 × f_id + 14 × 80 × 8 × ul_carrier_id

where,

- s_id is the index of the first OFDM symbol of the PRACH occasion (0 ≤ s_id < 14)

- t_id is the index of the first slot of the PRACH occasion in a system frame (0 ≤ t_id < 80)

- f_id is the index of the PRACH occasion in the frequency domain (0 ≤ f_id < 8)

- ul_carrier_id is the UL carrier used for Random Access Preamble transmission (0 for NUL carrier, and 1 for SUL carrier)

For PRACH configuration Option 2 (i.e., an SBFD-aware UE is configured with a legacy and additional PRACH configurations), there is a possibility that the SBFD ROs configured by the additional configurations may overlap with the legacy ROs. This could happen in flexible symbols (as both legacy and SBFD ROs can be configured in flexible symbols). When this overlapping happens, there is a non-negligible possibility that at least s_id and t_id of the overlapping legacy RO and SBFD RO are the same (and of course, the ul-carrier_id is the same since we are talking about the same carrier). Now the tricky part is the f_id. If the legacy specs is kept as is, then at the point when there is such overlapping, there is a mismatch between the frequency indices being used by legacy UEs and the ones being used by SBFD UEs. This leads to the same RA-RNTI and confuses these UE when receiving the same RA-RNTI.

Observation 2: In PRACH configuration Option 2, RA-RNTI collision happens when the SBFD ROs configured by the additional configurations may overlap with the legacy ROs in flexible symbols.

In this scenario, collision can be avoided by defining a new rule for indexing of f_id when there is overlapping without modifying the existing RA-RNTI formula. Therefor we propose:

Proposal 4: When there is overlapping, the SBFD-aware UE determines the frequency index (f_id) by:

- Firstly, determining the frequency index of the legacy ROs by ignoring the SBFD ROs (i.e., aiming at keeping the same legacy index for the legacy ROs).

- Secondly, determining the frequency index of SBFD ROs by continuing the indexing starting from the index used for the last legacy RO plus one (and ignoring the legacy ROs).

## RO type switching from non-SBFD symbols to SBFD symbols.

In RAN2#129 RAN2 made following agreement on initial attempt and re-attempt of the RACH procedure for an SBFD-aware UE.

RAN2#129 Agreements FFS if switching from the PRACH resources in non-SBFD symbols to the PRACH resources in SBFD symbols is supported.

Concerning the FFS, there was some discussion between companies in case the first attempt was on legacy ROs, whether there is any reason to fall back to SBFD-ROs for the re-attempt since legacy ROs already had a better interference condition compared to the additions ROs.

In fact, a successful initial access may not only depend on the interference condition in the RO. Indeed, an SBFD-aware UE could fail to achieve the initial access using the legacy symbols due to two main reasons:

- The SBFD-aware UE did not receive an RAR response from the NW (Msg1 failed to be received by the NW). A failure in the RAR response is an indication of bad coverage of PRACH.

- The SBFD-aware UE did not receive the contention resolution message (Msg4). A failure in the Msg4 response is an indication of a collision happening between different UEs.

  - Note that the probability of collision in the RACH procedure may depend at least on the number of users trying to perform random access on RACH resources. For SBFD, collision probability may be very different between SBFD ROs and legacy ROs considering only SBFD-aware (Rel-19 or later) UEs may be capable of transmitting in the SBFD ROs.

It can be observed that UE can distinguish between the two reasons of failure. For each reason of failure, UE may decide whether it would fall back to SBFD RO (from UL RO) or not, based on coverage condition and configuration.

Proposal 5: For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in legacy RACH occasions, UE is allowed to switch to SBFD RACH occasions.

The following agreements are made in RAN2#127bis.

RAN2#127Bis Agreements - For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in SBFD RACH occasions, UE is allowed to switch to legacy RACH occasions. FFS about the case when UE select legacy ROs first.

However, the agreed way of RO type switching has the issue mentioned below.

If a UE initially selects an SBFD RO, but after multiple attempts, the NW observes that the UL signal strength does not improve and is unlikely to change in the next several attempts (e.g., 5 attempts), or NW observes that there is preamble congestion in the selected RO type,  the NW realize that a legacy RO would provide a better chance of success. However, the current procedure requires the UE to exhaust all preambleTransMax attempts before switching to the more suitable legacy RO.  This results in unnecessary latency, as the UE keeps retrying in an inefficient RO type before switching, even when the network has a better alternative available.

Proposal 6: Discuss whether to allow the NW to indicate to the SBFD UEs via RAR message to switch the RO type.

## RACH Signalling Aspects.

The following open issues were identified in the Post129][217][SBFD] List of open issues of RRC impact (Huawei)

[Proposal 1] On RO type signaling for CFRA, discuss/conclude on: 1. For CFRA triggered by BFR, the RO type is indicated in BeamFailureRecoveryConfig. 2. For CFRA triggered by ReconfigurationwithSync, the RO type is indicated in RACH-ConfigDedicated. 3. Within the corresponding RACH configuration for the additional ROs, one bit is used to indicate the RO type for CFRA. 4. For CFRA triggered by PDCCH order including UL early sync, RAN2 can wait/follow RAN1 agreement. [Proposal 2] : Discuss option issue on the UE behaviour when the signalling on the RO type and RSRP threshold are absent. Consider solution options 1) either NT indication on RO type or on RSRP threshold is madatorily present. 2) When both NT indication on RO type and RSRP threshold are absent, the SBFD-aware UE should prioritize the additional RO over the legacy RO. [Proposal 3] : Choose one option below on whether confirm WA "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. " and discuss related RRC impact: Option 1: Confirm the WA "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. ". Option 2: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for Position SI request and SI request. [Proposal 4] RAN2 waits for conclusion from RAN3 and RAN1 on NZP CSI-RS resources configuration and SBFD frequency configuration information.

On [Proposal 2]:  Discuss option issue on the UE behaviour when the signalling on the RO type and RSRP threshold are absent. Consider solution options 1) either NT indication on RO type or on RSRP threshold is madatorily present. 2) When both NT indication on RO type and RSRP threshold are absent, the SBFD-aware UE should prioritize the additional RO over the legacy RO.

It seems logical that if no limitations (RO type/RSRP threshold) are configured by network for RO selection then UE selects first available resource regardless of RO type in order not to delay RACH procedure.

Proposal 7: UE selects the first available RO type (legacy RO or additional RO) among all of the RACH resources valid to the UE for its CBRA based access, when the indication (RO type/RSRP threshold) from the network is absent.

Regrading [Proposal 3] : Choose one option below on whether confirm WA "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. " and discuss related RRC impact:

Option 1: Confirm the WA "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. ".

Option 2: Random access procedure in SBFD symbols is supported for all the existing RACH trigger events except for Position SI request and SI request.

We have not identified reasons not to support random access procedure in SBFD symbols is supported for all the existing RACH trigger events. There were comments that it would be complex to support SBFD symbols for SI request procedures but we are not yet seen what is the difficulty. In RRC the indication of the RO type may be given in the RACHconfigCommon., which is easily conveyed in dedicated signalling as well. Thus the UE can choose the SBFD symbol to send the SI request as well similar to other RACH triggering scheme, and can have a unified solution for the RA. Thus in fact it may be  easier to have a common SBFD RACH framework for all RACH trigger events. Although there will be additional specification efforts for SI request, the changes will be similar/same as other RACH trigger events. Thus, we would like to confirm WA unless we identify that it requires great complexity in the MAC.

Proposal 8: Confirm WA "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. " unless there will be big impact to MAC to support it.

# Conclusion

Observation 1: For CBRA and CFRA, from the SBFD-aware UE perspective and to have better load balancing across different RACH resources,  SBFD ROs shall have higher priority than legacy (ROs). The priority between SBFD and legacy ROs can be based on the UE’s coverage conditions.

Observation 2: In PRACH configuration Option 2, RA-RNTI collision happens when the SBFD ROs configured by the additional configurations may overlap with the legacy ROs in flexible symbols.

Observation 3: It is not possible for the gNB to distinguish between legacy UEs and SBFD aware UEs based on the legacy RACH resources used by the UEs.

Proposal 1: For Option 2, in case the legacy ROs and the additional ROs have the same value for one of the RACH configuration mandatory parameters, only the legacy RACH configuration will be configured with this parameter.  For Option 2, in case the SBFD aware UEs did not receive a dedicated mandatory parameter in the additional RACH configuration, the SBFD-aware UEs determine this dedicated parameter from the legacy RACH configuration.

Proposal 2: Since the RACH configuration Option 1 is focused on reducing the latency of RA for SBFD UEs in good coverage area, we propose SBFD UEs selects the additional RO for initial access when the measured RSRP is above the RSRP threshold.

Proposal 3:  For RACH configuration Option 2, in case the additional prachConfigurationIndex indicates long PRACH preamble formats, the UE selects the additional ROs for initial access when the measured RSRP is below the RSRP threshold.

Proposal 4: When there is overlapping, the SBFD-aware UE determines the frequency index (f_id) by:

- Firstly, determining the frequency index of the legacy ROs by ignoring the SBFD ROs (i.e., aiming at keeping the same legacy index for the legacy ROs).

- Secondly, determining the frequency index of SBFD ROs by continuing the indexing starting from the index used for the last legacy RO plus one (and ignoring the legacy ROs).

Proposal 5: For the PRACH transmission re-attempt in one RACH procedure, after certain (configured) number of times of RACH attempt in legacy RACH occasions, UE is allowed to switch to SBFD RACH occasions.

Proposal 6: Discuss whether to allow the NW to indicate to the SBFD UEs  via RAR message to switch the RO type.

Proposal 7: UE selects the first available RO type (legacy RO or additional RO) among all of the RACH resources valid to the UE for its CBRA based access, when the indication (RO type/RSRP threshold) from the network is absent.

Proposal 8: Confirm WA "Random access procedure in SBFD symbols is supported for all the existing RACH trigger events. " unless there will be big impact to MAC to support it.