3GPP TSG-RAN WG2 #129bis	R2-2502321

Wuhan, China, 7 – 11 April 2025

Agenda item:		8.6.3	L1 event triggered measurement reporting

Title:		Details of MAC CE for event-triggered L1 measurement report

Source:		NEC

Document for:		Discussion and Decision

# 1. Introduction

In RAN2#129, a lot of agreements below were reached for event-triggered L1 measurement report [1]. Post-meeting email discussion has been done for MAC running CR [2].

Agreements on L1 event-triggered MR:

- Confirm on the previous agreement that "For measurement resource configuration, R18 LTM CSI resource configuration is reused".

- Continue to carry the bits for SSB resource set when CSI-RS resource set is configured. FFS whether CSI-RS can be configured without SSB (up to R1 to decide).

- Do not create a new resource configuration just for the event triggered measurements.

- Confirm on the previous agreement "For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused".

- Ask RAN1 what should be the maximum number of beam measurement results that can be reported in event-triggered measurement report.

- Reuse the existing RSRP values and RSRP offset values and their interpretations in TS 38.133 for event-triggered measurement report. Can revisit if RAN4 identifies any issue.

- TimeToTrigger and reportInterval of L3 measurement report can be reused. FFS on additional value(s) of reportInterval.

- Cell specific offset is not supported for LTM L1 event evaluation.

- The trigger quantity for LTM L1 event triggered measurement is the same as the report quantity.

- Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

- Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

- R18 differential RSRP encoding is used as baseline.

- LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant.

- Select the beam not satisfying the event condition, based on the measured quality (i.e. according to the best beam(s))

- Truncated MR MAC CE includes the triggered beam information:

- One report config ID (event config ID)

- At least one triggered beam id (SSBRI or CRI) + L1 RSRP

- Common format for normal MR MAC CE and truncated MR MAC CE is used. Details are handled by MAC running CR rapporteur.

- For truncated LTM MR MAC CE, an LCID separate from that for non-truncated LTM MR MAC CE is used.

- S-measureConfig is applicable to in the L1 event-triggered MR. No additional change is expected on RRC and MAC.

In this contribution, we discuss some remaining issues on signalling details and provide our views.

# 2. Discussion

2.1	Remaining issues on MR MAC CE

There were many agreements on event-triggered L1 measurement report and MR MAC CE, while there are some remaining issues (FFS). We discuss those in the following.

- Report Interval

TTT and reportInterval of L3 measurement report can be reused basically. It is FFS whether to introduce additional value of reportInterval, where currently the minimum value is 120ms.

- TimeToTrigger and reportInterval of L3 measurement report can be reused. FFS on additional value(s) of reportInterval..

For Rel-18 LTM, periodic CSI report reuses the legacy CSI-ReportPeriodicityAndOffset, where the periodicity values are {slots4,slots5,slots8,…, slots320}. For the event-triggered L1 measurement report, lower values of these periodicity values look so short. Considering that the motivation of event-triggered reporting is basically the same as L3 measurement report, there seems to be no strong reason to introduce additional (shorter) values for report interval. Note that if necessary, the lower values like {40ms,80ms} may be candidate values, which are similar to the shorter values of TTT.

Proposal 1: RAN2 to confirm that existing reportInterval of L3 measurement report can be reused for event-triggered L1 measurement report.

- Explicit indication for beam(s) meeting (entering) event condition

As agreed in the last meeting, MR MAC CE can include both beam(s) meeting the event condition with TTT and beam(s) not meeting the event condition (if configured by the NW). It is FFS on the detailed signaling.

- Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling.

For the explicit indication, there can be two options: 1) explicit indication per beam or 2) explicitly indicate the number of beams meeting the condition.

The Option 1 is very simple, and one bit is reserved for any beam to be included. There is slight overhead especially when only one beam out of N beams meets the condition.

The Option 2 is to fill the beam(s) meeting the condition at first, and then fill other beam(s) in descending order of report quantity (L1 RSRP). In the beginning of the MAC CE, a few bits are reserved to indicate how many beams meet the condition. For example, if maximum number of beams to be reported “N” is 8, then 3 bits are reserved. Compared with the Option 1, 5 bits can be saved.

It is noted that currently RAN2 is asking the value “N” to RAN1 [3] and thus RAN2 may decide the option based on a reply, if necessary.

Proposal 2: For explicit indication for beams meeting the event condition, RAN2 to discuss two options  and then decide either of two.

- Option 1: Add 1 bit indication per beam

- Option 2: Indicate the number of beams meeting the event condition with including the corresponding beams at the beginning of the MR MAC CE.

- Explicit indication for beam(s) meeting leaving event condition

Firstly, there were some divergent views among companies on the need of this explicit indication in the last meeting. It seems good to introduce a simpler way (if possible).

- Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

The basic assumption is that the common MR MAC CE format is used also for the reporting of beam(s) meeting the leaving condition. The simplest solution is to give a restriction that the UE can only report for beam(s) meeting the entering condition or beam(s) meeting the leaving condition in single MR MAC CE. If this restriction is feasible, the same mechanism for the entering condition can be reused by NW interpretation, i.e. if the NW knows the corresponding beam already met the entering condition and was reported already, the NW can recognise the beam meets the leaving condition now.

Another simple solution is to use the same mechanism as the reporting of the beam(s) meeting the entering condition, i.e. Option 1/2 above. If the number “N” is 8, the Option 1 reserves total 16 bits (2 bits per beam) and the Option 2 reserves total 6 bits (3 bits each). As we assume the restriction above seems not so feasible, this latter solution would be more reasonable.

Proposal 3: For explicit indication for beams meeting the leaving condition, RAN2 to apply the same way as the beams meeting the entering condition, which means the following way.

- Option 1 direction: Add 1 bit indication per entering/leaving condition per beam.

- Option 2 direction: Indicate the number of beams meeting entering/leaving condition respectively with including the corresponding beams meeting the entering condition and then the corresponding beams meeting the leaving condition at the beginning of the MR MAC CE.

2.2	Prioritization among MR MAC CEs

As agreed in the last meeting, “LTM MR MAC CE only include one triggered measurement event and the LTM MR MAC CE can be multiplexed in any available UL grant”. This means if more than one triggered MR MAC CEs exist while the UE is waiting for the UL grant and the received UL grant is not sufficient to send all the MR MAC CEs, the UE may need to select one or some of MR MAC CEs. Example scenarios are:

- The same event triggers again the L1 measurement report by other beam(s).

- A different event also triggers the L1 measurement report by the same or other beam(s).

- Expiry of event-periodic timer for a different event also triggers the L1 measurement report again.

As there are many scenarios are possible, it may not be so easy or simple to define a certain rule. The simplest solution is that the UE reports the MR MAC CE triggered at first or earlier, i.e. first-in first-out.

Another simple solution is that the network gives a priority order among configured events. For example, the network may give higher priority for the event configured mainly for the LTM cell switch decision (e.g., Event LTM3), while the network may give lower priority for the event configured mainly for the early sync decision (e.g., Event LTM2). This can be realized by adding one flag (e.g., reportPriority) for each event. Instead of priority among the events, the network may give priority order among reportConfig. This is useful to avoid undesirable delay to trigger the LTM cell switch.

We are also open for further discussions on other ways including the way leaving it up to UE implementation which MR MAC CE(s) is reported with priority, i.e. UE reports which one at first or earlier.

Proposal 4: RAN2 to discuss the case where more than one triggered MR MAC CEs are pending but not all of them can be reported in the received UL grant, including the solution options:

- UE reports from the first/earlier triggered MR MAC CE(s).

- Network gives priority order for each configured event/reportConfig and the UE reports MR MAC CE(s) following the priority order.

- it is up to the UE implementation which MR MAC CE(s) is reported.

Fig.1: Example scenarios of triggering MR MAC CE

# 3. Conclusion

In this contribution we discussed some remaining issues on signalling detail for event-triggered L1 measurement report and made the following proposals.

Proposal 1: RAN2 to confirm that existing reportInterval of L3 measurement report can be reused for event-triggered L1 measurement report.

Proposal 2: For explicit indication for beams meeting the event condition, RAN2 to discuss two options (1 bit indication per beam or a few bits to indicate the number of beams meeting the event condition) and then decide either of two.

Proposal 3: For explicit indication for beams meeting the leaving condition, RAN2 to apply the same way as the beams meeting the entering condition, which means the following way.

- Option 1 direction: Add 1 bit indication per entering/leaving condition per beam.

- Option 2 direction: Indicate the number of beams meeting entering/leaving condition respectively with including the corresponding beams meeting the entering condition and then the corresponding beams meeting the leaving condition at the beginning of the MR MAC CE.

Proposal 4: RAN2 to discuss the case where more than one triggered MR MAC CEs are pending but not all of them can be reported in the received UL grant, including the solution options:

- UE reports from the first/earlier triggered MR MAC CE(s).

- Network gives priority order for each configured event/reportConfig and the UE reports MR MAC CE(s) following the priority order.

- it is up to the UE implementation which MR MAC CE(s) is reported.

# References

[1] R2-2501331, R17/18 V2X/SL, R18/19 MOB and R19 NES, Vice Chairman (Samsung)

[2] R2-250xxxx, Running MAC CR for enhanced mobility Ph4 ([POST129][109][MOB] vivo)

[3] R2-2501485, LS on number of beam measurements in the measurement report MAC CE, To RAN1

# Annex for reference

# A1. RAN2 agreements

Agreements on L1 event-driven MR @RAN2#128

- TTT is evaluated per beam, and measurement report is only triggered by beam that has satisfied the condition (entering/leaving) for the whole duration of TTT.

- TTT timer is not restarted if the current beam changes and the entering condition is still met with the new current beam.

- TTT is applied to the leaving condition.

- Network can configure which RS type (SSB or CSI-RS) is used for LTM event evaluation.

- Either CSI-RS or SSB could be configured as candidate beam and the measurement RS of the serving cell beam is determined based on the candidate beam to ensure same RS type, i.e. the RS for current beam of serving cell is same as or QCLed with the QCL RS of the indicated TCI state, according to what is agreed in RAN1.

- SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

- For event-triggered L1 LTM measurement reporting, max N is total number of all beams included into MR MAC CE.

- For event-triggered L1 LTM measurement reporting, NW controls if the beam(s) not satisfying the event could be reported according to N beams in MR MAC CE.

- A single MAC CE format for the event-triggered L1 measurement report is used for both the SSB and CSI-RS reference signals.

- Support the truncated measurement report MAC CE.

Agreements on L1 event triggered MR @RAN2#127bis

- MR can be sent when the leaving condition is met, based on NW configuration.

- Event triggered periodic MR can be supported, based on NW configuration.

- For measurement resource configuration, R18 LTM CSI resource configuration is reused if possible. If CSI-RS resource only IE needs to be defined, we can revisit it in the stage 3.

- For measurement reporting configuration, R18 LTM-CSI-ReportConfig is reused if possible. We can revisit it in the stage 3 if needed.

- For association between measurement resource configuration and measurement reporting configuration, R18 LTM way is reused if possible. We can revisit it in the stage 3 if needed.

- The entire event evaluation procedure is handled by MAC based on the latest L1 measured results reported by L1.

- TTT operates only based on a timer (like TTT used in L3 event triggered MR).

- Confirms WA (Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5).

- Basic information included in MR MAC CE:

- Beam information: FFS if SSBRI/CRI of N beams or (LTM configuration id + SSB/CSI-RS id)

- Beam quantity: L1-RSRP or SINR (up to RAN1) of N beams

- Triggered event information (e.g., ReportConfigID)

- MR MAC CE can include up to N beams (FFS whether the beam should satisfy the event or not). N is configurable by NW.

- Additional information included in MR MAC CE:

- The information and quantity of current beam, based on NW configuration.

11. The legacy SR procedure for resource allocation is the baseline to send the event-triggered L1 measurements MAC CE.

12. NW can configure a dedicated SR configuration for MR MAC CE transmission.

Agreements on L1 MR event evaluation @RAN2#127

- Event LTM1 is not defined.

- Current beam (i.e. a beam corresponding to the indicated TCI state) is used for event evaluation in L1 measurement reporting for serving cell.

- Any beam in candidate RS configuration can be used for LTM event evaluation.

- Beam level measurement result, not cell level measurement result, is used LTM event evaluation. FFS on the conditional LTM case.

- R19 LTM event triggered measurement configuration can be taken as baseline:

- LTM measurement resource configuration is provided in LTM-config.

- Event triggered report config is provided in serving cell config.

6. 	MAC layer handles the event evaluation and measurement report triggering.

Agreements on L1 measurement reporting

- Event-triggered L1-measurements are reported by the UE to the network via MAC CE.

Agreements on measurement enhancements for LTM @RAN2#126

- Event triggered L1 measurement should be designed for the following LTM purposes:

- Select the candidate beam/cell to trigger early synchronization.

- Select the target beam/cell and trigger LTM cell switch procedure.

2. 	For event triggered L1 measurement, use of beam level measurement result for event evaluation is baseline. FFS for the cell level measurement.

3.	Support the following LTM events based on beam specific quality of serving cell and candidate cells as the L1 LTM measurement events.

- Event LTM2: Beam of serving cell becomes worse than absolute threshold;

- Event LTM3: Beam of candidate cell becomes amount of offset better than beam of serving cell;

- Event LTM4: Beam of candidate cell becomes better than absolute threshold;

- Event LTM5: Beam of serving cell becomes worse than absolute threshold1 AND Beam of candidate cell becomes better than another absolute threshold2.

FFS on what beam(s) of the serving cell and neighboring cell is used for event evaluation. FFS on the need of Event LTM1.

4.	Support the beam config of both SSB and CSI-RS in L1 measurement resource configuration in LTM config. Working assumption: Same RS type should be used for both serving and neighbouring cell for event LTM3 and event LTM5.

5.	RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

6.	For LTM event evaluation, TTT, hysteresis for entering/leaving, and/or beam specific (FFS for cell specific) offset can be applied. FFS on the need of measurement reporting once leaving condition is met.

Agreements on measurements @RAN2#125bis

- L1 LTM measurement event configuration is associated with L1 measurement resource configuration provided in LTM configuration via RRC signaling.