3GPP TSG-RAN WG2 #129bis	R2-2502322

# Wuhan, China, 7th – 11th April 2025

Agenda Item:	7.6

Source:	Ericsson, Apple, Qualcomm Incorporated, vivo

Title:	Discussion on resource re-selection from SL LBT Failure indication

Document for:	Discussion, Decision

# 1	Introduction

we discuss the issues on resource re-selection from SL LBT Failure indication in this paper.

# 2	Discussion

The issue on resource re-selection from SL LBT failure indication has been disagreed in RAN#129. The below progress was made

R2-2500875     SL Resource reselection in case of LBT failure Ericsson              discussion        Rel-18  NR_SL_enh2-Core

Proposal 1: In the current MAC procedure specified in clause 5.22.1.2c (which is specified for non MCSt case), “the selected number of HARQ retransmissions” need to be deleted in the procedure texts.

Proposal 2: add a note in clause 5.22.1.2c stating that the procedure specified in clause 5.22.1.2c is only applicable for non MCSt. FFS if more changes are needed for the case of multiple TB MCSt using single slot candidate resources.

Proposal 3: add “as specified in clause 5.22.1.1” in the procedure texts in clause 5.22.1.2

[Huawei, Lenovo, Apple, Ericsson]: For proposal 2, it needs to be specified as normative text. [Vivo]: With normative text, “FFS if more changes are needed for the case of multiple TB MCSt using single slot candidate resources” may need to be discussed. It is good to have complete texts all together. [Session chair]: Let’s postpone it to the next meeting.

We’ll capture proposal 2 as normative text.

CR will be revisited next meeting (also with the consideration of FFS).

Based on the above discussion progress and agreements, below observations can be made.

RAN2 agreed to address the issue by capturing normative texts in the spec.

Based on online and offline discussions, there are two different understanding for the way forward to address the issue.

Understanding 1 - the resource reselection behavior for single slot-based Multi-TB MCSt cases is the same as non-MCSt case, since the processes of different TBs are independently handled, even though the tx occasions of different TBs are consecutive to form an MCSt. So, for any tx occasion of any TB, the UE triggers resource reselection only for the occasion where LBT failure occurs.

Understanding 2 - the UE will only perform resource selection for a TB or multiple TBs after LBT failures happen for all slots, which is same as what agreed for single TB MCSt case:

It is worth noting that there may be two scenarios for single slot candidate resources based Multi-TB MCSt cases, as shown in the below Figure 1.

Figure 1. single slot candidate resource based Multi-TB MCSt

Multi-TB MCSt case using single slot candidate resources comprise two scenarios

  - Scenario 1 – the MCSt resources includes consecutive slots for one TB followed by another TB e.g., TB1-TB1-TB2-TB2

  - Scenario 2 - the MCSt resources includes consecutive slots for initial TBs followed by retransmission TBs e.g., TB1-TB2-TB1-TB2

Therefore for Understanding 2, resource reselection is performed for the above scenarios

Scenario 1 – trigger resource reselection after slot2 (for TB1) and trigger resource reselection after slot 4 (for TB2), which means that the MAC entity needs to perform resource reselection for different HARQ processes at different time instants

Scenario 2 – trigger resource reselection after slot 4 (for both TB1 and TB2), which means that the MAC entity needs to perform resource reselection for multiple HARQ processes at the same time

For Scenario 1, it is obviously that, with understanding 2, the intention is to extend the (confirmed) working assumption made at RAN2#123 to scenario 1, i.e.,

Working assumption: Trigger resource (re)selection if all initial transmission and retransmission within MCSt fail due to LBT failure. It should provide minimum specification change.

For Scenario 2, the understanding 2 would require the MAC entity to trigger resource reselection for multiple HARQ processes at the same time.

Understanding 2 requires below handling procedures for the different scenarios

  - Scenario 1 – RAN2 needs to extend the WA “Trigger resource (re)selection if all initial transmission and retransmission within MCSt fail due to LBT failure” to be applicable to both single-slot single-TB case and multi-slot single-TB case.

  - Scenario 2 - the MAC entity to trigger resource reselection for multiple HARQ processes at the same time.

RAN2 needs to conclude which understanding should be the way forward.

RAN2 to discuss the below understandings for the Multi-TB MCSt case using single slot candidate resources:

  - Understanding 1 - the resource reselection behavior for single slot-based Multi-TB MCSt cases is the same as non-MCSt case.

  - Understanding 2 – trigger resource reselection for one TB or multiple TBs after LBT failures happen for all slots.

RAN2 to discuss the below RAN2 impacts for Understanding 2:

  - For the scenario where the MCSt resources include consecutive slots for one TB followed by another TB e.g., TB1-TB1-TB2-TB2, RAN2 needs to extend the WA “Trigger resource (re)selection if all initial transmission and retransmission within MCSt fail due to LBT failure” to be applicable to both single-slot single-TB case and multi-slot single-TB case.

  - For the scenario where the MCSt resources include consecutive slots for initial TBs followed by retransmission TBs e.g., TB1-TB2-TB1-TB2, the MAC entity needs to trigger resource reselection for multiple HARQ processes at the same time.

For both options, we have prepared corresponding text proposals in section 4.

Adopt one of the understandings and the corresponding TP captured in Appendix.

# 3 Conclusion

In the previous sections we made the following observations:

Observation 1	RAN2 agreed to address the issue by capturing normative texts in the spec.

Observation 2	Multi-TB MCSt case using single slot candidate resources comprise two scenarios

a.	Scenario 1 – the MCSt resources includes consecutive slots for one TB followed by another TB e.g., TB1-TB1-TB2-TB2

b.	Scenario 2 - the MCSt resources includes consecutive slots for initial TBs followed by retransmission TBs e.g., TB1-TB2-TB1-TB2

Observation 3	Understanding 2 requires below handling procedures for the different scenarios

a.	Scenario 1 – RAN2 needs to extend the WA “Trigger resource (re)selection if all initial transmission and retransmission within MCSt fail due to LBT failure” to be applicable to both single-slot single-TB case and multi-slot single-TB case.

b.	Scenario 2 - the MAC entity to trigger resource reselection for multiple HARQ processes at the same time.

Based on the discussion in the previous sections we propose the following:

Proposal 1	RAN2 to discuss the below understandings for the Multi-TB MCSt case using single slot candidate resources:

a.	Understanding 1 - the resource reselection behavior for single slot-based Multi-TB MCSt cases is the same as non-MCSt case.

b.	Understanding 2 – trigger resource reselection for one TB or multiple TBs after LBT failures happen for all slots.

Proposal 2	RAN2 to discuss the below RAN2 impacts for Understanding 2:

a.	For the scenario where the MCSt resources include consecutive slots for one TB followed by another TB e.g., TB1-TB1-TB2-TB2, RAN2 needs to extend the WA “Trigger resource (re)selection if all initial transmission and retransmission within MCSt fail due to LBT failure” to be applicable to both single-slot single-TB case and multi-slot single-TB case.

b.	For the scenario where the MCSt resources include consecutive slots for initial TBs followed by retransmission TBs e.g., TB1-TB2-TB1-TB2, the MAC entity needs to trigger resource reselection for multiple HARQ processes at the same time.

Proposal 3	Adopt one of the understandings and the corresponding TP captured in Appendix.

# 4 Appendix - Text proposal

## 4.1 Understanding 1 - Text proposal

Start of the first change

#### 5.22.1.2	TX resource (re-)selection check

If the TX resource (re-)selection check procedure is triggered on the selected pool of resources for a Sidelink process according to clause 5.22.1.1, the MAC entity shall for the Sidelink process:

1>	if PSCCH duration(s) and 2nd stage SCI on PSSCH for all transmissions of a MAC PDU of any selected sidelink grant(s) are not in SL DRX Active time as specified in clause 5.28.3 of the destination that has data to be sent; or

1>	if SL_RESOURCE_RESELECTION_COUNTER = 0 and when SL_RESOURCE_RESELECTION_COUNTER was equal to 1 the MAC entity randomly selected, with equal probability, a value in the interval [0, 1] which is above the probability configured by RRC in sl-ProbResourceKeep; or

1>	if the pool of resources is configured or reconfigured by RRC; or

1>	if there is no selected sidelink grant on the selected pool of resources; or

1>	if neither transmission nor retransmission has been performed by the MAC entity on any resource indicated in the selected sidelink grant during the last second; or

1>	if sl-ReselectAfter is configured and the number of consecutive unused transmission opportunities on resources indicated in the selected sidelink grant, which is incremented by 1 when none of the resources of the selected sidelink grant within a resource reservation interval is used, is equal to sl-ReselectAfter; or

1>	if the selected sidelink grant cannot accommodate a RLC SDU by using the maximum allowed MCS configured by RRC in sl-MaxMCS-PSSCH associated with the selected MCS table and the UE selects not to segment the RLC SDU; or

NOTE 1:	If the selected sidelink grant cannot accommodate the RLC SDU, it is left for UE implementation whether to perform segmentation or sidelink resource reselection.

1>	if transmission(s) with the selected sidelink grant cannot fulfil the remaining PDB of the data in a logical or the remaining SL-PRS delay budget for SL-PRS transmission, if available channel, and the MAC entity selects not to perform transmission(s) corresponding to a single MAC PDU or SL-PRS transmission; or

1>	if Sidelink consistent LBT failure is detected as specified in clause 5.31.2 in some RB set(s) of the selected resource pool that spans multiple RB sets for the logical channel, if single carrier frequency is configured; or

1>	if a MAC PDU is not transmitted (i.e. initial transmission or retransmission) in any of the resources for this MAC PDU that are associated with the sidelink process for Multi-consecutive slots transmission due to the Sidelink LBT failure:

NOTE 2:	If the remaining PDB is not met, it is left for UE implementation whether to perform transmission(s) corresponding to single MAC PDU or sidelink resource reselection.

NOTE 3:	It is left for UE implementation whether to trigger the TX resource (re-)selection due to the latency requirement of the MAC CEs triggered according to clause 5.22.1.7 and clause 5.22.1.10.1.

2>	if multiple carrier frequencies are configured:

3>	trigger the TX carrier (re-)selection procedure as specified in clause 5.22.1.11.

NOTE 3a:	It is up to UE implementation to avoid triggering the TX carrier (re-)selection procedure again if it has just performed TX carrier (re-)selection procedure.

2>	clear the selected sidelink grant associated to the Sidelink process, if available;

2>	trigger the TX resource (re-)selection as specified in clause 5.22.1.1.

NOTE 4:	Void.

NOTE 5:	Void.

End of the first change

Start of the second change

#### 5.22.1.2c	Resource re-selection from SL LBT Failure indication

If the MAC entity has been configured with Sidelink resource allocation mode 2 to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] based on sensing or random selection the MAC entity shall for each Sidelink process:

1>	if SL LBT failure indication is received from lower layers:

2>	for the resource(s) where SL LBT failure is detected from the selected sidelink grant associated to the Sidelink process:

3>	if transmission based on random selection is configured by upper layers:

4>	randomly select the time and frequency resources for one transmission opportunity from the resource pool excluding all RB sets for which Sidelink consistent LBT failures were detected and not cancelled, if configured according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources.

3>	else:

4>	randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources.

The MAC entity shall only apply the above procedure for non MCSt case and Multi-TB MCSt case using single slot candidate resources (if reselected single slot candidate resources based on the above procedure are consecutive).

End of the second change

## 4.1 Understanding 2 - Text proposal

Start of the first change

#### 5.22.1.2	TX resource (re-)selection check

If the TX resource (re-)selection check procedure is triggered on the selected pool of resources for a Sidelink process according to clause 5.22.1.1, the MAC entity shall for the Sidelink process:

1>	if PSCCH duration(s) and 2nd stage SCI on PSSCH for all transmissions of a MAC PDU of any selected sidelink grant(s) are not in SL DRX Active time as specified in clause 5.28.3 of the destination that has data to be sent; or

1>	if SL_RESOURCE_RESELECTION_COUNTER = 0 and when SL_RESOURCE_RESELECTION_COUNTER was equal to 1 the MAC entity randomly selected, with equal probability, a value in the interval [0, 1] which is above the probability configured by RRC in sl-ProbResourceKeep; or

1>	if the pool of resources is configured or reconfigured by RRC; or

1>	if there is no selected sidelink grant on the selected pool of resources; or

1>	if neither transmission nor retransmission has been performed by the MAC entity on any resource indicated in the selected sidelink grant during the last second; or

1>	if sl-ReselectAfter is configured and the number of consecutive unused transmission opportunities on resources indicated in the selected sidelink grant, which is incremented by 1 when none of the resources of the selected sidelink grant within a resource reservation interval is used, is equal to sl-ReselectAfter; or

1>	if the selected sidelink grant cannot accommodate a RLC SDU by using the maximum allowed MCS configured by RRC in sl-MaxMCS-PSSCH associated with the selected MCS table and the UE selects not to segment the RLC SDU; or

NOTE 1:	If the selected sidelink grant cannot accommodate the RLC SDU, it is left for UE implementation whether to perform segmentation or sidelink resource reselection.

1>	if transmission(s) with the selected sidelink grant cannot fulfil the remaining PDB of the data in a logical or the remaining SL-PRS delay budget for SL-PRS transmission, if available channel, and the MAC entity selects not to perform transmission(s) corresponding to a single MAC PDU or SL-PRS transmission; or

1>	if Sidelink consistent LBT failure is detected as specified in clause 5.31.2 in some RB set(s) of the selected resource pool that spans multiple RB sets for the logical channel, if single carrier frequency is configured; or

1>	if a MAC PDU is not transmitted (i.e. initial transmission or retransmission) in any of the resources for this MAC PDU that are associated with the sidelink process for Multi-consecutive slots transmission due to the Sidelink LBT failure:

NOTE 2:	If the remaining PDB is not met, it is left for UE implementation whether to perform transmission(s) corresponding to single MAC PDU or sidelink resource reselection.

NOTE 3:	It is left for UE implementation whether to trigger the TX resource (re-)selection due to the latency requirement of the MAC CEs triggered according to clause 5.22.1.7 and clause 5.22.1.10.1.

2>	if multiple carrier frequencies are configured:

3>	trigger the TX carrier (re-)selection procedure as specified in clause 5.22.1.11.

NOTE 3a:	It is up to UE implementation to avoid triggering the TX carrier (re-)selection procedure again if it has just performed TX carrier (re-)selection procedure.

2>	clear the selected sidelink grant associated to the Sidelink process, if available;

2>	trigger the TX resource (re-)selection as specified in clause 5.22.1.1.

NOTE 4:	Void.

NOTE 5:	Void.

End of the first change

Start of the second change

#### 5.22.1.2c	Resource re-selection from SL LBT Failure indication

If the MAC entity has been configured with Sidelink resource allocation mode 2 to transmit using pool(s) of resources in a carrier as indicated in TS 38.331 [5] based on sensing or random selection the MAC entity shall for each Sidelink process:

1>	if SL LBT failure indication is received from lower layers:

2>	for the resource(s) where SL LBT failure is detected from the selected sidelink grant associated to the Sidelink process:

3>	if transmission based on random selection is configured by upper layers:

4>	randomly select the time and frequency resources for one transmission opportunity from the resource pool excluding all RB sets for which Sidelink consistent LBT failures were detected and not cancelled, if configured according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources.

3>	else:

4>	randomly select the time and frequency resources for one transmission opportunity from the resources indicated by the physical layer as specified in clause 8.1.4 of TS 38.214 [7], according to the amount of selected frequency resources and the remaining PDB of SL data available in the logical channel(s) by ensuring the minimum time gap between any two selected resources of the selected sidelink grant in case that PSFCH is configured for this pool of resources.

The MAC entity shall only apply the above procedure for non-Multi-consecutive slots transmission case. If a MAC PDU is not transmitted (i.e. initial transmission or retransmission) in any of the resources for this MAC PDU that are associated with the sidelink process for Multi-consecutive slots transmission due to the Sidelink LBT failure, the MAC entity shall reselect the entire Multi-consecutive slots transmission resources according to clause 5.22.1.1.

End of the second change