3GPP TSG-RAN WG2 Meeting #129bis	R2-2502351

Wuhan, China, April. 7th – 11th, 2025

Agenda item:		8.4.3

Source:		Lenovo

Title:	RRM measurement relaxation and offloading in RRC_IDLE/INACTIVE

Document for:		Discussion and Decision

- Introduction

In RAN#103 meeting, WID of Rel-19 Low-power wake-up signal and receiver for NR (LP-WUS/ WUR) [1] was discussed and agreed, the objectives related to RRM measurement for RRC IDLE/INACTIVE modes of the work items are given as following.

- For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2)

In the previous RAN2 meetings [2]-[5], RRM measurement relaxation and offloading in RRC_INACTIVE/IDLE state has been discussed and following agreements have been reached:

RAN2#126 Agreements: - For serving cell measurement offloading (i.e., serving cell measurement fully offloaded to LR and no serving cell measurement via MR is required), RAN2 should focus on specifying the offloading criterion for serving cell for UEs supporting LP-WUS, and assume that RAN4 will define the measurement offloading requirements for serving cell. - RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. - RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’. RAN2#127 Agreements： - RAN2 only discuss RRM measurement offloading/relaxation for LP-WUS UEs. - For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. - The exit condition is based on the LR measurement results. RAN2#127bis Agreements: Working assumption: - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available. RAN2#128 Agreements: - The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed. - FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements. - FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbor cell RRM measurement relaxation. RAN2#129 Agreements： - The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. - If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

This contribution aims to present our views on the RRM measurement relaxation and offloading from MR to LR for UEs in RRC_IDLE/INACTIVE mode.

- Discussion

According to the latest WID, both RAN4 and RAN2 are required to specify RRM measurement relaxation of UE MR for both serving cell and neighboring cell measurements, and UE serving cell RRM measurement offloading from MR to LR, including the necessary conditions. Currently, the potential cases to be considered for RRM relaxation and serving cell measurement offloading is under discussion in RAN4. It has been approved by RAN4 that the ‘fully offloading case’ and ‘relaxed case b’ are considered as feasible while other cases are pending for further study, the details can be seen as below:

Approved cases on RRM measurement relaxation/offloading.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | OFF | OFF: FFS the conditions and details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

FFS cases on RRM measurement relaxation/offloading.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | OFF | ON |
| #4 Relaxed case c | OFF | ON, FFS the condition and the details | ON |
| #5 Extra Relaxed case | On with relaxation measurement | Off | OFF |
| #6 Extra Relaxed case | On with relaxation measurement | On with relaxation measurement | OFF |

RAN4 is required to define the RRM measurement relaxation/offloading requirements while RAN2 focuses on specifying the relaxation/offloading criterion for UEs supporting LP-WUS. Based on RAN4’ conclusion, it is suggested RAN2 gives priority to discuss the entry/exit conditions for the following offloading/relaxation cases:

- Fully offloading case：serving cell measurement is fully offloaded from MR to LR, while MR performs no measurement on serving cell, FFS on neighboring cell measurement,

- Relaxed case b (Partial offloading case): serving cell measurement and neighboring cell measurement are relaxed on MR, while LR also performs serving cell measurement, this case is also called as partial offloading case.

Observation 1: RAN4 has approved fully offloading case and relaxed case b as below while other cases on RRM measurement relaxation and offloading are pending for further study:

- Fully offloading case: serving cell measurement is fully offloaded from MR to LR while MR performs no measurement on serving cell.

- Relaxed case b: serving cell measurement is relaxed on MR while LR also performs the serving cell measurement.

2.1 Serving cell measurement offloading

According to latest RAN2 agreements [3], the entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. Currently, the agreed entry condition aims to guarantee the UE with a good coverage performance for both MR and LR. However, the serving measurement offloading aims to achieve UE power saving gain but also needs to limit the impacts on mobility performance as much as possible. For this, the UE needs to at least stay in a relatively stable status and have a low possibility of performing cell (re)selection, then the serving measurement offloading from MR to LR could be considered. To achieve this, in addition to a great MR threshold, additional entry conditions for the fully serving cell measurement offloading can be considered. For example, the low mobility condition, which can be defined as the difference of RSRP threshold being lower than a (pre-)configured threshold within a time duration, and MR measurements are used for the entry condition evaluation for low mobility criterion.

Proposal 1: RAN2 to discuss additional entry condition for fully offloading case of serving cell measurement, e.g., low mobility conditions evaluated by the MR.

Additionally, due to the minimalist design of the low power receiver, RRM measurement results performed by the LR may have loss on accuracy to some extent compared to the MR measurement results. For the fully offloading case, the exit conditions are defined to make UE to stop the fully offloading and fallback to legacy RRM measurements on MR to guarantee the mobility performances. In the last RAN2 meeting, it has been agreed that the exit condition is based on the LR measurement results. If the low mobility condition is supported as one of the entry conditions for the offloading, then the exit conditions should also consider the high mobility criterion evaluated by the LR.

Proposal 2: The exit conditions for serving cell measurement offloading considers high mobility condition evaluated by the LR.

2.2 Serving cell measurement relaxation

As defined by RAN4, for the relaxed case b of serving cell measurement relaxation, MR performs relaxation measurement on serving cell while LR is ON and performs the serving cell measurement, which can also be defined as the partial offloading case. Regarding the entry conditions for the relaxed case, in the last RAN2 #129 meeting [6], it has been agreed that the entry conditions for serving cell RRM relaxation can include both MR and LR measurements, if LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

Compared to fully offloading serving cell measurement to the LR, since both MR and LR can perform serving cell measurement for the relaxed case, the entry conditions can be defined to be less stringent than the fully offloading case, for example, the UE needs to at least stay in the coverage of LP-WUS, then UE may partially offload the measurement from MR to LR. But only in the case of the coverage performances are good enough, i.e., more stringent requirement than LR to perform serving cell measurement relaxation, MR may fully offload the serving cell measurement to LR. Therefore, we proposed that for the relaxed case, the MR and LR thresholds for entry conditions need to be configured lower than the serving cell measurement offloading case.

Proposal 3: The MR and LR thresholds on serving cell measurement relaxation entry conditions are configured lower than the thresholds on serving cell measurement offloading case.

Regarding the exit conditions for the relaxed case b, i.e., MR stops performing the relaxation measurement on serving cell and falls back to the legacy RRM measurement, while LR also stops performing the measurement on serving cell. According to the discussion in RAN2#128 meeting [5], it is FFS on whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed for serving cell RRM measurement relaxation. In our understanding, since LR measurement result can also be included in the entry conditions, it makes sense to use ‘not fulfilling the entry conditions’ as the exit condition and no additional exit conditions are needed for serving cell measurement relaxation.

Proposal 4: RAN2 to consider ‘not fulfilling the entry condition’ as the exit condition for serving cell measurement relaxation.

Additionally, another FFS issue on relaxed case is that if the entry condition for serving cell RRM measurement relaxation is the same as neighboring cell RRM measurement relaxation. Since in RAN4’ s defined scenarios, both MR serving cell measurement and neighboring cell measurement are on with relaxation measurement, then some companies thought it should be triggered at the same time. However, in our view, the serving cell measurement relaxation implies that UE may have a good MR coverage, whereas neighboring cell measurement is performed when UE moves toward to the cell-edge and has possibility to perform cell (re)selection. Those two measurements target to different UE behaviors. It is possible that neighboring cell measurement is not triggered when serving cell measurement relaxation happens. And in legacy, the neighboring cell measurement relaxation is performed based on ‘not-at-cell-edge’ and ‘low-mobility’ conditions. We understood that same relaxation criterion can still be applied to the neighboring cell measurement for LP-WUS UEs. Then we prefer to configure separate entry conditions for serving cell RRM measurement relaxation and neighboring cell RRM measurement relaxation.

Proposal 5: The entry condition for serving cell RRM measurement relaxation is configured independently from the neighboring cell RRM measurement relaxation.

Based on above analysis for the serving cell measurement offloading case and relaxation case, we assumed that the entry conditions define the criterion for UEs to start performing the serving cell measurement offloading or relaxation from the legacy RRM measurement mode. While the exit conditions define the criterion for UEs to stop performing the serving cell measurement offloading or relaxation and fallback to legacy RRM measurement mode directly. However, whether the UE can perform measurement transition between the fully offloading case and relaxation  case can be further discussed. For example, UE is performing the serving cell measurement relaxation and then transits to the fully offloading case directly, or UE is performing the fully offloading measurement on the serving cell and then falls back to the serving cell measurement relaxation. If the transition is identified feasible, then corresponding conditions for the transition between the two cases may need to be further defined, or the existing defined entry/exit conditions for serving cell offloading case and relaxation case can still be applied to the transition case based on UE implementation. Therefore, it is suggested RAN2 to discuss the feasibility of the measurement status transition between the serving cell measurement offloading and serving cell measurement relaxation directly, if it is identified as feasible, whether additional conditions are needed should be further discussed.

Proposal 6: RAN2 to discuss whether additional conditions are needed for the transition between the serving cell measurement offloading and serving cell measurement relaxation directly if it is feasible.

2.3 Neighboring cell measurement relaxation

As concluded in RAN2#126 meeting [2], the RRM measurement of the neighboring cell can only be performed by MR. And in RAN2#127bis meeting, RAN2 assumes ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. In our understanding, ‘low mobility’ criterion guarantees that the UE has low possibility to perform the cell reselection, then the neighboring cell measurement can be relaxed from the MR side. It is suggested RAN2 to agree that ‘low mobility’ criterion is reused for neighboring cell measurement relaxation for UEs capable of LP-WUS in addition to ‘not at cell edge’ criterion.

Proposal 7: RAN2 to agree the ‘low mobility’ criterion is reused for neighboring cell measurement relaxation for UEs capable of LP-WUS.

More specifically, according to the neighboring cell measurement behaviors defined by RAN4, for the fully offloading case MR neighboring cell measurement is off and FFS on the conditions and details. In this case, the baseline UE behavior is that LR is performing serving cell measurement while the MR may not perform any neighbor cell measurements. In such a case, we think that it may be necessary to define LR based measurements (e.g., PSS/SSS/PBCH DMRS, LP-SS, etc.) to re-evaluate whether neighbor cell measurement on the MR need to be triggered.

Therefore, for fully offloading case, MR may trigger the neighbor cell measurement when the LR measurement results on serving cell satisfy the configured criteria, whether the neighbor cell measurement needs to be relaxed or performed as legacy may be left to FFS.

Proposal 8: For fully offloading case, RAN2 may consider the feasibility of the case that neighboring cell measurement on MR is triggered by the LR measurement result on the serving cell.

For the relaxed case b, as defined by RAN4, the MR neighboring cell is ON with the relaxation measurement. In this case, MR serving measurement is also ON to perform the relaxation measurement. Then the neighboring cell measurement relaxation conditions can at least follow legacy, when MR measurement results satisfy ‘low mobility’ or ‘not-at-cell-edge’ criterion if configured, the neighboring cell measurement relaxation is performed.

Proposal 9: For the relaxed case b, MR performs neighboring cell measurement relaxation at least follow legacy, i.e., when MR measurement results satisfy ‘not-at-cell-edge’ criterion and/or ‘low mobility’ criterion if supported.

- Conclusion

Observation 1: RAN4 has approved fully offloading case and relaxed case b as below while other cases on RRM measurement relaxation and offloading are pending for further study:

- Fully offloading case: serving cell measurement is fully offloaded from MR to LR while MR performs no measurement on serving cell.

- Relaxed case b: serving cell measurement is relaxed on MR while LR also performs the serving cell measurement.

Proposal 1: RAN2 to discuss additional entry condition for fully offloading case of serving cell measurement, e.g., low mobility conditions evaluated by the MR.

Proposal 2: The exit conditions for serving cell measurement offloading considers high mobility condition evaluated by the LR.

Proposal 3: The MR and LR thresholds on serving cell measurement relaxation entry conditions are configured lower than the thresholds on serving cell measurement offloading case.

Proposal 4: RAN2 to consider ‘not fulfilling the entry condition’ as the exit condition for serving cell measurement relaxation.

Proposal 5: The entry condition for serving cell RRM measurement relaxation is configured independently with the neighboring cell RRM measurement relaxation.

Proposal 6: RAN2 to discuss whether additional conditions are needed for the transition between the serving cell measurement offloading and serving cell measurement relaxation directly if it is feasible.

Proposal 7: RAN2 to agree the ‘low mobility’ criterion is reused for neighboring cell measurement relaxation for UEs capable of LP-WUS.

Proposal 8: For fully offloading case, RAN2 may consider the feasibility of the case that neighboring cell measurement on MR is triggered by the LR measurement result on the serving cell.

Proposal 9: For the relaxed case b, MR performs neighboring cell measurement relaxation at least follow legacy, i.e., when MR measurement results satisfy ‘not-at-cell-edge’ criterion and/ or ‘low mobility’ criterion if supported.

- References

3GPP RAN#103, RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR).

3GPP RAN2#126, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

3GPP RAN2#127, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

3GPP RAN2#127bis, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

3GPP RAN2#128, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS.

[5]	3GPP RAN2#129, Report from session on R18 MIMOevo, R18 MUSIM, and R19 LP-WUS