3GPP TSG-RAN WG2 Meeting #129bis	R2-2502352

Wuhan, China, Apr. 7th – 11th, 2025                           Revision of R2-2500615

Agenda item:		8.8.2

Source:		Lenovo

Title:		Further considerations on NR NTN DL-CE

Document for:		Discussion

- Introduction

The latest RAN2 meetings made the following agreements regarding DL-CE (Downlink Coverage Enhancement) in NTN:

RAN2#127bis Agreements - If it turns out that there is a need to bar UEs not supporting DL-CE, Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, in the same way as pre-Rel19 NTN UEs (this is an extension of the previous agreement to include also Rel-19 UE not supporting DL-CE) - If it turns out that there is a need to bar UEs not supporting DL-CE, then we need to introduce a barring mechanism to control access of UEs supporting Rel19 NTN DL-CE. FFS on the details. (This also implies that UEs supporting Rel19 NTN DL-CE will not consider the existing NTN barring bit) - (also depending on the details of the RAN1 solution) we can further consider methods to allow UEs not supporting DL CE to down-prioritize or prevent re-selection to cells operating with DL CE. RAN2#128 Agreements: - If we need to bar UEs not supporting DL-CE (via legacy mechanism), we introduce a new barring in SIB1 to be able to selectively bar “UEs supporting DL-CE” (FFS if we will finally refer to “UEs supporting extended SSB periodicity” instead). FFS on the details (e.g. how many bits, etc.) - As part of the work on DL-CE, RAN2 investigates related UE power consumption impact (including legacy UEs) - RAN2 will consider whether to introduce separate signalling (e.g. new SMTC5 list) for DL CE cells SMTCs, e.g. if different periodicities need to be signalled or to prevent reselection to specific cells RAN2#129 Agreements - RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer - RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame - RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell) - The number of SMTC/gaps a UE needs to consider at any time will not be increased further

In this contribution we discuss issues and potential solutions in cell barring, reselection and timing of satellite beam activation/deactivation for DL-CE in NTN.

- Discussion

## Cell barring for Rel-19 NTN UEs

Due to the current working direction of SSB periodicity extension for NTN DL-CE, the potential impacts were identified in RAN2#127 meeting focusing on the case that SSB periodicity is larger than 160ms. If the SSB periodicity is extended to a value larger than 160ms, existing Rel-17 and Rel-18 NTN UEs as well as Rel-19 NTN UEs incapable of such enhancement will be severely impacted and therefore access control may be necessary.

Based on current RAN2 discussion, cellBarredNTN introduced in Rel-17 NR NTN can be reused to restrict Rel-17 & Rel-18 NTN UEs from accessing an NTN cell operating in DL-CE (e.g., SSB periodicity is extended to a value larger than 160ms). However, the access control strategy for Rel-19 NTN UEs needs further discussion, i.e., how to allow/restrict access of NTN UE capable/incapable of DL-CE to an NTN cell operating in DL-CE. It is obvious that the cellBarredNTN alone cannot facilitate the expected access control for Rel-19 NTN UEs. For example, if cellBarredNTN is used to prevent access of Rel-17 & Rel-18 NTN UEs to an NTN cell operating in DL-CE, it has to be set to barred, which also prevents access of all Rel-19 NTN UEs. In the latest RAN2#127, there was consensus that Rel-19 UEs not supporting DL-CE can be barred from accessing a cell operating with DL-CE using the existing NTN bar bit, while whether there is a need to bar Rel-19 UEs supporting DL-CE is still uncertain.

Observation 1: Using cellBarredNTN alone is not sufficient to implement cell barring of Rel-19 NTN UEs supporting DL-CE.

Following this assumption, Rel-19 NTN UEs capable of DL-CE needs a new scheme to avoid from being barred by cellBarredNTN set to barred because of operating in DL-CE (while it may still need to be barred for other reasons when cellBarredNTN is set to barred). RAN2#127bis agreed to have an explicit indication (e.g., new cell barring bit) in SIB1 to selectively bar “UEs supporting DL-CE”.

Observation 2: Explicit indication beyond cellBarredNTN shall be considered to facilitate access control for Rel-19 NTN UEs in an NTN cell operating in DL-CE.

For an NTN-capable UE supporting DL-CE (e.g., SSB periodicity extension larger than 160ms), the following options of NTN cell barring logic can be considered:

- Option 1 [2 bits]: A new cell barring indication cellBarredNTN_DLCE is introduced with configurable value “barred” and “not barred”. An NTN-capable UE supporting DL-CE ignores cellBarredNTN, and determines cell barring status following cellBarredNTN_DLCE value.

- Option 2A [1 bit]: A new flag indication cellBarredNTN_DLCE is introduced. If cellBarredNTN_DLCE is present, an NTN-capable UE supporting DL-CE considers the cell as barred. If cellBarredNTN_DLCE is absent, an NTN-capable UE supporting DL-CE follows cellBarredNTN.

- Option 2B [1 bit]: A new flag indication cellAllowedNTN_DLCE is introduced. If cellAllowedNTN_DLCE is present, an NTN-capable UE supporting DL-CE ignores cellBarredNTN and considers the cell as not barred. If cellAllowedNTN_DLCE is absent, an NTN-capable UE supporting DL-CE follows cellBarredNTN value.

Proposal 1: RAN2 to discuss the following options of cell barring for an NTN-capable UE supporting DL-CE.

- Option 1: [2 bits] By a new cell barring indication cellBarredNTN_DLCE with configurable value “barred” and “not barred”.

- Option 2A: [1 bit] By presence of a new flag indication cellBarredNTN_DLCE.

- Option 2B: [1 bit] By presence of a new flag indication cellAllowedNTN_DLCE is introduced.

The legacy cell barring schemes including cellBarredNTN are solely based on the corresponding bit status due to that the network type (e.g., NTN or TN) is fixed, and for cell reselection the barring status is fixed to 300s. UE can only re-acquire the SIB1 to understand if the cell barring status has changed.

Meanwhile, in Rel-19 NTN the access control could be time sensitive as the NTN cell may change between modes enabling and disabling DL-CE (SSB periodicity extension). And it is possible that some prior information can be determined by network and even be indicated to UE to implement optimized access control e.g., based on flexible time duration in coordination with NTN DL-CE operations.

Observation 3: The cell barring status regarding NTN DL-CE is associated to the operation duration or cycle which can be flexible and pre-determined.

Therefore, if UE can be informed about the NTN DL-CE operation duration or cycle, it may also determine the validity duration of a cell barring status determination, which can prevent UE from unnecessary SIB1 acquiring for cell barring status or allow UE to acquire SIB1 for cell barring status in time.

Proposal 2: RAN2 to consider cell barring enhancement based on the validity duration of cell barring status for NTN DL-CE.

## Cell reselection for Rel-19 NTN UEs

Based on the above discussions for NTN cell barring, Rel-19 NTN UEs also face the similar issues during the cell reselection procedures. More specifically, if a neighbour NTN cell is operating in DL-CE, the corresponding cell barring scheme shall be considered by the UE when performing cell reselection. The determination of cell barring status for a neighboring NTN cell operating in DL-CE can either follow an option in Proposal 1 or an indicated list of neighbour cells operating in DL-CE.

Proposal 3: If a neighboring NTN cell operating in DL-CE is considered as not barred, an NTN-capable UE supporting DL-CE may prioritize the frequency of the cell for cell reselection.

Proposal 4: If a neighboring NTN cell operating in DL-CE is considered as barred, an NTN-capable UE supporting DL-CE may deprioritize the frequency of the cell for cell reselection.

## Timing of satellite beam activation/deactivation

With the agreement that “beam hopping” applies equally to all the satellite beams of a given cell, the beam activation/deactivation pattern is the same within a cell, which is a mechanism similar to cell DTX/DRX. Due to the large propagation delay and propagation delay difference in NTN, the configuring gNB and the configured UEs may have different understanding about the timing of satellite beam activation/deactivation. For DL satellite beams being activated or deactivated, UE may either miss possible transmission from network or monitor PDCCH too early with unnecessary power consumption.

Proposal 5: If a beam activation/deactivation pattern is configured by gNB, UE shall consider the propagation delay (e.g. half of UE-gNB RTT) when determine the timing of satellite beam activation/deactivation.

- Conclusion

In this contribution we discuss issues and potential solutions in cell barring, cell reselection and timing of satellite beam activation/deactivation for DL-CE in NTN. The following observations are given:

Observation 1: Using cellBarredNTN alone is not sufficient to implement cell barring of Rel-19 NTN UEs supporting DL-CE.

Observation 2: Explicit indication beyond cellBarredNTN shall be considered to facilitate access control for Rel-19 NTN UEs in an NTN cell operating in DL-CE.

Observation 3: The cell barring status regarding NTN DL-CE is associated to the operation duration or cycle which can be flexible and pre-determined.

And we propose:

Proposal 1: RAN2 to discuss the following options of cell barring for an NTN-capable UE supporting DL-CE.

- Option 1: [2 bits] By a new cell barring indication cellBarredNTN_DLCE with configurable value “barred” and “not barred”.

- Option 2A: [1 bit] By presence of a new flag indication cellBarredNTN_DLCE.

- Option 2B: [1 bit] By presence of a new flag indication cellAllowedNTN_DLCE is introduced.

Proposal 2: RAN2 to consider cell barring enhancement based on the validity duration of cell barring status for NTN DL-CE.

Proposal 3: If a neighboring NTN cell operating in DL-CE is considered as not barred, an NTN-capable UE supporting DL-CE may prioritize the frequency of the cell for cell reselection.

Proposal 4: If a neighboring NTN cell operating in DL-CE is considered as barred, an NTN-capable UE supporting DL-CE may deprioritize the frequency of the cell for cell reselection.

Proposal 5: If a beam activation/deactivation pattern is configured by gNB, UE shall consider the propagation delay (e.g. half of UE-gNB RTT) when determine the timing of satellite beam activation/deactivation.