3GPP TSG-RAN WG2 Meeting #129bis	R2-2502353

Wuhan, China, Apr. 7th – 11th, 2025

Agenda item:		8.8.4

Source:		Lenovo

Title:		Service continuity for MBS broadcast in NR NTN

Document for:		Discussion

- Introduction

The latest RAN2 meetings made the following agreements regarding broadcast services in NTN:

RAN2#128 Agreements: - The encoding of TN coverage introduced in Rel-18 in TS38.331, including tn-ReferenceLocation-r18 and tn-DistanceRadius-r18, is reused for the geographical area of the circle. - The encoding of Polygon in TS37.355 is reused for the geographical area of the Polygon. - The IntendedServiceArea is considered as the IE name of the geographical area (we can still update the name in the CR implementation if needed) - A signalled intended service area for a MBS BC service may include geographic areas across the current serving cell and overlapping neighbor cell(s). - RAN2 understands that the geographic area information for the intended service areas can be semi-static and not cause frequent updates. - Introduce a new SIB to include a list of intended service areas and related pointer (FFS if we point to the intended services areas via the index in the list or with an ID or another way) - The legacy SIB modification procedure is applied to update the intended service area information in the new SIB. RAN2#129 Agreements: - In the new SIB, explicit network-indicated area ID is used to label an intended service area in the list - It shall be possible to signal multiple service area IDs to one MBS service (we Insert a list of service area IDs in MCCH) - Introduce “warning area coordinates” in ETWS Primary Notification (SIB6) and in ETWS Secondary Notification (SIB7). FFS on the signalling details for “warning area coordinates” (SIB6 is not segmented)

In this contribution we discuss MBS broadcast service continuity based on broadcast service area information in NTN.

- Discussion

For an NTN cell that could be much larger than an MBS broadcast service area, a UE may not be automatically authorized to receive an MBS broadcast service by the serving NTN cell it is in, i.e., not all UE within the same NTN cell is authorized to receive the same MBS broadcast service. For this reason, RAN2 have agreed to introduce broadcast service area information and area ID in a new SIB for NTN, and further impacts and issues on MBS broadcast service continuity are to be discussed.

## MBS broadcast service continuity for RRC_IDLE/INACTIVE

For IDLE/INACTIVE MBS broadcast service continuity in TN, mobility procedures for MBS reception allow the UE to start or continue receiving MBS service when changing cells. The gNB may indicate in SIB21 the mapping between the current and/or neighboring carrier frequencies and MBS Frequency Selection Area Identities (FSAI). The FSAI is derived from the serviceArea and radiofrequency elements received in the USD (User Service Description) via NAS signalling from network, wherein the serviceArea element declares one or more service areas designated by the SAI (Service Area Identity) which is mapped onto one or more cells.

If UE is receiving or interested to receive an MBS broadcast service and can only receive this MBS broadcast service by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session. The UE may also consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session.

This legacy mechanism works well under the assumption that an MBS broadcast service area is mapped to one or more TN cells. The UE can simply determine the frequency priority based on the receiving or interested MBS broadcast service and the frequency it associates to, and reselects to a cell that supports that service within its whole coverage. When considering that an NTN cell coverage could be larger than an MBS broadcast service area, which cannot be mapped to the cell’s whole coverage, UE cannot simply determine the frequency priority only based on the association between service and frequency, and service area information of neighbour cell may be needed.

Figure 1. Example of relation between cell coverage and MBS broadcast service area in NTN

For NTN, MBS broadcast service #A is supported in partial coverage of Cell #1, #2 and #3, and UE1 from Cell #1 to Cell #2 is expected to continue to receive MBS broadcast service #A as UE is still in its service area, while UE2 from Cell #1 to Cell #3 is expected not to continue to receive MBS broadcast service #A as UE is moving out its service area. If the legacy mechanism for TN applies, UE2 will also prioritize a frequency associated to the broadcast service but it will not be authorized to receive as it moves out of service area.

Although RAN2#128 agreed that broadcast service area information may include geographic areas across the current serving cell and overlapping neighbor cell(s), a further question is how to let UE know this and how to use it for service continuity guaranteeing. A straight-forward way is to have a pointer to the corresponding broadcast service area in SIB21 so that UE can associate the area with a neighbor cell or its frequency. Noticing that in RAN2#129 the service area ID was agreed in the new SIB, it can also be used for service continuity.

Proposal 1: The MBS broadcast service area ID(s) can be included in SIB21 to associate the neighbor cell frequency with the broadcast service area information for service continuity.

Once the neighbour cell MBS broadcast service area information is indicated to UE, the next step is the corresponding frequency priority handling for service continuity. For TN, the legacy handling logic is as follows:

- If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, the UE may consider that frequency to be the highest priority during the MBS broadcast session.

- If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service, the UE may consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session.

Considering that an NTN cell coverage could be larger than an MBS broadcast service area, the above priority handling also needs to consider neighbour cell MBS broadcast service area information if provided. For example:

- If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service(s) and can only receive this MBS broadcast service(s) by camping on a frequency on which it is provided, and UE is in the MBS broadcast service area associated to that frequency, the UE may consider that frequency to be the highest priority during the MBS broadcast session.

- If the MBS broadcast capable UE is receiving or interested to receive an MBS broadcast service, and UE is in the MBS broadcast service area associated to that MBS broadcast service, the UE may consider cell reselection candidate frequencies at which it cannot receive the MBS broadcast service to be of the lowest priority during the MBS broadcast session.

Proposal 2: For a UE receiving or interested to receive an MBS broadcast service associated to a MBS broadcast service area and a frequency, UE can prioritize that frequency for cell reselection if UE is in that broadcast service area.

Proposal 3: For a UE receiving or interested to receive an MBS broadcast service associated to a MBS broadcast service area and a frequency, UE can deprioritize other frequency not associated for cell reselection if UE is in that broadcast service area.

MBS broadcast service continuity for RRC_CONNECTED

For CONNECTED MBS broadcast service continuity in TN, the UE in RRC_CONNECTED state may send MBS Interest Indication to the gNB, including the list of MBS frequencies and interested services, as well as frequency priorities. The gNB may use this indication when providing handover configuration to allow the UE to receive the MBS services the UE is interested in. Due to the same assumption that an MBS broadcast service area is mapped to one or more TN cells, UE does not need to provide its current or expected MBS broadcast service area information in the MBS Interest Indication.

When considering that an NTN cell coverage could be larger than an MBS broadcast service area, UE’s current/expected MBS broadcast service area information is necessary at gNB for further handover decisions e.g., to judge whether UE is staying/moving into or moving out of an MBS broadcast service area and configure an appropriate target cell.

Proposal 4: UE may report its current broadcast service area information or area ID to gNB for service continuity in RRC_CONNECTED.

- Conclusion

In this contribution we discuss the further issues of broadcast service area indication and service continuity in NTN. It is proposed that:

Proposal 1: The MBS broadcast service area ID(s) can be included in SIB21 to associate the neighbor cell frequency with the broadcast service area information for service continuity.

Proposal 2: For a UE receiving or interested to receive an MBS broadcast service associated to a MBS broadcast service area and a frequency, UE can prioritize that frequency for cell reselection if UE is in that broadcast service area.

Proposal 3: For a UE receiving or interested to receive an MBS broadcast service associated to a MBS broadcast service area and a frequency, UE can deprioritize other frequency not associated for cell reselection if UE is in that broadcast service area.

Proposal 4: UE may report its current broadcast service area information or area ID to gNB for service continuity in RRC_CONNECTED.