3GPP TSG-RAN WG2 Meeting #129bis	R2-2502356

Wuhan, China, Apr. 7th – 11th, 2025                           Revision of R2-2500619

Agenda item:		8.9.3

Source:		Lenovo

Title:		EDT for uplink capacity enhancement in NTN

Document for:		Discussion

- Introduction

The latest RAN2#129 meeting made the following agreements and working assumptions regarding uplink capacity enhancements in Rel-19 IoT NTN:

RAN2#129 agreements: - It is FFS if separate CB-msg3 resources would be needed for CB-msg-3 using OCC or if the same CB-msg3 resources could be used - RAN2 assumes that one possibility to take power imbalance under control is to define RSRP ranges that need to be respected to transmit CB-msg3 using OCC - RAN2 assumes that at least the following will be part of the shared resources configuration for CB-msg3 (FFS on other aspects) - Time domain resources for (N)PUSCH occasions: periodicity and start time (e.g., start subframe, start SFN) - Frequency domain resources for (N)PUSCH occasions - repetition number - (N)PDCCH resource - MCS - For CB-msg3 transmission, for eMTC NTN, up to three separate RSRP thresholds (on top of the minimum RSRP threshold and possibly different from the thresholds for PRACH) can be supported for achieving at most 4 CE levels; for NB-IoT NTN, up to two separate RSRP thresholds (on top of the minimum RSRP threshold possibly different from the thresholds for PRACH) can be supported for achieving at most 3 repetition levels. - The UE shall at most have one ongoing CB EDT procedure at any time. - The CB EDT Config has one minimum RSRP threshold (as agreed in RAN2#128) to use CB EDT. - The CB EDT Config has two RSRP thresholds for NB-IoT for the three CE levels. - CB EDT Config has three RSRP thresholds for eMTC for the four CE levels. - As Signalling design Baseline RAN2 assumes the PUR config and the NPRACH config for shared (N)PUSCH config can be used and some of the parameters can be included in a new CB EDT config. - RAN2 consider a new CBEDT-ConfigSIB-NB IE for configuring the CB EDT feature Working assumption: - One CB-MSG4 can target multiple UEs simultaneously (FFS on the details) - For CB-MSG3, the Transmission window is configured by the network with a starting point (e.g. H-SFN offset), a window length, and a window periodicity (window length and periodicity could be the same). For k=1 the window length can be equal to 1: same behaviour as today The UE first selects the next DSA transmission window and then randomly select K replicasinside the window. RAN2 assumes that a pointer solution is not needed in Rel-19

In this contribution we discuss the further issues and solutions for EDT enhancements in IoT NTN.

- Discussion

Considering the need of supporting massive capacity in terms of number and types of UE, there is a need to identify methods to reduce the necessary uplink and downlink signaling. From this perspective there are two directions that are to be studied: Msg3 transmission without Msg1/2 and reduced overhead reduction for Msg4 delivery.

## Msg3 transmission without Msg1/2 for EDT

Based on the progress and discussions in previous RAN2 meetings, the RSRP thresholds for using CB EDT and other metrics are almost determined. Moreover, in RAN2#129 it was also agreed that RAN2 assumes the time/frequency domain resources for (N)PUSCH occasions, repetition number, (N)PDCCH resource and MCS as part of for the shared resources configuration for CB-msg3. Noting that although it has been agreed that the RNTI used to schedule Msg4 is derived from the PUSCH resource corresponding to the Msg3 transmission occasion, RAN2 have not decided on the RNTI used for CB-Msg3 PUSCH transmission, while without Msg2 such RNTI is necessary for Msg3 transmission in CB-EDT. A straight forward way is that the RNTI used for CB-Msg3 PUSCH transmission can also be derived from the PUSCH resource corresponding to the Msg3 transmission occasion.

Proposal 1: The RNTI used for CB-EDT Msg3 transmission is derived from the PUSCH resource corresponding to the Msg3 transmission occasion (same as that for Msg4 scheduling).

In legacy EDT, UE fallbacks to RRC connection establishment if EDT operation is not successful. If Msg3 transmission without Msg1(preamble)/Msg2(RAR) is supported and not successful, the followed operation including possible backoff or fallback needs to be specified, with more options than the legacy fallback mechanism, including:

- Option 1: UE suspends the EDT operation without Msg1(preamble)/Msg2(RAR) and waits for a next attempt e.g., when UE receives indication of contention from eNB or UE has a timer for contention resolution (Msg4 reception) expired. UE may backoff for a time duration to use the next available resource.

- Option 2: UE fallbacks to normal EDT (with random access) e.g., when UE receives indication of contention or fallback from eNB or UE has a timer for contention resolution (Msg4 reception) expired.

- Option 3: As in legacy EDT, UE directly fallbacks to connection establishment to transit to CONNECTED state e.g., when UE receives indication of contention or fallback from eNB or UE has a timer for contention resolution (Msg4 reception) expired.

Proposal 2: RAN2 to discuss the fallback schemes for Msg3 transmission failure in CB EDT.

Another fact for NTN is that UE needs to pre-compensate the timing advance between UE and the network reference point for UL synchronization before initiating random access. For an EDT operation in such deployment, it is also required for the UE to calculate the pre-compensated timing advance, while the procedures to acquire ephemeris or UE position may interrupt the initiation of EDT.

For a CB-EDT initiation that collisions with GNSS position fix or ephemeris acquiring, wither the initiation of an EDT operation without Msg1(preamble)/Msg2(RAR) can delayed until UE completes GNSS position fix or ephemeris (SIB31) acquiring, or GNSS position fix or ephemeris is suspended (no UL sync loss after validity duration) when an EDT operation without Msg1(preamble)/Msg2(RAR) is triggered.

Proposal 3: RAN2 to discuss how to handle the CB-EDT initiation that occurs during GNSS position fix or ephemeris (SIB31) acquiring.

## Efficient delivery of Msg4 for EDT

In legacy EDT the delivery of Msg4 (e.g., RRCEarlyDataComplete for CP-EDT) is unicast via dedicated RRC signalling, and its included fields (e.g., dedicatedInfoNAS, extendedWaitTime, idleModeMobilityControlInfo, redirectedCarrierInfo) are specified as optional. From network perspective eNB can reduce overhead simply by omitting some unnecessary fields or restricting the optional fields in Msg4.

Moreover, these fields of Msg4 are not all UE-specific e.g., extendedWaitTime, idleModeMobilityControlInfo, and redirectedCarrierInfo could be common for multiple or even all UEs in the cell, and dedicatedInfoNAS could also be common when it is empty or zero (i.e., no DL data for EDT from core network). Therefore it is not necessary to always unicast all these fields with duplicated signalling overhead. Based on the above analysis, at least the following two directions can be considered for efficient Msg4 delivery:

- Case A: At least part of Msg4 is common for multiple UEs initiated EDT in the cell. In this case the common part of Msg4 may be delivered in a non-unicast manner.

- Case B: No DL data or other configuration is to be delivered to UE (i.e., empty Msg4). In this case an alternative indication other than Msg4 via RRC can be considered e.g., the Layer 1 or MAC CE acknowledgement used for PUR.

Therefore the following directions can be further discussed:

Proposal 4: RAN2 to discuss the following enhancements for Msg4 delivery in EDT:

- Msg4 delivery in non-unicast ways (e.g., multicast);

- Msg4 delivery using lower layer indication (e.g., Layer 1 indication or MAC CE as in PUR).

- Conclusion

In this contribution we discuss the potential issues and solutions for EDT enhancements in IoT NTN. The following proposals are given:

Proposal 1: The RNTI used for CB-EDT Msg3 transmission is derived from the PUSCH resource corresponding to the Msg3 transmission occasion (same as that for Msg4 scheduling).

Proposal 2: RAN2 to discuss the fallback schemes for Msg3 transmission failure in CB EDT.

Proposal 3: RAN2 to discuss how to handle the CB-EDT initiation that occurs during GNSS position fix or ephemeris (SIB31) acquiring.

Proposal 4: RAN2 to discuss the following enhancements for Msg4 delivery in EDT:

- Msg4 delivery in non-unicast ways (e.g., multicast);

- Msg4 delivery using lower layer indication (e.g., Layer 1 indication or MAC CE as in PUR).