3GPP TSG-RAN WG2 #129bis	R2-2502370

Wuhan, China, April 7th – 11th, 2025

Agenda item:	   8.1.4

Source:		Lenovo

Title:	UE requests data collection for AI based positioning

Document for:		Discussion and Decision

# Introduction

In the previous RAN2 meeting, it was agreed to extend the following principles for general UE side data collection configuration:

RAN2#129 Agreement - Extend the following agreements on data collection configuration in AI/ML based beam management to general UE-side data collection configuration: - Data collection related configuration(s) and associated ID(s)(if needed) can be included in training data collection configuration. - For data collection configuration UE-side model training, the UE can send a request for data collection (e.g. start/stop). FFS whether a suggested data collection configuration/associated IDs (if specified)/parameters can be provided to the network. - The network can provide or release the data collection configuration (at any point in time), with or without UE request. - The following methods for network control of the initiation and configuration for data collection: - The network can decide when to start/stop the data collection and send configuration. - The network can configure whether UE is allowed to initiate request for data collection (e.g. start/stop indication).

In this paper, we discuss the relevant issues for AI based positioning.

# Discussion

RAN1 Agreement For training data collection of Case 1 and 2a, in terms of DL PRS configuration for collecting training data, RAN1 study the following options on assistance data, using legacy mechanisms as a starting point: Option A. (UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE. Option B. (LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE. Note: the UE can be a PRU and/or a Non-PRU UE. Note: as in existing specification, the DL PRS configurations in the assistance data from LMF to UE are based on DL PRS configuration coordinated between LMF and gNB

In the previous RAN1 discussion, RAN1 has considered using the following two options to provide necessary DL PRS configuration. And in our view, both can be supported based on legacy LPP Assistance Data Transfer procedure (RequestAssistanceData/ProvideAssistanceData) or LPP Assistance Data Delivery Procedure (ProvideAssistanceData)

Option A. 	(UE initiated) UE makes a request to LMF on the preferred DL PRS configuration for training data collection, e.g., on-demand PRS. LMF makes the decision on determining the DL PRS configuration for training data collection and provides the assistance data to the UE.

For Option A, UE can send a RequestAssistanceData message to LMF requesting on-demand DL PRS configuration, including relevant information, e.g., time duration, PRS characteristics. Then LMF will provide the demanded DL PRS configuration to UE via ProvideAssistanceData message.

Option B. 	(LMF initiated) LMF determines the DL PRS configuration for training data collection and provides the assistance data to the UE.

For Option B, LMF could provide the DL PRS configuration to UE via ProvideAssistanceData message directly.

The legacy LPP Assistance Data Transfer procedure (via RequestAssistanceData/ ProvideAssistanceData messages) is taken as the baseline for UE to request DL PRS configuration from LMF for training data collection.

The legacy LPP Assistance Data Delivery procedure (via ProvideAssistanceData messages) is taken as the baseline for LMF to proactively provide DL PRS configuration to UE for training data collection.

RAN1#120 Agreement For AI/ML based positioning Case 1, from RAN1 perspective, when the label data of location is generated by LMF and transferred from LMF to UE, label and quality indicator of label can be provided by reusing existing IEs. - From RAN1 perspective, the existing IE can use one of the geographic shapes defined in TS 23.032. The location estimate uncertainty and confidence (if included with the geographic shapes) can serve as quality indicator of the label.

Also, RAN1 supports the scenario where the label data of location is generated by LMF and transferred from LMF to UE in the DL. However, there is no existing LPP procedure can be directly reused for such purpose. Therefore, RAN2 is suggested to discuss if LMF can transfer the label data of location to UE via existing LPP message (e.g., ProvideAssistanceData) or a new LPP message proactively/reactive.

RAN2 discusses if LMF can transfer the label data of location to UE via existing LPP message (e.g., ProvideAssistanceData) or a new LPP message proactively/reactively.

# 3	Conclusion

Based on the discussion above, we observe:

No table of figures entries found.

Based on the discussion above, we propose:

Proposal 1	The legacy LPP Assistance Data Transfer procedure (via RequestAssistanceData/ ProvideAssistanceData messages) is taken as the baseline for UE to request DL PRS configuration from LMF for training data collection.

Proposal 2	The legacy LPP Assistance Data Delivery procedure (via ProvideAssistanceData messages) is taken as the baseline for LMF to proactively provide DL PRS configuration to UE for training data collection.

Proposal 3	RAN2 discusses if LMF can transfer the label data of location to UE via existing LPP message (e.g., ProvideAssistanceData) or a new LPP message proactively/reactively.

# Reference