3GPP TSG-RAN WG2 Meeting #129bis                                 	    R2- 2502372

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.5.4

Source: 	Lenovo

Title:  	Paging and PRACH adaptation

Document for:	Discussion and decision

# Introduction

In RAN2#129 [1], Adaptation of common signal/channel transmissions was discussed, and following agreements were achieved:

Agreements on RACH adaptation

- Legacy UEs and UEs non-capable of time domain PRACH adaptation are expected to use legacy PRACH resources per legacy configuration and procedures. No barring is needed.

2a.	RAN2 starts CBRA for RACH adaptation.

2b. 	RAN2 starts 4-step RACH adaptation.

3. 	From R2 perspective, not apply time-domain RACH adaptation to RACH resources for MSG1-based SI request.

- From R2 perspective, not apply time-domain RACH adaptation to IAB RACH resources.

5a. 	From R2 perspective, RACH adaptation is not modelled as RA feature(s).

5b.	From R2 perspective, RACH partitioning with all the features, i.e. RedCap, SDT, and Slicing, and feature combinations, are supported for PRACH adaption in time domain.

- Will follow legacy mechanism regarding how to select RACH resource.

This paper intends to check if it is possible to achieve better energy saving with respect to RACH enhancement and access restriction. Also, PRACH adaptation related issues are discussed in this paper.

# Discussion

## RACH enhancement

In RAN1#118bis discussion on the RACH adaptation for Rel-19 took place with the following agreements:

Agreement

For adaptation of PRACH in time-domain, the same PRACH preamble format is used for the additional RACH resources and legacy PRACH resources.

Agreement

For adaptation of PRACH in time-domain, support both of the following

- Alt 1: The PRACH configuration index for the additional PRACH resources is same as the PRACH configuration index for the legacy resources

- Alt 2: The PRACH configuration index for the additional PRACH resources is different from the PRACH configuration index for the legacy resources

- FFS: Additional details

Agreement

For adaptation of PRACH in time-domain, the frequency domain resources for the additional PRACH resources and legacy PRACH resources can be same or different

- FFS: applicable case(s) (i.e. case(s) from the RAN1#117 agreement).

- Discuss further following options for signaling

- Option 1: at least the following parameter(s) can be configured separately for the additional PRACH resources.

  - msg1-FrequencyStart at least for 4-step RACH

  - FFS: other applicable legacy frequency domain parameter(s)

- Option 2: Offset to legacy frequency domain parameter(s) are configured for the additional PRACH resources

  - Note: Offset to legacy frequency domain parameter(s) is a new parameter

  - FFS: applicable legacy frequency domain parameter(s)

According to RAN1 agreement, the configuration of additional PRACH resources is provided by semi-static signaling. From point view of RAN2 specification, the configuration of additional PRACH resources is separately configured from the legacy RACH configuration in SIB 1, wherein the configuration of additional PRACH resources needs to indicate at least the PRACH configuration index, Number of SSBs per RACH occasion. The other config(s) e.g., msg1-FDM, msg1-FrequencyStart, may be also separately configured from the legacy RACH configuration for flexibility. If the other config(s) are not separately configured, UE default reuses the config in legacy RACH configuration.

Proposal 1: RAN2 to agree the additional RACH occasion for adaptation in time domain is separately configured in RACH configuration from legacy RACH configuration in SIB1.

Proposal 2: RAN2 to agree the separate RACH configuration indicates at least Number of SSBs per RACH occasion and the other config(s) e.g., msg1-FDM, msg1-FrequencyStart, may be also separately configured. If the other config(s) are not separately configured, UE default reuses the config in legacy RACH configuration.

## Access restriction considering feature combination

Based on the RAN2 agreement, the NES-capable UE e.g. the UE is a RedCap UE checks the system information on NES related configuration e.g. additional RACH configuration to obtain the access information and also considers the common resources as available for NES-capable UE.

Observation: a NES-capable UE which supports the feature e.g. RedCap, SDT, etc. needs to check the system information on NES related configuration to obtain the access information and also considers the common resources as available for NES-capable UE.

Further, there could be a case that the cell may not support RedCap UE, while supports NES-capable UE. Then, the cellBarred in the acquired MIB is set to barred and the cellBarredNES is set to notBarred. How does the UE consider the access restriction? Shall the UE determine the access restriction based on the feature and ignore the indication related with NES?

Proposal 3: RAN2 to discuss UE with any the feature, i.e. RedCap, SDT, etc. determines the access restriction based on the feature or UE determines the access restriction based on the indication related with NES.

# Conclusion

In summary, we have following observations and proposals for adaptation of common signal/channel transmissions:

Proposal 1: RAN2 to agree the additional RACH occasion for adaptation in time domain is separately configured in RACH configuration from legacy RACH configuration in SIB1.

Proposal 2: RAN2 to agree the separate RACH configuration indicates at least Number of SSBs per RACH occasion and the other config(s) e.g., msg1-FDM, msg1-FrequencyStart, may be also separately configured. If the other config(s) are not separately configured, UE default reuses the config in legacy RACH configuration.

Observation: a NES-capable UE which supports the feature e.g. RedCap, SDT, etc. needs to check the system information on NES related configuration to obtain the access information and also considers the common resources as available for NES-capable UE.

Proposal 3: RAN2 to discuss UE with any the feature, i.e. RedCap, SDT, etc. determines the access restriction based on the feature or UE determines the access restriction based on the indication related with NES.

# References

[1] RAN2#129 chairman notes.

[2] RAN1#118bis chairman notes.