3GPP TSG-RAN-WG2 Meeting #129bis	R2- 2502377

Wuhan, China, Apr. 7th – 11th , 2025

Agenda item	:		8.8.2

Source	: 	Sharp

Title	: 	Discussion on DL coverage enhancement

Document for	:	Discussion and Decision

Introduction

In the RAN2 #129 meeting, RAN2 made some agreements/conclusions as follows [1].

Agreements:

- RAN2 assumes it will be possible to have different SSB periodicity among neighbour cells in the same frequency layer

- RAN2 assumes that in a NR NTN cell, SSB beam sweeping in different spatial directions is possible as in a NR TN cell: the whole cell is covered by the different SSB beams in half-frame

- RAN2 also assumes that, with the current status of RAN1 discussion, if one cell is defined by multiple “satellite beams”, the satellite beams are all simultaneously active or inactive (“beam hopping” applies equally to all the satellite beams of a given cell)

The number of SMTC/gaps a UE needs to consider at any time will not be increased further.

- From RAN2 perspective, we support option a (clustered cells illuminated) and option b (scattered cells illuminated) for further discussion on SMTC. No LS is sent to RAN1. - RAN2 observes that if the cells active simultaneously are in clusters, existing SMTC mechanism (a maximum of 4 SMTCs per frequency) may be sufficient (FFS if any solution identified for option b applies to option a as well). - RAN2 observes that if the cells active simultaneously are scattered, (for the case of intra-frequency neighbouring cells), the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough.

In this paper, we further discuss the impacts for SSB periodicity and SMTC configuration.

Discussion

[Access Control]

In the previous meeting, RAN2 agreed to introduce new barring in SIB1 to be able to selectively bar UEs supporting DL CE if it is needed to bar UEs not supporting DL-CE via legacy mechanism. In this case, to avoid the useless cell detection/reselection of legacy UE, NW should also broadcast barring information for neighbour cells. In the current spec, SIB3/SIB4 includes the excluded cell list for intra-frequency/inter-frequency neighbouring cells. To prevent re-selection of legacy UE, it is natural to include cells operating with DL CE into intraFreqExcludedCellList (in SIB3) and interFreqExcludedCellList (in SIB4). In addition, new information to indicate whether listed cell is barred or not for UEs supporting DL CE should be introduced.

Observation 1: To avoid the useless cell detection/reselection of legacy UE, NW should also broadcast barring information for neighbour cells.

Proposal.1: If it is needed to bar UEs not supporting DL-CE (via legacy mechanism), cells operating with DL CE are included into intraFreqExcludedCellList (in SIB3) and interFreqExcludedCellList (in SIB4). In addition, new information to indicate whether listed cell is barred or not for UEs supporting DL CE is introduced.

[SMTC]

It was agreed that RAN2 can further consider SMTC impacts due to beam-hopping / larger SSB periodicity. SMTC is a parameter used to neighbour cell measurements and can specify the window of the SSB to be measured per frequency/cell by using the parameters of periodicity, offset, and duration.

The maximum measurement window for one SSB burst is currently 5ms, and SSB periodicity and offset can be supported up to 160ms in the current specification. For DL CE, there is no requirements to extend the number of SSB index (i.e. extension of duration), and current assumed extended SSB periodicity is 160ms. In the current spec, 4 SMTCs (smtc1 + 3 x smtc4) can be configured per frequency for NR NTN. Though this is originally introduced to cover differential propagation delay between satellites, if the hopping pattern is as shown in Figure.1, 4 SMTC configuration should be enough even if UE is located among multiple satellites. Therefore, SMTC parameters (periodicity, offset, and duration) are not needed to extend for option a (clustered cells illuminated) case.

Figure 1

On the other hand, option b (scattered cells illuminated) was also discussed, and it was suggested for this option that the SSBs of surrounding neighbour cells may be transmitted at different times and existing SMTC mechanism may not be enough.

For option b, in the current spec, location-based measurement reporting / conditional reconfiguration mechanism is specified, and the location of the serving cell and the target cell can be recognized by UE using SIB19. So, if the relation info between neighbour cell (physical cell id and frequency) and SMTC pattern is informed to UE, UE can determine the SMTC to use. In this case, more than 4 SMTC patterns are used for neighbour cells. However, they are linked to the location, and it is possible NW configures SMTCs not to exceed four for any locations.

Therefore, we think the location-based SMTC selection is appropriate. In addition, all necessary information can be notified by SIB, this can be adopted not only to connected mode UEs but also to idle/inactive mode UEs.

Observation 2: For the location-based SMTC, NW can configure SMTCs not to exceed 4 for any locations.

Proposal.2: The location-based SMTC selection is appropriate for option b.

Conclusion

In this paper, we made following observations and proposals;

[Access Control]

Observation 1: To avoid the useless cell detection/reselection of legacy UE, NW should also broadcast barring information for neighbour cells.

Proposal.1: If it is needed to bar UEs not supporting DL-CE (via legacy mechanism), cells operating with DL CE are included into intraFreqExcludedCellList (in SIB3) and interFreqExcludedCellList (in SIB4). In addition, new information to indicate whether listed cell is barred or not for UEs supporting DL CE is introduced.

[SMTC]

Observation 2: For the location-based SMTC, NW can configure SMTCs not to exceed 4 for any locations.

Proposal.2: The location-based SMTC selection is appropriate for option b.

Reference

- Draft Report of 3GPP TSG RAN WG2 meeting #129

- R2-2501415 Report of [AT129][304][R19 NR NTN] LS to RAN1 on DL coverage enhancements (Huawei)