3GPP TSG-RAN-WG2 Meeting #129bis	R2-2502380

Wuhan, China, 7th– 11th Apr, 2025

Agenda item	:		8.13.4

Source	: 	Sharp

Title	: 	discussion on service continuity for multi-hop relay

Document for	:	Discussion and Decision

Introduction

In RAN2#128/129 meeting, RAN2 reached the agreement as below [1][2];

RAN2#128 - For scenario B, RAN2 supports the case that the target U2N relay UE is a new relay UE which is not on the source relay path, and existing Rel-18 indirect path to indirect path switching can be reused. - For scenario B, RAN2 will support the case that the target single-hop relay UE is the last relay UE on the source path using the existing Rel-18 i2i path switching. Spec impact will be minimised. - For scenario B, RAN2 does not specify anything to support the case that that the target U2N relay UE is an intermediate relay UE which is on the source relay path using a single procedure. This case can be handled in the baseline control plane model, if necessary and subject to network implementation, by sequential path switches for the remote UE and the target intermediate relay UE. - The intra-gNB switching from indirect to direct path procedure defined in TS 38.300 for single hop case can be used as the baseline for multi-hop indirect to direct path switching. - Event X1 /X2 can be reused in multi-hop indirect to direct path switching with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported. - The intra-gNB switching from indirect path to indirect path procedure defined in TS 38.300 in single hop case can be used as baseline for multi-hop indirect to single hop indirect path switching procedure. - The following measurement events can be reused in multi-hop indirect to single-hop indirect path switching: - Event Y2 - Event Z1 with the understanding that the “first relay UE” in multi-hop relay link is “serving L2 U2N Relay UE” to be reported. - Figure 1 of R2-2410589 can be used as a baseline for intra-gNB multi-hop indirect to direct path switching procedure to be captured in TS 38.300. - Figure 2 of R2-2410589 can be used as a baseline for intra-gNB multi-hop indirect to single-hop indirect path switching to be captured in TS 38.300. NOTE: The current existing measurement framework and existing data forwarding mechanism should be reused. RAN2#129 - For multi-hop i2d, Remote UE evaluates both relay link (for the link between remote UE and serving first relay UE) and Uu link, where the sidelink relay measurement report shall include at least serving first relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result. - For multi-hop i2d, first relay UE evaluates adjacent upstream relay link and Uu link, where the sidelink relay measurement report shall include at least measured relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result. FFS if intermediate relay UE reports. - For multi-hop i2i, the remote UE reports serving first relay UE and candidate single-hop relay UE(s), including at least a source L2 ID, serving cell ID, and a sidelink measurement quantity information. - For multi-hop i2i, first evaluates adjacent upstream relay link and candidate relay link, where the sidelink relay measurement report shall include at least serving intermediate/ last relay UE's source L2 ID, serving cell ID and sidelink measurement quantity result. FFS if intermediate relay UE reports.

In this paper, we discuss some potential issues and solutions for it.

Discussion

  - mechanism enhancement for Scenario A and B

Scenario A and B are below;

- Intra-gNB multi-hop indirect to direct path switching using existing framework

- Intra-gNB multi-hop indirect to single-hop indirect path switching using existing framework

In RAN2#128 meeting, following procedures were agreed as baseline procedures for scenario A and B.

Figure1. baseline procedure for intra-gNB multi-hop indirect to direct path switching, i.e. scenario A

(from figure 1 of R2-2410589)

Here, we need to discuss details of the procedures. For multi-hop indirect to direct switching procedure, discussion points are follows;

Step1: Measurement configuration and report

- Trigger of measurement report

  - [Agreement]: X1/X2 are used with terminology changes

- Contents of measurement report [TBD]

Step2: Decision of switching to a direct cell

Step3: RRC Reconfiguration message (to Remote UE)

- Contents of RRC Reconfiguration [TBD]

Step4: RA to gNB

Step5: RRC Reconfiguration Complete message

- Completion of path switching procedure [TBD]

Step6: RRC Reconfiguration message (to source Relay UEs)

Step7: PC5 link release (between Remote UE and source First Relay UE)

- When/who to release the PC5 link [TBD]

Figure2. baseline procedure for intra-gNB multi-hop indirect to single-hop indirect path switching, i.e. scenario B

(from figure 2 of R2-2410589)

For multi-hop indirect to single-hop indirect switching procedure, discussion points are follows;

Step1: Measurement configuration and report

- Trigger of measurement report

  - [Agreement]: Y2/Z1 are used with terminology changes

    - [TBD]: Whether Z2 can be used

- Contents of measurement report [TBD]

Step2: Decision of switching to a target relay UE

- Whether the UE not in RRC_CONNECTED is selected as a target relay UE [TBD]

Step3: RRC Reconfiguration for Remote UE

- Contents of RRC Reconfiguration [TBD]

Step4: RRC Reconfiguration message (to Remote UE)

- Contents of RRC Reconfiguration [TBD]

Step5: PC5 connection establishment

Step6: RRC Reconfiguration Complete message

- Completion of path switching procedure [TBD]

Step7: RRC Reconfiguration message (to source Relay UEs)

- Whether the each PC5 link should be released [TBD]

Step8: PC5 link release (between Remote UE and source First Relay UE)

- When/who to release the PC5 link [TBD]

Then, RAN2 need to discuss following topics.

- What information should be included in the measurement report.

We will discuss this issue in clause 2.3.

- Whether the UE not in RRC_CONNECTED is selected as a target relay UE (multi-hop to single-hop specific issue).

In scenario B, i.e. intra-gNB multi-hop indirect to single-hop indirect path switching, the target relay UE is always potential last relay UE. The role of last relay UE is very similar with Rel-17 U2N relay UE. And RAN2 already supported the i2i path switching procedure with service continuity in Rel-18. In Rel-18, the UE not in RRC_CONNECTED is reported to the gNB and can be selected as a target U2N relay UE. Therefore, there shouldn’t be any issue to support the case the UE not in RRC_CONNECTED is selected as a target relay UE in intra-gNB multi-hop indirect to single-hop indirect path switching scenario (scenario B).

Proposal 1. In scenario B, the UE not in RRC_CONNECTED can be selected as a target relay UE.

- What information should be included in the RRCReconfiguration to be sent to the last relay UE (multi-hop to single-hop specific issue).

As described above, the role of last relay UE is very similar with Rel-17 U2N relay UE. And the target UE should be the Rel-17 U2N relay UE in scenario B. Therefore, to configure the target UE for the remote UE, legacy RRCReconfiguration message should be reused.

Proposal 2. To configure the target UE for the remote UE, legacy RRCReconfiguration message should be reused.

- What information should be included in the RRCReconfiguration to be sent to the remote UE.

In scenario A, the remote UE switches to the target cell. It is same with i2d path switching which is specified in Rel-17. And the RRCReconfiguration (with sync) message should be same as legacy handover command. Based on the same logic, the Timer T304 should be reused. Therefore, in scenario A, the legacy handover command (i.e. RRCReconfiguration with sync) is reused (including T304).

Proposal 3. In scenario A, the legacy handover command (i.e. RRCReconfiguration with sync) is reused (including T304).

In scenario B, the remote UE switches to the target UE which is acting as a Rel-17 U2N relay UE. It is same with i2i path switching which is specified in Rel-18. So, the RRCReconfiguration message should be same as legacy path switch command (reconfigurationWithSync including sl-PathSwitchConfig) including legacy T420.

Proposal 4. In scenario B, the path switch command (i.e. RRCReconfiguration with sync including sl-PathSwitchConfig) is reused (including T420).

- Whether the completion condition of path switching procedure in Rel-17/18 can be reused.

There’s no difference between multi-hop indirect to direct/single-hop indirect path switching and legacy i2d/i2i path switching from the perspective of completion phase. So, the completion condition of path switching procedure in Rel-17/18 should be reused for scenario A and B.

Proposal 5. The completion condition of path switching procedure in Rel-17/18 should be reused for scenario A and B.

- Whether/when the each PC5 link on source path should be released.

On the source path, after remote UE switches from the source path to the target path, PC5 link(s) and related configuration for the remote UE are not needed. In Rel-17/18, RAN2 already discussed service continuity issue. Based on the discussion,

- For DL, based on remote UE’s PDCP status report, NW should guarantee lossless path switching.

- For UL, remote UE only performs PDCP recovery-based re-transmission, and remaining packet at the source relay UE should be sent to the (source) gNB. After the gNB receives all remaining packets, the gNB will configure the source relay UE to release a configuration for the remote UE.

These principles can be applied to the scenario A and B. In other words, from the DL point of view, the source path can be released if remote UE switches, and from the UL point of view, at least the connection between the relay and remote can be released if remote UE switches. The exact timing can be up to the remote UE.

Proposal 6. The PC5 connection between the remote UE and the first relay UE can be released after the remote UE switches from the multi-hop indirect path to direct/single-hop indirect path. The exact timing is up to the remote UE.

For the configurations for the remote UE in the first relay UE and the last relay UE on the source path and the PC5 connection between the first relay UE and the last relay UE, for lossless path switching for UL data, it should be up to gNB whether/when to be released. Of course, if the last relay UE is acting as a remote UE, its PC5 connection may not be released (if same RSC is used for single-hop/multi-hop relaying).

Proposal 7. For lossless path switching for UL data, it is up to gNB whether/when the configurations for the remote UE in the first relay UE and the last relay UE on the source path and the PC5 connection between the first relay UE and the last relay UE are released.

Figure3. Intra-gNB multi-hop indirect to single-hop indirect path switching by first relay UE, i.e. scenario B

For scenario B, we can consider another procedure, i.e. the first relay UE switches the path while keeping the path with the remote UE as shown in fig.3. The procedure shown in fig.3 is used, for example, in case of wearable device including medical device. Furthermore, most of the steps in the fig.3 are supported by legacy I2D (indirect to direct) path switching procedure. Therefore, RAN2 should also consider the procedure as shown in fig. 3 for scenario B.

Proposal 8. RAN2 also considers the procedure as shown in fig. 3 for scenario B.

  - . mechanism enhancement for Scenario C and D

Scenario C and D are below;

- Intra-gNB direct to multi-hop indirect path switching

- Intra-gNB single-hop indirect to multi-hop indirect path switching

The scenarios C and D are limited to path switching to a target indirect path consisting of the last relay UE in “direct” RRC Connected mode and all the other intermediate relay(s) in “indirect” RRC Connected mode to the same cell.

Following procedures are example of the switch procedures of scenario C.

Figure 3. Intra-gNB direct to multi-hop indirect path switching procedure (scenario C)

First, RAN2 needs to decide an overall procedure to discuss details. So, we propose that figure3 should be considered as baseline for the D2M (direct to multi-hop indirect) path switching procedure.

Proposal 9. Figure 3 should be considered as baseline for the D2M (direct to multi-hop indirect) path switching procedure (scenario C).

For the measurement reporting event, Event Y1 (PCell becomes worse than threshold1 and candidate L2 U2N Relay UE becomes better than threshold2) and Event Y2 (Candidate L2 U2N Relay UE becomes better than threshold) can be used for this scenario since this path switching scenario is switching to target relay UE from source PCell.

Proposal 10. Current measurement reporting event Y1 and Y2 can be reused for D2M path switching procedure to report candidate relay UEs.

For scenario D, following procedures can be a baseline.

Figure 4-a. Intra-gNB single-hop indirect to multi-hop indirect path switching procedure by remote UE (scenario D)

Figure 4-b. Intra-gNB single-hop indirect to multi-hop indirect path switching procedure by relay UE (scenario D)

To clarify the scenario D, we consider whether the source last relay UE can be the target first relay UE. In a U2N relay use case, the remote UE may be a low-power device, e.g. wearable. In this case, remote UE may move with the source last relay UE. Therefore, there exists a scenario where the source last relay UE can be the target first relay UE and also be a remote UE of the target last relay UE. Not covering this scenario would limit the use cases of Rel-19 multi-hop relay.

Figure 5. One possible case of scenario D

Proposal 11. RAN2 confirm that the source last relay UE can be the target first relay UE and also be a remote UE of the target last relay UE.

We illustrated the case that the source last relay UE is the target first relay UE in fig.4-b. In fig.4-b, most of the procedures are supported by legacy D2I (direct to indirect) path switching procedure. If P11 is confirmed, RAN2 should consider fig. 4-a/4-b as baseline of scenario D.

Proposal 12. Figure 4-a/4-b should be considered as baseline for the I2M (single-hop indirect to multi-hop indirect) path switching procedure (scenario D).

For the measurement reporting event, Event X2 (Serving L2 U2N Relay UE becomes worse than threshold), event Y2 (Candidate L2 U2N Relay UE becomes better than threshold) and event Z1 (Serving L2 U2N Relay UE becomes worse than threshold1 and Candidate L2 U2N Relay UE becomes better than threshold2) can be used for this scenario since this path switching scenario is switching to target relay UE from source relay UE. This scenario is similar to I2I path switching as with M2I path switching procedure.

Proposal 13. Current measurement reporting event X2, Y2 and Z1 can be reused for I2M path switching procedure to report candidate relay UEs.

In R17, RAN2 introduced the Timer (T420) to manage whether the path switch is successful or not. We think T420 can be reused for MH U2N relay path switch. In U2U relay, the need for a Timer (T400) with double the existing value was recognized to consider the E2E delays caused by the increased number of hops. However, stop condition of T420 is sending RRCReconfigurationComplete message, i.e. when remote UE establishes PC5-RRC connection with relay UE and complete configuring PC5 RLC channel. It means that the increase in hop count has no relation to the value of T420.

Proposal 14. T420 can be reused for path switch to multi-hop indirect path without value expansion.

  - . Contents of candidate relay UE(s) report

In Rel-18 U2N relay, (candidate) remote UE sends measurement report to send candidate relay UEs information to the gNB. The measurement report includes following contents;

SL-MeasResultListRelay-r17 ::= SEQUENCE (SIZE (1..maxNrofRelayMeas-r17)) OF SL-MeasResultRelay-r17

SL-MeasResultRelay-r17 ::= SEQUENCE {

cellIdentity-r17			CellAccessRelatedInfo,

sl-RelayUE-Identity-r17	SL-SourceIdentity-r17,

sl-MeasResult-r17	SL-MeasResult-r16,

...,

[[

sl-MeasQuantity-r18	ENUMERATED { sl-rsrp, sd-rsrp }	OPTIONAL,

sl-RelayIndicationMP-r18	SL-RelayIndicationMP-r18		OPTIONAL

]]

}

For the cell access information, the gNB needs to find whether the candidate first relay UE reaches the same gNB. In legacy, remote UE reports the serving cell information of the candidate relay UE to the gNB. And the remote UE considers the serving cell of the relay UE as PCell. Same as the legacy, in multi-hop system, remote UE should report the serving cell information of last relay UE connecting with the candidate first relay UE.

Observation 1. gNB can decide whether the reported first relay UE can reach the gNB since first relay UE contains cellAccessRelatedInfo of serving cell of last relay UE in RRC container included in discovery message.

Proposal 15. The cellIdentity in sl-MeasResultRelay without enhancement can be used for decision whether the candidate first relay UE is connecting with the same gNB.

These information except MP indication is the minimum required information. So, RAN2 can reuse them.

Proposal 16. Candidate UE’s cell info, Candidate relay UE’s source ID, Measurement result and information indicating whether the measurement result is SL-RSRP or SD-RSRP are reported to the gNB as with legacy.

We also consider what information gNB needs to select the appropriate PC5 links. We assume the AS layer evaluation metrics that the gNB uses to select optimal PC5 links are PC5 link quality of each PC5 link, number of hops, and QoS profile for the (potential) remote UE. This allows the gNB to select a path with low latency and/or robustness (i.e., less likely to be disconnected) depending on the QoS and radio resource management of the remote UE. According to the current SA2 discussion, the discovery message may include hop counter, ID of last relay UE and accumulated QoS for the PC5 links (i.e. Achievable PDB) in addition to the legacy contents. Therefore, the remote UE knows these metrics. Then, the gNB knows that the reported first relay UE is connecting with the same gNB or not, the number of hops of the PC5 links to which the reported candidate first relay UE belongs, and the QoS profile for the (potential) remote UE. So, for selecting appropriate path (first relay UE), UE should send the information of PC5 link quality of the path the first relay UE belongs. Furthermore, the greater number of hops there are, the more the quality of each hop affects the delay and the efficiency of radio resource utilization. So, we consider that the UE needs to report to the gNB the PC5 link qualities of each hops of the path first relay UE belongs or accumulated QoS for the PC5 links (i.e. Achievable PDB).

Observation 2. In order to select appropriate path, from the perspective of QoS and radio resource management, gNB needs to know PC5 link quality of the PC5 links, number of hops, and QoS profile for the (potential) remote UE.

Observation 3. gNB already knows number of hops, and QoS profile for the (potential) remote UE.

Observation 4. The greater number of hops there are, the more the quality of each hop affects the delay and the efficiency of radio resource utilization.

Proposal 17. Potential remote UE reports to the gNB the PC5 link qualities of each hops of the path first relay UE belongs or accumulated QoS for the PC5 links (i.e. Achievable PDB).

Conclusion

Proposal 1. In scenario B, the UE not in RRC_CONNECTED can be selected as a target relay UE.

Proposal 2. To configure the target UE for the remote UE, legacy RRCReconfiguration message should be reused.

Proposal 3. In scenario A, the legacy handover command (i.e. RRCReconfiguration with sync) is reused (including T304).

Proposal 4. In scenario B, the path switch command (i.e. RRCReconfiguration with sync including sl-PathSwitchConfig) is reused (including T420).

Proposal 5. The completion condition of path switching procedure in Rel-17/18 should be reused for scenario A and B.

Proposal 6. The PC5 connection between the remote UE and the first relay UE can be released after the remote UE switches from the multi-hop indirect path to direct/single-hop indirect path. The exact timing is up to the remote UE.

Proposal 7. For lossless path switching for UL data, it is up to gNB whether/when the configurations for the remote UE in the first relay UE and the last relay UE on the source path and the PC5 connection between the first relay UE and the last relay UE are released.

Proposal 8. RAN2 also considers the procedure as shown in fig. 3 for scenario B.

Proposal 9. Figure 3 should be considered as baseline for the D2M (direct to multi-hop indirect) path switching procedure (scenario C).

Proposal 10. Current measurement reporting event Y1 and Y2 can be reused for D2M path switching procedure to report candidate relay UEs.

Proposal 11. RAN2 confirm that the source last relay UE can be the target first relay UE and also be a remote UE of the target last relay UE.

Proposal 12. Figure 4-a/4-b should be considered as baseline for the I2M (single-hop indirect to multi-hop indirect) path switching procedure (scenario D).

Proposal 13. Current measurement reporting event X2, Y2 and Z1 can be reused for I2M path switching procedure to report candidate relay UEs.

Proposal 14. T420 can be reused for path switch to multi-hop indirect path without value expansion.

Observation 1. gNB can decide whether the reported first relay UE can reach the gNB since first relay UE contains cellAccessRelatedInfo of serving cell of last relay UE in RRC container included in discovery message.

Proposal 15. The cellIdentity in sl-MeasResultRelay without enhancement can be used for decision whether the candidate first relay UE is connecting with the same gNB.

Proposal 16. Candidate UE’s cell info, Candidate relay UE’s source ID, Measurement result and information indicating whether the measurement result is SL-RSRP or SD-RSRP are reported to the gNB as with legacy.

Observation 2. In order to select appropriate path, from the perspective of QoS and radio resource management, gNB needs to know PC5 link quality of the PC5 links, number of hops, and QoS profile for the (potential) remote UE.

Observation 3. gNB already knows number of hops, and QoS profile for the (potential) remote UE.

Observation 4. The greater number of hops there are, the more the quality of each hop affects the delay and the efficiency of radio resource utilization.

Proposal 17. Potential remote UE reports to the gNB the PC5 link qualities of each hops of the path first relay UE belongs or accumulated QoS for the PC5 links (i.e. Achievable PDB).

Reference

- R2-2410914, Report from session on positioning and sidelink relay

- R2-2501334, Report from session on positioning and sidelink relay