3GPP TSG-RAN WG2 Meeting #129bis	R2-2502386

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.5.4

Source:	vivo

Title:	Discussion on adaptation of common signal transmissions

Document for:	Discussion and Decision

Introduction

In this contribution, we will discuss the remaining open issues on paging adaptation and SSB adaptation in time domain.

Discussion

## 2.1 Paging adaptation in the time domain

In this section, we will discuss the following open issues:

- Signaling overhead optimization of configuring Rel-19 firstPDCCH-MonitoringOccasionOfPO;

- Remaining open issues on UE capability reporting;

- Whether to introduce a separate pei-ConfigBWP for paging adaptation;

- UE behavior if pagingAdaptationPEI-Config is configured.

Signaling overhead optimization of configuring Rel-19 firstPDCCH-MonitoringOccasionOfPO:

In the legacy firstPDCCH-MonitoringOccasionOfPO configuration, each PO (first PDCCH MO of the PO) can be configured on any symbol between two adjacent PFs. Thus, the maximum signaling overhead of legacy firstPDCCH-MonitoringOccasionOfPO is 4*17 = 68 bits, given by the configuration shown below.

firstPDCCH-MonitoringOccasionOfPO   CHOICE {

sCS15KHZoneT                                                                SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                                  SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                                 SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT             SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                                    SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..17919)

}      OPTIONAL,           -- Need R

firstPDCCH-MonitoringOccasionOfPO-v1710  CHOICE {

sCS480KHZoneEighthT        SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT     SEQUENCE (SIZE (1..maxPO-perPF)) OF INTEGER (0..71679)

In Rel-19 NES, N = T/32, Ns = 8 are supported. Follow the legacy logical, the following table 1 analyzes the signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO, assuming N = T/32, Ns = 8.

Table 1: signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO

| SCS | Total OFDM symbols per radio frame | Total OFDM symbols across T (T = 32) | Overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO (Ns = 8) |
| --- | --- | --- | --- |
| 15KHz | 140 | 4480 (13 bits) | 104 bits |
| 30KHz | 280 | 8960 (14 bits) | 112 bits |
| 60KHz | 560 | 17920 (15 bits) | 120 bits |
| 120KHz | 1120 | 35840 (16 bits) | 128 bits |
| 480KHz | 4480 | 143360 (18 bits) | 144 bits |

It can be observed that the maximum signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO configuration exceeds 2 times that of the legacy. Additionally, it should be noted that both the legacy configuration and the Rel-19 configuration can be configured at the same time, meaning only firstPDCCH-MonitoringOccasionOfPO configuration would occupy 68 + 144 =  212 bits, and the maximum SIB1 size is only 2976 bits. We think such signaling overhead is unacceptable and should be optimized.

TS 38.331 NOTE 2: The physical layer imposes a limit to the maximum size a SIB can take. The maximum SIB1 or SI message size is 2976 bits.

In fact, the value range of Rel-19 firstPDCCH-MonitoringOccasionOfPO corresponding to N = 1/32T does not need to span the whole 32 radio frames, considering the target scenario is for NES. Otherwise, the network can simply configure N = 1/16T or 1/8T etc. Therefore, one solution is to restrict the maximum value range of firstPDCCH-MonitoringOccasionOfPO. Table 2 summarized the signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO if different maximum value range of Rel-19 firstPDCCH-MonitoringOccasionOfPO is used.

Table 2: signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO for different maximum value range

| SCS | Total OFDM symbols per radio frame | Overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO (value range = 32 frames) | Overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO (value range = 16 frames) | Overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO (value range = 2 frames) |
| --- | --- | --- | --- | --- |
| 15KHz | 140 | 104 bits | 96 bits | 72 bits |
| 30KHz | 280 | 112 bits | 104 bits | 80 bits |
| 60KHz | 560 | 120 bits | 112 bits | 88 bits |
| 120KHz | 1120 | 128 bits | 120 bits | 96 bits |
| 480KHz | 4480 | 144 bits | 136 bits | 112 bits |

Another approach is to sacrifice some unnecessary flexibility in case of NES by having the network configure evenly spaced POs. For example, the network only needs to configure the starting PO position among 8 POs and the interval between them. Both the starting PO position and the interval require up to 18 bits from the overhead listed in Table 1 for 480KHz, thus the signaling overhead of Rel-19 firstPDCCH-MonitoringOccasionOfPO can be limited to 36 bits. Given the above, we have the following proposal:

Proposal 1: For signaling overhead optimization of Rel-19 firstPDCCH-MonitoringOccasionOfPO, RAN2 to discuss the following options:

- Option 1: restrict the maximum value range of Rel-19 firstPDCCH-MonitoringOccasionOfPO for different SCS corresponding to the Rel-19 nAndPagingFrameOffset (i.e. N);

- Option 2: support configuring evenly distributed POs, i.e., the first PO position among 8 POs and the interval between them.

Remaining open issues on UE capability reporting:

In the last RAN2 meeting, RAN2 made the following FFS when discussing the Rel-19 NES UE radio paging capability:

A new UE capability is added for R19 NES paging enhancement, and the new capability is included in UE-RadioPagingInfo. FFS on whether we have a common capability for all NES features.

RAN1 send an LS [R1-2501399] to RAN2 on Rel-19 RAN1 UE features list for NR after RAN1#120, which includes the UE feature list for Rel-19 NES WI. It can be observed from it that basically all the Rel-19 NES feature is optional with/without capability signaling. Each feature can be controlled by the network according to the corresponding UE capability signaling, so we don’t see the need to define a common UE capability for all Rel-19 NES features.

Proposal 2: No need to define a common capability for all Rel-19 NES features.

Whether to introduce a separate pei-ConfigBWP for paging adaptation?

In the current RRC Running CR, a separate PEI-Config-r17 is introduced.

There is an Editor’s note in the CR:

FFS: Do we need to introduce a separate pei-ConfigBWP for paging adaptation?

In the existing PEI scheme, the UE determines its PEI-O based on both PEI-Config-r17 and pei-ConfigBWP-r17 configurations, where PEI-Config-r17 provides a reference point that is a frame-level offset from the start of the first PF of the PF(s) associated with the PEI-O, and pei-ConfigBWP-r17 provides a symbol-level offset from the reference point to the start of the first PDCCH MO of PEI-O. In case of Rel-19 NES, if Ns = 8 is configured, all 8 POs belongs to a single PF and will be linked to a single reference time (pei-FrameOffset-r17) and 1~4 PEI-O(s) which is based on the configured po-NumPerPEI, please note that po-NumPerPEI cannot be set to 1, since the maximum of PEI-O per PF (maxPEI-perPF-r17) is 4. And the corresponding PDCCH MO of 1~4 PEI-O(s) is according to the pei-ConfigBWP-r17. Therefore, to determine the PEI-O of Rel-19 PO, both PEI-Config-r17 and pei-ConfigBWP-r17 configurations are required.

Proposal 3: Introduce a separate pei-ConfigBWP for paging adaptation. FFS whether to introduce a new maxPEI-perPF-r19.

Moreover, similar signaling overhead issue exists also for firstPDCCH-MonitoringOccasionOfPEI-O-r17, thus the similar handling of Rel-19 firstPDCCH-MonitoringOccasionOfPO can be considered for firstPDCCH-MonitoringOccasionOfPEI-O-r17.

Proposal 4: If P1 is agreed, similar handling of Rel-19 firstPDCCH-MonitoringOccasionOfPO is also applied to Rel-19 firstPDCCH-MonitoringOccasionOfPEI-O-r17.

PEI-Config-r17 ::=                        SEQUENCE {

po-NumPerPEI-r17                          ENUMERATED {po1, po2, po4, po8},

payloadSizeDCI-2-7-r17                    INTEGER (1..maxDCI-2-7-Size-r17),

pei-FrameOffset-r17                       INTEGER (0..16),

subgroupConfig-r17                        SubgroupConfig-r17,

lastUsedCellOnly-r17                      ENUMERATED {true}                                                OPTIONAL,  -- Need R

...

}

pei-ConfigBWP-r17      SEQUENCE {

pei-SearchSpace-r17                 SearchSpaceId,

firstPDCCH-MonitoringOccasionOfPEI-O-r17  CHOICE {

sCS15KHZoneT                                                     SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..139),

sCS30KHZoneT-SCS15KHZhalfT                                       SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..279),

sCS60KHZoneT-SCS30KHZhalfT-SCS15KHZquarterT                      SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..559),

sCS120KHZoneT-SCS60KHZhalfT-SCS30KHZquarterT-SCS15KHZoneEighthT  SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..1119),

sCS120KHZhalfT-SCS60KHZquarterT-SCS30KHZoneEighthT-SCS15KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..2239),

sCS480KHZoneT-SCS120KHZquarterT-SCS60KHZoneEighthT-SCS30KHZoneSixteenthT SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..4479),

sCS480KHZhalfT-SCS120KHZoneEighthT-SCS60KHZoneSixteenthT         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..8959),

sCS480KHZquarterT-SCS120KHZoneSixteenthT                         SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..17919),

sCS480KHZoneEighthT                                           SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..35839),

sCS480KHZoneSixteenthT                                        SEQUENCE (SIZE (1..maxPEI-perPF-r17)) OF INTEGER (0..71679)

}

}

pei-FrameOffset Offset, in number of frames from the start of a reference frame for PEI-O to the start of a first paging frame of the paging frames associated with the PEI-O, see TS 38.213 [13], clause 10.4A.

firstPDCCH-MonitoringOccasionOfPEI-O Offset, in number of symbols, from the start of the reference frame for PEI-O to the start of the first PDCCH monitoring occasion of PEI-O on this BWP, see TS 38.213 [13], clause 10.4A. For the case po-NumPerPEI is smaller than Ns, UE applies the (floor(i_s/po-NumPerPEI)+1)-th value out of (N_s/po-NumPerPEI) configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset. When po-NumPerPEI is one or multiple of Ns, UE applies the first configured value in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset.

UE behavior if pagingAdaptationPEI-Config is configured.

In the post RAN2#129 email discussion [POST129][104][NES] (Apple), the following issue was pointed out by the companies.

| Nokia002 | PEI – You seem to have capture incorrectly agreement in RAN2 meeting – agreement is “ Introduce a separate PEI configuration” – there is no agreement UE only monitors Pei derived from this set of PEI parameters. Please remove that. [Huawei] Agree with Nokia. There are two possible behaviours for R19 NES UEs: Option 1) R19 NES UEs only monitor R19 PEI Option 2) R19 NES UEs monitor legacy PEI for legacy POs and R19 PEI for R19 POs It should be further discussed in the next meeting which is the intended UE behaviour. |
| --- | --- |

Thus, there is an Editor’s note in the current 38.304 Running CR:

For a UE supporting paging adaptation and PEI, if pagingAdaptationPEI-Config is signaled in system information, its behavior is FFS. Editor’s note: FFS the UE behavior if pagingAdaptationPEI-Config is configured.

From our understanding, if pagingAdaptationPEI-Config is configured for the UE supporting paging adaptation and PEI, it is clear that the intended UE behavior is the above option 1. We don’t see any motivation to monitor legacy PEI for the UE supporting paging adaptation in this case. This UE behaviour against with the following RAN2 agreement as mentioned by the rapporteur, and will cause more UE power consumption and UE complexity as the UE needs to read the legacy PEI configuration and calculate the legacy PEI-O.

- From the UE point of view, UE will monitor one PEI/PO every paging DRX cycle, i.e. the UE doesn’t skip PO in paging DRX cycle.

Proposal 5: If pagingAdaptationPEI-Config is configured for the UE supporting paging adaptation and PEI, the UE is only required to monitor Rel-19 PEI calculated by pagingAdaptationPEI-Config.

In addition, we think it is unclear that the UE behavior if pagingAdaptationPEI-Config is NOT configured for the UE supporting paging adaptation and PEI. There are two UE behaviors:

- UE behavior 1: the UE does not monitor PEI.

- UE behavior 2: the UE monitors the PEI based on the legacy PEI configuration.

We believe more specification work is needed in RAN1 and RAN2 to support the UE behavior 2, thus, we understand the UE behavior 1 is the expected UE behavior.

Proposal 6: If pagingAdaptationPEI-Config is not configured and NES PO is configured, the Rel-19 NES UE does not monitor legacy PEI.

## 2.2 SSB adaptation in the time domain

In the last RAN2 meeting, RAN2 made the following agreements for SSB adaptation:

Agreements on SSB adaptation - RAN2 confirms SSB adaptation in time domain is not supported for RRC idle/inactive UEs and Rel-19 NES-capable UE’s PCell. - RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation. - For L3 measurement, RAN2 assumes the adapted SSB on neighbor cell is measured based on legacy SMTC.

Moreover, RAN1 has agreed in RAN1#120 meeting to support group common DCI signalling for switching the SSB burst periodicity, and leave MAC-CE design for SSB periodicity adaptation to RAN2.

Agreement For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

If MAC CE based SSB periodicity adaptation is supported, the signaling framework for OD-SSB SCell and SSB periodicity adaptation is similar from our perspective. That is, the network configures multiple sets of OD-SSB configurations for OD-SSB or multiple SMTC configurations for different SSB periodicities via RRC, and uses MAC CE to dynamically indicate which configuration to use. Therefore, we think it is simple to design an unified MAC CE to support both features, allowing the UE to choose supporting only MAC CE based SSB periodicity adaptation without supporting group common DCI based SSB periodicity adaptation. The detailed design can wait further RAN1 progress on both OD-SSB and SSB adaptation.

Proposal 7: Support an unified MAC CE design to support both OD-SSB SCell and SSB periodicity adaptation.

Conclusion

Based on the discussion, we have the following proposals:

Adaptation of paging occasions in the time domain

Proposal 1: For signaling overhead optimization of Rel-19 firstPDCCH-MonitoringOccasionOfPO, RAN2 to discuss the following options:

- Option 1: restrict the maximum value range of Rel-19 firstPDCCH-MonitoringOccasionOfPO for different SCS corresponding to the Rel-19 nAndPagingFrameOffset (i.e. N);

- Option 2: support configuring evenly distributed POs, i.e., the first PO position among 8 POs and the interval between them.

Proposal 2: No need to define a common capability for all Rel-19 NES features.

Proposal 3: Introduce a separate pei-ConfigBWP for paging adaptation. FFS whether to introduce a new maxPEI-perPF-r19.

Proposal 4: If P1 is agreed, similar handling of Rel-19 firstPDCCH-MonitoringOccasionOfPO is also applied to Rel-19 firstPDCCH-MonitoringOccasionOfPEI-O-r17.

Proposal 5: If pagingAdaptationPEI-Config is configured for the UE supporting paging adaptation and PEI, the UE is only required to monitor Rel-19 PEI calculated by pagingAdaptationPEI-Config.

Proposal 6: If pagingAdaptationPEI-Config is not configured and NES PO is configured, the Rel-19 NES UE does not monitor legacy PEI.

Adaptation of SSB adaptation in the time domain

Proposal 7: Support an unified MAC CE design to support both OD-SSB SCell and SSB periodicity adaptation.

# 4. Reference