3GPP TSG-RAN WG2 Meeting #129bis                              	  R2-2502392

Wuhan, China, April. 7th – 11th, 2025

Agenda Item:	8.12.2

Source: 	OPPO

Title:  	Discussion on the remaining issues of the pathloss offset update

Document for:	Discussion and Decision

# Introduction

According to the RAN1 discussion on the asymmetric UL/DL TRP, RAN1 agreed to use both RRC and MAC CE to configure the pathloss offset as follows:

RAN1#116bis Agreements For FR1, a joint TCI state can be associated with a PL offset. - When a joint TCI state associated with a PL offset is applied for the PUSCH/PUCCH/SRS transmission, the UE shall calculate the Tx power of the PUSCH/PUCCH/SRS based on the DL PL RS and PL offset associated with this joint TCI state. - Reuse the legacy uplink power control formulation by replacing legacy PL with a PL which is derived from the DL PL RS and the PL offset. - FFS: The UE can update UL PL in a way that new UL PL = current UL PL + an update delta indicated by the NW. For the association between PL offset and joint/UL TCI state, consider and down-select one from the following Alts: - Alt1a: One PL offset value is configured in a joint or UL TCI state by RRC only - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s). - Alt2a: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value. One new RRC parameter is introduced in a joint or UL TCI state to indicate one of the configured PL offset configurations. - Alt2b: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value. One new RRC parameter is introduced in a joint or UL TCI state to indicate one of the configured PL offset configurations. A MAC CE can update the association between a joint or UL TCI state and PL offset configuration - Alt3: A list of PL offset configurations is configured by RRC in BWP/CC and each PL offset configuration contains one PL offset value. A MAC CE can activate/indicate one PL offset configuration for each activated joint or UL TCI state. In each joint or UL TCI state, the initial PL offset value is 0dB. - Alt4: A list of PL offset values is provided in a joint or UL TCI state by RRC. Each PL offset value is applied to a corresponding measured PL range. Other alternatives are not precluded.

RAN1#117 Agreements For the association between PL offset and joint/UL TCI state, support the following - Alt1b: One PL offset value is configured in a joint or UL TCI state by RRC, where different PL offset values can be configured to different joint or UL TCI states. A MAC CE can update the PL offset value(s) for joint or UL TCI state(s).

RAN1#118 Agreements For the asymmetric DL sTRP/UL mTRP scenarios, the value range of PL offset includes at least [-10, 60] dB. - FFS: Extending the range to lower than -10dB - Step size is 4dB For the asymmetric DL sTRP/UL mTRP scenarios, support to include PL offset in the calculation of Type 1 PHR based on actual PUSCH transmission and Type 1 PHR based on reference PUSCH.

RAN1#118bis Agreements The lower limit of the value range of PL offset is extended to -12 dB.

According to the previous RAN2 discussion on how the PL offset is configured and updated via both RRC and MAC CE, RAN2 made the following agreements:

RAN2#128 Agreements: - New MAC CE is introduced for PL offset update for asymmetric DL sTRP/UL mTRP. This new MAC CE is identified by new eLCID. - Absolute value of PL offset is indicated in the new MAC CE. For the offset value, the value range is [-12, 60] dB and the step size is 4dB. - In the MAC CE, PL offset value can be updated for any configured TCI states with RRC configured PL offset, i.e., not limited to the activated TCI states

RAN2#129 Agreements: - One PL offset value is indicated for each TCI state included in the new MAC CE. - The new MAC CE contains one serving cell ID and one BWP ID - TCI state ID is used to indicate a TCI state in the new MAC CE (i.e., no bitmap for TCI states is needed) - The new MAC CE can include flexible number of PL offset values. - Working assumption: UE applies the latest PL offset value received in RRC or MAC CE. Can revisit if new issue is found. - RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

In this contribution, we discuss the remaining issues regarding the PL offset configuration.

# Discussion

## PHR

RAN2#129: - RAN2 understands that if a joint/UL TCI state is configured with a PL offset, PHR trigger is based on the PL change of the PL-RS associated to the joint/UL TCI, where the PL change takes into account the PL offset. FFS whether/how to capture this.

TS 38.321: ### 5.4.6 Power Headroom Reporting … A Power Headroom Report (PHR) shall be triggered if any of the following events occur: - phr-ProhibitTimer expires or has expired and the path loss has changed more than phr-Tx-PowerFactorChange dB for at least one RS used as pathloss reference for one activated Serving Cell of any MAC entity of which the active DL BWP is not dormant BWP since the last transmission of a PHR in this MAC entity when the MAC entity has UL resources for new transmission; NOTE 1: The path loss variation for one cell assessed above is between the pathloss measured at present time on the current pathloss reference and the pathloss measured at the transmission time of the last transmission of PHR on the pathloss reference in use at that time, irrespective of whether the pathloss reference has changed in between. The current pathloss reference for this purpose does not include any pathloss reference configured using pathlossReferenceRS-Pos in TS 38.331 [5]. …

As quoted above, it is RAN2 common understanding that for the PHR trigger condition of the pathloss change, the pathloss result calculated by the UE includes both the measurement result of the pathloss reference and the pathloss offset provided by the RRC configuration and the pathloss offset update MAC CE. However, according to the specification text as quoted above from TS 38.321, the NOTE 1 in section “5.4.6 Power Headroom Reporting” could cause some misunderstanding that the pathloss used for the PHR trigger only considers the pathloss result measured on the pathloss reference, and the pathloss offset configured via RRC or MAC CE is not included.

Observation 1: The current NOTE 1 in the TS 38.321 causes the following misunderstandings:

- The pathloss measured at present time only includes the measurement result of “the current pathloss reference”, and does not include the pathloss offset.

- the pathloss measured at the transmission time of the last transmission of PHR only includes the measurement result of “the pathloss reference in use at that time”, and does not include the pathloss offset.

To avoid the misunderstanding in the NOTE 1 of TS 38.321 and the subsequent IoT test issues, we think that the NOTE 1 in 5.4.6 of TS 38.321 should be clarified that the pathloss used for triggering PHR includes both the measurement result of the pathloss reference and the pathloss offset configured by RRC and MAC CE.

Proposal 1: To clarify the NOTE 1 in 5.4.6 of TS 38.321 that the pathloss used for triggering PHR includes both the measurement result of the pathloss reference and the pathloss offset.

## Need Code

According to the latest RAN1 parameter list, pathlossOffset is configured in TCI-State and TCI-UL-State.

TCI-State ::= SEQUENCE { tci-StateId TCI-StateId, qcl-Type1 QCL-Info, qcl-Type2 QCL-Info OPTIONAL, -- Need R ..., [[ additionalPCI-r17 AdditionalPCIIndex-r17 OPTIONAL, -- Need R pathlossReferenceRS-Id-r17 PathlossReferenceRS-Id-r17 OPTIONAL, -- Cond JointTCI1 ul-powerControl-r17 Uplink-powerControlId-r17 OPTIONAL -- Cond JointTCI ]], [[ tag-Id-ptr-r18 ENUMERATED {n0,n1} OPTIONAL -- Cond 2TA ]], [[ pathlossOffset-r18 ENUMERATED {dB-12, dB-10, dB-6, dB-2, dB0, dB4, dB8, dB12, dB16, dB20, dB24, dB28, dB32, dB36, dB40, dB44, dB48, dB52, dB56, dB60} OPTIONAL -- Need R? ]] }

In general, when the pathlossOffset is configured/updated via RRC or MAC CE, the UE stores the pathlossOffset configuration and use the latest pathlossOffset value for the uplink-only TRP. This means that the UE should change its stored RRC configuration of pathlossOffset, when MAC CE provides a new value of pathlossOffset. However, according to the TS 38.331 guidance of using “need code”, which can be also found in the Annex A, the description of the meaning of “need code” only refers to the configuration provided by the RRC message, not the MAC CE. Then, we could have the following issues of using the “need code” for pathlossOffset updated via both RRC message and MAC CE.

| Need Code | Meaning in TS 38.331 | Issues |
| --- | --- | --- |
| Need S | Specified Used for (configuration) fields, whose field description or procedure specifies the UE behavior performed upon receiving a message with the field absent (and not if field description or procedure specifies the UE behavior when field is not configured). | Issue 1: It is unclear what UE behaviours need to be specified regarding the configuration, e.g. whether/how to store the field and the UE behaviour when field is not absent. |
| Need M | Maintain Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE maintains the current value. | Issue 1: If Need M is used, an explicit release for the pathlossOffset is required. Issue 2: If the pathlossOffset is absent, it is unclear whether the UE should apply the last value configured by RRC or the latest value updated by MAC CE. The meaning of “the current value” used to refer to only the RRC configuration (i.e. not MAC CE). |
| Need N | No action (one-shot configuration that is not maintained) Used for (configuration) fields that are not stored and whose presence causes a one-time action by the UE. Upon receiving message with the field absent, the UE takes no action. | Issue 1: Need N is not applicable, as pathlossOffset should be stored for the use of many cases, e.g. power control and RACH. |
| Need R | Release Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE releases the current value. | Issue 1: The pathlossOffset should be always provided in TCI-state when TCI-state is configured, as the absence of pathlossOffset requires the UE to release it. Issue 2: If the pathlossOffset is absent, it is unclear whether the UE should release the last value configured by RRC or the latest value updated by MAC CE. The meaning of “the current value” used to refer to only the RRC configuration (i.e. not MAC CE). |

Observation 2: In 38.331, when the pathlossOffset is absent in RRC configuration, it can be misunderstood that the “current” pathlossOffset configuration stored at the UE only refers to the last RRC configuration.

Before RAN2 decides which “need code” is used, we think that RAN2 should firstly confirm whether the pathlossOffset updated by MAC is stored in RRC. If the pathlossOffset updated by MAC is not stored in RRC, the UE implementation would need two separate internal variables of pathloss offset, i.e. one variable for RRC and one variable for MAC. The UE would compare the two values and use the latest one. This will complicate the UE implementation. If the pathlossOffset updated by MAC is stored/updated in RRC variable, the UE implementation can be simplified that each time when a new value of pathlossOffset is provided by either RRC or MAC CE, the UE updates the RRC pathlossOffset variable. If RAN2 confirmed that the pathlossOffset updated by MAC CE also updates the pathlossOffset value stored in RRC, we think that either “Need R” or “Need M” can be used, and “Need R” is simpler from the specification.

Proposal 2: To clarify in TS 38.331 that when the pathlossOffset is absent in RRC configuration, the “current” pathlossOffset configuration stored at the UE refers to the latest configuration provided by RRC message or MAC CE.

Proposal 3: “Need R” is used for pathlossOffset in RRC.

## Processing delay

According to the processing delay of RRC configuration as quoted in Annex B from TS 38.331, in addition to the 10ms of normal RRC processing delay, extra 6ms are added for scell addition/release and SCG establishment/ modification/ release due to the change of the PHY configuration. The RRC processing delay include the decoding time of the RRC message and the time required for applying the RRC configuration. When the pathlossOffset is updated via MAC CE, it is unclear what the processing delay requirement for updating pathlossOffset configuration is. From our understanding, the expected delay should be less than 16ms, as the decoding time required for MAC CE is lower than the decoding time required for RRC message. We think that to allow sufficient processing time in the UE, we could choose 10ms or “leave to UE implementation” for the processing delay requirement for updating pathlossOffset configuration via MAC CE.

Proposal 4: RAN2 is kindly requested to confirm one of the following options for the maximum processing delay requirement of updating pathlossOffset configuration via MAC CE.

- Option 1: 10ms, as RRC reconfiguration

- Option 2: up to UE implementation

# Conclusion

According to the analysis provided above, we have the following observation and proposals:

Observation 1: The current NOTE 1 in the TS 38.321 causes the following misunderstandings:

- The pathloss measured at present time only includes the measurement result of “the current pathloss reference”, and does not include the pathloss offset.

- the pathloss measured at the transmission time of the last transmission of PHR only includes the measurement result of “the pathloss reference in use at that time”, and does not include the pathloss offset.

Observation 2: In 38.331, when the pathlossOffset is absent in RRC configuration, it can be misunderstood that the “current” pathlossOffset configuration stored at the UE only refers to the last RRC configuration.

Proposal 1: To clarify the NOTE 1 in 5.4.6 of TS 38.321 that the pathloss used for triggering PHR includes both the measurement result of the pathloss reference and the pathloss offset.

Proposal 2: To clarify in TS 38.331 that when the pathlossOffset is absent in RRC configuration, the “current” pathlossOffset configuration stored at the UE refers to the latest configuration provided by RRC message or MAC CE.

Proposal 3: “Need R” is used for pathlossOffset in RRC.

Proposal 4: RAN2 is kindly requested to confirm one of the following options for the maximum processing delay requirement of updating pathlossOffset configuration via MAC CE.

- Option 1: 10ms, as RRC reconfiguration

- Option 2: up to UE implementation

# Reference

[1] 3GPP TS38.331, “NR; Radio Resource Control (RRC) protocol specification”

# Annex A

The follows are quoted from TS 38.331:

Table 6.1.2-1: Meaning of abbreviations used to specify the need for fields to be present

| Abbreviation | Meaning |
| --- | --- |
| Cond conditionTag | Conditionally present Presence of the field is specified in a tabular form following the ASN.1 segment. |
| CondC conditionTag | Configuration condition Presence of the field is conditional to other configuration settings. |
| CondM conditionTag | Message condition Presence of the field is conditional to other fields included in the message. |
| Need S | Specified Used for (configuration) fields, whose field description or procedure specifies the UE behavior performed upon receiving a message with the field absent (and not if field description or procedure specifies the UE behavior when field is not configured). |
| Need M | Maintain Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE maintains the current value. |
| Need N | No action (one-shot configuration that is not maintained) Used for (configuration) fields that are not stored and whose presence causes a one-time action by the UE. Upon receiving message with the field absent, the UE takes no action. |
| Need R | Release Used for (configuration) fields that are stored by the UE i.e. not one-shot. Upon receiving a message with the field absent, the UE releases the current value. |

# Annex B

The followings are quoted from TS 38.331:

12	Processing delay requirements for RRC procedures

The UE performance requirements for RRC procedures are specified in the following tables. The performance requirement is expressed as the time in [ms] from the end of reception of the network -> UE message on the UE physical layer up to when the UE shall be ready for the reception of uplink grant for the UE -> network response message with no access delay other than the TTI-alignment (e.g. excluding delays caused by scheduling, the random access procedure or physical layer synchronisation). In case the RRC procedure triggers BWP switching, the RRC procedure delay is the value defined in the following table plus the BWP switching delay defined in TS 38.133 [14], clause 8.6.3.

Figure 12.1-1: Illustration of RRC procedure delay

Table 12.1-1: UE performance requirements for RRC procedures for UEs

| Procedure title: | Network -> UE | UE -> Network | Value [ms] | Notes |
| --- | --- | --- | --- | --- |
| RRC Connection Control Procedures |  |  |  |  |
| RRC reconfiguration | RRCReconfiguration | RRCReconfigurationComplete | 10 |  |
| RRC reconfiguration (scell addition/release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration (LTE/NR SCG establishment/ modification/ release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration (Intra-NR mobility with LTE/NR SCG establishment/ modification/ release) | RRCReconfiguration | RRCReconfigurationComplete | 16 |  |
| RRC reconfiguration | DLDedicatedMessageSegment | RRCReconfigurationComplete | 16+( Nseg -1)*10 | Nseg is number of RRC segments |