3GPP TSG-RAN WG2 Meeting #129bis                              	  R2-2502393

Wuhan, China, Apr. 7th – 11th, 2025

Agenda Item:	8.12.3

Source: 	OPPO

Title:  	Clarification on the uplink grant used for the UE initiated beam report

Document for:	Discussion and Decision

# Introduction

According to the RAN1 discussion on the UE initiated beam report, RAN1 agreed to use PUSCH to carry the UCI of the UE initiated beam report.

RAN1#118bis agreements: [1] Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, regarding the triggering procedure in Step-2 of Mode-A - Reuse CSI request field in DCI format 0_1/0_2 to trigger the transmission of the UEI beam report - If a CSI trigger state associated with UEI beam report configuration(s) is indicated by the CSI request field in DCI format 0_1/0_2, the UE transmits the corresponding UEI beam report(s) in the second PUSCH scheduled by the DCI format 0_1/0_2 - FFS: DCI format 0_3 - FFS: Whether a CSI trigger state should be dedicated to UE-initiated/event-driven beam reporting, i.e., not associated with legacy AP-CSI report configuration. Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH does NOT carry the beam report, for the second UL channel in Mode-B, Option-2 is supported: - Option-2: Type-1 CG PUSCH can NOT be transmitted Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least option3 is supported: - Option-1: The same Type-1 CG PUSCH can carry UL-SCH, any other UCI, and the beam report. - Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI. - Option-3: The Type-1 CG PUSCH is a type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can carry any other UCI. FFS: whether Type-1 CG PUSCH can be transmitted if the pre-configured Type-1 CG PUSCH does NOT carry the beam report.

In this contribution, we discuss the MAC specification impacts regarding the PUSCH carrying the UCI of the UE initiated beam report.

# Discussion

## UCI for PUSCH without data

RAN1#118bis agreements: Agreement On beam report transmission procedure for UE-initiated/event-driven beam reporting, for the case the pre-configured Type-1 CG PUSCH carry the beam report, for the second UL channel in Mode-B, at least option3 is supported: - Option-1: The same Type-1 CG PUSCH can carry UL-SCH, any other UCI, and the beam report. - Option-2: The Type-1 CG PUSCH is a dedicated type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can NOT carry any other UCI. - Option-3: The Type-1 CG PUSCH is a type-1 CG PUSCH for carrying the beam report - Note: This PUSCH can NOT carry UL-SCH. This PUSCH can carry any other UCI. FFS: whether Type-1 CG PUSCH can be transmitted if the pre-configured Type-1 CG PUSCH does NOT carry the beam report.

According to the RAN1 agreement quoted above, the pre-configured CG PUSCH for the UE initiated beam report in Mode-B is only used to carry UCI, and UL-SCH (i.e. MAC PDU) cannot be included. This is to improve the transmission reliability of the UCI as transmitting UCI-only in PUSCH can provide more RE(s) and UL power for the UCI transmission.

Observation 1: According to the RAN1 agreement, the pre-configured CG PUSCH for the UE initiated beam report in Mode-B cannot carry MAC PDU.

##### TS 38.321: ##### 5.4.3.1.3 Allocation of resources … The MAC entity shall: 1> if the MAC entity is configured with enhancedSkipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or if the MAC entity is configured with enhancedSkipUplinkTxConfigured with value true and the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no UCI to be multiplexed on this PUSCH transmission as specified in TS 38.213 [6]; and 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. 1> else if the MAC entity is configured with skipUplinkTxDynamic with value true and the grant indicated to the HARQ entity was addressed to a C-RNTI, or the grant indicated to the HARQ entity is a configured uplink grant: 2> if there is no aperiodic CSI requested for this PUSCH transmission as specified in TS 38.212 [9]; and 2> if the MAC PDU includes zero MAC SDUs; and 2> if the MAC PDU includes only the periodic BSR and there is no data available for any LCG, or the MAC PDU includes only the padding BSR: 3> not generate a MAC PDU for the HARQ entity. …

According to the current MAC specification as quoted above, when there is an UCI to be multiplexed in the CG PUSCH, the MAC entity needs to generate a MAC PDU.

Observation 2: According to TS 38.321, for configured uplink grant, if a UCI is to be multiplexed in PUSCH, the MAC entity needs to always generate a MAC PDU. This conflicts with the RAN1 agreement.

To align with the RAN1 agreement, we think that the MAC UE behaviours of the pre-configured CG PUSCH for the UE initiated beam report in Mode-B can be clarified to “not generate a MAC PDU”.

Proposal: RAN2 is kindly requested to confirm the understandings that the pre-configured CG PUSCH for the UE initiated beam report in Mode-B does not carry MAC PDU.

# Conclusion

According to the analysis provided above, we have the following observation and proposal:

Observation 1: According to the RAN1 agreement, the pre-configured CG PUSCH for the UE initiated beam report in Mode-B cannot carry MAC PDU.

Observation 2: According to TS 38.321, for configured uplink grant, if a UCI is to be multiplexed in PUSCH, the MAC entity needs to always generate a MAC PDU. This conflicts with the RAN1 agreement.

Proposal: RAN2 is kindly requested to confirm the understandings that the pre-configured CG PUSCH for the UE initiated beam report in Mode-B does not carry MAC PDU.

# Reference

[1] RAN1#118bis meeting minutes