3GPP TSG-RAN WG2 Meeting #129bis                              	  R2-2502395

Wuhan, China, April. 7th – 11th, 2025

Agenda Item:	8.11.3

Source: 	OPPO

Title:  	Discussion on the SBFD related measurement and BFR

Document for:	Discussion and Decision

# Introduction

According to the discussion in RAN1 and RAN4, the UE requirements on the L3 measurement and the BFR procedure were discussed. In this contribution, we provide our understandings on the RAN2 related aspects of the SBFD operation for the L3 measurement and BFR procedure.

# Discussion

## L3 measurement configuration

RAN4#112bis: Issue 1-2-2: Requirements for CSI-RS based measurement - Agreements (from online session) - RAN4 to discuss whether and how L1 CSI-RS measurement requirements would be impacted by the following SBFD operations. - CSI-RS colliding with dynamically scheduled UL transmission - CSI-RS puncturing in frequency domain in SBFD symbols - CSI-RS distributed over SBFD and non-SBFD symbols - Align the understanding whether above SBFD operations are applicable for CSI-RS for RLM, BFR and L1-RSRP, or they are only applicable for CSI-RS for CSI. - RAN4 to discuss whether and how L3 CSI-RS measurement requirements would be impacted by SBFD operations, and further RAN1/2 conclusions can be considered.

RAN4#113: Issue 1-2-3: Requirements for CSI-RS based L3 measurement - Agreements (from online session) - Companies to check the existing requirement, and discuss whether they can be applied to SBFD operation and discuss any additional aspect needs to be addressed.

RAN4#114: Issue 2-1-2: Requirements for CSI-RS based L3 measurement Agreements (from Wed AH session) - For DU case: - UE measurement requirements would be same as in legacy with the condition that all CSI-RS resources configured for measurement are available in DL subband of SBFD symbols and/or on non-SBFD DL symbols. - FFS on DUD case

According to the RAN4 agreements on the L3 measurement as quoted above, the UE measurement requirement for the CSI-RS based L3 measurement is not changed, and the CSI-RS resource configured for the L3 measurement can be in DL subband of SBFD symbol and non-SBFD DL symbol. There is no restriction of using separate requirements or separate configurations for the L3 measurement configuration on the SBFD symbol.

Observation 1: The RAN4 agreements achieved so far do not differentiate the UE measurement requirements/configurations for DL subband of SBFD symbols and non-SBFD DL symbols.

RAN1#117: Agreement For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options. - Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. - Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

According to the above RAN1 agreements, RAN1 is discussing whether/how to support separate CSI reports on SBFD and non-SBFD, as the CSI report needs to reflects the dynamic channel status so that the gNB can adjust its radio configuration (e.g. MCS) in time. As the interference conditions for SBFD symbol and non-SBFD symbol are quite different, it is reasonable to use separate configuration for CSI report associated with periodic/semi-persistent CSI-RS for SBFD symbol and non-SBFD symbol. However, the L3 measurement is used for the mobility purpose (i.e. handover), in which the L3 measurement results need to reflect the overall cell channel conditions during a relatively longer time period so that the ping-pong handover can be avoid. The benefit of using a separate L3 measurement report configuration only for the SBFD symbol is unclear. On the other hand, it is also not clear whether the use case of using only the SBFD symbol for the target cell via the SBFD-only measurement is common.

Observation 2: RAN1 discussion is focusing on the separate CSI reports on SBFD symbol and non-SBFD symbol, not on the separate L3 RRM measurement on SBFD symbol and non-SBFD symbol.

As such, we think that RAN2 can confirm the above RAN4 agreement that the CSI-RS resource configured for a L3 measurement can be in both DL subband of SBFD symbol and non-SBFD DL symbols. RAN2 can wait for more inputs from RAN1/RAN4 regarding whether a separate L3 measurement for SBFD symbol is needed.

Proposal 1: RAN2 is kindly requested to confirm that the CSI-RS resource configured for a L3 measurement can be in both DL subband of SBFD symbols and non-SBFD DL symbols.

Proposal 2: Wait for RAN1/RAN4 inputs on whether a separate L3 measurement report for SBFD symbol is configured.

## Measurement gap

RAN1#118bis: Working Assumption For SSB symbols configured with SBFD subbands, - Option 1: The SSB symbols configured with SBFD subbands are SBFD symbols. Only DL receptions within DL usable PRBs are allowed for SBFD aware UEs. - Note: The SSB block is assumed to be within DL subband

According to the RAN1 agreement as quoted above, SSB can be configured in the SBFD symbol, with the restriction that the SSB is within DL suband. The case that SSB outside of the DL suband of SBFD symbol has not been agreed yet. Form our understanding, the UE may/may not require a measurement gap when the SSB is outside of the DL subband of the SBFD symbol, depending on the UE capability and the measurement requirements defined in RAN4. RAN2 can wait for more inputs from RAN4 on whether a separate gap is needed for the SBFD symbol.

Observation 3: The SSB in SBFD symbol is within DL subband.

Proposal 3: Wait for RAN1/RAN4 inputs on whether a separate measurement gap for SBFD symbol is needed.

## BFR

TS 38.321: The following UE variables are used for the beam failure detection procedure: - BFI_COUNTER (per Serving Cell or per BFD-RS set of Serving Cell configured with two BFD-RS sets): counter for beam failure instance indication which is initially set to 0. … 2> if beam failure instance indication has been received from lower layers: 3> start or restart the beamFailureDetectionTimer; 3> increment BFI_COUNTER by 1;

According to the BFR procedure defined in TS38.321, the UE uses the beam failure instance indication to calculate the BFI_COUNTER, and the beam failure instance indication is provided by PHY based on the PHY measurement on the BFD reference signals. The BFI_COUNTER is per Serving Cell or per BFD-RS set of Serving Cell configured with two BFD-RS sets, i.e. does not differentiate between different beams or different reference signal configuration in different symbols. On the other hand, as the signal transmission from/to a specific beam should also be allowed in both the SBFD symbol and the non-SBFD symbol, the failure of one beam should be the combination of the channel conditions of both the SBFD symbol and the non-SBFD symbol. Thus, there is no need to differentiate the BFD/BFD procedure for the SBFD symbol and the non-SBFD symbol.

Observation 4: The current BFD procedure in MAC is per Serving Cell or per BFD-RS set of Serving Cell configured with two BFD-RS sets, i.e. BFI_COUNTER does not reset when the serving beam changes.

Proposal 4: The BFR procedure does not differentiate between the SBFD symbol and the non-SBFD symbol.

# Conclusion

According to the analysis provided above, we have the following observations and proposals:

Observation 1: The RAN4 agreements achieved so far do not differentiate the UE measurement requirements/configurations for DL subband of SBFD symbols and non-SBFD DL symbols.

Observation 2: RAN1 discussion is focusing on the separate CSI reports on SBFD symbol and non-SBFD symbol, not on the separate L3 RRM measurement on SBFD symbol and non-SBFD symbol.

Observation 3: The SSB in SBFD symbol is within DL subband.

Observation 4: The current BFD procedure in MAC is per Serving Cell or per BFD-RS set of Serving Cell configured with two BFD-RS sets, i.e. BFI_COUNTER does not reset when the serving beam changes.

Proposal 1: RAN2 is kindly requested to confirm that the CSI-RS resource configured for a L3 measurement can be in both DL subband of SBFD symbols and non-SBFD DL symbols.

Proposal 2: Wait for RAN1/RAN4 inputs on whether a separate L3 measurement report for SBFD symbol is configured.

Proposal 3: Wait for RAN1/RAN4 inputs on whether a separate measurement gap for SBFD symbol is needed.

Proposal 4: The BFR procedure does not differentiate between the SBFD symbol and the non-SBFD symbol.

# Reference

[1] R2-2500554 Other aspects of SBFD Nokia discussion Rel-19 NR_duplex_evo-Core

[2] R2-2500607 Discussion on L3 and L1 measurement in SBFD ZTE Corporation discussion Rel-19 NR_duplex_evo-Core

[3] R2-2501170 Other Aspects of SBFD Samsung discussion Rel-19 NR_duplex_evo-Core

[4] R2-2500885 CSI-RS measurements for RLM/BFD/CBD in SBFD Ericsson discussion Rel-19 NR_duplex_evo-Core