3GPP TSG-RAN WG2 Meeting #129bis	R2-2502401

Wuhan, China, Apr. 7th – Apr. 11th, 2025

Agenda Item:	8.7.5

Source: 	Canon Research Centre France

Title:	Discussion on RLC AM Enhancements

Document for:	Discussion

# Introduction

During the previous RAN2 meetings in Orlando [1] and Athens [3], RAN2 discussed RLC AM and reached the following agreements:

At RAN2#128 (Orlando) Agreements on unnecessary RLC retransmissions avoidance - There is no clear understanding on how the indication would look like or what problem it would solve that cannot be solved by the local timer - Unless critical issue is identified, no Tx to Rx indication will be introduced Special handling to avoid PDCP control PDU discard is not needed. A new RLC timer at the Rx is introduced to determine obsolete RLC SDUs. The timer starts when the gap is detected at RLC layer. The abandoned RLC SDUs determined by a new RLC timer are positively acknowledged in the STATUS report. - Timely RLC retransmissions – autonomous versus polling Timely RLC retransmission solution covers both autonomous retransmission and polling enhancement and NW can configure either or both of them. At RAN2#129 (Athens) Autonomous retransmissions and polling enhancements - Autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. FFS if remaining time is determined based on discardTimer at PDCP or new timer at RLC - Only a single autonomous retransmission will be triggered per RLC SDU. - There is no dynamic activation/deactivation of the autonomous retransmission mechanism. - We have separate thresholds for autonomous reTx and for polling Unnecessary retransmissions avoidance - When the TX RLC entity receives a discard indication of the SDU from PDCP, the TX RLC entity considers the SDU as an outdated SDU. The TX RLC entity does not perform any transmission and retransmission of such SDU/SDU segment. - A new RLC timer at the TX is not introduced to determine outdated RLC SDUs. - The new RLC timer at the RX is per RLC entity - The duration of the new RLC timer is not lower than that of t-reassembly - Proposals 4 and 6 from R2-2500380 and P3 and 4 from R2-2500401 will be discussed together with RLC CR review

In this contribution, we discuss further details of unnecessary retransmission avoidance (2.1) and of autonomous retransmission and enhanced polling mechanisms to ensure timely RLC retransmission (2.2).

# Discussion

## Avoid unnecessary retransmission

Based on the agreements taken at RAN2#129 meeting (Athens), a TX RLC entity does not perform any transmission and retransmission of RLC SDUs considered as outdated, a new RLC timer at the TX side is not introduced to determine outdated RLC SDUs, and the TX RLC entity considers a RLC SDU as outdated when it receives a discard indication of the RLC SDU from PDCP.

Besides, the RLC AM ARQ mechanism is based on perpetual retransmission of an RLC PDU, which may belong to a PDU Set. A radio link failure condition (RLF) occurs if the retransmission is stopped (e.g., if the number of retransmissions exceeds the maxRetxThreshold [2]). This may lead to the current session being reset, at which point the UE may reconnect to another gNB. Therefore, stopping the retransmission of a RLC PDU due to a remaining time condition shall not trigger an RLF.

Proposal 1: Stopping the retransmission of a RLC PDU due to a remaining time condition shall not trigger an RLF.

## Timely RLC retransmission

Remaining time to trigger autonomous retransmission and/or polling

Based on the agreements taken at RAN2#129 meeting (Athens), autonomous retransmission and/or polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. Considering separate thresholds for autonomous retransmission and for polling, one question is whether the remaining time is determined based on discardTimer at PDCP layer or on a new timer at RLC layer.

It seems appropriate to link discard operation at PDCP layer and trigger for retransmission at RLC layer. Indeed, at TX RLC side, retransmissions that occur after the PSDB shall be avoided because the retransmitted RLC PDUs will likely be discarded (e.g., even if successfully received by the network) and yet they occupy valuable transmission time that could otherwise be allocated to another PDU Set (e.g., transmission of the next PDU Set). Such unnecessary retransmissions can delay the subsequent transmission of data.

Observation 1: Any retransmission that occurs after the PSDB shall be avoided.

Proposal 2: Remaining time of an RLC SDU is determined based on discardTimer at PDCP.

RLC polling enhancements

From the previous meeting, it is agreed that polling should be triggered when the remaining time of an RLC SDU falls below a specified threshold. This is a new triggering condition of RLC polling at the TX side but it does not speed up the whole RLC polling mechanism. Indeed, sending a RLC status report at the RX side is still pending to t-StatusProhibit timer, and a RLC SDU (or RLC SDU segment) will be reported as lost (i.e. NACK_SN, to trigger a retransmission at TX side) only at the expiry of t-Reassembly timer.

Proposal 3: Enhancements are also introduced at the RX RLC side to speed up RLC polling.

As a first possible enhancement, it is proposed to allow configuring a RX RLC entity to not take into account the t-Reassembly timer to report a RLC SDU (or RLC SDU segment) as lost. With this enhancement, a SDU that is not yet received (i.e. with t-Reassembly timer still running) would be reported as lost with the legacy NACK_SN. To avoid generating such “false NACK”, another option would be to add a new indication in the RLC status report to differentiate a NACK_SN corresponding to a lost SDU to a NACK_SN corresponding to a SDU with unknown status (i.e. a SDU which has not yet been received). For instance, the reserved bit along with the NACK_SN field may be used to indicate the unknown status. Then, the TX RLC entity can consider retransmitting the SDUs with an unknown status in addition to the SDUs that are reported as lost.

As a second possible enhancement, a polling bit sent based on remaining time information is enhanced with dedicated signalling that allows the receiving entity to handle the reassembly and prohibit timers differently. When detecting this enhanced polling, the RX RLC entity may skip the t-StatusProhibit timer to send a RLC status report. It may also report “false NACK” or unknown status by skipping the t-Reassembly timer as described above.

Proposal 4: A RX RLC entity can be configured to not take into account t-Reassembly timer to report a RLC SDU (or RLC SDU segment) as lost or having an unknown status.

Proposal 5: Enhance the polling bit signalling to indicate a low remaining time condition.

Combined autonomous retransmission and polling enhancements

Each retransmission solution (autonomous retransmission, polling enhancement) presents unique advantages. With two retransmission solutions allowed, UEs with different capabilities will coexist.

Observation 2: UEs can be sorted into 5 categories:

- UEs with no retransmission enhancement capacity

- UEs with polling enhancement capability

- UEs with autonomous retransmission capability

- UEs with both polling and autonomous capabilities, but configurable with only one capability active at a time

- UEs with both polling and autonomous capabilities able to be simultaneously configured with both capabilities active.

Proposal 6: RAN2 to specify UE configuration by the network according to the 5 UE categories.

For UEs of the fourth and fifth categories it may be useful to let the network control the activation and deactivation of each of the retransmission solutions. As agreed at the previous meeting for the autonomous retransmission, the activation/deactivation of a retransmission solution is not dynamic, it corresponds to the configuration of the UE through e.g. a RRC Reconfiguration message.

Besides, the network should also configure the thresholds triggering autonomous retransmission and enhanced polling. With this flexibility, different options are possible to take benefit of the two retransmission solutions with UEs of the fifth category,

For instance, it is possible to set the triggering thresholds so that the enhanced polling is triggered before the autonomous transmission. Then, an autonomous retransmission of an RLC SDU would occur if the remaining time falls below the threshold for autonomous retransmission, and if no RLC status report was received from the enhanced polling or if the RLC status report was received but with an unknown status for this RLC SDU.

Proposal 7: The network configures the thresholds triggering autonomous retransmission and enhanced polling without constraints.

# Conclusion

In this contribution, we conclude the following observations and proposals:

Proposal 1: Stopping the retransmission of a RLC PDU due to remaining time condition shall not trigger an RLF.

Observation 1: Any retransmission that occurs after the PSDB shall be avoided.

Proposal 2: Remaining time of an RLC SDU is determined based on discardTimer at PDCP.

Proposal 3: Enhancements are also introduced at the RX RLC side to speed up RLC polling.

Proposal 4: A RX RLC entity can be configured to not take into account t-Reassembly timer to report a RLC SDU (or RLC SDU segment) as lost or having an unknown status.

Proposal 5: Enhance the polling bit signalling to indicate a low remaining time condition.

Observation 2: UEs can be sorted into 5 categories:

- UEs with no retransmission enhancement capacity

- UEs with polling enhancement capability

- UEs with autonomous retransmission capability

- UEs with both polling and autonomous capabilities, but configurable with only one capability active at a time

- UEs with both polling and autonomous capabilities able to be simultaneously configured with both capabilities active.

Proposal 6: RAN2 to specify UE configuration by the network according to the 5 UE categories.

Proposal 7: The network configures the thresholds triggering autonomous retransmission and enhanced polling without constraints.

# References

- R2-2410915, Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei)

- TS 38.322, Radio Link Control (RLC) protocol specification

- R2-2501335, Report from session on R18 MBS, R18 QoE and R19 XR, Session chair (Huawei)