3GPP TSG-RAN WG2#129bis   									        R2-2502403

Wuhan, China, Apr. 7th – 11st, 2025

Agenda Item:	8.18

Source: 	NEC

Title:	        BWP restriction for SDT

Document for:	 Discussion and Decision

- Introduction

In RAN2 #129 [1], BWP restriction for MT-SDT was discussed, and two solutions could be considered for improving network capacity. However, there is no consensus on whether to send two solutions to RAN1 or downselect between two solutions first in RAN2. We will discuss two solutions and provide our views in this contribution.

- Discussion

Two solutions discussed in the last meeting are listed below:

- Solution 1: Currently the unicast downlink data scheduling during the SDT procedure is restricted to the size of CORESET#0 on the initial DL BWP. In order to enable data transmission/reception over a larger bandwidth, a separate UL and DL BWP for SDT can be configured. This DL BWP for SDT overlaps/includes with initial DL BWP.

- Solution 2: A MAC CE to signal channel quality measurement (yet to be decided e.g. SS-RSRQ, CQI) can be defined. This MAC CE may be sent (e.g. periodically/one shot etc.) in the UL grant during the SDT procedure This would enable the network to use a more appropriate MCS for SDT data transfer (depending on the channel quality measurement). In addition to feasibility, RAN2 would like to know the channel quality metric available in the UE during the SDT procedure which could be helpful to network select an appropriate MCS.

Regarding solution 1, separate BWP with a larger bandwidth than the size of COREST #0 can indeed improve network capacity. However, for (e)Redcap UE, only the maximum 20MHz bandwidth in FR1 may be used, i.e. when (e)RedCap UE performs SDT procedure, separate BWP with larger 20MHz bandwidth is not available. Moreover, some specification impacts also need to be considered, such as configuring separate BWP, UE capability indication, and BWP switch when the UE starts to use separate BWP or receive paging/SSB/SIB.

Besides the specification impacts above, if BWP with larger bandwidth is configured, maybe this leads to a different DCI 1_0 size since the frequency domain resource size is calculated based on CORESET#0 size or separate DL BWP size, which can have some impacts on RAN1.

Regarding solution 2, channel quality measurement can be sent to the UE periodically/one shot by MAC CE. Some specification impacts also need to be considered. Besides some specification impacts mentioned in [2], other impacts also need to be considered. Firstly, UE capability indication needs to be informed to the network. Based on this indication, the network may know the UE can support channel quality reporting. Secondly, the network also needs to indicate the UE to report channel quality. Considering the maximum duration of the SDT procedure is 4s, we think channel quality reporting during subsequent transmission procedure is also required, such as periodic reporting. Then the reporting periodicity also needs to be configured. Of course, whether channel quality reporting is beneficial for SDT procedure also needs to be confirmed by RAN1.

Based on the analysis of two solutions above, we can see two solutions have their pros and cons. Considering solution 2 may be used for all scenarios, including (e)RedCap UE, we slightly prefer to introduce channel quality reporting by MAC CE. However, this needs to be confirmed by RAN 1 whether channel quality reporting is feasible and beneficial from RAN1 perspective. Of course, if solution 1 is the majority’s selection, we think it is okay.

Proposal 1: Select the solution of introducing channel quality reporting (such as CQI) by MAC CE to improve network capacity.

Proposal 2: Send an LS to ask RAN1 whether introducing channel quality reporting for the SDT procedure is feasible and beneficial.

- Conclusion

This contribution provides our analysis and views for BWP restriction, the following proposals are listed below:

Proposal 1: Select the solution of introducing channel quality reporting (such as CQI) by MAC CE to improve network capacity.

Proposal 2: Send an LS to ask RAN1 whether introducing channel quality reporting for the SDT procedure is feasible and beneficial.

- References

[1] Draft_RAN2_129_Meeting_Report_v1.

[2] R2-2500776 Addressing Bandwidth Restrictions Ericsson.