3GPP TSG-RAN WG2 Meeting #129bis	R2-2502408

Wuhan, China, 07 – 11 April 2025

Agenda item:	8.10.2.1

Source:	Nokia

Title:	MRO for LTM and near failure handling for CHO with candidate SCGs

WID/SID:	NR_ENDC_SON_MDT_Ph4-Core – Rel-19

Document for:	Discussion and Decision

# 1	Introduction

This paper discusses details regarding the optimization of early TA acquisition, possible TA related SHR enhancements and open issues relating to near failure handling for CHO with candidate SCGs.

# 2	MRO for LTM

The following agreements were reached on the topic of MRO for LTM before this meeting:

- RAN2#125bis

- RAN2 considers SHR, RA report and RLF for MCG LTM SON.

- RAN2#127

RAN2 include the specific access type in the RLF report, i.e. whether it is RA-based or RA-less cell switch. FFS details, e.g. if explicit or implicitly signalled.

RA-based access will not be a new triggering condition for the SHR report.

HO interruption time will not be a new triggering condition for the SHR report.

Unless RAN3 defines a NW-based solution: The UE logs and reports whether and how the UE got the TA value used for a failed LTM switch (gNB indicated or UE determined).

Include an explicit indicator in SHR whether the successful LTM execution was RACH-less or RACH-based. Can sort out the details during stage-3 implementation.

## 2.1	Early TA acquisition optimization

The main rational behind and strength of LTM is the RACH-less cell change, which is only possible with knowing the timing advance (TA) when cell switch is triggered. One critical part of the LTM is therefore the phase called early TA acquisition, which the UE may be instructed to perform before receiving the cell switch command. If the ETA is successful, the interruption time during the cell switch will be significantly reduced as RA can be skipped. If the UE received the cell switch command that does not include a valid TA previously acquired for the target cell, the UE can still access the cell via regular RA procedure, but the benefits of LTM compared to the legacy L3 handover types are lost.

In case of the fully defined network-triggered early TA acquisition, the two trigger criteria for early TA acquisition and cell switch command need to be properly synced for a successful RACH-less LTM. Firstly, the TA once acquired may become invalid because of changed UE position or changed propagation conditions with respect to the candidate cell and the network will need to trigger early TA acquisition again. Secondly, the early TA acquisition procedure should not be performed too often as it is associated with some interruption time.

Observation 1: Timely and correct early TA acquisition is a crucial part of LTM to guarantee the benefits of LTM, namely a RACH-less cell switch for minimum handover interruption time.

For example, the network should avoid unnecessary early TA acquisition (e.g. obtain the TA for cells that are then not used for cell switch) and should be able to identify instances where the TA was acquired but became invalid by the time cell switch command was sent.

Proposal 1: Log additional information regarding TAs that were unnecessarily acquisitioned.

In other cases, the TA may be obtained and included in the cell switch command but upon UE’s attempt to use it, access to the target cell fails. The UE then has to switch RACH based access via LTM recovery/re-establishment procedure when new TA value is provided by the network in RAR message. For such cases, reporting of the information about the difference between the TA reported in the Cell switch command for which the RACH-less LTM failed, and TA received consequently in RAR for recovery/re-establishment may help to optimize the wrong cell switch timing.

Proposal 2: RAN2 to discuss adding the following information into RLF report: TA difference between TA provided by network in Cell Switch Command for which the RACH-less LTM failed, and TA received in RAR for recovery/re-establishment.

As an alternative to the network triggered TA, the UE may be instructed to obtain the TA itself based on measurements. The two methods may even be configured in parallel. However, in such cases, if the network includes a TA in the Cell switch command the UE must use it. It may happen that the TA obtained based on UE measurements is very close to TA received consequently in RAR for recovery/re-establishment in case of RACH less LTM failure for TA reported in the Cell switch command. Consequently, the UE, based on some similar observation of other LTM cell switch procedures may prove the trend of better precision of UE measurement-based TA. Then it may be desirable that the UE indicates to the network that the use of the UE measurement-based TA is preferred.

Proposal 3: RAN2 to discuss signalling that allows the UE  to indicate that the use of the UE measurement-based TA is preferred.

## 2.2	Enhancement of SHR for LTM

Successful HO report (SHR) was introduced in Rel-17 to capture near failure events that happen during a successful HO. SHR is triggered based on running timers (T310/T312/T304) exceeding a preconfigured threshold. In some cases, additional SHR triggering conditions have been defined. For example, in case of DAPS HO, SHR is also generated if the UE experiences an RLF on the source link during the DAPS HO as this is not the desired behaviour.

One optimization aspect with respect to LTM is to identify the unnecessary configurations. As a reminder, in LTM, the UE may receive multiple configurations but will only execute one of them once it received the cell switch command. Moreover, the UE does not release the configuration and the prepared candidate PCells after LTM cell switch execution or LTM recovery execution. This enables fast triggering of cell changes and reduces the signalling overhead that is associated with the preparation of the LTM configurations.

There is currently however no way for the network to detect any unnecessary configurations and implicitly blocked resources. Information that would help the network to identify these cases could be added to the SHR.

Proposal 4: RAN2 to enhance SHR with new information and triggering criteria that allow the network to identify unnecessary LTM configurations.

No new LTM specific SHR triggers have been agreed in RAN2 and some like RACH based access and handover interruption time have even been excluded. However, we believe that, in case of RACH-less access the ACK for the first UL transmission taking longer than expected would constitute an undesired behaviour (near failure scenario). The LTM handover will still be successful (e.g. by means of fallback to RACH-based access or since the number of RLC retransmissions remain below the maximum admissible number) but the interruption time would be significantly increased.

Observation 2: The ACK for first UL transmission taking longer than expected during an LTM HO represents an undesired behaviour.

Proposal 5: RAN2 to define a threshold related to the number of RLC retransmissions of the first UL transmission in RLC AM for RACH-less LTM as a new SHR trigger.

# 3		Near failure handling for CHO with candidate SCGs

The following agreements were reached on the topic of MRO for CHO with candidate SCG with regard to information to be logged in case of near failure in SHR:

RAN2#125bis :RAN2 to study failure and near failure scenarios for CHO with candidate SCGs.

RAN2#128 - Measurement results of PCells and PSCells and the time information (as agreed for RLF) are included in SHR and SCGFailureInformation also. We will check what the spec impact of this is, e.g. something in the spec today may already make the UE log this.

At the start of the release both failure and near failure scenarios were agreed to be studied for CHO with candidate SCG. However, no near failure scenarios were discussed or defined in RAN3 until now and RAN2 also did not clearly decide on when and how near failure related information is to be logged.

In the spirit of aligning the report content with RLF report and legacy SHR, some agreements were made in RAN2 on possible content. Starting from the above agreements the following implementation of SHR was included in the running RRC CR (outcome of [Post129][601][SONMDT] Running NR RRC CR (Ericsson)]. It can be easily observed that new information elements were added as part of the ChoWithCandidateSCGInfo IE in addition to the initially agreed information to be logged:

#### 5.7.10.6	Actions for the successful handover report determination

The UE shall for the PCell:

[...]

2>	store the successful handover information in VarSuccessHO-Report and determine the content in VarSuccessHO-Report as follows:

[...]

3>	if the procedure is triggered due to successful completion of reconfiguration with sync and if the UE was configured with condExecutionCond and condExecutionCondPScell, for each ChoWithCandidateSCGInfo in choWithCandidateSCGInfoList:

4>	set firstFulfilledConfig to cho or cpc, whichever was fulfilled first in time;

4>	if all triggering conditions of both condExecutionCond and condExecutionCondPSCell are fulfilled:

5>	set timeBetweenFulfillment to the elapsed time between the fulfillments of the last triggering events of the two execution conditions;

4>	else if all triggering conditions of one of the condExecutionCond or condExecutionCondPSCell are fulfilled:

5>	set timeBetweenLastFulfillmentAndEvent to the elapsed time between the point in time of fulfilling the last triggering event of the fulfilled execution condition and handover execution;

4>	include the global cell identity and tracking area code, if available, and otherwise the physical cell identity and carrier frequency of the candidate PCell and candidate PSCell;

4>	include the available measurement quantities (SS/PBCH block or CSI-RS) in the MeasResults;

But in the case of CHO with candidate SCGs execution, both handovers on the PCell and PSCell are executed, which means near failure can occur on both links. While we do agree that the primary link is more important in terms of connectivity retention, we believe that to truly optimize this feature, near failures on the secondary link need to also be logged.

Proposal 6: RAN2 to agree to also log near failure information for the secondary link in case of CHO with candidate SCG.

So far RAN2 has not discussed or decided on any enhancements for SPR to capture near failure events on the secondary link. The majority of the companies seem to agree that SPR should be used in case of CHO with candidate SCG. Moreover, current specification text still allows for SPR to the logged based on legacy triggers (T310/T304/ T312) and containing legacy information which does not cover CHO with candidate SCGs. This means that SPR may be generated in the case of CHO with candidate SCG but will be useless to the network.

Proposal 7: RAN2 to agree to extend SPR with the same information agreed for SHR for CHO with candidate SCGs.

In the case that both SHR and SPR are generated for the same CHO with candidate SCG, correlation of the two reports is need as they are both pointing to the same execution and in this way, the maximum information is gathered and can be used by the network for optimization actions. Such correlation may be easiest achieved by including a coexistence flag in each report pointing to the existence of the other report.

Proposal 8: RAN2 to discuss and decide on a correlation mechanism for SHR and SPR generated for the same CHO with candidate SCG execution, for example by introducing a coexistence flag in each report.

Furthermore, in order to decide on corrective actions, the network needs to identify the cell pairs that underperformed. Currently the SON reports include information on all the configured cell pairs but no clear indication on the target pair selected by the UE in case multiple pairs fulfilled both execution conditions. As of now, SHR includes Source and Target PCell IDs and Source PSCell ID and SPR contains the (source) PCell ID and the source and target PScell ID. Therefor in the case of CHO with candidate SCG, we propose adding Target PScell ID in SHR and Target PCell ID in SPR in order to identify the cell pairs relating to the execution.

Proposal 9: RAN2 to agree on adding Target Pscell ID in SHR and Target Pcell ID in SPR to identify the cell pairs linked to an execution of CHO with candidate SCG.

Another interesting aspect in case of SHR/SPR triggering based on T310 would be knowing at which point T310 was started in relation to the two execution conditions’ fulfillment. For example, if T310 on either link was started after the fulfillment of one execution condition but before fulfilment of the second execution condition, this information could more clearly point to the root cause of the near failure, e.g. the second fulfilled condition needs to be updated.

Proposal 10: RAN2 to agree that there is a need to distinguish if the T310 started before or after fulfilment of the first execution conditions for CHO with Candidate SCG(s).

# 4	Conclusion

This document has made the following observation and proposals for TA optimization:

Observation 1: Timely and correct early TA acquisition is a crucial part of LTM to guarantee the benefits of LTM, namely a RACH-less cell switch for minimum handover interruption time.

Proposal 1: Log additional information regarding TAs that were unnecessarily acquisitioned.

Proposal 2: RAN2 to discuss adding the following information into RLF report: TA difference between TA provided by network in Cell Switch Command for which the RACH-less LTM failed, and TA received in RAR for recovery/re-establishment.

Proposal 3: RAN2 to discuss signalling that the UE indicates that the use of the UE measurement-based TA is preferred.

This document has made the following observation and proposals for SHR enhancement for LTM:

Proposal 4: RAN2 to enhance SHR with new information and triggering criteria that allow the network to identify unnecessary LTM configurations.

Observation 2: The ACK for first UL transmission taking longer than expected during an LTM HO represents an undesired behaviour.

Proposal 5: RAN2 to define a threshold related to the number of RLC retransmissions of the first UL transmission in RLC AM for RACH-less LTM as a new SHR trigger.

This document has made the following proposals for MRO for CHO with candidate SCGs:

Proposal 6: RAN2 to agree to also log near failure information for the secondary link in case of CHO with candidate SCG.

Proposal 7: RAN2 to agree to extend SPR with the same information agreed for SHR for CHO with candidate SCGs.

Proposal 8: RAN2 to discuss and decide on a correlation mechanism for SHR and SPR generated for the same CHO with candidate SCG execution, for example by introducing a coexistence flag in each report.

Proposal 9: RAN2 to agree on adding Target Pscell ID in SHR and Target Pcell ID in SPR to identify the cell pairs linked to an execution of CHO with candidate SCG.

Proposal 10: RAN2 to agree that there is a need to distinguish if the T310 started before or after fulfilment of the first execution conditions for CHO with Candidate SCG(s).