3GPP TSG-RAN WG2 Meeting #129b                        R2-2502422

Wuhan, China, Apr. 7th – 11st, 2025

Agenda Item:	8.2.2

Source: 	Transsion Holdings

Title:  	Discussion on Paging for A-IoT

Document for:	Discussion and Decision

- Introduction

In the last RAN2#129 meeting, the A-IoT Paging was discussed, and the following agreements were made[1]:

- Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

- 1 bit solution is excluded.   FFS the size.  Aim to have a reasonable size.

- RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

Agreements on paging ID

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.   FFS if there is really a need for visibility in the MAC layer

Agreements

- The A-IoT paging message can include a number of msg1 resources

- From RAN2 perspective, after initial paging message, the R2D transmission   which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later:

Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or

Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field

3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

In this contribution, we provide our views on Paging to proceed the work item.

- Discussion

  - A-IoT all device paging

It has been agreed in the WID that A-IoT Paging message contains no identifier will be supported which means all A-IoT devices that can receive the A-IoT paging message need to respond. Moreover RAN2 has also confirm that slotted-ALOHA is the baseline for A-IoT random access and the A-IoT device will randomly choose a slot based on a Q-like value to access. A appropriate Q-like value is both good for reader’s resource allocation and device’s access efficiency. To achieve this, it has been agreed that the approximate number of target devices is considered useful to be visible to the reader from CN. With the approximate number of target devices, the reader can set a appropriate Q-like value. However for all devices paging, if the number of devices is huge, the Q-like value has to be set large enough to avoid conflict. However a large Q-like value may cause resource wasting if many slots are not been selected by devices and may make device wait long if a device selects a large slot. To improve the access efficiency, the multi-round or sub-grouping paging can be considered. For all device paging, if the number of target devices is huge, the devices can be further grouped and the devices only respond to the paging belong to its own round or sub-gruop. For example, the reader can include the number of groups N and the current paging group i in the paging message, the device can choose the paging group index based on the number of group N and its device id, and when the current paging group i in the paging equals its paging group index, it responds to the paging message.

Proposal 1: For all device paging, if the number of target devices is huge, multi-round or sub-grouping paging can be considered, the devices only respond to the paging belong to its own round or group.

2.2 Simultaneous Paging Procedures

In last meeting the simultaneous paging is discussed and made the agreement that parallel service requests by the same reader is not supported and the device is expected to only perform one procedure at a time, FFS device behaviour if multiple requests are received in parallel. In the offline email discussing, the cases are divided into 2 scenarios: the requests are from the same reader or different reader, the requests are the same service request or different service request. First of all ,from our understanding as we has already agreed parallel service requests by the same reader is not supported, so a smart reader will not request a service when there is a ongoing one no matter the service is a different or a same one. So we can make a assumption that if there is another service request when the device is in a ongoing procedure, the service request is from a different reader. Regarding the device behaviour if the requests are the same service request or different service request. We think that we should follow the agreement that if the service request is the same one(thus have the same transaction ID), the device ignores the request. If the service request is the new one(thus have the new transaction ID) when there is an ongoing procedure the device follows the latest one. The device decides whether to respond the service request(paging message) only based on the transaction ID is a same or new one.

Proposal 2:  RAN2 to make a assumption that if the device receives a service request(paging message) when there is an ongoing procedure, the service request is from a different reader.

Proposal 3: The device only response the service request if the transaction ID is different from the previous/current one.

- 3 Conclusions

In this contribution we discussed issues related to A-IoT paging and made the following proposals:

Proposal 1: For all device paging, if the number of target devices is huge, the multi-round paging can be considered, the devices only respond to the paging belong to its own round.

Proposal 2:  RAN2 to make a assumption that if the device receives a service request(paging message) when there is an ongoing procedure, the service request is from a different reader.

Proposal 3: The device only response the service request if the transaction ID is different from the previous/current one.

- 4 References

- “Draft_RAN2_129_Meeting_Report_v3”