3GPP TSG-RAN WG2 Meeting #129bis 	R2-2502429

Wuhan, China, Apr, 7th – 11th, 2025

Agenda Item:	8.2.2

Source:	Spreadtrum, UNISOC

Title:	Discussion on paging procedure of A-IoT

Document for:	Discussion and Decision

Introduction

In RAN2 #129 meeting, the following agreements were achieved for AIoT paging aspects, with yellow highlights added to the FFSs [1].

Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

- 1 bit solution is excluded.   FFS the size.  Aim to have a reasonable size.

- RAN2 acknowledges that multi-reader scenario may exist but we will not specify something specific for this purpose.  We can rely on transaction ID and implementation to handle it.

Agreements on paging ID

- The “one identifier” in the paging message includes both the case of “one single device identifier” and “one group identifier”/”filtering criteria”, while the exact format of latter is supposed to be designed by SA2.

- The current assumption is that the paging identifier is transparent to the A-IoT MAC Layer and carried by upper layer.   FFS if there is really a need for visibility in the MAC layer

Agreements - The A-IoT paging message can include a number of msg1 resources - From RAN2 perspective, after initial paging message, the R2D transmission which determines the Msg1 resource(s), can be achieved by one of the below two ways, unless RAN1 concludes to use L1 signaling later: Way-1: introducing new R2D message other than the paging message, e.g., QueryRep-like; or Way-2: reusing the same paging message, using field(s) to indicate it is only to determine the Msg1 resource(s) and omitting the paging identifier (device ID/group ID) field 3. The service type of A-IoT (e.g., inventory only, inventory + command) is not included in paging message.

In this contribution, we analyses the parallel services, transaction ID and paging message contents and provide our opinions.

Discussion

2.1 Parallel/Interleaved Services from Multiple readers

In the last RAN2 meeting, the following agreements were reached. The device is expected to perform only one procedure at a time. Therefore, we need to discuss the device behaviour when multiple requests are received in parallel from different readers. This is because the same reader will not trigger a new service request if there is an ongoing procedure.

For the multi-reader scenario, if different readers belong to different BSs, to mitigate interference caused by concurrent paging transmissions from multiple readers, non-overlapping time-domain resources may be allocated by OAM. Regarding the multi-reader scenario for proximity service, the device is expected to respond to multiple readers for the same service. As agreed, the CN can assign distinct correlation IDs to different readers. Therefore these readers need to generate different transaction IDs.

Agreements

- Parallel service requests by the same reader is not supported.

- The device is expected to only perform one procedure at a time.   FFS device behaviour if multiple requests are received in parallel (if needed).

From device perspective, considering the above scenarios, what would be the device behaviour if another service request is received while there is one ongoing procedure? There are two options in email discussion of AIoT paging:

Option a: ignore all new requests and R2D messages addressed to itself but not associated with the ongoing procedure (17);

Option b: terminate the ongoing procedure and respond to the latest request (7);

For option a, the device needs to determine whether the ongoing procedure is completed. However, from the device’s perspective, this determination is challenging due to the inability to confirm whether subsequent command instructions exist for the ongoing procedure. Additionally, the device also needs to ignore other R2D messages if those are addressed to itself but not relevant to the ongoing procedure. However, how the device determines this distinction requires further discussion.

For option b, if the transaction ID in a received paging message differs from the currently stored value, the device will update its stored transaction ID to the newly received value. It means that device will execute the instructions provided in the latest paging message. So the device does not need to determine whether the ongoing procedure is finished.

Considering the additional specification efforts required to define the criteria for procedure completion, option b is slightly preferred.

Proposal 1: If a device receives a new service request while a procedure is still ongoing (i.e. not completed or failed yet), the device will terminate the ongoing procedure and respond to the latest request.

2.2 Transaction ID

- The “transaction ID” can be generated by reader based on CN corelation ID.  FFS how reader will generate “transaction ID”.  FFS the size of transaction ID

- 1 bit solution is excluded.   FFS the size.  Aim to have a reasonable size.

For transaction ID, we agreed that the transaction ID can be generated by the reader based on CN correlation ID. But how to generate the transaction ID is FFS. In our opinion, Somehow network needs to guarantee the transaction ID different if the correlation ID is different. Regarding the multi-reader scenario for the proximity service, the device is expected to respond to multiple readers for the same service. As the agreement enlightened, the CN can assign distinct correlation IDs to different readers. Therefore these readers need to generate different transaction IDs. There are several ways to generate the transaction ID. For example, 1) reader can truncate correlation ID as transaction ID, e.g., take the lowest few bits of correlation ID; 2) reader randomly select X bits from the correlation ID to generate transaction ID.

We can rely on predefined rules or reader implementation to generate the transaction ID. But if we just rely on reader implementation to generate the transaction ID without the coordination between the readers, it is difficult to ensure the transaction ID between multiple readers is unique. Because the different readers may use different rules to generate transaction ID. Therefore, it is preferable to specify a simple generation rule to generate the transaction ID. But we can wait for the RAN3/SA2 progress.

For the size of transaction ID, considering to avoid wraparound and the maximum number of neighbouring readers covering an overlapping area. 3 bits can be reasonable size.

Proposal 2: Specify a simple rule to generate the transaction ID.

Proposal 3: For the size of transaction ID, 3 bits can be reasonable size.

2.3 Paging message content

- Identifier of A-IoT device(s)

For AIoT paging identifier, there are several possible: permanent ID, temporary ID, mask/filter/group ID. From SA2 progress, the length of permanent ID is variable [1]. For temporary ID, we need to wait for SA3 progress of SA3. And for mask/filter/group ID, it also needs to wait for SA2 progress. Therefore, the paging message include a Length Field to indicate the length of paging identifier.

Proposal 4: AIoT paging message includes a Length Field indicating the length of paging identifier.

- The identify of CBRA and CFRA

The last meeting has agreed that AIoT paging message can include a number of Msg1 resources. So whether the device trigger the contention-based or contention-free random access, it depends on the number of access occasion(s). For CBRA, the number of access occasion (s) is more than one. For CFRA, R19 AIoT only support to page one device. So the number of access occasion for CFRA is only one. So no explicit indication of the access type is required.

Proposal 5: The reader implicitly indicates the random access type (CBRA and CFRA) through the number of allocated access resource(s) in AIoT paging message.

Conclusions

In this contribution, we discuss the Rel-19 paging procedure and message content. And the proposals are given as follows:

Proposal 1: If a device receives a new service request while a procedure is still ongoing (i.e. not completed or failed yet), the device will terminate the ongoing procedure and respond to the latest request.

Proposal 2: Specify a simple rule to generate the transaction ID.

Proposal 3: For the size of transaction ID, 3 bits can be reasonable size.

Proposal 4: AIoT paging message includes a Length Field indicating the length of paging identifier.

Proposal 5: The reader implicitly indicates the random access type (CBRA and CFRA) through the number of allocated access resource(s) in AIoT paging message.

References

[1] Draft RAN2 #129 meeting report v3.

[2] TR 23.700-13, SA2 Technical report of Study on architecture support of ambient power-enabled IoT.