3GPP TSG-RAN WG2 #129bis	R2-2502445

Wuhan, China, April 7th – 11th, 2025

Agenda item:	8.6.3

Source: 	Ericsson

Title:  	Discussion on event-based L1 measurements

Document for:	Discussion and Decision

# Introduction

Based on the status of the discussions in RAN2, here is our view on the remaining issues to discuss, including some aspects of the design of the MAC CE.

# Discussion

## MAC CE Design

Based on the running CR for 38.321 and the discussions in previous RAN2 meetings we think there are some questions that requires discussions. Based on these RAN2 could go ahead with an improved formatting of the L1 event-based MR MAC CE.

### Beam identifier in MAC CE

Previous agreements in RAN2 have stated that the SSBRI should be used for identifying the beam that is reported by the UE:

RAN2#128: SSBRI and CRI is used to represent the candidate beam ID in LTM MR MAC CE.

However, during our discussions it is apparent that using the SSBRI (which requires 9 bits) has a greater impact to the MAC CE design that RAN2 considered when the agreement was taken. Consider Figure 1 and Figure 2 below:

| SSBRI (9 bits) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | R | R | R | Diff RSRP (4 bits) |

Figure 1; SSBRI as beam identifier

| R | R | R | R | R | LTM candidate ID1 (3 bit) | Common for entire MAC CE |
| --- | --- | --- | --- | --- | --- | --- |
| … |  |  |  |  |  |  |
| R | R | SSB1 Index (6 bits) |  |  |  | Beam #1 |
| R | R | R | R | Diff RSRP (4 bits) |  |  |
| R | R | SSB2 Index (6 bits) |  |  |  | Beam #2 |
| R | R | R | R | Diff RSRP (4 bits) |  |  |
| R | R | SSB3 Index (6 bits) |  |  |  | Beam #4 |
| R | R | R | R | Diff RSRP (4 bits) |  |  |
| R | R | SSB4 Index (6 bits) |  |  |  | Beam #4 |
| R | R | R | R | Diff RSRP (4 bits) |  |  |

Figure 2; SSB Index as beam identifier

It can be seen that using the SSB Index is a more efficient way of signaling the beam, as it leaves room for more spare bits for explicit signaling purposes as discussed in later chapters 2.1.2. For the SSB Index to be valid it does however require the LTM Candidate ID to be included per MAC CE (3 bits), but this is only required once per MAC CE.

Using SSBRI or CRI as beam identifier for the MAC CE consumes 9 bits per beam

Using a combination of LTM Candidate ID and SSB Index consumes 6 bits per beam, and an additional 3 bits for the MAC CE to indicate the LTM Candidate.

LTM Candidate ID in combination with SSB Index will use fewer bits in the MAC CE.

Revert the previous agreement of using SSBRI/CRI as beam identifier and use LTM Candidate ID for the measurement report and then SSB Index/CRI Index to identify each beam in the MAC CE to reduce the total number of bits carried.

### Indication for triggered beams, beams that meet the event condition, and beams that meet leaving condition and entering condition

In the latest RAN2 meeting we discussed indication of beams and how they are fulfilling entering and/or leaving conditions and took the following agreements.

RAN2#129 agreements: - Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling. - Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

This is currently reflected in the running CR as editor’s notes:

Editor’s NOTE: The detailed field(s) and corresponding description will be further updated based on the discussion progress. Editor’s NOTE: Explicit indication in MR MAC CE to inform which beam(s) meets the event condition with TTT. FFS on the detailed signalling. Editor’s NOTE: Explicit indication in MR MAC CE to inform the leaving event condition is met. FFS whether it is per beam.

Based on the agreements in RAN2 above, and the comments in the running CR we can see to alternatives to fulfill this given the assumption that RAN1 might support 16 beams in the report (still being discussed):

Alternative A – A bitmap in the beginning of the MAC CE to indicate which of the reported beams fulfill entering vs leaving conditions (16 bits per indication, total of 16-48 bits increased size of MAC CE depending on outcome of discussion if both entering and leaving conditions can be present as discussed in Proposal 2 regardless of how many beams are reported).

In the below example the following fields are present:

EBX;	Indication if beam X meets the entering condition.

LBX;	Indication if beam X meets the leaving condition.

| EB1 | EB2 | EB3 | EB4 | EB5 | EB6 | EB7 | EB8 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| EB9 | EB10 | EB11 | EB12 | EB13 | EB14 | EB15 | EB16 |
| LB1 | LB2 | LB3 | LB4 | LB5 | LB6 | LB7 | LB8 |
| LB9 | LB10 | LB11 | LB12 | LB13 | LB14 | LB15 | LB16 |
| … |  |  |  |  |  |  |  |
| SSBRI (9 bits) |  |  |  |  |  |  |  |
|  | R | R | R | Diff RSRP (4 bits) |  |  |  |

Figure 3; Alternative A, bitmap for beams

Alternative B – One bit per beam reported to indicate entering, leaving and fulfillment of execution condition (3 bits per beam, which is available in the above proposal, total of 0 bits increased MAC CE).

In the below example the following fields are present:

T;	Indication if the beam has fulfilled the triggering condition.

E;	Indication if the beam meets the entering condition.

L;	Indication if the beam meets the leaving condition.

| SSBRI (9 bits) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | T | E | L | Diff RSRP (4 bits) |

Figure 4; Alternative B, per beam indication

Alternative C – An enhancement of Alternative B, one bit per beam reported to indicate whether a beam is a triggering beam and one bit that depending on if it is a triggering beam indicates entering or leaving, or in the case of the beam is not triggering is just a reserved bit. (2 bits per beam, which is available in the proposal, total of 0 bits increased MAC CE)

In the below example the following fields are present:

T;	Indication if the beam has fulfilled the triggering condition.

E;	Indication if the beam meets the entering condition.

L;	Indication if the beam meets the leaving condition.

| SSBRI (9 bits) |  |  |  |  |
| --- | --- | --- | --- | --- |
|  | R | T | E/L | Diff RSRP (4 bits) |

Figure 5; Alternative C, per beam indication

Alternative C above seems like the most efficient encoding.

RAN2 to consider the following options to indicate the report-on-leave and event-triggered beams in the MR MAC CE:

- A bitmap to indicate whether the reported beam is an event-triggered or a non-triggered beam and whether the reported beam fulfills the entering condition(s) or the leaving condition(s) for the given LTM event.

- A single-bit per beam to indicate whether the beam is a triggering beam and an additional single-bit indication per reported beam to distinguish the event-triggered and non-triggered beams for the reported beam.

### Conclusion on the MAC CE design aspects

Based on the discussion in 2.1.1, 2.1.2 and we propose the following example of a MAC CE to be used as baseline for the running MAC CR.

Event ID; The ID of the event (LTM2/3/4/5)

S; Indicates if RSRP for serving beam is reported or not

LX; Indicates if LTM candidate X is reported or not (max 4 candidates)

NY: Indicates how many beams are reported for each LTM candidate (max 4 beams)

| Event ID |  |  | R | R | R | R | S | Common MAC CE information |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| L1 | L2 | L3 | L4 | L5 | L6 | L7 | L8 |  |
| N1 |  | N2 |  | N3 |  | N4 |  |  |
| … |  |  |  |  |  |  |  |  |
| T | E/L | SSB/CSI Index (6 bits) |  |  |  |  |  | First beam (highest RSRP) |
| R | Full RSRP (7 bits) |  |  |  |  |  |  |  |
| … |  |  |  |  |  |  |  |  |
| T | E/L | SSB/CSI Index (6 bits) |  |  |  |  |  | Each remaining beam |
| R | R | R | R | Diff RSRP (4 bits) |  |  |  |  |
| … |  |  |  |  |  |  |  |  |
|  | Serving Cell RSRP (7 bits) |  |  |  |  |  |  | Serving beam |

## RAN1 LSs

Two LSs has been received from RAN1 and they are discussed in this chapter.

### Filtering

During previous RAN2 meeting the following agreement was taken:

RAN2#126 agreement: RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation.

Later this issue was discussed in RAN1 with the following conclusion and reply to RAN2:

RAN1 has discussed the necessity of the specified L1 filtering for event triggered reporting based on the RAN2 agreement at RAN2#126 below: Agreements on measurement enhancements for LTM: 5. RAN2 assumes filtering of the L1 measure results is needed. It’s up to RAN1 whether the specified L1 filtering is needed or ok to leave it to UE implementation. There is no consensus in RAN1 to support specified filtering of L1-RSRP for LTM event triggered reporting, and hence L1 filtering is up to UE implementation.

Since there is no consensus in RAN1 to specify a filter, we think it is up to RAN2 to specify the filter since the RAN2 have earlier assumed it is necessary.

RAN2 specifies filtering of L1 measurement results in MAC.

### Activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells

An LS has been received from RAN1 about activation/deactivation of semi-persistent CSI-RS resource for LTM candidate cells (R1-2501500). This LS specifically asks RAN2 to act on the following:

MAC CE is used to activate/deactivate the semi-persistent CSI-RS resource similarly to the legacy mechanism for a serving cell which will be specified in RAN2

The existing mechanism is described for the serving cell in 3GPP TS 38.321:

The corresponding MAC CE is described in 3GPP TS 38.321 chapter 6.1.3.12

RAN1 wants RAN2 to expand the activation/deactivation procedure from the serving cell to also candidate cells.

RAN1 asks RAN2 to specify procedures to activate/deactivate semipersistent CSI-RS resource set for candidate cells.

While the existing procedures covers only serving cell ID in the existing MAC CE, it seems suitable to apply similar procedures either by reusing the same MAC CE or define a new one for Rel-19 purposes. The only difference between the Rel-18 procedure and Rel-19 procedure is the cell identifier. Since the CSI-RS resource set may contain a number of resources for several LTM candidates, it seems like a possible solution to use the LTM Candidate ID for this purpose instead of the Serving Cell Id. Conveniently enough the LTM Candidate Id is fewer bits than the Serving Cell. From this point of view, it would be possible to reuse the MAC CE as a baseline for a new MAC CE for the Rel-19 purposes for candidate cells.

All other information fields (SP CSI-RS resource set ID and TCI state ID) would be enough to point out the specific resource within the resource set, but one field would not be used for this purpose (SP CSI-IM resource set ID).

It is a possible to reuse the layout of the existing MAC CE for Rel-19 purposes for activation/deactivation of SP CSI-RS resource set.

Define a new MAC CE that activates/deactivates the SP CSI-RS resources in a candidate cell (Rel-18 MAC CE for activation/deactivation of SP CSI-RS resources in the serving cell can be baseline).

## Details of LTM2 Event

For LTM2 the measuring quantity is not defined and could be either CSI-RS or SSB.

Event LTM2: Beam of serving cell becomes worse than absolute threshold;

There is an ambiguity in which RS to use for LTM2, which could be either of SSB or CSI-RS.

For this reason we think there needs to be a definition on which RS the UE bases the measuring quantity on and eventually reports in the MR.

For LTM2, if the CSI-RS Resource set is present in LTM-CSI-ResourceConfig, the UE evaluates CSI-RS as the RS. Otherwise the UE evaluates SSB.

Furthermore, for the case where LTM2 trigger the MR, even though there is no candidate cell to report the UE could still include measurements for LTM candidates.

The event LTM2 has no relation to candidate cells.

If an MR is triggered for LTM2, the UE could still include measurements for LTM candidates.

For LTM2, the UE can still include measurements for LTM candidates.

# Conclusion

In the previous sections we made the following observations:

Observation 1	Using SSBRI or CRI as beam identifier for the MAC CE consumes 9 bits per beam

Observation 2	Using a combination of LTM Candidate ID and SSB Index consumes 6 bits per beam, and an additional 3 bits for the MAC CE to indicate the LTM Candidate.

Observation 3	LTM Candidate ID in combination with SSB Index will use fewer bits in the MAC CE.

Observation 4	Alternative C above seems like the most efficient encoding.

Observation 5	RAN1 asks RAN2 to specify procedures to activate/deactivate semipersistent CSI-RS resource set for candidate cells.

Observation 6	It is a possible to reuse the layout of the existing MAC CE for Rel-19 purposes for activation/deactivation of SP CSI-RS resource set.

Observation 7	There is an ambiguity in which RS to use for LTM2, which could be either of SSB or CSI-RS.

Observation 8	The event LTM2 has no relation to candidate cells.

Observation 9	If an MR is triggered for LTM2, the UE could still include measurements for LTM candidates.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Revert the previous agreement of using SSBRI/CRI as beam identifier and use LTM Candidate ID for the measurement report and then SSB Index/CRI Index to identify each beam in the MAC CE to reduce the total number of bits carried.

Proposal 2	RAN2 to consider the following options to indicate the report-on-leave and event-triggered beams in the MR MAC CE:

a)	A bitmap to indicate whether the reported beam is an event-triggered or a non-triggered beam and whether the reported beam fulfills the entering condition(s) or the leaving condition(s) for the given LTM event.

b)	A single-bit per beam to indicate whether the beam is a triggering beam and an additional single-bit indication per reported beam to distinguish the event-triggered and non-triggered beams for the reported beam.

Proposal 3	RAN2 specifies filtering of L1 measurement results in MAC.

Proposal 4	Define a new MAC CE that activates/deactivates the SP CSI-RS resources in a candidate cell (Rel-18 MAC CE for activation/deactivation of SP CSI-RS resources in the serving cell can be baseline).

Proposal 5	For LTM2, if the CSI-RS Resource set is present in LTM-CSI-ResourceConfig, the UE evaluates CSI-RS as the RS. Otherwise the UE evaluates SSB.

Proposal 6	For LTM2, the UE can still include measurements for LTM candidates.

# References

R2-25xxxxx- Running RRC CR for inter-CU and conditional LTM, Ericsson, RAN2#129bis, April 2025.