3GPP TSG-RAN WG2 Meeting #129bis	R2-2502448

China, Wuhan, April. 7th – 11th, 2025

Source:                Qualcomm Incorporated

Title:	LP-WUS operation in CONNECTED state

Agenda Item:	8.4.4

Document for:	Discussion and Decision

# Introduction & Background

• Initiate the discussion on the configuration, activation/deactivation.

• Initiate the discussion on the procedure of using LP-WUS with/without C-DRX.

# Discussion

## 2.1 Short DRX configuration

For option 1-1, since legacy C-DRX is configured, and active time is based on legacy on-duration timer. Similar with DCP, short DRX can be configured for option 1-1,and LP-WUS is only enabled during long DRX is started.

- For Option 1-1, short DRX can be configured, and UE will stop LP-WUS monitor when short DRX is started.

For option 1-2, the intention is to reduce the user plane latency of DL data, and it is assumed the LP-WUS periodicity is shorter or equivalent with short DRX. Similarly the purpose of short DRX is also to reduce DL data latency, so there is no need to configure two features for the same purpose. If introduce short DRX for option 1-2, RAN2 has to discuss the relationship between short DRX and LP-WUS.

- For Option 1-2, short DRX is not configured.

## 2.2 Switching between LP-WUS and legacy PDCCH monitoring

As discussed in IDLE/Inactive state, it is possible the LP-WUS coverage is smaller than MR coverage. In this case, UE should be able to fall back to PDCCH monitoring on MR using the configured C-DRX configuration.

Considering different LP-WUS options and different legacy PDCCH there are two fallback cases to be considered.

Case 1: UE is using LP-WUS option 1-1 and fallback to MR PDCCH monitoring with/without DCP configured

In this case, the UE is using LP-WUS option 1-1, when the UE detects moving outside of LP-WUS coverage, UE can automatically switch to legacy C-DRX PDCCH monitoring. There is no essential need for the network to know the exact moment that the UE autonomously fallback to PDCCH monitoring using C-DRX configuration. In option 1-1, the network anyway transmits both LP-WUS and PDCCH in the corresponding C-DRX active time. The UE monitors either LP-WUS or PDCCH in the corresponding C-DRX active time, depending on fallback status. Therefore, the network does not need to know the particular moment that the UE switches back to legacy C-DRX operation. However, it would be beneficial for network to know whether the UE is able to detect the LP-WUS or not, then gNB can determine whether to send LP-WUS any more.

Observation 1: When UE is using LP-WUS option 1-1 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB can stop sending LP-WUS indication and enable DCP functionality if configured.

Case 2: UE is using LP-WUS option 1-2 and fallback to MR PDCCH monitoring with/without DCP configured

For Option 1-2, if UE autonomous fallback to legacy C-DRX is enabled for Option 1-2, network does not know whether/when the UE leaves LP-WUS mode. There can be misalignment between the UE and the gNB when the UE monitors legacy PDCCH and gNB will continue to send LP-WUS indication and PDCCH in the new active time window, and then PDCCH could be missed. So in this case, UE should report to the gNB before switching to legacy PDCCH monitoring. And gNB should send command to the UE to switch to legacy PDCCH monitoring.

Observation 2: When UE is using LP-WUS option 1-2 and fallback to MR PDCCH monitoring with/without DCP configured, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on PDCCH monitoring to avoid potential PDCCH missing.

Case 3: UE is using legacy PDCCH monitoring with/without DCP configured and enters LP-WUS mode

In this case, the UE is monitoring PDCCH as legacy, and UE detects entering LP-WUS coverage. Since for LP-WUS, gNB should firstly indicate to UE in LP-WUS and then send PDCCH on active time window. Then UE should report to gNB. And gNB should send switching command to the UE to enable LP-WUS. Then there is no mis-alignment between the gNB and the UE, and there is no potential PDCCH monitoring missed.

Observation 3: When UE is using legacy PDCCH monitoring and enters LP-WUS, it is beneficial that UE reports to the gNB so that gNB and the UE can be aligned on LP-WUS enable to avoid potential PDCCH missing.

Considering the above cases, UE should indicate to gNB when enters or leave LP-WUS mode. The condition for entering or leaving LP-WUS mode could be based on MR measurement, it can be FFS for other conditions, and it can be FFS on whether the entry/exist condition need to be configured to the UE or can be left UE implementation.

- For both of Option 1-1 and Option 1-2, UE needs to indicate to the gNB when the UE enters or leave LP-WUS mode.

- For option 1-1, the UE switches from LP-WUS mode to legacy PDCCH monitoring, RAN2 discusses whether gNB needs to send a switching command to UE.

- For option 1-2, gNB needs to send a switching command to UE in order for UE switching from LP-WUS mode to legacy PDCCH monitoring with/without DCP.

## 2.3 LP-WUS activation/deactivation

RAN1 decided to study activation/deactivation of LP-WUS monitoring as highlighted below in the RAN1#116 agreement.

Agreement For RRC CONNECTED mode, from RAN1 perspective, - PDCCH monitoring triggered by LP-WUS is enabled/disabled by gNB RRC signaling - FFS whether to support UE assistance. - LP-WUS monitoring by UE is known to gNB. - FFS whether implicit/explicit indication from UE is necessary - In case LP-WUS monitoring is enabled, following options are further studied - Option 1: No additional indication/condition are introduced for activation/deactivation of LP-WUS monitoring - Option 2: Activation/deactivation of LP-WUS monitoring by gNB L1/L2 signaling with or without UE assistance. - Option 3: Activation/deactivation of LP-WUS monitoring based on condition(s), such as timer. - Option 4: Activation/deactivation of LP-WUS monitoring based on implicit indication/condition, e.g. UL transmission.

It could be possible that a network configures one or multiple LP-WUS configurations, and one of the LP-WUS configuration is activated by the MAC-CE. The MAC-CE can also deactivate LP-WUS monitoring (i.e., it activates none of the LP-WUS configurations), in which case the UE should switch to legacy C-DRX configuration.

In each of the LP-WUS configurations, various RRC parameters can be provided, then the MAC-CE activation can control the overhead/efficiency, associated beam, etc, in dynamic manner.

- gNB can configure multiple LP-WUS configurations to the UE and activate one of the configurations to the UE via MAC CE.

## 2.4 LP-WUS in CA

For CA, from Rel-16, dual DRX groups has been supported. A UE can be configured with different values of drx-onDurationTimer and drx-InactivityTimer for one DRX group and for the other DRX group. Long DRX cycle is common for both DRX groups.

Dual DRX groups

Rel-16 DCP was enabled only on PCell or PSCell and indicates to start drx-onDurationTimer of all the DRX groups. In other words, it was not possible to trigger PDCCH monitoring of different DRX groups independently. This is not beneficial for UE power saving. For Rel-19 LP-WUS in connected mode, per DRX group PDCCH monitoring triggering should be considered. There are two options to enable this:

- Opt.1: LP-WUS is configured to be monitored per DRX group

- Opt.2: LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring

Option 1 is enabled if LP-WUS is monitored in two different serving cells in different DRX groups. The LP-WUS in each serving cell is nothing to do with PDCCH monitoring for the cell(s) in the other DRX group. Option 2 is enabled without LP-WUS monitoring on multiple serving cells. However, it has to reserve a couple of bits in the LP-WUS to indicate DRX group. Further study is necessary on which option to support.

- For a UE configured with CA without dual-DRX groups, support LP-WUS monitoring on a serving cell that triggers PDCCH monitoring on all the serving cells

- For a UE configured with CA with dual-DRX groups, support LP-WUS monitoring on a serving cell per DRX group

    - Opt.1: LP-WUS is configured to be monitored per DRX group

      - For dual DRX groups, LP-WUS monitoring is configured per DRX group

      - LP-WUS monitored on a cell within a DRX group triggers PDCCH monitoring on the cell of the DRX group

    - Opt.2: LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring

## 2.5 LP-WUS for other PDCCH monitoring

In the WID, it is only mentioned that LP-WUS triggers MR PDCCH monitoring, however it does not mention whether it is only for unicast PDCCH monitoring or not.

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1)

Currently, DRX in CONNECTED state can also be used for MBS broadcast and multicast, sidelink, paging for SI notification. It should be clarified whether LP-WUS can also be used for these features.

- RAN2 discusses whether LP-WUS can be applied for MBS broadcast and/or MBS multicast DRX, sidelink DRX.

For paging, since the paging is cell specific and can be applicable for all RRC state. For CONNECTED state UE, it is possible to monitor paging DRX cycle for SI notification if the UE is in RRC_CONNECTED with an active BWP with common search space configured by searchSpaceSIB1 and pagingSearchSpace. There are two options for paging DRX handling in CONNECTED state.

Option 1: UE in CONNECTED state monitors paging as legacy. That LP-WUS is not used for paging DRX in CONNECTED state.

Option 2: IDLE/Inactive LP-WUS can be used for the UE in CONNECTED state for paging monitoring. In this option, UE needs to monitor LP-WUS as in IDLE/Inactive state.

Both options can work with less specification clarification. RAN is proposed to discuss which option should be used.

- RAN2 discusses the following two options for paging monitoring in CONNECTED state

Option 1: UE in CONNECTED state monitors paging as legacy, i.e. no LP-WUS is used.

Option 2: UE in CONNECTED state monitors paging using IDLE/Inactive LP-WUS monitoring.

# Conclusion

Short DRX

- For Option 1-1, short DRX can be configured, and UE will stop LP-WUS monitor when short DRX is started.

- For Option 1-2, short DRX is not configured.

Switching between LP-WUS and legacy PDCCH monitoring

- For both of Option 1-1 and Option 1-2, UE needs to indicate to the gNB when the UE enters or leave LP-WUS mode.

- For option 1-1, the UE switches from LP-WUS mode to legacy PDCCH monitoring, RAN2 discusses whether gNB needs to send a switching command to UE.

- For option 1-2, gNB needs to send a switching command to UE in order for UE switching from LP-WUS mode to legacy PDCCH monitoring with/without DCP.

LP-WUS activation/deactivation

- gNB can configure multiple LP-WUS configurations to the UE and activate one of the configurations to the UE via MAC CE.

LP-WUS in CA

- For a UE configured with CA without dual-DRX groups, support LP-WUS monitoring on a serving cell that triggers PDCCH monitoring on all the serving cells

- For a UE configured with CA with dual-DRX groups, support LP-WUS monitoring on a serving cell per DRX group

    - Opt.1: LP-WUS is configured to be monitored per DRX group

      - For dual DRX groups, LP-WUS monitoring is configured per DRX group

      - LP-WUS monitored on a cell within a DRX group triggers PDCCH monitoring on the cell of the DRX group

    - Opt.2: LP-WUS carries an indication for which DRX group(s) it triggers PDCCH monitoring

LP-WUS for other PDCCH monitoring

- RAN2 discusses whether LP-WUS can be applied for MBS broadcast and/or MBS multicast DRX, sidelink DRX.

- RAN2 discusses the following two options for paging monitoring in CONNECTED state

Option 1: UE in CONNECTED state monitors paging as legacy, i.e. no LP-WUS is used.

Option 2: UE in CONNECTED state monitors paging using IDLE/Inactive LP-WUS monitoring.

# Reference

[1]    RP-234056: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)