3GPP TSG-RAN WG2 Meeting #129bis	R2-2502454

China, Wuhan, April 7th – 11th, 2025

Source:	Qualcomm Incorporated

Title:	Discovery and Relay (re)selection for multi-hop U2N relay

Agenda Item:	8.13.2

Document for:	Discussion and Decision

# Introduction & Background

In the WID [1], multi-hop U2N relay discovery and Relay (re)selection needs to discuss. This contribution discusses the related aspects.

- Specify mechanisms to support up to two additional hops relays on top of Rel-17 U2N relay. The work starts with one additional hop relay (i.e., remote UE -> first relay UE -> last relay UE -> gNB) until RAN#107 and further check will be made in RAN#107 if it can be easily extended to two additional hops relays (i.e., remote UE -> first relay UE -> second relay UE -> last relay UE -> gNB). A necessary criterion for the specified mechanisms is easy extensibility to support two additional hop relays for the work done until RAN#107 and to be forward compatible for future extensions for additional relays.

  - Relay discovery and (re)selection [RAN2]

  - Signalling support for relay UEs and remote UE authorization if SA2 concludes it is needed [RAN3]

  - Impact on SRAP and QoS handling for multi-hop [RAN2]

  - Control plane procedures [RAN2, RAN3]

# Discussion

## 2.3 Discovery

For Model B discovery response transmission, one issue is whether last Relay UE needs to check the PC5 link quality of the receiving link satisfies a PC5 threshold. There are two options to be considered.

Option 1: The last Relay UE checks the PC5 link quality of the receiving link and determines to send discovery response message. If the receiving link is above the PC5 threshold, then the last Relay UE sends discovery response message.

Option 2: The last Relay UE does not check the PC5 link quality of the receiving link as single-hop Relay UE. It is up to the intermediate Relay checks the PC5 link quality of the receiving link for discovery response message and determines whether to forward the discovery response message.

Option 2 will waste the discovery resource if the link between the intermediate Relay UE and the last Relay UE does not satisfy the PC5 threshold. It is better that the last Relay UE check PC5 link quality of the receiving link.

- For Model B discovery response transmission, the last Relay UE should only send Model B discovery response message if the PC5 link quality of the receiving link satisfies a PC5 threshold.

For the AS container included in discovery message, it is assumed that the NCGI and RRC Container defined in Rel-17 are also supported in the multi-hop Relay discovery message.

- It is assumed that the NCGI and RRC Container defined in Rel-17 are also supported in the multi-hop Relay discovery message.

## 2.4 Relay (Re)selection

In existing U2N or U2U relay, the Remote UE (re)selects candidate Relay UE considering the hop link quality (SD-RSRP) between the Remote UE and the Relay UE, similar mechanism can be reused for Remote UE to (re)select the candidate Intermediate Relay UEs. And the Remote UE considers an Intermediate Relay UE for which SD-RSRP exceeds a threshold as candidate Intermediate Relay UE.

There are some FFS issues based on the last RAN2 progress as following about intermediate Relay relay reselection.

L2 multi-hop U2N Remote UE in idle/inactive triggers relay reselection upon PC5-RRC signaling from the first relay UE indicating (FFS on the detailed PC5-RRC signaling design):

-	cell reselection, handover, Uu RLF, or Uu RRC connection establishment/resume failure between the last relay UE and network.

-	release of RRC connection between an intermediate relay UE and the network.

-	PC5 RLF or PC5-S connection release between the multi-hop U2N Relay UE(s).

-	FFS: link quality degradation of the upstream links resulting in a notification.

-	FFS if any of these conditions can occur without notifying the remote UE (e.g., allowing recovery by the intermediate relay UE).

-	FFS on the signalling contents of the PC5-RRC indication from the first relay UE.

-	FFS applicability to L3.

Agreements:

The intermediate relay UE can adopt the same AS triggering condition of relay (re)selection of the R17 remote UE. FFS upper layer conditions and whether the spec impact will be modelled as a separate relay UE behaviour or through functioning as a remote UE.

Upon intermediate relay UE performing relay (re)selection, it can notify its child node.  If the cause of (re)selection already causes a notification, there is no double-trigger of the notification.

FFS if (re)selection as such is captured as a cause of notification (intention to maintain no double-trigger)

FFS if the notification is required in all cases

FFS the content in the notification

FFS the child node behaviour after receiving this notification

FFS other condition/timing that the intermediate relay UE notifies its child node

The highlight FFS issues are related to whether and when the intermediate relay UE should send notification to the Remote UE/Child UE. When the intermediate Relay UE performs relay (re)selection and selects a new relay path which can satisfy the relay requirement for the Remote UE from both AS an upper layer perspective, the intermediate Relay UE does not need to notify the Remote UE. The benefit of not notifying the Remote UE is that Remote UE does not need to perform relay reselection from starting.

AS relay requirement is candidate PC5 link quality is above a threshold, and upper layer requirement is up to SA2 because Model A and Model B discovery procedure are different from Remote UE.

Then from AS layer perspective, if the intermediate Relay reselect a candidate relay path, the intermediate Relay UE does not need to notify the Remote UE.

From upper layer, whether the intermediate Relay UE needs to notify the Remote UE from upper layer perspective is up to SA2.

- From AS layer perspective, if the intermediate Relay reselects a candidate relay path for the Remote UE considering AS relay reselection criterion, the intermediate Relay UE does not need to notify the Remote UE.

- From upper layer perspective, whether the intermediate Relay UE needs to notify the Remote UE is up to SA2.

- Send LS to ask SA2 whether the intermediate Relay UE needs to notify the Remote UE is the intermediate Relay UE reselects a relay path for the Remote UE.

When the intermediate Relay UE cannot reselect a relay path for the Remote UE, the intermediate Relay UE should notify the Remote UE.

Based on SA2 discussion, L3 based multi-hop U2N relay is structured as multi-hop L3 U2U relay and U2N relay. From AS layer perspective, Remote UE is behind the Relay UE without connecting with network. When the last relay UE performs cell reselection, handover, re-establishment after radio link failure, the last relay UE’s RRC connection can be kept. In case the last relay UE connection is released, the last relay UE will trigger PC5 connection towards the child relay UE to the Remote UE, then no notification is needed.

In case RLF or connection release between the relay UEs happens, the relay UE will trigger PC5 connection release towards the Remote UE, then no notification is needed.

- For L3 based multi-hop U2N relay, there is no need to send notification from the relay UE to the Remote UE in AS layer.

- Conclusion

Discovery

- For Model B discovery response transmission, the last Relay UE should only send Model B discovery response message if the PC5 link quality of the receiving link satisfies a PC5 threshold.

- It is assumed that the NCGI and RRC Container defined in Rel-17 are also supported in the multi-hop Relay discovery message.

Relay UE (re)selection

- From AS layer perspective, if the intermediate Relay reselects a candidate relay path for the Remote UE considering AS relay reselection criterion, the intermediate Relay UE does not need to notify the Remote UE.

- From upper layer perspective, whether the intermediate Relay UE needs to notify the Remote UE is up to SA2.

- Send LS to ask SA2 whether the intermediate Relay UE needs to notify the Remote UE is the intermediate Relay UE reselects a relay path for the Remote UE.

- For L3 based multi-hop U2N relay, there is no need to send notification from the relay UE to the Remote UE in AS layer.

# Reference

[1]. RP-241609    New WID on NR sidelink multi-hop relay    LG Electronics, InterDigital, FirstNet