3GPP TSG-RAN WG2 Meeting #129-bis				   		R2-2502465

Wuhan, China, 7 – 11 April, 2025

Title: 	Discussion on NW-sided data collection for training

Source: 	Huawei, HiSilicon

Agenda item:	8.1.3

Document for:	Discussion/Decision

# Introduction

In the previous meetings, the following agreements were achieved for NW-sided data collection :

Agreements from RAN2#127bis

1	Periodic logging is supported for training data collection procedure in R19

2	Event-triggered data logging will be supported.  At least radio condition based event triggered logging will be supported.  FFS the details of radio condition based event.  FFS if other events are supported.

3	Periodic reporting of logged data is not supported.

4	On-demand reporting of the logged measurements will be specified

5	UEInformationRequest/UEInformationResponse is used for on-demand reporting of AI/ML training data collection.   FFS of details of the message

6	The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

7	Low priority SRB will be used.    FFS new SRB or use of SRB4

8	For data collection for both NW-sided/UE sided BM model training, at least L1-RSRPs and/or beam-IDs needs to be collected by UE.  FFS if other data needs to be collected based on RAN1 progress.

Agreements from RAN2#128

1. Focus on the following three radio condition event based logging

§ L3 serving cell measurement based (e.g. X1/X2 similar to A1/A2)

§ Beam based events (e.g. beam becomes top-1 beam and number of measurements is less than configured value)

§ L1 beam level measurement

2. Measurements on aperiodic CSI resources are not reported for NW sided data collection.

3. Data collection is controlled by the network.   The UE will not autonomously stop when low power state is detected.

4. The UE reports to the network when the power state is low.  We will not specify how the UE determines low power state.   The network should de-configure the data collection (this can be captured in stage 2).

5. The UE reports to the network when buffer is or may become full.  FFS when it reports (before and/or after).

6. The UE can report the reason for triggering of indication for the status (e.g. low power state, low memory).  FFS how this is signalled and if the reporting can be part of availability indication.

7. Duration is not supported

Agreements from RAN2#129

1. Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not.  L1 measurement event triggered will not be supported.    FFS what to log

2. Low power bit indication is supported

3. Data availability indication is supported.  FFS when this would be triggered

4. As baseline, the UEInformationResponse contains one or more logged measurement entries in chronological order (i.e. starting from the oldest measurement entries stored in the UE memory), and an availability indication if there are further data available for transmission. Same principles as for logged MDT.

5. UE retains logged data during handover (HO).  FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed

6. UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

7. FFS how to handle idle/inactive and RLF cases and whether we have a unified.

This paper analyzes the network-sided data collection topic further, with a focus on the FFS points mentioned above.

# Discussion

## Event-triggered logging details

In the last meeting, the following agreements were made for event triggered logging:

1. Support the use of L3 measurement event triggered (i.e. L3 serving cell measurements becoming worse/better than a threshold for TTT) to determine whether the UE performs logging or not. L1 measurement event triggered will not be supported. FFS what to log

As mentioned above, L3 serving cell measurements becoming worse/better than a threshold for TTT can be used as the triggering event which corresponds to the definitions of existing events A1 and A2. Thus, RAN2 should confirm that the definitions of events A1 and A2 should be reused for event triggered logging.

Proposal 1: For the event-triggered logging, RAN2 confirms that the definitions and ASN.1 structure of events A1 and A2 will be reused.

Regarding the FFS on what information to log, L1-RSRP should be logged as agreed in RAN1. Some companies propose time stamps should also be logged, but we do not see any benefits for the model training if the timestamp is included in logged data. Furthermore, at least in case of periodic reporting, there is nothing that prevents the gNB to add time stamps to the measurements by itself.

Proposal 2: RAN2 should clarify what the benefits is of including a timestamp together with the measurement sample.

## Data Availability Indication

Regarding the on-demand reporting, it has been agreed that the UE should send the data available indication to the NW when the collected data is available on the UE side. There is an FFS about the trigger condition for the data available indication as follows in [2].

- The UE can indicates the availability of logged data to the network to assist network to trigger UEInformationRequest.  FFS trigger/definition of availability indication.   and FFS how data availability indication is sent to the network.

Furthermore, in RAN2#128, some further agreements were made:

- The UE reports to the network when buffer is or may become full. FFS when it reports (before and/or after).

In the last meeting, the data available indication is supported in RAN2:

3. Data availability indication is supported. FFS when this would be triggered

As RAN2 agreed previously, the main trigger for sending data availability indication by the UE is based on UE’s memory status, i.e. the indication is sent “when buffer is or may become full“.

When it comes to the FFS on the exact time when the report from the UE should be sent, there are several factors to consider. Firstly, it has to be sufficiently early to allow the UE to continue data collection without an interruption, if needed. If the UE reported data availability only once its buffer becomes full, the UE would have to stop collecting additional data until the network collects the buffered one.

Observation 1: The data collected by the UE needs to be indicated to the network before UE’s buffer becomes full and early enough for the network to be able to collect it and allow the UE continue data collection without an interruption.

Furthermore, data volume is the most important metric for data logging and the NW can use it to control how much data the UE should collect, e.g. depending on how much data the NW needs to receive from the UE to efficiently train a model. Additionally, the network may have a preference to collect data more frequently, not waiting for the UE’s buffer to become full, e.g. to avoid having to collect big amount of data at the same time.

Observation 2: The network may have a preference to collect data more frequently, not waiting for the UE’s buffer to become full, e.g. to avoid having to collect big amount of data at the same time.

For the reasons mentioned above, we think it is reasonable to introduce a configurable data volume threshold used by the UE to send a data available indication to the NW when the volume of collected data exceeds this threshold. With such threshold, the NW would have flexibility to set it either to a value lower than or equal to UE’s maximum buffer size.

Proposal 3: For the on-demand data retrieval mechanism, the UE should send the data availability indication to the NW when the volume of the collected data reaches a threshold configured by the network.

We suppose the network can provide the UE with multiple sets of configurations, each corresponding to different scenarios, as discussed in section 2.4. These configurations or scenarios may have different data collection requirements. The UE performs data collection based on the provided configuration information and logs the collected data locally. When the stored data volume meets the specified requirements, the UE reports a data availability indication. Since the speed of data collection may vary for each configuration or scenario, the UE needs to include the corresponding data collection configuration identifier in the data availability indication. This allows the network to obtain more complete information about the data availability and accurately request the data collected by the UE for the specific configuration or scenario.

Proposal 4: The UE should include the corresponding configuration identifier in the data available indication to avoid NW requests unavailable data or support NW to request a certain data collected by UE.

There is an FFS about data available indication in running CR as follow:

Editor's Note: FFS the need to introduce further procedures, e.g. prohibit timers, indication that battery state is not low any longer, etc.

Once the data is reported to the network, it should be discarded from the UE’s memory, so that more data can be logged and the memory can be used for another purpose..

Proposal 5: The UE should discard the logged data after it is reported to the NW.

## Low power state indication

In the previous meeting, RAN2 agreed that the UE reports to the network when its power state is low and then the NW should de-configure the data collection [1].

In the last meeting, RAN2 agreed low power state indication is supported:

2. Low power bit indication is supported

Similarly, as data available indication, we believe low power state indication can be sent by the UE using UEAssistanceInformation message.

Proposal 6: The UEAssistanceInformation message is used for sending low power state indication.

After receiving the low power indication from the UE, there may be two ways for the NW to de-configure the data collection job. Firstly, the network may remove the measurement and logging configuration on the UE side through the RRC reconfiguration message.

Proposal 7: When the NW receives the low power state indication from a UE, the NW may use RRC Reconfiguration to either release the data collection or suspend it without removing it from UE’s RRC configuration.

There is an FFS about low power indication in running CR as follow:

Editor's Note: FFS the need to introduce further procedures, e.g. prohibit timers, indication that battery state is not low any longer, etc.

The NW may indicate the UE to suspend the running data collection procedure and recover it when the UE exits low power state. In case the UE recovers from the low power state quickly, then the NW would not have to configure the data collection configuration again when the UE exits the low power state, so a process can be continued.

Proposal 8: When the UE exits low power state, it indicates it to the network and the network may either configure a new data collection configuration or resume a suspended configuration.

Another issue may happen if the NW tries to configure data collection configuration to the UE which is already in a low power state. In this case, the UE will have a preference not to collect the data. Thus, the UE that is already in a low power state should send low power state indication to the NW when it receives data collection configuration.

Proposal 9:  The UE that is in a low power state should send a low power state indication to the NW when it receives the data collection configuration (e.g. using the RRCReconfigurationComplete or UAI message).

Finally, since the low power state determination is up to UE implementation, the network needs to have a possibility to control the amount of related signalling, Therefore, it is necessary to specify a prohibit timer for the low power state indications.

Proposal 10: Prohibit timer for low power state indications is introduced.

## Configuration details

Another FFS is whether multiple configurations can be provided to the UE as follows and whether they can be dynamically activated and deactivated [3].

- As the baseline approach, the UE receives the measurement configuration for AI/ML-enabled features/FGs for data collection and logging of measurements.  The network can explicitly configure the UE whether the corresponding data collection and logging (if supported) should be immediately started.  FFS if multiple configurations can be provided to the UE.  FFS if dynamic activation/deactivation is support.

A single UE may support data collection for multiple AI use cases, e.g. CSI prediction, beam management, and positioning. Thus, we think it should be possible for the network to configure one data collection configuration per use case. We currently do not see a need to have multiple configurations for the same use case simultaneously at a single UE.

Proposal 11: The NW can configure one data collection configuration per use case, i.e. multiple configurations for the same use case are not supported.

Regarding whether dynamic activation/deactivation of data collection should be supported. Since we do not believe multiple configurations for the same use case are needed, then also dynamic activation/deactivation is not required. Even when multiple configurations per use case would be supported, dynamic activation/deactivation is still not needed in our view. Data collection for model training is normally a continuous process and it is unclear what the purpose would be of activating and deactivating it dynamically. If the network no longer requires more data from the UE for a certain data collection configuration, the network may simply de-configure the data collection configuration using RRC signaling. Another potential reason to have dynamic activation/deactivation would be to allow the network to rely on semi-persistent CSI resources for data collection. Related to this, RAN2 made the following agreement during RAN2#128:

2. Measurements on aperiodic CSI resources are not reported for NW sided data collection.

Similarly, we do not see a reason to rely on semi-persistent resources and believe periodic CSI resources fit data collection purpose best. Based on the above consideration, we propose the following:

Proposal 12: Only periodic CSI resources are used for NW sided data collection, i.e. neither aperiodic nor semi-persistent CSI resources are supported for this purpose.

There is an FFS in the running CR about whether periodicity of the logging should be configurable as follows:

Editor's Note: FFS whether the periodicity of the logging is configurable.

It should be noted that the UE gathers data through RS measurements, and it is most efficient to align the logging periodicity with the CSI resource periodicity. If the network requires data with a specific periodicity, it can configure the CSI resource accordingly and there is no need for configuring logging periodicity separately.

Proposal 13: There is no need to configure logging periodicity separately, i.e. the measurements are logged according to the periodicity of the CSI resource used for data collection.

## SRB for data reporting

At RAN2#127bis meeting, it was agreed that low priority SRB will be used and whether to use new SRB or existing SRB4 will be further discussed.

The feature of NR QoE measurement reporting was introduced since Rel-17. RRC messages containing the QoE reports (generated in UE application layer) may be quite large, and then it may lead to frequent RRC message transmission in Uu interface. In LTE Rel-15, a separate SRB4 was introduced, and the network is able to configure SRB4 to a different logical channel group with the lowest priority among SRBs and DRBs. For NR QoE, SRB4 was also introduced for the same purpose.

For SRB4, currently it is mainly for QoE reporting. If SRB4 is also supported for NW-sided data reporting, our analysis is shown as below:

- Pros:  very few specification impacts

- Cons:  if both QoE reporting and NW-sided data reporting are enabled at the same time, there is still competition between the two types of reports, which results in more latency. Furthermore, there are some UE actions specified in relation to QoE based on whether SRB4 is configured or not, so this may cause some misinterpretations, e.g. when the NW wants to use SRB4 for AI, but the UE performs some actions related to QoE.

For new SRB approach, it can follow the principle of SRB4 where the LCH priority of the new SRB is configurable. Our analysis is shown as below:

- Pros:  only NW-sided data is transmitted via the new SRB, so it avoids competition with other reports. It also avoids potential issues with AIML/QoE feature interworking, as mentioned above.

- Cons: some specification impacts

In the future, there may be more use cases for NW-sided data reporting, and the requirements may be different from QoE reporting. In this case, we think new SRB approach may be better and it has better scalability and backwards/forwards compatibility.

Proposal 14: RAN2 to adopt a new SRB for NW-sided data reporting with a configurable priority.

## Collected data handling during handover and state transitions

It may happen that while the UE is collecting the data, a handover needs to be performed. In such case, the source gNB may not have time to fetch the collected data from the UE as delaying a handover procedure may lead to a handover failure.

Observation 3: When a handover needs to be performed, the source gNB may not have time to fetch the collected data from the UE as delaying the handover procedure may lead to a handover failure.

In the last meeting, RAN2 agreed UE retains logged data during handover:

5. UE retains logged data during handover (HO). FFS if there is scenarios where the UE needs to release the data and how does the UE know and if control from network is needed 6. UE indicates availability of logged data during handover (i.e., within the RRCReconfigurationComplete message) (if data is retained in the UE).

As we can see, RAN2 agreed UE indicates availability of logged data during handover. However, RAN2 did not discuss what the network does based on such indication. Depending on whether gNB-centric or OAM-centric data collection was performed, the collected data needs to be either forwarded to the source gNB or to the OAM. To achieve this, after receiving the data available indication from the UE, the target gNB should fetch the data and forward it to the source gNB or OAM. It seems RAN3 involvement is needed to handle this aspect.

Proposal 15: After receiving the data available indication from the UE, the target gNB should fetch the data from the UE and forward it to the source gNB or OAM.

Regarding the FFS on whether there is a scenario where the UE needs to release the data collected in the source gNB, we see two situations where this may be needed:

- The data collected in the source gNB may be seen by the network vendor as proprietary/sensitive data and sometimes it may be preferred that such data does not cross equipment of other vendors, e.g. in case it may disclose some sensitive configuration or algorithm data. In such case, it should be possible for the source gNB to command the UE to discard the data and not report it to the target gNB.

- When the full configuration is performed, the UE should release the logged data as the target gNB cannot understand the configuration from the source gNB and will not be able to fetch the data anyway.

Proposal 16: To protect the privacy of the data collected by the source gNB in some situations, source gNB should be able to indicate the UE to discard the logged data before or during the handover to prevent it from being reported and sent to the target gNB.

Proposal 17: The UE should release the logged data when it receives HO command with fullConfig indication.

To address the above scenario, the source gNB needs to be aware of whether there is data available at the UE when a HO is about to happen. Hence, it should be possible for the data avialbility indication to be sent in the measurement result sent for handover purposes, e.g., based on event A3 or A5. In this case, the indication should be sent regardless of the amount of stored data. When the source gNB receives the indication, it can indicate the UE to delete data before sending the handover command if inter-vendor handover occurs, or if there is sufficient time, the source gNB may still try to fetch the data.

Proposal 18: The UE should include the logged data availability indication in the L3 measurement report sent to the network.

The data privacy issue of source gNB also needs to be considered for RRC state transition and RLF case. However, in this case the gNB may not always have a chance to request the UE to discard the collected data as RLF happens unexpectedly. To allow for data being retained in these scenarios while respecting the potential data privacy concerns, the gNB could pre-configure a list of cells to which the UE can provide the collected data. When the UE re-establishes, sets up or resumes a connection in one of such cells, the UE can indicate the data availability to the new cell and if the connection is being established in a cell which is not on the list, the UE should discard the collected data.

Proposal 19: For the RLF case and for RRC state transition from RRC CONNECTED to RRC IDLE or RRC INACTIVE, the gNB can pre-configure a UE with a list of cells to which the UE can provide the collected data:

- When the UE re-establishes, sets up or resume a connection in one of such cells, the UE can indicate the data availability to the new cell.

- When the connection is being established in a cell which is not on the list, the UE should discard the collected data.

If the data is retained in the scenarios described above, after receiving the data available indication, the new serving gNB should fetch and forward the logged data to the last serving gNB or to OAM, similarly as in the HO case.

Proposal 20: After receiving the data availability indication, the new serving gNB should fetch and forward the logged data to the last serving gNB or to OAM, similarly as in the HO case.

# Conclusion

Based on the discussion in the Tdoc the following, we provided the following observations and proposals:

Event-triggered logging details

Proposal 1: For the event-triggered logging, RAN2 confirms that the definitions and ASN.1 structure of events A1 and A2 will be reused.

Proposal 2: RAN2 should clarify what the benefits is of including a timestamp together with the measurement sample.

Data Availability Indication

Observation 1: The data collected by the UE needs to be indicated to the network before UE’s buffer becomes full and early enough for the network to be able to collect it and allow the UE continue data collection without an interruption.

Observation 2: The network may have a preference to collect data more frequently, not waiting for the UE’s buffer to become full, e.g. to avoid having to collect big amount of data at the same time.

Proposal 3: For the on-demand data retrieval mechanism, the UE should send the data availability indication to the NW when the volume of the collected data reaches a threshold configured by the network.

Proposal 4: The UE should include the corresponding configuration identifier in the data available indication to avoid NW requests unavailable data or support NW to request a certain data collected by UE.

Proposal 5: The UE should discard the logged data after it is reported to the NW.

Low power state indication

Proposal 6: The UEAssistanceInformation message is used for sending low power state indication.

Proposal 7: When the NW receives the low power state indication from a UE, the NW may use RRC Reconfiguration to either release the data collection or suspend it without removing it from UE’s RRC configuration.

Proposal 8: When the UE exits low power state, it indicates it to the network and the network may either configure a new data collection configuration or resume a suspended configuration.

Proposal 9:  The UE that is in a low power state should send a low power state indication to the NW when it receives the data collection configuration (e.g. using the RRCReconfigurationComplete or UAI message).

Proposal 10: Prohibit timer for low power state indications is introduced.

Configuration details

Proposal 11: The NW can configure one data collection configuration per use case, i.e. multiple configurations for the same use case are not supported.

Proposal 12: Only periodic CSI resources are used for NW sided data collection, i.e. neither aperiodic nor semi-persistent CSI resources are supported for this purpose.

Proposal 13: There is no need to configure logging periodicity separately, i.e. the measurements are logged according to the periodicity of the CSI resource used for data collection.

SRB for data reporting

Proposal 14: RAN2 to adopt a new SRB for NW-sided data reporting with a configurable priority.

Collected data handling during handover and state transitions

Observation 3: When a handover needs to be performed, the source gNB may not have time to fetch the collected data from the UE as delaying the handover procedure may lead to a handover failure.

Proposal 15: After receiving the data available indication from the UE, the target gNB should fetch the data from the UE and forward it to the source gNB or OAM.

Proposal 16: To protect the privacy of the data collected by the source gNB in some situations, source gNB should be able to indicate the UE to discard the logged data before or during the handover to prevent it from being reported and sent to the target gNB.

Proposal 17: The UE should release the logged data when it receives HO command with fullConfig indication.

Proposal 18: The UE should include the logged data availability indication in the L3 measurement report sent to the network.

Proposal 19: For the RLF case and for RRC state transition from RRC CONNECTED to RRC IDLE or RRC INACTIVE, the gNB can pre-configure a UE with a list of cells to which the UE can provide the collected data:

- When the UE re-establishes, sets up or resume a connection in one of such cells, the UE can indicate the data availability to the new cell.

- When the connection is being established in a cell which is not on the list, the UE should discard the collected data.

Proposal 20: After receiving the data availability indication, the new serving gNB should fetch and forward the logged data to the last serving gNB or to OAM, similarly as in the HO case.

# References

- [1] RAN2#128 meeting minutes

- [2] RAN2#127bis meeting minutes

- [3] RAN2#127 meeting minutes