3GPP TSG-RAN WG2 #129bis	R2-2502466

# Wuhan, China, 7th – 11th April, 2025

Agenda Item:	8.2.3

Source:	Ericsson

Title:	UL multiple access for Ambient IoT

Document for:	Discussion/Decision

# 1	Introduction

A new work item for A-IoT has been recently agreed in RP#106 [1]. The following objectives are defined for RAN2 scope:

  - Specify the necessary functions and procedures for an Ambient IoT compact protocol stack and lightweight signalling procedure to enable DO-DTT and DT data transmission:

    - A-IoT Paging, including subsequent paging for the same service. Support the options that a paging message contains one identifier, and that a paging message contains no identifier. Temporary identifier is not supported, unless required by SA WGs.

Note: RAN2 aims to design a paging message format such that multiple identifiers can be contained in one paging message, for forward compatibility purposes.

    - A-IoT Random access, including re-access for failure handling. Contention-based and contention-free cases are supported. For the contention-based random access, only Solution 1 (3-step only) is included (unless RAN2 decides to use Solution 3 (unified solution) by RAN2#129).

    - A-IoT data transmission, including data (re-)transmission for failure handling. Segmentation is supported at least in D2R.

    - Only MAC layer is included

We focus on the objective related to random access in this paper.

# 2	Discussion

RAN2 has made below progress on random access related aspects in RAN2#129

- For Rel-19, only 3-step CBRA is supported for A-IoT

- We will specify both CBRA and CFRA.

- Re-use the subsequent paging message to trigger re-access.  There is no need to differentiate msg1 resource for initial access vs re-access.

- NACK based mechanism is supported for D2R messages to determine re-access for at least msg3.  FFS details including whether we need a timer or explicit message and when reader sends feedback

- RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

## 2.1	Re-access

In RAN#127bis, it was agreed to support optional explicit R2D failure/success feedback indication for at least Msg3 for re-access purpose, and  FFS for following D2R data. However, it is important to understand and agree on termninologies surrounding retransmission and re-access. Note, we TDMA as baseline for feedback. We will update he discussion once concerete behavior from RAN1 and RAN2 on FDMA basline is specified.

During the SI phase, RAN1 has agreed the below timing requirements for D2R transmission and its subsequent R2D transmission

TD2R_max:	Maximum time between the D2R transmission and the corresponding R2D transmission following it, so that the R2D transmission timing is expected to be within [TD2R_min, TD2R_max], when a R2D transmission in response to a D2R transmission is expected for A-IoT Msg2 response to A-IoT Msg1 for the A-IoT device.

TR2D_R2D_min:	Minimum time between two different consecutive R2D transmissions to the same A-IoT device.

For a device with failed transmission attempt, reader can trigger either

- Retransmission, or

- Re-access

where the trigger (R2D transmission) is within R2D transmission timing, i.e., between TD2R_min and TD2R_max.

The retransmission behavior was already agreed in SI, where reader can retransmit Msg2 or R2D message to trigger retransmission of Msg3 or subsequent D2R transmission.

For re-access, e.g., in next paging round, a feedback is needed at least for inventory use case. The reader determines to send feedback (i.e., ACK and/or NACK) message to targeted devices when one of the conditions is met:

- The on-going service (procedure) will not contain/trigger subsequent R2D transmission after the reader has received data transmission from devices (e.g., the data transmission sent by the devices contains inventory response, the reader will not trigger subsequent command transmission. The service procedure may be also referred to as inventory only)

- The reader has decided to not rely on upper layer (device NAS layer or application layer) and/or the CN entity (AIoTF or AF) to trigger retransmission or trigger a new service/procedure towards the same target devices. In this case, the reader has decided to enable AS feedback to improve the transmission reliability in AS.

Regarding feedback, we belive NACK based feedback is more suitable as it can resaonably assumed, majority of devices will positive access in the same round, which means fewer number of NACKs are needed compared to ACKs.

In case only NACK based feedback is employed, the ACK is implcit. It means, if device receives no NACK, then is equivalent to ACK. However, the behvior should be understood in accordance with timings RAN1 has agreed. The implicit ACK should be considered if

- No NACK is received until TD2R_max, or

- An indicator is recived indicating begining the start of next CBRA occasion before the end of TD2R_max.

For data transmission failure, the device may be triggered to perform re-access by the reader after a number of times of (re-)transmissions failed attempts.

The indication of next CBRA occasion by explicit or implicit means would mean an ACK in case on no explicit NACK is received by the device within the D2R timer.

An explicit ACK signalling for indicating a D2R transmission was successful is not needed.

Upon transmission of a D2R message (e.g., Msg3), the device starts the D2R timer, and the device monitors for NACK signalling until the D2R timer is expired or beginning of next CBRA occasion whichever occurs first.

The device interprets its D2R transmission (e.g., Msg3) is successful if the device has not received NACK signalling until the D2R timer is expired or the beginning of next CBRA occasion whichever occurs first.

For inventory only use case, there is no subsequent R2D transmission after Msg3, which can be used by the reader to indicate success of the Msg3 transmission. In this case, it is helpful to let the reader to apply AS feedback to the device indicating whether re-access is needed or not.

For inventory use case, the reader may determine to send NACK signalling to devices for triggering re-access to resolve data transmission failure (i.e., Msg3).

For inventory plus command use case, there is subsequent R2D transmission, e.g., command request after Msg3. The device can derive whether its Msg3 transmission is successful based on whether the subsequent R2D transmission is received or not. If a Msg3 transmission has failed, the reader can just allocate new resources to the device (e.g., via Msg4) for triggering retransmissions. The retransmission opportunties can be provided by the reader to the device in the current access occasion or access occasions in a CFRA procedure after the current CBRA access procedure. It is not needed for the reader to trigger re-access to resolve the data transmission failure in this case, since the device is already known to the reader and the CN (i.e., the device has already successfully transmit Msg3).

Based on above discussion, we propose following proposal.

For inventory plus command use case, the reader only triggers retransmissions to devices for resolving data transmission failure (i.e., Msg5 and beyond) without triggering re-access.

## 2.2 FDMA and Msg2

On issues pertaining to FDMA resource selection, in the last RAN2 meeting 129, it was agreed

RAN2 assumes that device randomly selects among FDMA occasions as the baseline.

Subsequently, the reader needs to address these devices in Msg2 which have successfully transmitted Msg1s indicating their RN16s in FDMed manner.

One solution can be based on RAR functionality where Msg2 can be addressed to multiple devices with their RN16s as their sub-headers and corresponding resource allocation information for their Msg3 and Msg4 transmissions as MAC sub-payloads. The major issue is the message size would increase as it would include all the successfully transmitted RN16s which a device may not be interested in the message.

Other solution can be based on L1 mapping where a preceding DL control information is sent before individual Msg2 transmissions. Each Msg2 is addressed to single device and the preceding DL control information provides mapping information of Msg2 resource allocation, see below figure. The individual message size is decreased compare RAR inspired option, however, there is a need of additional DL control information prior to Msg2 transmissions which provides the resource mapping.

In addition, RAN1 already agreed in the last meeting RAN1#120 that Msg2 can heave one or more RN16s subject to Msg2 size limitation.

Support following two options for Msg2 transmission in response to multiple Msg1 transmissions, which are initiated by a R2D transmission triggering random access.

- Option 1: A PRDCH for Msg2 transmission corresponds to a A-IoT Msg1 received from one device

- Option 2: A PRDCH for Msg2 transmission corresponds to multiple A-IoT Msg1 received from different devices

- FFS the maximum number of Msg1 responses that can be carried within a PRDCH for Msg2.

In our opinion, RAR functionality based Msg2 is more suitable as this behaviour already implemented in legacy solutions, and thus, for A-IoT, a simplified version for RAR message for Msg2 is very appropriate, and accordingly we capture following proposal.

The Msg2 echoes the list RN16s for the devices which have successfully accessed the FDMA occasions within a TDMA occasion limited by grant size.

For each echoed RN16, there should have some mapping between the echoed RN16 and FDMA resource. This allows devices not to parse every RN16 and look for own RN16 only subject to mapping behaviour between the FDMA resource and indicated RN16. The mapping can be either:

- Static, or

- Dynamic,

  - Bitmap based,

  - Index based.

In case of static mapping, the Msg2 contains the same number of placeholders as number of available FDMA access occasions. The FDMA occasion, which is accessed successfully, the corresponding placeholder in Msg2 indicates RN16 which is transmitted over the access FDMA occasion. Other placeholders may indicate dummy RN16 where no device is identified. The drawback is the Msg2 length is long even if the number of accessed devices is small.

In the other option, the Msg2 content can be dynamic and include only successful RN16s. Compared to static scenario, the message length is small as its size is limited by the number of devices responded to in a particular Msg2, not by total number of FDMA occasions configured. For a dynamic mapping, there can two options:

One option is bitmap based: For each FDMA occasion, there is a corresponding bit in the bit string which represents an association with FDMA occasion. If the bit is set, then corresponding payload contains successful RN16 transmitted over FDMA occasion which maps to the set bit in the string. One issue with this approach is that the devices need to know the order of bits mapped to the FDMA occasions a-priory, and if Msg1 resource is updated, then bitmap needs to be updated as well.

In the other option, the FDMA occasion index value is included in the header and the corresponding payload contains the accessed device’s RN16 transmitted over the FDMA occasion denoted by the same FDMA index value.

We believe a dynamic option based on FDMA index as an additional header field associated with a corresponding RN16 in the mag payload is most suitable. The option with bitmap can also work but the mapping order needs to be specified (which bit in a bit string maps to which FDMA resource).

Based on above discussion, we propose following observation and proposals.

Under dynamic option, the successful random ID correspond to selected Msg1 FDMA occasion can be indicated in Msg2 as

- Option 1: Bitmap fashion,

- Option 2: Include FDMA occasion index alongside.

In case there are multiple FDMA occasions available, reader signals to the devices a mapping between FDMA occasions and received random IDs in Msg2. FFS if the mapping is conveyed via L1 control info or MAC layer signalling.

RAN2 to consider dynamic option based on FDMA occasion index for Msg2 format i.e., Msg2 includes random IDs and associated FDMA occasion indices.

## 2.3 Contention-Free Access

To provide CFRA, the reader needs to allocate dedicated (collision-free) resources to the devices. We have named two methods, where in one method, the device IDs are know to reader and in another, they are kept hidden.

One of the methods is to allocate resources by mapping them to their device IDs. It means the paging message includes non-transparent device ID (IDs are not part of NAS container) and the corresponding resources. It is not yet agreed what are these device IDs based on, e.g., AS ID, original device ID (CN allocated), or temporary ID (CN allocated) and whether it will be transparent or non-transparent to reader. This discussion we leave it to SA2 first except the part with AS ID which is a focus of agenda item dealing with protocol stack. In essence, in this method, the reader needs to know the IDs and provide mapping of resources.

In another method, the device IDs are part of NAS container or kept hidden from reader in paging message. In order to allocate CFRA resource, the

- the reader needs to know the group size or number of devices from CN and this information can be retrieved by reader from CN via NGAP signaling, and based on this information, the reader provides group of resources to group of devices without any explicit mapping;

- A implicit mapping rule is defined by the reader, for instance, the devices can arrange themselves to a group of resources in increasing sort manner. For instance, device with lower ID have a right o transmit on resource with lower resource index;

- In order of implicit mapping to work, a device must be able to other device IDs in paging message so the device knows where it should position itself on resource w.r.t. other devices

Below is summary of two methods.

| CFRA Allocation |  |
| --- | --- |
| Method 1: Device IDs are non-transparent to reader | Method 2: Device IDs are transparent to reader |
| There is direct/explcit mapping between IDs and resource | There is implicit mapping between IDs and resource as IDs are not known to reader The resources are provided by reader based on group size information which reader can retrive from CN via NGAP message The devices must be able to know other devices indicatd paging message in order to implcit map to pool of rsources |

Further, we believe CFRA should be permitted for multiple devices, similar to CBRA based allocation (can be considered as baseline). The allocation behaviour can be inspired from CBRA where instead the occasions are allocated in a dedicated manner. We do not see reason on limiting CFRA to single device.

Next, we address failure of CFRA Msg1 transmission. In case, the device fails to deliver Msg1, then reader triggers retransmission using CFRA as device identity is already known. There is no need for CBRA based re-access for the same device from the same reader. The re-access is only employed if current reader unable to access the device, and thus re-access happens via different reader.

For CFRA based allocation, two allocation methods are observed, option (a) device IDs are known to reader, and option (b) device IDs are transparent to reader.

For CFRA, it is assumed that the reader knows at least the number of devices (e.g., provided by CN) to allocate contention free resources corresponding to the number of devices.

Dedicated allocation for retransmission is possible for erroneous case where

- CFRA Msg1 or subsequent message transmission fails,

The CFRA for multiple devices triggered by a single paging message from reader is supported.

For CFRA, reader indicates a mapping rule between the set of allocated resources and the corresponding devices explicitly or implicitly to devices.

# 3	Conclusion

Based on the discussion in the previous sections we propose the following:

Observation 1	For data transmission failure, the device may be triggered to perform re-access by the reader after a number of times of (re-)transmissions failed attempts.

Observation 2	The indication of next CBRA occasion by explicit or implicit means would mean an ACK in case on no explicit NACK is received by the device within the D2R timer.

Observation 3	The Msg2 echoes the list RN16s for the devices which have successfully accessed the FDMA occasions within a TDMA occasion limited by grant size.

Observation 4	Under dynamic option, the successful random ID correspond to selected Msg1 FDMA occasion can be indicated in Msg2 as

	Option 1: Bitmap fashion,

	Option 2: Include FDMA occasion index alongside.

Observation 5	For CFRA based allocation, two allocation methods are observed, option (a) device IDs are known to reader, and option (b) device IDs are transparent to reader.

Observation 6	For CFRA, it is assumed that the reader knows at least the number of devices (e.g., provided by CN) to allocate contention free resources corresponding to the number of devices.

Observation 7	Dedicated allocation for retransmission is possible for erroneous case where

	CFRA Msg1 or subsequent message transmission fails,

Based on the discussion in the previous sections we propose the following:

Proposal 1	An explicit ACK signalling for indicating a D2R transmission was successful is not needed.

Proposal 2	Upon transmission of a D2R message (e.g., Msg3), the device starts the D2R timer, and the device monitors for NACK signalling until the D2R timer is expired or beginning of next CBRA occasion whichever occurs first.

Proposal 3	The device interprets its D2R transmission (e.g., Msg3) is successful if the device has not received NACK signalling until the D2R timer is expired or the beginning of next CBRA occasion whichever occurs first.

Proposal 4	For inventory use case, the reader may determine to send NACK signalling to devices for triggering re-access to resolve data transmission failure (i.e., Msg3).

Proposal 5	For inventory plus command use case, the reader only triggers retransmissions to devices for resolving data transmission failure (i.e., Msg5 and beyond) without triggering re-access.

Proposal 6	In case there are multiple FDMA occasions available, reader signals to the devices a mapping between FDMA occasions and received random IDs in Msg2. FFS if the mapping is conveyed via L1 control info or MAC layer signalling.

Proposal 7	RAN2 to consider dynamic option based on FDMA occasion index for Msg2 format i.e., Msg2 includes random IDs and associated FDMA occasion indices.

Proposal 8	The CFRA for multiple devices triggered by a single paging message from reader is supported.

Proposal 9	For CFRA, reader indicates a mapping rule between the set of allocated resources and the corresponding devices explicitly or implicitly to devices.

# 4	References

RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, Huawei, 3GPP TSG RAN Meeting #106, Madrid, Spain, December 9-12, 2024.