3GPP TSG-RAN WG2 Meeting #129bis	R2-2502472

Wuhan, China, April 07th – 11th, 2025

Agenda item:	6.1.3.1

Source:	Huawei, HiSilicon

Title:	Discussion on the issue of UE radio paging capability loss

Document for:	Discussion and Decision

- Introduction

SA2 sent a LS to RAN2 with the following question on the issue of UE radio paging capability loss, and requested feedback from RAN2 [1].

2) Initial Registration may be performed on a pre-Rel 19 gNB (or a later gNB that does not support LP-WUS) that analyses the UE’s UE Radio Access Capabilities to create the UE Radio Capability for Paging Information and send it to the AMF, see clause 5.4.4.3 of TS23.501. It will be important that LP-WUS related aspects of UE Radio Capability for Paging Information are not lost when Initial Registration is performed on a pre-Rel-19 gNB. If the LP-WUS aspects are lost, then there can be paging problems, when for example, paging is performed by a gNB that supports LP-WUS. SA2 invites RAN2 and RAN3 to investigate this and provides solution if any issue is identified. SA2 was not sure whether this issue also impacts RedCap, eRedCap and other features that are signalled within the UE Radio Capability for Paging Information. SA2 invites RAN2 and RAN3 to investigate this.

In RAN2#129 meeting, the issue was discussed in LP-WUS WI and the following agreement was reached:

- Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.

In this contribution, we further discuss the issue.

- Discussion

  - LP-WUS and PEI capabilities in UE-RadioPagingInfo-r17

UERadioPagingInformation-v1700-IEs ::= SEQUENCE {

ue-RadioPagingInfo-r17                 OCTET STRING (CONTAINING UE-RadioPagingInfo-r17)     OPTIONAL,

inactiveStatePO-Determination-r17      ENUMERATED {supported}                               OPTIONAL,

numberOfRxRedCap-r17                   ENUMERATED {one, two}                                OPTIONAL,

halfDuplexFDD-TypeA-RedCap-r17         SEQUENCE (SIZE (1..maxBands)) OF FreqBandIndicatorNR OPTIONAL,

nonCriticalExtension                   UERadioPagingInformation-v1800-IEs                   OPTIONAL

}

It was agreed that “LP-WUS related capability” will be added in UE-RadioPagingInfo-r17 container, which already includes PEI capability. RAN2 confirmed that there is no issue for NW from Rel-17 since Rel-17 gNB can forward the UE-RadioPagingInfo-r17 to the CN via UERadioPagingInformation message. The LP-WUS capability and PEI capability for a UE is aligned among UE, gNB and CN. We further analyse the case for Rel-15 and Rel-16 gNBs.

As illustrated in Figure 1, if the UE reports UE-RadioPagingInfo-r17 to a Rel-15/16 gNB, the gNB cannot include this field in UERadioPagingInformation inter-node message as it cannot recognize it. So the CN does not have the possibility to know about UE’s LP-WUS/PEI capabilities. When there is a paging for the UE, CN does not include any LP-WUS/PEI capability even for a new gNB that supports LP-WUS/PEI. So, even if gNB configures LP-WUS/PEI in SIB, but still sends paging message to the UE directly without using LP-WUS/PEI. However, LP-WUS/PEI UE is not aware of this and will monitor LP-WUS/PEI. As there is no wake-up indication, UE does not wake up the MR and thus miss paging.

Fig.1 An example for the LP-WUS capability maintenance and mismatch issue

Proposal 1: RAN2 to confirm that UE will miss paging when camps in the cell with LP-WUS/PEI enabled, if the UE reports UE-RadioPagingInfo-r17 to a Rel-15/16 gNB that cannot forward this field to CN.

  - Other features outside UE-RadioPagingInfo-r17

In the same LS, SA2 was not sure if the issue exists in other features:

SA2 was not sure whether this issue also impacts RedCap, eRedCap and other features that are signalled within the UE Radio Capability for Paging Information. SA2 invites RAN2 and RAN3 to investigate this.

In UERadioPagingInformation message, as shown below, there are several capabilities outside UE-RadioPagingInfo-r17 for other features introduced after Rel-15, e.g., inactiveStatePO-Determination, (e)RedCap, 2 Rx XR, DL scheduling offset for NTN. RAN2 is requested to further investigate whether the same issue exists for these features.

inactiveStatePO-Determination

The same issue exists for inactiveStatePO-Determination. If the UE reports this capability to a Rel-15/16 gNB that cannot forward this field to CN, it causes the misalignment. When RRC_INACTIVE UE camps in the cell with ranPagingInIdlePO-r17 indicated, the UE will use the updated i_s to determine PO but the gNB still uses the legacy i_s. The determined PO is misaligned between UE and gNB, which will lead to missing paging message.

Proposal 2: RAN2 to confirm that RRC_INACTIVE UE will miss paging when camps in the cell with ranPagingInIdlePO-r17 indicated, if the UE reports inactiveStatePO-Determination-r17 to a Rel-15/16 gNB that cannot forward this field to CN.

(e)RedCap, 2 Rx XR, DL scheduling offset for NTN

For (e)RedCap, the (e)RedCap UEs can only camp on/access to a cell that supports (e)Redcap, so the gNBs can only be the upgraded gNBs. Thus, (e)RedCap UEs will only camp in the cells that support (e)Redcap. Thus, the issue of UE radio paging capability loss does not exist for (e)RedCap.

As for 2 Rx XR and DL scheduling offset for NTN, it is an enhancement for paging transmission. If the gNB does not know these capabilities, it uses the legacy way to page UEs. There is no negative impact on the paging reception for UEs, but it’s not the best use of paging resources from NW perspective.

Proposal 3: RAN2 to confirm that:

- For (e)RedCap, UE radio paging capability loss issue does not exist;

- For 2 Rx XR and DL scheduling offset for NTN, UE will not miss any paging but it’s not the best use of paging resources from NW perspective.

  - Solutions

To address the capability mismatch issue above for UE-RadioPagingInfo-r17 and inactiveStatePO-Determination, the following possible solutions are provided.

- Solution 1: Up to deployment to upgrade the gNBs to support corresponding ASN.1 simultaneously

One simple approach is to leave it to the network vendors to upgrade their gNBs to support corresponding ASN.1 simultaneously in a given area, e.g., across the entire TA area. In this case, mismatch issue would no longer exist. This solution does not have any impact on the specifications. However, SA2 discussions emphasized multi-vendors scenario and achieving simultaneous upgrades for multi-vendors may be challenging. If the operators confirm that the issue can be addressed by deployment, then the specified solution is not needed.

- Solution 2: UE gets information from the gNB whether corresponding capability can be maintained by the NW

If Solution 1 cannot be confirmed, another solution is to inform the UE whether gNB will/is capable of forwarding the corresponding capability, UE-RadioPagingInfo-r17 or inactiveStatePO-Determination-r17, to the CN. Only when gNB enquires the UE about these capabilities or indicates the UE that it can forward these capabilities, the UE will report these capabilities via UECapabilityInformation message and can use corresponding function in a cell. It is a usual way to avoid NBC issue for UE capabilities reporting. For example, the Rel-15/16 gNB won’t provide this information to the UE, so the UE won’t report UE-RadioPagingInfo-r17/inactiveStatePO-Determination-r17, or use the function even if it detects a cell with this function enabled. However, the upgraded gNB will indicate this new information, enabling the UE to clearly use the function.

Proposal 4: RAN2 to discuss the following solutions:

- Solution 1: Up to deployment to upgrade the gNBs to support corresponding ASN.1 simultaneously

- Solution 2: UE gets information from the gNB whether corresponding capability can be maintained by the NW

- Conclusion

In this paper, we further discuss the question 2) from SA2, and have the following proposals.

Proposal 1: RAN2 to confirm that UE will miss paging when camps in the cell with LP-WUS/PEI enabled, if the UE reports UE-RadioPagingInfo-r17 to a Rel-15/16 gNB that cannot forward this field to CN.

Proposal 2: RAN2 to confirm that RRC_INACTIVE UE will miss paging when camps in the cell with ranPagingInIdlePO-r17 indicated, if the UE reports inactiveStatePO-Determination-r17 to a Rel-15/16 gNB that cannot forward this field to CN.

Proposal 3: RAN2 to confirm that:

- For (e)RedCap, UE radio paging capability loss issue does not exist;

- For 2 Rx XR and DL scheduling offset for NTN, UE will not miss any paging but it’s not the best use of paging resources from NW perspective.

Proposal 4: RAN2 to discuss the following solutions:

- Solution 1: Up to deployment to upgrade the gNBs to support corresponding ASN.1 simultaneously

- Solution 2: UE gets information from the gNB whether corresponding capability can be maintained by the NW

- References

R2-2500050/S2-2412876, LS Reply on LP-WUS subgrouping.