3GPP TSG RAN WG2 Meeting #129bis			R2-2502486

Wuhan, China, April 7th – 11th 2025

Agenda Item: 	8.4.2

Source:		Sony

Title:			RAN2 aspects on LP-WUS/WUR in RRC Idle/Inactive mode

Document for:	Discussion & Decision

# Introduction

At RAN#103 meeting, a revised version for the WID for Low Power Wake-up Signal and Receiver was approved including the following objectives [1].

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline Specify necessary RRM requirements-

At RAN2#125bis [2], the following agreements were made related to RRC Idle/Inactive mode.

The LP-WUS related configuration for IDLE/INACTIVE state is provided via system information. FFS if dedicated configuration is needed.

Working assumption: the LP-WUS configuration in SIB at least includes the following information:

- -	LP-SS configuration

- -	LP-WUS configuration

- -	FFS on Entry/exit condition for LP-WUS monitoring

The PEI subgrouping method is taken as baseline for LP-WUS subgrouping, i.e. CN assigned and UE_ID based subgrouping. FFS the maximum number of subgroups.

At RAN2#126 [3], the following agreements were made related to RRC Idle/Inactive mode;

RAN2 will further discuss the details about LP-WUS monitoring entry/exit conditions based on RAN1’s existing working assumptions.

The LP-WUS related configuration in SIB at least include the following information for IDLE/INACTIVE:

    - LP-SS configuration

    - LP-WUS configuration

    - Entry/exit condition for LP-WUS monitoring (FFS if it always configured)

Baseline for entry condition definition: If the serving cell quality, e.g. RSRP, RSRQ from MR, is above threshold(s) (if configured), UE may start to monitor LP-WUS, if UE monitors LP-WUS, it may stop monitoring the legacy PO. FFS if any measurement from LR is needed.

Baseline for exit condition definition: If the serving cell measurement result based on LR is below a threshold (if configured), UE monitors PO as in legacy and it may stop monitoring the LP-WUS.

RAN2 understand that if UE is configured with CN-based LP-WUS subgrouping, it is up to CN to assign the LP-WUS subgroup ID to the UE.

RAN2 assume the maximum number of subgroups that can be configured for LP-WUS subgrouping is no less than 8

From RAN2 perspective, no new procedure is introduced for SI reception/updates.

At RAN2#127 [4], the following agreements were made:

Baseline: The network does not need to be aware of whether the UE is monitoring LP-WUS or not in RRC_IDLE/INACTIVE

Separate entry/exit thresholds can be configured for OFDM-based and OOK-based WUR if a cell supports both types of LRs. Signalling details are FFS.

Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds.

The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ.

The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

At RAN2#127bis [5], the following agreements were made:

If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

The LPWUS monitoring exit condition does not include MR measurements.

For CN assigned LP-WUS subgrouping, RAN2 assumes similar procedure for PEI will be used for LP-WUS subgrouping. Final design is up to SA2/CT1/RAN3 discussion.

For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping.

RAN2 inform this conclusion to SA2/CT1/RAN3.

This LS is approved in R2-2409226

At RAN2#128 [6], the following agreements were made:

Agreements on subgrouping

FFS whether/how to handle the case for UE-ID based subgrouping when the UE has an emergency PDU session. FFS on the following options - - Option 1: The subgrouping number for UE_ID based PEI subgrouping is considered in the formula for UE_ID based LP-WUS subgrouping. - - Option 2 The subgrouping number for UE_ID LPWUS subgrouping includes an offset K which is configurable or fixed. - - Option 3: The formula for UE_ID based PEI subgrouping is reused.

Agreements on RRM measurement relaxation

The entry condition for serving cell RRM relaxation is at least ‘if serving cell quality measured by MR is higher than relaxation threshold, e.g. RSRP and/or RSRQ’. FFS if LR measurement is needed.

FFS on exit condition for serving cell RRM relaxation, e.g., whether a separate exit condition other than ‘not fulfilling the entry condition’ is needed, or whether exit condition include MR and/or LR-based measurements

FFS if the entry condition for serving cell RRM measurement relaxation is the same as neighbour cell RRM measurement relaxation.

The agreements on RRM measurements entry/exit conditions seems also be relevant under the Idle/Inactive mode AI, therefor we elaborate on that in this tdoc.

Agreements made during RAN2#129 [9], related to Idle/Inactive…

Agreements on procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE

On separate band issue RAN2 understands that UE can report which band(s) is supported by LR to NW. RAN2 understands that any potential overload issues could be addressed by current mechanism in spec. Send LS to RAN1 and RAN4 to inform the agreements. On sub-grouping For UE_ID based subgrouping, similar formula defined for PEI subgrouping is reused for LP-WUS subgrouping, i.e., SubgroupID = (floor (UE_ID/(N*Ns*Np)) mod subgroupsNumForUEID) + (subgroupsNumPerPO – subgroupsNumForUEID), where - UE_ID is related to 5G-S-TMSI, - N is the number of total paging frames in one DRX cycle, - Ns is the number of the PO for a PF, - Np is the number of subgroupNumForUEID for PEI, if configured and UE supports PEI; otherwise, Np is 1, - subgroupsNumForUEID and subgroupsNumPerPO are the subgroup number for UE_ID based subgrouping for LP-WUS and the total subgroup number for LP-WUS, respectively. On SA2 LS (R2-2500050/S2-2412876) RAN2 sends reply LS to SA2/RAN3/CT1, and CC RAN1 and RAN4 to confirm the “LP-WUS” terminology. Regarding the SA2 raised issue on UE Radio Capability for Paging Information, R2 understand that there is no issue for NW after Release 17 (in which case the LP-WUS UE-ID based subgrouping UE capability is included in the UE-RadioPagingInfo container). Whether there is issue for the other cases (for the features mentioned by SA2 LS R2-2500050) can be further discussed in the main session.

Agreements related to RRM relaxation:

The entry condition for MR serving cell RRM relaxation can include both MR and LR measurements. If LR threshold is configured, the entry condition is when both MR and LR measurement are above the configured thresholds.

In this contribution, we discuss our view on some RAN2 aspects related to subgrouping, entry/exit criteria and latency aspects.

# Discussions on RAN2 aspects for LP-WUS/WUR in RRC Idle/Inactive

In RRC Idle/Inactive UE power saving gain comes from that the UE can operate with DRX operation, i.e., only periodically monitoring for paging, System update and performing cell measurements. Still the monitoring using the Main Radio, (MR), consumes a non-negligible power consumption, especially for small devices, that requires rather short wake-up times. For this purpose, the use of a low power wake-up radio has been discussed and agreed to be of this work item as a solution to reduce power consumption even further.

Introducing such a low power wake-up radio (LP-WUR), requires that it is reachable from the network, i.e. it can react to paging or some wake-up signal. Furthermore, in order to not create false wake-up, some sort of subgrouping is needed, as well as a concept of whether the UE is in capacity to operate using LP-WUR or not.

## Subgrouping

In earlier release starting from Rel-15, subgrouping have been defined for both LTE, NB-IoT, and NR (Rel-17). The concept of subgrouping may also be applicable for Rel-18/19 WUS/LP-WUR.

After waking up from LP-WUS, in case UE needs to monitor PO, the time offset between LP-WUS and the PO should be long enough to include the transition time to start up the main receiver to be ready for PDCCH monitoring, either all UEs could be triggered to wake up to monitor paging in associated PO(s), or just a subset of the UEs sharing the PO, so called LP-WUS UE subgrouping. The purpose of sub-grouping is to reduce the number of false wake-up of UEs that the paging was not intended for. [7]

In order to reduce the cost of overhearing, one option is where the UEs monitoring the same PO are divided into multiple subgroups, where the LP-WUS can provide wake-up indication for each subgroup.

At RAN1#118bis [8], a working assumption was agreed to support maximum 32 subgroups per PO. Accommodating 32 subgroups for the same PO via time-multiplexing of MOs may result in an increased delay as the wake-up delay of the last subgroup may not allow the UE to receive/monitor the upcoming PO and the UE needs to listen to the next PO instead.

Observation 1: Large number of subgroups may delay the wake up of the last subgroups due to short time to PO.

In order to increase the subgrouping space, instead of time-multiplexing of different sub-groups as discussed above, a grid of time and frequency where multiple LP-WUS can be accommodated when assigned through time-frequency multiplexing, for LP-WUS operation. With this, different UEs can be assigned to different resources (MO) and minimizing the difference of the delay between the UEs, before a UE can monitor the PO after LP-WUS detection.

Proposal 1: Support time-frequency multiplexing of LP-WUS for different UEs to increase LP-WUS address/identification space and minimize the difference of the delay between UEs

## Latency (duty-cycled vs always on)

When operating based on a duty-cycled scheme, the average communication delay from when data is available at the gNB until paging/data is correctly received by the UE depends on the sleep duration in the duty-cycle length, contributing error events such as false-alarm or miss-detection, and the transition time before the main receiver is ready for PDCCH reception.

The average communication delay in the LP-WUS/WUR scheme would be longer than the baseline scheme, i.e., Rel. 15/Rel. 16 scheme, if both are configured with the same duty-cycle length to fulfill a certain delay requirement. This is since the main radio goes to ultra-deep sleep state to further save power and therefore a longer transition time is needed to wake-up the main radio from ultra-deep sleep. In certain scenarios with extremely tight delay requirements, the communication delay time can become very significant and even the shortest DRX cycle can prevent the UE from meeting the tight delay requirement. This can happen, for instance, if the communication delay is required to be shorter than the length of “existing shortest possible” DRX configuration (320 msec) + the main radio transition and synchronization time. In these types of scenarios, the LP-WUR can instead be configured to monitor the channel continuously.

To address this, the operation of the LP-WUR can be configured as an adaptive operation where the LP-WUR can be adaptively operated according to a duty-cycled or an always-on scheme based on the communication delay requirement.

Observation 2: The long transition time to wake-up the main radio from ultra-deep sleep state together with the sleep time of the duty-cycle can prevent some UEs from meeting the delay requirement.

In the study [2], it was agreed that at least duty-cycled monitoring of LP-WUS is supported but depending on use case and capabilities it should be possible to configure either always on or duty cycled monitoring.

Proposal 2: Support an adaptive configuration where the UE, depending on its delay requirement, can operate based on either always-on or a duty-cycle scheme.

To give more freedom for configuring the LP-WUS monitoring, and for subsequent action after detecting the LP-WUS, the LP-WUR could be configured with a DRX cycle that is totally independent from the legacy Paging DRX cycle that the Main Radio (MR) is configured with. This should also make it possible to trigger other activities after detection the LP-WUS, e.g., to data transmission/reception or initiating RACH procedure directly.

Proposal 3: Support a configurable operation of duty-cycled LP-WUS monitoring to be done with respect to the existing DRX configuration and/or a stand-alone DRX configuration for LP-WUR.

## LP-WUS and entry/exit condition for LP-WUS monitoring

There may be a situation where the UE is in the border line between LR coverage and only MR coverage, with the risk of ping-pong behaviour, exiting and entering the LP-WUS monitoring mode and MR mode, leading to extra power consumption at the UE.

The border line may not necessarily have to be distance from base station, but the device can be in an area, e.g. city center, or being mobile within the cell, and where channel conditions are changing rather rapidly.

This may lead to a ping-pong scenario, where the device is shifting back and forth between MR and LR usage.

A first step to this can be by introducing a power level hysteresis, where the entry condition has another threshold value than the exit conditions.

Observation 3: A power level hysteresis would be relevant to prevent ping-pong behavior

Though if the variations are rather large, this hysteresis may not be enough. For solving this , a time window together with power level hysteresis can be used. If the UE has entered LR monitoring, and stopped using the MR, there is a time configured before the device triggers the MR again. And if the device has switched from using LR and using MR, there is a time period configured before it can move back to LR monitoring again.

Then depending on scenario, latency requirements or DRX configurations, the time window for moving back to LR monitoring should likely be longer, than the time for moving back to MR usage. This to be confident that the conditions are stable for LR usage.

Proposal 4: To further avoid ping-pong behaviours, a time window can be used in addition to power level hysteresis.

# Summary

In this contribution, we have discussed our views on aspects related to subgrouping, entry/exit criteria and latency aspects.

Observation 1: Large number of subgroups may delay the wake up of the last subgroups due to short time to PO.

Proposal 1: Support time-frequency multiplexing of LP-WUS for different UEs to increase LP-WUS address/identification space.

Observation 2: The long transition time to wake-up the main radio from ultra-deep sleep state together with the sleep time of the duty-cycle can prevent some UEs from meeting the delay requirement.

Proposal 2: Support an adaptive configuration where the UE, depending on its delay requirement, can operate based on either always-on or a duty-cycle scheme.

Proposal 3: Support a configurable operation of duty-cycled LP-WUS monitoring to be done with respect to the existing DRX configuration and/or a stand-alone DRX configuration for LP-WUR.

Observation 3: A power level hysteresis would be relevant to prevent ping-pong behavior

Proposal 4: To further avoid ping-pong behaviours, a time window can be used in addition to power level hysteresis.

# References

- RP-240801, Revised WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

- Minutes RAN2#125bis, Changsha

- Minutes RAN2#126, Fukuoka

- Minutes RAN2#127

- RAN2#127bis, Hefei

- RAN2#128, Orlando

- TR 38.869, Study on low-power wake up signal and receiver for NR (Release 18

- RAN1#118bis

- RAN2#129, Athens