3GPP TSG-RAN WG2 Meeting #129bis  	                            R2-2502488

Wuhan, China, Apr. 7th – 11th, 2025			                                (Revision of R2-2500743)

Agenda Item:	8.5.3

Source: 	Sony

Title:	Remaining details for on-demand SIB1

Document for:	Discussion

# Introduction

In RAN2#129 meeting, the agreements on OD-SIB1 were agreed as follows:

Agreements on OD-SIB1

- There is no need for additional barring mechanisms (in addition to the k_ssb signaling “no SIB1” indication in MIB) to handle legacy to be able to bar cell using OD-SIB1.

- Specify the following UE behavior to allow the UEs in RRC_CONNECTED state to acquire OD-SIB1 when T311 is running:

- When T311 is running, the UE can trigger the OD-SIB1 acquisition procedure with stored UL WUS configuration in SIB-X, if it is still valid.

- The legacy cell selection criteria are reused as the trigger condition of OD-SIB1 acquisition.

- The OD-SIB1 acquisition behavior is same as that of RRC_IDLE/IANCTIV UEs.

- The UE follows the legacy validity principle of stored SIB.

- For Rel-19 NES UE in RRC_CONNECTED, rely on the NW dedicated RRC for SIB1 delivery if searchSpaceSIB1 is not configured. It is legacy UE behavior and no spec change is expected.

- SIB-x can be cell specific or area specific, as legacy.

- Upon reception of RAR, the UE monitors OD-SIB1 in the window agreed by RAN1.

In this contribution, we discuss remaining aspects on OD-SIB1.

# Discussion

In RAN1#116 meeting, it was agreed that for study of UL WUS design, consider at least PRACH as a starting point. In Rel-15, on-demand SIB transmission is supported via IDLE/INACITVE UE to trigger a random access procedure. In MSG1 solution, a subset of the PRACH resources is associated with the requested SI, and network will reply a MAC subPDU with RAPID only for acknowledgement.

It is important that network can identify the request to transmit on-demand SIB1 from very beginning, in order to reduce the access delay for UE. And it was agreed in RAN2#125bis that UL WUS configuration includes at least RACH configuration. Therefore, the network should reserve PRACH resources that are associated with on-demand SIB1 transmission request.

Proposal 1: The network reserves PRACH resources associated with on-demand SIB1 transmission request. UE transmits UL WUS on the reserved PRACH resources to request the on-demand SIB1 transmission.

In RAN2#127bis meeting, it was agreed that if UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for the cell before requesting SIB1 of that cell.

If UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell.

And further in RAN1#120 meeting, some options regarding UE monitoring SIB1 transmissions are listed as follows:

If a UE has SIB1 request configuration of a cell and before transmitting UL WUS, - If the UE detects a SSB where K_SSB>=24 for FR1 or K_SSB>=13 for FR2, down select from the following: - Alt. 1: Only if K_SSB=30 for FR1 and K_SSB=14 for FR2, UE monitors Type 0 PDCCH for SIB1 transmission; Otherwise, UE doesn’t monitor Type 0 PDCCH - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified. - Alt. 2: UE monitors Type 0 PDCCH for SIB1 transmission - FFS: Whether monitoring time window of Type 0 PDCCH is up to UE implementation or specified. - Alt. 3: It is up to UE implementation on whether to monitor Type 0 PDCCH for SIB1 transmission - Alt. 4: UE doesn’t monitor Type 0 PDCCH for SIB1 transmission. - Alt. 5: UE monitors Type 0 PDCCH for SIB1 based on some repurposed parameters in MIB or PBCH

With Alt. 3, it implies the UE’s behaviour to monitor the SIB1 transmission doesn’t need to be specified. And Alt. 4 means UE doesn’t monitor SIB1 transmission before sending the OD-SIB1 request. Alt. 5 implies that explicit indication of OD-SIB1 is being broadcasted or not needs to be specified. Depending on the decision to be made in RAN1, RAN2 may need to revisit previous agreement regarding whether to monitor the OD-SIB1 is being broadcasted before sending the request.

Observation 1: RAN2 may need to revisit the previous agreement regarding UE’s behaviour on monitoring OD-SIB1 is being broadcasted before sending the request, after RAN1 makes decision on this issue.

It is better for RAN2 to inform RAN1 of their decision so that RAN1 does not pick a solution against RAN2 decision.

Proposal 2: Send LS to RAN1 informing RAN2 decision on “if UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell”.

# Conclusion

We have observation as follows.

Observation 1: RAN2 may need to revisit the previous agreement regarding UE’s behaviour on monitoring OD-SIB1 is being broadcasted before sending the request, after RAN1 makes decision on this issue.

We propose RAN2 to discuss following proposals:

Proposal 1: The network reserves PRACH resources associated with on-demand SIB1 transmission request. UE transmits UL WUS on the reserved PRACH resources to request the on-demand SIB1 transmission.

Proposal 2: Send LS to RAN1 informing RAN2 decision on “if UE has SIB1 request configuration of a cell, UE needs to check if SIB1 is currently being broadcasted or provided on demand for that cell before requesting SIB1 of that cell”.